3GPP TSG-RAN WG2 Meeting #115-e	R2-2108413

Online, August, 2021

Agenda item:       8.10.3.2 Idle/Inactive mode

Source: 	Convida Wireless

Title: 	NTN Cell (re)selection enhancements

Document for:	Discussion and Decision

# 1  Introduction

For Rel-17, a work item for NR to support NTN (Non-Terrestrial Networks) has been approved [1] with some initial progress in RAN2. One of the primary RAN2 objectives for such enhancements is to address idle mode procedures as highlighted below:

The following control plane procedures enhancements should be specified (see TR 38.821) [2]

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

RAN2 has made some agreements in previous meetings for control plane idle mode procedures related to cell (re)selection as shown below with the following was incorporated into the endorsed running CR for 38.300 [3] from 16.x.4.1.1 Mobility in RRC_IDLE from RAN2#114e:

An NTN UE may perform cell selection and reselection based on the NTN (satellite/HAPS) ephemeris provisioned.

Editor’s note: FFS on the definition of satellite/HAPS ephemeris. FFS under what circumstance UE will perform cell selection/reselection based on satellite/HAPS ephemeris.

Editor’s note: FFS whether UE location (and/or other information) based cell selection/reselection should be introduced for NTN.

The UE is made aware that a cell is provided by NTN.

Editor’s note: FFS on whether to make the UE aware of NTN in an implicit or explicit way.

For cell reselection, based on configuration criteria, an NTN UE may prioritise a network type (e.g., TN or NTN).

Editor’s note: Configuration details are FFS.

Based on the above endorsements, the endorsed TS 38.304 running CR was also updated with the following Editor’s Notes in RAN2#114e, as shown below [4]:

Editor’s note: For cell selection and reselection in NTN, the satellite/HAPS ephemeris may be used by UE.  The ephemeris is divided into serving cell’s ephemeris and neighbour cells’ ephemeris. FFS on the definition of satellite/HAPS ephemeris, under what circumstance UE will perform cell selection/reselection based on satellite/HAPS ephemeris and how would the serving cell’s ephemeris and neighbour cells’ ephemeris differ regarding e.g. the required accuracy or signalling impact.

Editor’s note: FFS whether UE location (and/or other information) based cell selection/reselection should be introduced for NTN.

Editor’s note: For intra-NTN and TN/NTN cell selection and reselection, UE needs to be aware whether a cell is an NTN cell or not no later than SIB1 reception. Further details are FFS..

Editor’s note: FFS on any further enhancement on cell reselection priority configuration in NTN.

For quasi earth fixed cell, the information on when a cell is going to stop serving the area is broadcast to UE via system information and UE will use such timing information to decide when to perform measurement on neighbor cells.

Editor’s note: FFS on whether the information on when a cell is going to stop serving the area is also needed for earth moving cell.

Editor’s note: FFS on whether the timing information about new upcoming cell is needed for quasi earth fixed cell and/or earth moving cell. FFS if such information is known from system information and/or the ephemeris. FFS on the utilization of such information.

The objective of this paper is to progress these agreements further, and address the FFSs related to cell (re)selection in TS 38.300 and TS 38.304.

# 2	 Discussion

#### 2.1 Rationale for NTN Cell (Re)selection Enhancements

In this section, we discuss the open issue regarding cell (re)selection enhancements over the existing NR idle mode procedures, which should be used as a baseline per previous RAN2 agreements.

Additionally, RAN2 has agreed Satellite/HAPS ephemeris-based cell selection and reselection should be defined for NTN and consistent language in the NTN WID [1]:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

To address dissimilar characteristics for NTN platforms versus existing TN platforms, such as uniform received power across NTN cell coverage, and mixed NTN/TN coverage and deployment scenarios, TS 38.304 requires enhancements.

Characteristics of NTN platforms deviate from existing TNs, including the relatively small changes in UE received signal strength from cell center to cell edge. Based on the Figure 2.1-1, using measured RSRP/RSRQ of the NTN cells would not be sufficient criteria for cell (re)selection and lead to a high number of reselections (and power consumption) especially in the presence of multiple NTN cells (e.g. HAPS, NGSO and GEO) and TN platforms.

Figure 2.1-1 NTN vs TN Near-far effects [2]

For the mixed deployment scenarios defined by a deployment with Multiple NTN-platform coverage (e.g., earth-fixed and earth-moving NTN cells) and possibly TNs, there may be multiple platforms with overlapping RF coverage at certain instances (geo-locations and time). In this case, there may be insufficient information available to the UE to know which platform should be prioritized and selected. It can be envisioned that the network will prefer to establish priorities for cell (re)selection for the UEs in the presence of multiple TN/NTN architectures. The UE may also have requirements to operate on each of the NTN cell types (timing and frequency compensation requirements), therefore enhancements from existing TN methods are required.

Reselection priority handling should be enhanced to address these issues. Specifically, cell ranking (R-criterion) is strictly designed for TNs, and not optimized for fast-moving NTN cells or cells (and neighbor cells) that lack near-far signal strengths, making cell re-selection challenging.

Additionally, TN rules to limit intra- and inter-frequency measurements (relaxed measurements) are not valid for NTN platforms. Furthermore, the criteria to determine mobility states in earth-moving cells deployments should be redefined as there will be high number of re-selections with moving cells. Mobility aspects in these cases will have an impact on measurement rules in these deployments as defined for TN and should also be enhanced from the existing procedures.

It can be envisioned that the network will prefer to establish priorities for cell reselection for the UEs in the presence of multiple TN/NTN platforms. The UE may also have requirements to operate on each of the NTN cell types (timing and frequency compensation requirements), therefore enhancements from existing TN methods are required.

Observation 1: Per WID and RAN2 agreements, existing NR cell (re)selection procedures should be enhanced for NTN with additional assistance information.

Observation 2: The measured RSRP/RSRQ of the NTN cells alone is not sufficient criteria for cell (re)selection in mixed deployments and mixed coverage.

Observation 3: Existing Rel-16 cell reselection priorities, measurement rules and mobility states for TN are not sufficient for NTN platforms.

#### 2.2 Potential enhancements for Cell (Re)Selection

In one example, to address uniform received power across NTN cell coverage and mixed NTN/TN deployment scenarios, per TS 38.304 [5] 5.2.3.1 as a baseline with highlighted potential enhancements:

Cell selection is performed by one of the following two procedures:

a)	Initial NTN/TN cell selection (no prior knowledge of which RF channels are NR or NTN frequencies):

1.	The UE shall scan all RF channels in the NR bands according to its capabilities to find a suitable cell.

2.	On each frequency, the UE need only search for the strongest cell, except for operation with shared spectrum channel access where the UE may search for the next strongest cell(s).

2a. Additional exception for NTN operations, where UE may search for the next strongest cell(s) determined by satellite ephemeris (and UE location), NTN platform type, and UTC time.

2b. Based on the platform obtained from the System Information, UE may select the cell prioritized based on platform type in conjunction with signal strength.

3.	Once a suitable cell is found, this cell shall be selected.

b)	Cell selection by leveraging stored information:

1.	This procedure requires stored information of NTN/TN frequencies and optionally also information on NTN/TN cell parameters from previously received measurement control information elements or from previously detected cells.

1a. Ephemeris and orbital parameters for NTN cells stored from previously detected cells per time of day relative to e.g., UTC accurate time and optionally location (relative UE location based on previous cell selections).

2.	Once the UE has found a suitable cell, the UE shall select it.

3.	If no suitable cell is found, the initial cell selection procedure in a) shall be started.

UEs may be pre-configured with PLMN IDs associated with both TN and NTN bands, and MNOs can provide dynamic priorities based on criteria (signal strength, orbital parameters/time, UE location, etc.) stored in UE/uSIM. This may be coupled with frequency layer prioritization/selection. However, PLMN ID and frequency layer prioritization as a standalone solution may have shortcomings due to the fast-moving nature of LEO satellites and TN/NTN deployed on the same bands.

To aid in cell (re)selection for NTN and mixed NTN/TN environments, satellite ephemeris information and UE location information (derived from GNSS per [2]) can be used to help UEs perform measurements. Calculated/stored geographic UE position information, possibly coupled with other SI broadcasted (for example, cell radius/time), and RSRP/RSRQ measurements, can be considered for the UE to camp on an NTN cell. In turn, this could be stored and used by the UE for future (re)selections.

Observation 4: The existing TN cell (re)selection procedures have shortcomings for NTN and mixed deployment scenarios/coverage and should be enhanced in TS 38.304 to describe additional enhancements for NTN cell (re)selection.

Observation 5: Criteria for Cell (re)selection that includes NTN cell timing and UE location assistance information has support by a majority of companies based on the summary in R2-2104805.

Proposal 1: RAN2 agrees to enhance TS 38.304 with additional assistance information and enhancements, using TN cell (re)selection as a baseline and to update the running CR to include NTN cell timing and UE location assistance information for cell (re)selection.

#### 2.3 Cell Reselection Priorities Handling for NTN

Per Reselection priorities handling for TNs, per TS 38.304 [5]:

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection.

In the case of NTN-capable UEs, NTN-specific priorities for reselection may be provided to the UE in system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. These priorities may depend on several factors that do not depend on absolute NR frequency prioritization.

#### 2.4 Measurement rules for cell re-selection

Following rules are used by the UE to limit unnecessary measurements [5]:

-	If the TN serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

- Otherwise, the UE shall perform intra-frequency measurements.

Specifically for NTN serving cells, TN measurement rules should be enhanced.

-	If the NTN serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose to evaluate the type of NTN platform and coverage area/time, associated satellite ephemeris data, location to determine when to perform intra-frequency measurements.

- Otherwise, the UE shall perform intra-frequency measurements.

If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements [5]. Extended/enhanced relaxedMeasurement may be useful for the network to configure for geo-stationary (GEO) NTN platform deployments, where the NTN cell footprint may be quite large.

Observation 6: The existing TN cell reselection procedures have shortcomings for NTN and mixed deployment scenarios/coverage and should be enhanced in TS 38.304. Additionally, enhancements related to measurement rules and mobility states for NTN cell reselection should be investigated.

Proposal 2: RAN2 agrees to update the currently endorsed running TS 38.304 CR with additional assistance information and enhancements, including cell reselection priorities handling and measurement rules. An email discussion at RAN2#115e should commence to progress these aspects.

# 3  Conclusion

To summarize, the following observations and proposals were examined:

Observation 1: Per WID and RAN2 agreements, existing NR cell (re)selection procedures should be enhanced for NTN with additional assistance information.

Observation 2: The measured RSRP/RSRQ of the NTN cells alone is not sufficient criteria for cell (re)selection in mixed deployments and mixed coverage.

Observation 3: Existing Rel-16 cell reselection priorities, measurement rules and mobility states for TN are not sufficient for NTN platforms.

Observation 4: The existing TN cell (re)selection procedures have shortcomings for NTN and mixed deployment scenarios/coverage and should be enhanced in TS 38.304 to describe additional enhancements for NTN cell (re)selection.

Observation 5: Criteria for Cell (re)selection that includes NTN cell timing and UE location assistance information has support by a majority of companies based on the summary in R2-2104805.

Observation 6: The existing TN cell reselection procedures have shortcomings for NTN and mixed deployment scenarios/coverage and should be enhanced in TS 38.304. Additionally, enhancements related to measurement rules and mobility states for NTN cell reselection should be investigated.

Proposal 1: RAN2 agrees to enhance TS 38.304 with additional assistance information and enhancements, using TN cell (re)selection as a baseline and to update the running CR to include NTN cell timing and UE location assistance information for cell (re)selection.

Proposal 2: RAN2 agrees to update the currently endorsed running TS 38.304 CR with additional assistance information and enhancements, including cell reselection priorities handling and measurement rules. An email discussion at RAN2#115e should commence to progress these aspects.

# 4  References

[1] 3GPP RP-210908, "Revised WID: Solutions for NR to support non-terrestrial networks (NTN)," March 2021.

[2] 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), (Release 16).

[3] R2-2106539 NTN Stage2 running CR 38.300, THALES

[4] R2-2106540	Stage-3 running 304 CR for NTN	ZTE corporation, Sanechips

[5] 3GPP TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state, (Release 16).