3GPP TSG-RAN WG2 Meeting#115-e		       					       R2-2108763

Electronic, 9th - 27th August 2021

Source: 			ZTE Corporation, Sanechips

Title: 	UAC for minimization of service interruption when disaster condition applies

Agenda item:		8.22

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the UAC enhancements for minimization of service interruption when disaster condition applies.

- Discussion

An LS from CT1 [1] has arrived to ask RAN2 about UAC enhancements for minimization of service interruption when disaster condition applies:

Based on the stage 1 requirements on minimization of service interruption described in clause 6.31 of 3GPP TS 22.261, CT1 has been performing stage 2 study (see 3GPP TR 24.811 for further details).

Among the solutions that CT1 has not excluded to progress during the normative phase, there are two solutions impacting UAC after a disaster inbound roamer selects a PLMN without disaster condition: Solutions #38 and #40. These solutions require changes in the barring configuration in addition to introducing Access Identity 3 (see 3GPP TS 22.261), which are in the remit of RAN2

Solution #38 and 40 are preferred from CT1’s perspective.

- Solution #38: Introduce barring factor and timer for Access identity 3

Within UAC-BarringInfoSet, an NG-RAN node can include barring factor for Access Identity 3.

During the access barring check, if the UE NAS layer provides Access Identity 3 to the UE RRC layer together with an access category, the UE RRC layer decides whether the access attempt is allowed or not based on the value of the barring factor for Access Identity 3 associated with the access category and a random number drawn if none of the bit(s) for other access identity(ies) in uac-BarringForAccessIdentity is set to zero.

NOTE:	The change in the UAC-BarringInfoSet proposed in this solution is subject to RAN2 agreement.

The 5GSM level congestion can be prevented by properly setting the values of the barring factor for Access Identity 3 each of which is associated with an access category. For example, if access attempts of disaster inbound roamers related to DNN X should be reduced, the barring factor for Access Identity 3 associated with an operator-defined access category for DNN X can be adjusted.

- Solution #40 Introduce offset to adjust the barring factor for Access Identity 3

It is important for the PLMN providing disaster roaming to avoid degradation of service levels to its own subscribers due to the activities of incoming UEs. For this reason, the network needs a method by which it can set differential access barring levels for own subscribers and disaster roamers.

The usage of new Access Identity 3 allows network to differentiate inbound roamers from own subscribers. But this alone would not be useful to mitigate congestion caused by a sudden inflow of inbound roamers.

A new offset value is introduced to the unified access control barring information. A UE which is registered or attempting registration in a PLMN which is on the forbidden PLMN list or on the list of "forbidden tracking areas for roaming", but which is broadcasting "disaster roaming active", shall apply a uac-DisasterOffsetToBarringFactor to the uac-BarringFactor.

The uac-DisasterOffsetToBarringFactor indicates to the disaster roaming UEs the offset value by which the BarringFactor must be reduced when evaluating the access barring condition for that access category. The uac-DisasterOffsetToBarringFactor is set per access category.

The uac-DisasterOffsetToBarringFactor is defined as a range of s5 till s95 in steps of 5.

A disaster roaming UE computes the uac-BarringFactor for its access category as

uac-BarringFactor = max (p00, (uac-BarringFactor - uac-DisasterOffsetToBarringFactor))

Both solution#38 and 40 are feasible from RAN2’s perspective with considerate spec impact.

In the existing UAC configuration, the barring factor and timer is configured per access category. For UE with specific access identity (1,2, 11-15), NW can allow high priority access by configuring uac-BarringForAccessIdentity with value 0 for certain access identity, which means all the access attempts are allowed and UE with the corresponding access identity will not draw a random value and compare with the barring factor. UE with other access identities, for which the corresponding bit in uac-BarringForAccessIdentity is set to “1” will be treated as the same and perform barring check based on the barring factor configured for each access category.

UAC-BarringPerPLMN-List ::=         SEQUENCE (SIZE (1.. maxPLMN)) OF UAC-BarringPerPLMN

UAC-BarringPerPLMN ::=              SEQUENCE {

plmn-IdentityIndex                  INTEGER (1..maxPLMN),

uac-ACBarringListType               CHOICE{

uac-ImplicitACBarringList           SEQUENCE (SIZE(maxAccessCat-1)) OF UAC-BarringInfoSetIndex,

uac-ExplicitACBarringList           UAC-BarringPerCatList

}                                                                                                     OPTIONAL     -- Need S

}

UAC-BarringPerCatList ::=           SEQUENCE (SIZE (1..maxAccessCat-1)) OF UAC-BarringPerCat

UAC-BarringPerCat ::=               SEQUENCE {

accessCategory                       INTEGER (1..maxAccessCat-1),

uac-barringInfoSetIndex              UAC-BarringInfoSetIndex

}

UAC-BarringInfoSetList ::=          SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet

UAC-BarringInfoSet ::=              SEQUENCE {

uac-BarringFactor                   ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime                     ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

uac-BarringForAccessIdentity        BIT STRING (SIZE(7))

}

In summary, the existing UAC provides prioritization of certain access identities with a one bit indication but does not support barring factor and timer configuration per access category for each access identity.

For solution #38, barring factor per access category will be introduced for access identity 3 with considerate signaling overhead as there are at most 64 access categories. In solution#40, the signaling overhead would be acceptable with a common adjustment to the barring factor but requires more processing at UE side to drive the barring factor for each access category.

Observation: Solution#38 and #40 are both feasible but will have considerate RAN2 impact in signaling in the Uu interface and UE processing at AS layer.

From RAN2’s perspective, since the intention is to prevent congestion in disaster roaming PLMN, the simplest way to go is to broadcast a one bit indicator along with a common barring timer to bar all the access attempts from UE with access identity#3, except for MT access and emergency call, which is never barred for any access identities.

Proposal 1: A one bit indication along with a common barring timer should be introduced to bar all the access attempts from UE with access identity#3, except for MT access and emergency call.

Proposal 2: Agree the draft reply LS to CT1 [2].

- Conclusion and proposals

With the above analysis, we have the following conclusions and proposals:

Observation: Solution#38 and #40 are both feasible but will have considerate RAN2 impact in signaling in the Uu interface and UE processing at AS layer.

Proposal 1: A one bit indication along with a common barring timer should be introduced to bar all the access attempts from UE with access identity#3, except for MT access and emergency call.

Proposal 2: Agree the draft reply LS to CT1 [2].

- References

- R2-2106902/C1-213527_ LS on UAC enhancements for minimization of service interruption when disaster condition applies

- R2-2108763_DRAFT reply LS on UAC enhancements for minimization of service interruption when disaster condition applies