3GPP TSG-RAN WG2 Meeting #115 electronic   						    	R2-2108795

E-Meeting, 9th Aug – 27th Aug, 2021							    	 revision of R2-2105725

Source: 	Xiaomi Communications

Title:  	Clarification on the survival time requirement

Agenda Item:	8.5.4

Document for:	Discussion and Decision

# Introduction

According to the RAN2#113-e meeting discussion [1], RAN2 made the following agreements on survival time requirement:

- RAN2 confirms that specification enhancement for survival time support may only needed for uplink. Downlink is addressed by implementation and no specification impacts. - Support for survival time in UCE is up to network configuration. - Continue discussing whether burst spread and burst ending time is beneficial from RAN2 perspective, but trigger the discussion after SA2 progress in February. - Only periodic traffic is considered for survival time work in Rel-17. - RAN2 assumes one application message is conveyed by one PDCP SDU, and may further consider the cases where one application message is conveyed by varying number of PDCP SDUs depending on the progress

And RAN2 also made the following agreement [2] while selecting the survival time definition provided by the SA2:

- Time period during which “message loss” can be tolerated is adopted as the preferred format for Survival time.

According to the discussion in the RAN2#114-e meeting [7], RAN2 agreed to study the solutions for fulfilling the survival time requirements:

- No specific enhancements in support of Survival Time in UCE will be studied in R17, but we should aim for solutions for Survival time that also work in UCE - When Survival Time information is provided in TSC AI, RAN action (gNB and/or UE) can utilize it to improve the associated link reliability so that the survival time requirement is met - Study fast mechanisms for survival time handling and the need

According to the email discussion [8] on the fast mechanisms for survival time handling, two main solution directions are discussed.

According to the survival time definition provided by SA2 [3], the survival time is considered as “as a maximum time in units of “time” where each unit corresponds to the data burst periodicity defined in TSCAI in Rel-16”. In the email discussion [3] on the URLLC QoS requirements, companies discussed the solutions on how the RAN fulfils the survival time requirements. In this contribution, we provide some general design principles for the solutions fulfilling the survival time requirement.

# Discussion

## Survival time requirement

According to the offline discussion summary [4] in the RAN2#113e meeting, there are two steps to fulfil the survival time requirement:

- Step 1 (including 11 solutions): Monitoring of survival time state

- Step 2 (including 4 solution categories): Action after entering survival time state

The intention of step 1 is to detect/count the packet loss, and the intention of step 2 is to avoid consecutive packet loss. After the UE enters the survival time state, the UE can take certain actions (e.g. PDCP duplication, change L1/L2 parameters and report the failure to the gNB as given in [4]) to avoid the intolerable consecutive errors. However it seems that some solutions for monitoring the survival time state cannot fulfil all cases of survival time.

According to the Table quoted from TS 22.104 [5] and the survival timer definition given by SA2 [3], the survival time could be more than one period/interval of the traffic. This means that if there is only one packet loss, the UE should not enter the survival time state to take excessive scheduling strategies, as the excessive scheduling strategy as given in Step 2 would consume more network resources or take over the resource allocated for other services. Thus, the solutions of monitoring the survival time state should be able to count consecutive packet loss.

Observation: The UE can tolerate more than one consecutive packet loss before entering the survival time state.

According to [8], some solutions on the survival time trigger are discussed:

- Solution 1: UE-based reactive solution based on HARQ-NACK triggering

- Solution 2: UE-based reactive solution based on Tx-side timer

From our understanding, no matter which Option is selected by RAN2, the UE should not be forced to enter the survival time state when there is only one packet loss or when there is no packet loss. From our understanding, either Solution 1 or Solution 2 is able to count the consecutive packet loss, as only one application message per transmission interval is conveyed by one PDCP SDU for the survival time requirement in Rel-17.

Proposal 1: The UE should be able to count the number of consecutive packet loss.

As the consecutive packet loss is to count the number of lost PDCP SDUs (i.e. IP packets), the transmitting PDCP entity should be responsible to count the number of consecutive packet loss.

Proposal 2: The transmitting PDCP entity of the UE counts the number of consecutive packet loss.

Then the UE should enter the survival time state only when the number of the consecutive packet loss exceeds a configured threshold, so as to avoid the too early action of entering the survival time state.

Proposal 3: The UE enters the survival time state only when the number of the consecutive packet loss exceeds a configured threshold.

Regarding how the UE determines the packet loss, we consider that depending on the solution details, some cross layer interaction might be needed. For example, the packet loss can be determined by HARA/RLC feedbacks.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation: The UE can tolerate more than one consecutive packet loss before entering the survival time state.

Proposal 1: The UE should be able to count the number of consecutive packet loss.

Proposal 2: The transmitting PDCP entity of the UE counts the number of consecutive packet loss.

Proposal 3: The UE enters the survival time state only when the number of the consecutive packet loss exceeds a configured threshold.

# Reference

[1] RAN2#113-e meeting minutes.

[2] RAN2#112-e meeting minutes.

[3] R2-2010692, LSin from SA2 to RAN2, “LS on Use of Survival Time for Deterministic Applications in 5GS”

[4] R2-2102074	Offline on RAN enhancements QoS [AT113-e][506] 	Nokia

[5] 3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”.

[6] [Post113bis-e][506][R17 IIoT] Enhancements based on QoS (CATT)

[7] RAN2#114-e meeting minutes.

[8] [Post114-e][511][URLLC - IIoT] QoS Solutions (Samsung)