3GPP TSG-RAN WG2 #116-e                                                                                       R2-2111187

eMeeting, 1st – 12th November, 2021

Agenda Item:	    8.22

Source: 	    MediaTek

Title:		    Discussion on RAN2 impacts for MG enhancement WI

Document for:	    Discussion and decision

# 1 Introduction

In Rel-17, MG enhancement WI (led by RAN4) is agreed to be introduced as specified in RP-211591 [1]. There are all 3 objectives in this WI and some RAN2 work is required (listed as below) to complete the WI.

- Pre-configured MG pattern(s) (fast MG configuration) [RAN4, RAN2]

  - Procedures and signaling for pre-configured MG pattern(s) [RAN2]

    - Specification of protocol impacts of the mechanisms of activation/deactivation of MG following a DCI or timer based BWP switch, e.g., per BWP MG configuration based on RAN4 input

- Multiple concurrent and independent MG patterns [RAN4, RAN2]

  - Procedures and signaling for simultaneous RRC (re-)configuration of one or more gap patterns [RAN2]

    - Specification of protocol impacts for multiple concurrent and independent MG patterns based on RAN4 input

- Network Controlled Small Gap (NCSG) specification [RAN4, RAN2]

  - Procedures and signaling for NCSG patterns [RAN2]

For objective 1 and 2, RAN4 already provided initial input on current progress [2][3]. This paper discussed the RAN2 SPEC impact (mainly for RRC configuration part) for the 3 objectives.

# 2 Discussion

## 2.1 Pre-configured MG pattern(s)

For pre-configured MG, according to the LS [1] from RAN4, we have the following understanding.

- Pre-configured MG share the same configuration parameters as legacy MG

- An indication should be added in measurement gap configuration to indicate whether it is a pre-configured MG

- Pre-configured MG status (activated or not) is determined by which [downlink] BWP is activated.

  - Detail activation/deactivation rule to be discussed in RAN4

  - Note that legacy MG is activated all the time.

- For each [downlink] BWP, RRC configuration should indicate the pre-configured MG status (activated or not)

We think it would be good to check whether companies have common understanding on this. One thing to confirm with RAN4 is that whether “per BWP” in the incoming LS implies “downlink” BWP, not “uplink” BWP.

Proposal 1: RAN2 to confirm the following understandings for per-configured MG

- Pre-configured MG shares the same configuration parameters as legacy MG

- An indication should be added in measurement gap configuration to indicate whether it is a pre-configured MG

- Pre-configured MG status (activated or not) is determined by which [downlink] BWP is activated.

- For each [downlink] BWP, RRC configuration should indicate the pre-configured MG status (activated or not)

Proposal 2: Confirm with RAN4 that the pre-configured MG status is determined by which “downlink” BWP is activated.

With P1 above, we think that the RRC configuration change is quite straightforward. An indication on is pre-configured or not could be added in IE MeasConfig and an indication on the pre-configured MG status could be added in BWP-DownlinkDedicated. Draft CR is provided in [4] for reference.

## 2.2 Concurrent MG

For concurrent MG, according to the LS [2] from RAN4, we have the following understanding.

- Concurrent MG is a list of measurement gaps, where each measurement gap is associated with certain “purpose”.

- The “purpose” could be

  - SSB measurement in one NR frequency layer

  - CSI-RS measurement in one NR frequency layer

  - LTE CRS measurement in one LTE frequency layer

  - PRS measurement

- One measurement gap could be associated with multiple purpose but one purpose could only belong to one measurement gap.

Proposal 3: RAN2 to confirm the following understandings for concurrent MG

- Concurrent MG is a list of measurement gaps, where each measurement gap is associated with certain “purpose”.

- The “purpose” could be

  - SSB measurement in one NR frequency layer

  - CSI-RS measurement in one NR frequency layer

  - LTE CRS measurement in one LTE frequency layer

  - PRS measurement

- One measurement gap could be associated with multiple purpose but one purpose could only belong to one measurement gap.

With P3 above, it seems obvious that we need a new measurement gap configuration list (e.g. MeasGapConfigList) in RRC Signaling. Each gap configuration in this list is basically the same as legacy one with the additional parameters that it can be associated with some purposes. One way is to have measurement gap Id to identify the gap within the list and in NR or LTE MO, a pointer could be added to build the association. Draft CR is provided in [4] for reference.

## 2.3 Network Controlled Small Gap (NCSG)

For NCSG, there is no LS from RAN4 now. We understand the configuration part is not difficult. We could probably reuse all legacy MG configuration parameters with a new indication to tell UE that it is NCSG. We could wait RAN4 for additional information.

On the other hand, the capability reporting for NCSG seems non-trivial. RAN4 had some initial discussion but no conclusion yet. We believe that it could be discussed in RAN2 directly. In Rel-16, RAN2 agreed to report NeedForGap information in RRC Reconfiguration Complete message based on the resultant configuration. The NCSG is also kind of gap requirement information and there is no need to deviate from Rel-16 framework. We propose to reuse similar mechanism for NCSG capability reporting. In Rel-16, {gap, no-gap} is reported as gap requirement information. In Rel-17, the UE could report {gap, no-gap, NCSG} in same framework.

Proposal 4: The Rel-16 dynamic NeedForGap reporting framework is reused to report the NCSG capability.

- The UE reports which kind of gap is needed (gap, no-gap, NCSG) for each supported and requested frequency band in RRC Reconfiguration Complete message based on the resultant configuration.

# 3 Conclusions

Base on the discussion in section 2, we propose the following:

For pre-configured MG pattern(s)

Proposal 1: RAN2 to confirm the following understandings for per-configured MG

- Pre-configured MG shares the same configuration parameters as legacy MG

- An indication should be added in measurement gap configuration to indicate whether it is a pre-configured MG

- Pre-configured MG status (activated or not) is determined by which [downlink] BWP is activated.

- For each [downlink] BWP, RRC configuration should indicate the pre-configured MG status (activated or not)

Proposal 2: Confirm with RAN4 that the pre-configured MG status is determined by which “downlink” BWP is activated.

For concurrent gap

Proposal 3: RAN2 to confirm the following understandings for concurrent MG

- Concurrent MG is a list of measurement gaps, where each measurement gap is associated with certain “purpose”.

- The “purpose” could be

  - SSB measurement in one NR frequency layer

  - CSI-RS measurement in one NR frequency layer

  - LTE CRS measurement in one LTE frequency layer

  - PRS measurement

- One measurement gap could be associated with multiple purpose but one purpose could only belong to one measurement gap.

For NCSG

Proposal 4: The Rel-16 dynamic NeedForGap reporting framework is reused to report the NCSG capability.

- The UE reports which kind of gap is needed (gap, no-gap, NCSG) for each supported and requested frequency band in RRC Reconfiguration Complete message based on the resultant configuration.

# 4 References

[1] RP-211591, “Revised WID on NR and MR-DC measurement gap enhancements”, Intel Corporation, MediaTek Inc.

[2] R2-2109367/R4-2115438, “LS on R17 NR MG enhancements – Pre-configured MG”, RAN4

[3] R2-2109361/R4-2115343, “LS on R17 NR MG enhancements – Concurrent MG”, RAN4

[4] R2-2111189, “RRC signaling for measurement gap enhancement”, MediaTek