3GPP TSG-RAN WG2 Meeting #116 electronic	               R2-21xxxxx

Electronic meeting, 1st-12th November 2021

Agenda Item:	8.7.2.2

Source: 	Huawei, HiSilicon

Title:	Summary of [AT116-e][626][Relay] Direct-to-indirect path switch (Huawei)

Document for:	Discussion and Decision

# Introduction

This is to summarize the following offline discussion.

[AT116-e][626][Relay] Direct-to-indirect path switch (Huawei)

Scope: Discuss P14-1/P15/P16/P14-2/P17/P23 of R2-2111276, and attempt to converge the options.

Intended outcome: Report to CB session

Deadline:  Thursday 2021-11-11 0100 UTC

# Contact information

| Company | Name | E-mail |
| --- | --- | --- |
| Qualcomm | Peng Cheng | chengp@qti.qualcomm.com |
| Ericsson | Antonino Orsino | antonino.orsino@ericsson.com |
| OPPO | Boyuan Zhang | zhangboyuan@oppo.com |
| Xiaomi | Xing Yang | Yangxing1@xiaomi.com |
| ASUSTeK | Lider Pan | lider_pan@asus.com |
| Lenovo&MM | Lianhai Wu/Prateek | Wulh5@lenovo.com/ |
| Apple | Zhibin Wu | Zhibin_wu@apple.com |
| InterDigital | Martino Freda | martino.freda@gmail.com |
| Sharp | Chongming Zhang | Chongming.zhang@cn.sharp-world.com |
| Fujitsu | Guorong Li | liguorong@fujitsu.com |
| Samsung | Hyunjeong Kang | hyunjeong.kang@samsung.com |
| Spreadtrum | Xing Liu | xing.liu1@unisoc.com |
| Kyocera | Henry Chang | henry.chang@kyocera.com |
| ZTE | Mengzhen Wang | wang.mengzhen@zte.com.cn |
| CMCC | Xueyan Huang | huangxueyan@chinamobile.com |
| LG | Seoyoung Back | seoyoung.back@lge.com |
| Philips | Jesus Gonzalez Tejeria | Jesus.gonzalez.tejeria@philips.com |
| Intel | Sangeetha B | sangeetha.l.bangolae@intel.com |
| NEC | You Li | liyou@labs.nec.cn |
| Huawei, HiSilicon | Rui Wang | wangrui46@huawei.com |

# Discussion

## T304 applicability

Proposal 14-1: [Easy] A new T304-like timer is introduced for direct-to-indirect path switch. The Remote UE starts the timer upon reception of the RRC reconfiguration message indicating direct-to-indirect path switch, and the Remote UE initiates RRC re-establishment upon timer expiry.

- Q1: Do companies agree the above P14-1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | It is straight forward. |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| ASUSTeK | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| Apple | Yes |  |
| InterDigital | Yes |  |
| Sharp | Yes |  |
| Fujitsu | Yes |  |
| Samsung | Yes |  |
| Spreadtrum | Yes |  |
| Kyocera | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| LG | Yes |  |
| Philips | Yes |  |
| Intel | Yes |  |
| NEC | Yes |  |
| Huawei, HiSilicon | Yes |  |

Proposal 14-2: [To be discussed] FFS which option is taken as stop condition of the new T304-like timer in Remote UE:

- Option1: Upon successfully sending RRCReconfigurationComplete (i.e., lower layer acknowledge is received from target relay);

- Option2: Upon the PC5 unicast link is successfully established with the target Relay UE;

- Option3: Upon reception of RRCReconfigurationCompleteSidelink message from target Relay UE;

- Option4: Upon reception of an explicit indication from the target Relay UE.

- Q2: Which option do companies prefer in the above P14-2?

| Company | Option # | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | In legacy T304, the timer is stopped upon successful completion of random access on the corresponding SpCell. We think the new stop condition should be similar to the principle of legacy one. - Successful completion of RACH means that UE successfully sends preamble to gNB Then, we think Option 2 and 3 can be precluded because establishment of unicast PC5 link doesn’t mean that UE can successfully send ReconfigurationComplete to relay with configured PC5 RLC bearer for relaying in HO command. - Between Option 1 and Option 4, we prefer Option 1 because no new signalling is required to be introduced. Actually, in legacy Uu HO, the UE use lower layer acknowledge to determine whether ReconfigurationComplete is successfully send to gNB. We think same principle can be reused. - Some one may argue that in Option 1, it is possible that RRCReconfigurationComplete is sent to relay but not delivered to gNB (e.g., due to Uu RLF), we think it is a rare case, and remote UE can rely on failure handling mechanism when it happens. |
| Ericsson | Option 1 | We agree with QC analysis. |
| OPPO | Option 2/3 | Since the time being of completion of path switch is when remote UE finishes the unicast link setup procedure with relay UE can be the start point of performing UP delivery over the target path. In that case, we think both option 2 and option 3 works well. For option-1, we understand similar to legacy HO, there is no need to wait for the completion of HO-confirm delivery to send UP data. |
| Xiaomi | Option 1 | Option is closest to legacy T304 stop. |
| Vivo | Option 2 | It is simple. For Option 1, it is unclear to us what scheme can be used for the remote UE to confirm the successful delivery of an UL RRC message via indirect path. If this were to depend on feedback at PDCP layer, we would be not sure whether extra standard changes are needed. For option 3, we think the confirmation on the portion of PC5 does not reliably mean the Uu reconfiguration complete message is successfully delivered to the NW. Also, it seems we haven’t the conclusion that the Uu complete message is going to be carried by the PC5 reconfiguration complete message or a new message. For option 4, our concern is also the Spec impact. |
| ASUSTeK | Option 1 |  |
| Lenovo | Option 2 or Option 3 | In legacy, UE stops T304 Upon successful completion of random access (before UE transmits the reconfiguration message). In the relay case, we need to reuse this principle. Namely, the remote UE successfully establishes PC5 link. Therefore, option 2 or option 3 can be support. For Option 1, if the HARQ is disabled, how to determine ‘successfully’? |
| MediaTek | Option 1 |  |
| Apple | Option 2/3 | Follow the Uu design, similar to RACH, the remote UE only need to established PC5-RRC connection to the relay UE to stop this timer. |
| InterDigital | Option 1 | Stop condition should be closest to legacy |
| Sharp | Option 1 |  |
| Fujitsu | Option 1 |  |
| Samsung | Option 1 | We share the views from Qualcomm. |
| Spreadtrum | Option 1 |  |
| Kyocera | Option 1 |  |
| ZTE | Option 2/3 | Agree with Lenovo and Apple. |
| CMCC | Option 1 |  |
| LG | Option 1 | We share same view with QC |
| Philips | Option 1 |  |
| Intel | Option 1 |  |
| NEC | Option 1 | We share the same view with QC. |
| Huawei, HiSilicon | Prefer option4, can accept option1 with clarification | If we assume there is an available and stable connection between Relay UE (i.e. in connected state) and gNB, there is no big difference among the 4 options. Option1 is preferable to us. However, if Relay UE in idle/inactive can be selected as discussed in P23, then there would be one more question whether we need to consider the case that Relay UE fails its own the RRC connection setup towards gNB. If we intent to cover this case, the Remote UE should only stop the timer that Relay UE succeeds on its own RRC setup, e.g. via option 4. And if we go with option1, it needs to clarify whether Relay UE can only send the lower layer ACK for Remote UE’s Uu RRCReconfigurationComplete message after completion of its Uu RACH or not. Our view is left to UE implementation. And if Remote UE fails RACH procedure or RRC connection setup, it would trigger unicast release resulting in Remote UE’s reestablishment. However, the case of Relay UE’s RRC setup failure seems not covered by the agreed failure handling yet. |

## Signalling details

Proposal 15: [Easy] RRC reconfiguration message towards the Remote UE should include the Relay UE ID to indicate the target Relay UE for direct-to-indirect path switch which is the same Relay UE ID agreed to be included in SL measurement report.

- Q3: Do companies agree the above P15?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes (to change Proposal with “Relay UE L2 ID”) | The intention of including relay UE ID in RRCReconfiguration towards remote UE is for remote UE to establish unicast PC5 link. As we specified in Rel-16 V2X, the unicast PC5 link is identified by Source L2 ID and Destination L2 ID. We think it is important to keep this principle. Another option is to use C-RNTI of relay UE. However, it requires Gnb to provide the mapping from relay’s L2 source ID to C-RNTI to remote UE via another Uu RRC message, which is unnecessary spec change. Thus, we propose relay UE ID should be its source L2 ID. Thus, we suggest below wording change on proposal: Proposal 15: [Easy] RRC reconfiguration message towards the Remote UE should include the Relay UE L2 ID to indicate the target Relay UE for direct-to-indirect path switch, which is the same Relay UE ID agreed to be included in SL measurement report. |
| Ericsson | Yes | Ok with the clarification proposed by QC |
| OPPO | Yes | We share the view with Qualcomm |
| Xiaomi | Yes | We also think the second sentence is not needed. |
| Vivo | Yes, with the change proposed by QC above. | Same view as Qualcomm. |
| ASUSTeK | Yes | We agree with Qualcomm’s suggestion on Proposal 15. |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| Apple | Yes with comment | But we oppose the QC change the proposal text because Src L2 ID issue is captured in another separate proposal. “U2N relay UE ID” is a term used in TR and earlier agreements. We just need to stick to the same name. We are not sure why C-RNTI is related to this agreement. The proposal just means the Gnb selects the relay UE from the relay UE ID(s) reported in SL measurement reports by the remote UE and notify the selection relay UE ID to the remote UE. There is no need to remove the second part. |
| InterDigital | Yes |  |
| Sharp | Yes |  |
| Fujitsu | Yes |  |
| Samsung | Yes | Agree with the change by Qualcomm |
| Spreadtrum | Yes |  |
| Kyocera | Yes |  |
| ZTE | Yes | Agree with Qualcomm’s suggestion. |
| CMCC | Yes | Prefer QC’s modification. |
| LG | Yes |  |
| Philips | Yes, with comments | Agree with Apple to not make the change as Qualcomm suggested. The relay UE may have changed its self-assigned layer-2 ID used for discovery (e.g. for privacy reasons) by the time the RRCReconfiguration message is sent to the remote UE. It would be rather confusing for the remote UE to receive a different layer-2 identifier than the one it used in the measurement report, given that the remote UE may also use other criteria for its relay selection. One way to avoid such confusion would be to use a more stable higher-layer identifier than the layer-2 ID in the measurement reporting such as User Info ID received as announcer info/discoveree info parameters during discovery. Irrespective of which identity is used for measurement reporting, the situation that a remote UE uses an older layer-2 ID for connection setup whilst the relay UE has changed its layer-2 ID in the meantime, is likely something that will/should be handled anyway by the relay UE’s implementation. We could add a clarifying note to that effect |
| Intel | Yes | We understand that the proposed (but not agreed yet) Relay UE ID to be used in the SL measurement report is also source Layer-2 ID. |
| NEC | Yes | We share the same view with QC. |
| Huawei, HiSilicon | Yes | The original intention is to align the relay UE ID in measurement report from Remote UE and path switch command to the Remote UE. Then the premise of QC’s proposal would be RAN2 can also agree to include candidate Relays SRC L2 ID in measurement report. |

Proposal 16: [Easy] RRC reconfiguration message towards the target Relay UE should include the Remote UE’s local ID/AL ID and L2 ID when preparing the direct-to-indirect path switch.

- Q4: Do companies agree the above P16?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | It is straight forward |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| ASUSTeK | Yes |  |
| Lenovo | Yes |  |
| MediaTek | Yes |  |
| Apple | Yes |  |
| InterDigital | Yes |  |
| Sharp | Yes |  |
| Fujitsu | Yes |  |
| Samsung | Yes |  |
| Spreadtrum | Yes |  |
| Kyocera | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| LG | Yes |  |
| Philips | No, with comments | We don't agree that the gNB can/should provide the layer-2 identifier of the remote UE to the relay UE, because the layer-2 that the Remote UE uses is self-assigned and not known by the gNB, and for privacy reasons should be changed between discovery and connection setup (currently ongoing discussions in SA3 about that). Note that the gNB may not need to create a new local ID if the Remote UE's Source User Info ID that is used during connection setup will be used (as per section 6.4.3.6 of 23.304). |
| Intel | Yes |  |
| NEC | Yes |  |
| Huawei, HiSilicon | Yes | For Philips’s suggestion on source user info ID, our understanding is the user info ID is out of scope of SA2 spec or AS spec, we do think we can use it as identity of Remote UE in during AS path switch. |

Proposal 17: [To be discussed] FFS whether existing reconfigurationWithSync or new RRC signaling is used to indicate direct-to-indirect path switch to Remote UE.

- Q5: Which option do companies prefer in the above P17, i.e.:

- Option1: reconfigurationWithSync

- Option2: new RRC 8ignaling

- Option3: Just a normal RRCReconfiguration.

| Company | Option # | Comments |
| --- | --- | --- |
| Qualcomm | Slightly prefer Option 1 | We think the benefit of Option 2 is that TS 38.331 will be clearer because most of procedure text on ReconfigurationWithSync handling can’t be reused for direct-to-indirect path. However, we are not sure whether it is necessary to have a new Uu RRC message just for it. In all, we don’t have strong view (slightly prefer to reuse legacy signalling) and can follow majority. |
| Ericsson | Option 3 | The reconfigurationWithSync is used to trigger RACH on the UE and deviating from this principle is not something we should do (since this is adopted since Rel-15). Also, the impact on the RRC spec is quite substantial. Further, when performing path switch from direct to indirect path there is no RACH and the path switch can be triggered with a simple RRCReconfiguration message. Not sure why we need to use reconfigurationWithSync or even a new signaling. |
| OPPO | Option 1 | We support to reuse the IE, in order for forwards compatibility, since - the procedural text (5.3.5.5.2)that triggered by reconfigurationwithsync can be partly reused, e.g., (even though not all, but seems the handling of T310/316/312, together with mac-resetting, c-rnti configuration and etc.). And please note that RACH is not a mandatory result of reconfigurationwithsync, logically it is triggered since the UE is initially in un-sync state for target cell, so RACH is triggered as a result, which naturally is not applicable to SL-based scenario, and thus no need to be a reason to deviate from reconfigurewithsync. |
| Xiaomi | Option 1 |  |
| vivo | Option 1 | We can do extension on top of existing ReconfigurationWithSync, if needed. |
| ASUSTeK | Option 1 |  |
| Lenovo | Option 1 |  |
| MediaTek | Option 1 | We see some changes needed in order to reuse reconfigurationwithsync |
| Apple | Option 1 | It is logical to extend existing HO signaling instead of design a new one for SL relay purpose. |
| InterDigital | Option 1 or 3 |  |
| Sharp | Option 1 |  |
| Fujitsu | Option 1 or 3 |  |
| Samsung | Option 3 | We share the view from Ericsson. It should be avoided to use reconfigurationWithSync for relay path switch purpose. |
| Spreadtrum | Option 1 |  |
| Kyocera | Option 1 |  |
| ZTE | Option 1 | We think the existing reconfigurationWithSync can be reused to configure remote UE for direct-to-indirect path switch. Since it has agreed the reconfigurationWithSync is used for indirect-to-direct path switch, it is better to unify configuration way for both directions of path switch. |
| CMCC | Option1 |  |
| LG | Option 1 |  |
| Philips | Option 1 or 3 |  |
| Intel | Option 1 or 3 |  |
| NEC | Option 1 |  |
| Huawei, HiSilicon | Both ways are fine | Either reusing reconfigurationWithSync or introducing new fields/IE to indicate direct-to-indirect path switch can work. But the corresponding spec changes are different. - If go with existing reconfigurationWithSync, a new subsection of UE behavour upon reception of reconfigurationWithSync is needed for Remote UE as most of the Uu behavour is not applicable to Remote UE, e.g. RACH as mentioned by Ericsson. - If go with new field included in normal RRC reconfiguration message, then the UE handling of this new field would be new and naturally separated from legacy Uu UE. |

## Whether Relay UE in IDLE/INACTIVE can be selected as target Relay in direct-to-indirect path switch

Proposal 23 (reworded): [To be discussed] RAN2 to down select among the following options to handle the case of Relay UE in IDLE/INACTIVE during direct-to-indirect path switch:

- Option1: The target Relay UE of direct-to-indirect path switch must be in RRC_CONNECTED.

- Option2: Relay UE in IDLE/INACTIVE can be indicated as target Relay, and to support such case by the Remote UE oriented solution, i.e. after receiving the path switch command, Remote UE establishes PC5 link with the Relay UE and sends HO complete message via the Relay UE which will trigger the Relay UE to enter CONNECTED sate.

- Option3: Relay UE in IDLE/INACTIVE can be indicated as target Relay, and to support such case by the paging-based solution, i.e. the network sends paging message to the Relay UE which will trigger the Relay UE to enter CONNECTED sate before sending path switch command to the Remote UE.

To assist the suggested down-selection, based on Rapporteur’s understanding the comparison among the 3 options is given.

| Option # | Spec impact | Pros and cons |
| --- | --- | --- |
| Option1 | No spec impact. In this option, it is more up to NW implementation and some legacy procedures can be used. For instance: - NW implementation can make sure the target Relay is in connected state. - Or if there is no candidate Relay in connected state, NW can perform RRC release to idle/inactive, which results in remote UE’s RRC setup/resume to a suitable relay/cell. - In the worst case, the remote may experience SL RLF and perform RRC reestablishment as agreed. | Cons: The Relay candidates may need to be kept in RRC_CONNECTED, which have negative impact on UE power consumption. Pros: No extra spec impact, and direct-to-indirect path switch delay is minimum. |
| Option2 | Small spec impact, including: - Allow network to provide the Relay UE with the configuration associated to the Remote UE after the Remote UE access via the Relay UE. - No/minor new signaling is needed. | Cons: The path switch delay is added by the Relay UE’s RRC Setup/Resume procedure. Pros: The Relay candidates can be sent to IDLE/INACTIVE to save power. |
| Option3 | Bigger spec impact, including - Define new SL signaling to enable Relay UE inform it’s paging UE ID to the (Remote) UE before path switch. - Define new Uu signaling to enable the (Remote) UE to report Relay UE’s paging UE ID to gNB. - Define new NG interface signaling to enable gNB to ask CN to generate CN paging message of the Relay UE. | Cons: The path switch delay is added by Relay UE reporting its paging UE ID to gNB via the (Remote) UE and gNB requesting CN to page the Remote UE. Pros: The Relay candidates can be sent to IDLE/INACTIVE to save power. |

- Q6: Which option do companies prefer in the above P23?

| Company | Option # | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | We understand some companies’ concern that Option 1 is not good for relay UE power saving. However, we have two comments: - Even if only CONNECTED relay is allowed in this release, it is not the end of world. If gNB can’t find available CONNECTED relay, it can release the remote UE to IDLE/INACTIVE state and remote UE will perform RRC re-establishment. So, we do not think spec is broken if only CONNECTED relay is allowed in this release - We have concern on workload of Option 2 and Option 3, especially we don’t even have majority view on down-selection between Option 2 and Option 3. And if Option 3 is agreed, cross-WG effort of SA2/RAN3 is required. We are not sure whether RAN2 can complete all details in the remaining 2 meetings. Based on the concern, we would like to suggest below way-forward: - RAN2 first prioritize to complete remaining details on CONNECTED relay. It ensures RAN2 has a workable Spec in Rel-17. - Companies supporting Option 2 and Option 3 can have an offline discussion to have a single converged solution with detailed spec impacts in a co-sourced contribution for next RAN2 meeting. And RAN2 can evaluate the workload and determine whether to specify IDLE/INACTIVE relay in next RAN2 meeting. In our view, if the converged solution is simple and spec impacts are manageable, we are happy to accept it. |
| Ericsson | Option 3 preferred Option 2 second choice | Since the SI we decided to not have any restriction on the RRC state of the relay UE and remote UE. If we go for Option 1 this will have even a bigger impact than going for Option 2 and Option 3. We think that Option 3 and Option 2 are easly implementable and we don’t see any reason to revert a decision that we have taken since the SI. |
| OPPO | Option 2 | To consider the power consumption issue for relay UE, it is not reasonable to limit relay UE always being in RRC_CONNECTED, therefore, we prefer that RRC_IDLE/INACTIVE relay can also be selected. But on the other hand, we would like to avoid too much spec impact on this issue. Therefore, it seems option 3 is not such easy to be implemented since there is RAN3 impact involved in this option. Option-2 is mostly like the other RRC procedure, so we do not see clearly delta part / additional work due to this option, so suggest to pick-up this option as the way-out. |
| Xiaomi | Option 3 or 2 | Option 1 would require relay UE to be always in CONNECTED, which is power wasting for relay UE. Either option 2 and 3 are acceptable for us. |
| Vivo | See comments | Want to add an option like “Option X: First conclude all path switch related issues for the case of RRC_CONNECTED Relay UEs. If time allows, further discuss above options in this release”. Then we select this Option X. |
| ASUSTeK | Option 2 | We share the same view with OPPO. Besides, Option 2 is aligned with the way used for Remote UE connection establishment, where the relay UE performs its own RRC connection establishment with gNB upon reception of the RRCSetupRequest message from remote UE. |
| Lenovo | Option 2 | Agree with Oppo. Regarding Option 3, there’s higher latency since the gNB first needs to page the relay. |
| MediaTek | Option 1 | We agree with QC. |
| Apple | Option 1 | Option 2 means the remote UE and relay UE need support two different HO procedures for the same “direct-to-indirect switching” scenario with different RRC states of relay UE. That adds complexity with UE implementation. Also, relay UE in Option 2 is not properly vetted by gNB, as gNB is unable to check if there is really a reliable Uu link via the relay UE to continue the service. Option 3 does have cross-WG dependency and need more time to complete. Given that, we may have to accept Option 1 as the likely outcome of Rel-17. |
| InterDigital | Option 2, with comments | We do not see any spec impact with option 2, since it is the same as connection establishment at the relay caused by initial access by the remote UE. Then it can be upto NW implementation whether to use option 1 or option 2. And we do not need to discuss this further. |
| Sharp | Option 1 | For option 2 and 3 the delay for path switch will be extended and we think it should be avoided. |
| Fujitsu | Option 1 | To be simple, we suggest Option 1 for this release. |
| Samsung | Option 1 | We think that the simplest approach in R17 is to consider only RRC_CONNECTED Relay UEs as candidate Relay UE for RRC_CONNECTED Remote UE’s path switch operation. Both option 2 and option 3 have complexity to be solved as the Rapporteur analyzes. |
| Spreadtrum | Option 2 | Same view with OPPO. |
| Kyocera | Option 3 or 2 (Option 3 preferred) | We think either option can work well. For Option 3, we could potentially limit the case to INACTIVE relay UEs so that the gNB already has the relay UE’s context. We assume it’s up to the gNB to decide whether to release the relay UE to IDLE or INACTIVE. |
| ZTE | Option 2 | Generally gNB may prioritize the RRC_Connected Relay UE to be selected as the target Relay UE. But we should consider that, if no such suitable Relay UE exists, gNB may also configure the Remote UE to switch to a RRC_IDLE/INACTIVE Relay UE. Since we agreed that “relay UE shall enter CONNECTED to be able to forward remote UE’s initial RRC messages”, option 2 is actually the same as connection establishment at relay UE triggered by remote UE, we see no extra spec impact. |
| CMCC | Option 2 | Option 2 brings no spec impact and it allows power consumption for relay UE. |
| LG | Option 2 | We expect that gNB selects RRC_CONNECTED relay UE preferentially, but if there is no RRC_CONNECTED relay UE, gNB should select RRC_INACTIVE/IDLE relay UE for the service continuity of remote UE. The oriented solution described option 2 has some disadvantage in the aspects of latency, but it’s aligned connection establishment procedure when relay UE is RRC IDLE/INACTIVE. We think there is no big spec impact. |
| Philips | Option 2 or 3 | Option 1 is not acceptable due to the Relay UE energy consumption |
| Intel | Option 1 as baseline | As a baseline we prefer that the gNB select a target Relay UE that is visible in the RAN. While we prefer RRC_CONNECTED only, we can also consider Relay UE in inactive state which can be a compromise for companies concerned about power consumption. We do not prefer option 3 due to complexity and dependency on other working groups. |
| NEC | Option 2 or 3 | For connection setup, both RRC_IDLE UE and INACTIVE UE can be selected as relay. So it is straight forward that those UEs can be selected as relay for path switch. |
| Huawei, HiSilicon | Prefer Option 1, can accept Option 2 | We think the NW implementations in option 1 might be sufficient. However, if companies want to allow Relay UE in idle and inactive to be chosen as target, we can accept to support it via the solution with minimum spec effort, i.e. option 2. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Reference

- R2-2111276	Summary of Agenda item 8.7.2.2: Service continuity	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

# Annex RAN2 agreements on service continuity

RAN2 #114

Agreements:

Proposal 1 (easy) (19/19): The procedure of Figure 4.5.4.1-1 in TR38.836 and the procedure of Figure 4.5.4.2-1 in TR38.836 are the baseline for Remote UE’s intra gNB mobility in RRC_CONNECTED.

Proposal 2 (easy) (19/19): INM RRC and/or X2/Xn messages for inter-gNB handover are not used for the path switch procedures in intra gNB case.

Proposal 3 (easy) (19/19): DAPS-like path switch procedure for Remote UE is not considered in this release.

Proposal 6 (easy) (19/19): Legacy RRC Reconfiguration and Measurement Report signalling procedures can be used for path switch procedure with extension to evaluate relay link measurement and Uu link measurement.

Proposal 10 (easy) (19/19): In case of path switch from indirect to direct, detailed measurement results from Remote UE are reported when configured reporting criteria is met as legacy measurement report.

Proposal 11 (easy) (19/19): SL relay measurement report can include at least Relay UE ID, serving cell ID, RSRP information.

Proposal 13 (easy) (19/19): Remote UE in RRC_CONNECTED suspend Uu RLM when Remote UE is connected to gNB via Relay UE.

Proposal 14 (easy) (19/19): For indirect to direct path switch, Remote UE stops UP and CP transmission via relay link after reception of RRC Reconfiguration message from gNB (i.e., step 3).

Proposal 23 (easy) (19/19): For indirect to direct path switch, the timing of step 8 is independent of step 6 and step 7.

[Note: P23 refers to the step numbers from Figure 4.5.4-1 of TR 38.836]

Proposal 24 (easy) (19/19): For indirect to direct path switch, RLC and lower layers behaviours of a Remote UE can be similar with those of legacy UE in intra-gNB handover.

Proposal 29 (easy) (19/19): For direct to indirect path switch, Remote UE stops UP and CP transmission over Uu after reception of RRC Reconfiguration message from gNB (i.e., step 3).

Proposal 31 (easy) (19/19): For direct to indirect path switch, the contents in RRC Reconfiguration message for Remote UE can include at least Relay UE ID, PC5 RLC configuration for relaying and associated E2E RB.

RAN2 #115

Agreements:

Proposal 4 (easy) (18/19): CHO-like path switch procedure for Remote UE can be studied after the baseline design is finalized.

Proposal 5 (easy): The handling of RRC_CONNECTED Remote UE’s mobility due to SL RLF or Uu RLF notified by Relay UE can be discussed in CP agenda item.

Measurement events:

Agreement:

Proposal 7 (easy)(modified): New measurement events for the remote UE can be defined to compare SL relay link measurement with a threshold and/or to compare SL relay link measurement with threshold A and Uu link measurement with threshold B.

Procedures for service continuity:

Agreements:

Proposal 17 (easy) (18/19): For indirect to direct path switch, that PC5 connection reconfiguration can be executed between Remote UE and Relay UE to release PC5 RLC for relaying.

Proposal 19 (easy) (16/19) (modified): For indirect to direct path switch, PC5 unicast link can be released after Remote UE and Relay UE receive RRC reconfiguration from gNB (if there are no non-relaying PC5 RLC channels on the same PC5 unicast link, i.e. dedicated relaying link).  FFS details of inter-layer interaction.

Proposal 20 (easy): For indirect to direct path switch, layer 2 link release procedure as legacy can be used when Remote UE and Relay UE execute PC5 unicast link release procedure.

Proposal 26 (easy) (18/19): For indirect to direct path switch, the RRC Reconfiguration message for Relay UE is intended to release Uu and PC5 RLC configuration for relaying, and bearer mapping configuration between PC5 RLC and Uu RLC.

NOTE 1: P17 was edited after agreement for clarity (deletion marked with strikeout).  Checked in email discussion [AT115-e][600].

NOTE 2: P26 was edited after agreement for clarity (deletion marked with strikeout, insertion marked with underline).  Checked in email discussion [AT115-e][600].

Agreement:

Proposal 2	RAN2 to confirm that tThe Remote UE shall report only the Relay UE candidate(s) that fulfil the higher layer criteria. FFS is if also AS criteria should be taken into account.

NOTE: P2 was edited after agreement for clarity (deletions marked with strikeout, insertions marked with underline).  Checked in email discussion [AT115-e][600].

Lossless delivery/PDCP status report:

Agreement:

Proposal 13	RAN2 to confirm that tThe DL/UL lossless delivery during the path switch is done according to the PDCP status report. FFS if there is spec impact.

NOTE: P13 was edited after agreement for clarity (deletion marked with strikeout, insertion marked with underline).  Checked in email discussion [AT115-e][600].

Agreements:

Proposal-1:  Agree Proposal 15 within R2-2107710:  for indirect to direct path switch, RRC Reconfiguration message to Relay UE can be sent any time after step 3 based on gNB implementation, as in the Figure 4.5.4.1-1.

Proposal-2:  Agree reworded Proposal 16 within R2-2107710:  for indirect to direct path switch, either Relay UE or Remote UE can initiate the PC5 unicast link release (PC5-S) (i.e. for Remote UE it should be after step 3; for Relay UE it should be after step 6), and upon the initiation of link release, the timing to execute link release is up to UE implementation.

Proposal-3:  Agree reworded Proposal 18 within R2-2107710:  for indirect to direct path switch, Remote UE can execute PC5 connection reconfiguration to release PC5 RLC for relaying upon reception of RRC Reconfiguration by gNB in Step 3, and Relay UE can execute PC5 connection reconfiguration to release PC5 RLC for relaying upon reception of RRC Reconfiguration by gNB in Step 6.

Proposal-4:  Agree original Proposal 22 within R2-2107710:  for indirect to direct path switch, step 8 can be executed in parallel or after step 5.

Proposal-5:  Agree reworded Proposal 18 within R2-2107710: for direct to indirect path switch, the PC5 connection setup procedure is executed upon reception of RRC Reconfiguration for path switch in step 3 if the PC5 connection has not been setup yet.

Data forwarding:

Proposal-6:  Agree original Proposal 21 within R2-2107710:  for indirect to direct path switch, Relay UE does not perform data forwarding back to gNB for Remote UE.

Reconfiguration message contents (remote UE):

Proposal-7:  Agree original Proposal 25 within R2-2107710:  for indirect to direct path switch, the contents in RRC Reconfiguration message for Remote UE can be same as legacy NR RRC Reconfiguration with sync.

Trigger for relay UE reconfiguration:

Proposal-8:  Agree original Proposal 30 within R2-2107710:  for direct to indirect path switch, additional indication from RRC_CONNECTED Relay UE to gNB is not necessary to initiate Relay UE’s reconfiguration upon establishing unicast link with Remote UE.

Reconfiguration message contents (relay UE):

Proposal-9 (modified):  Agree original Proposal 32 within R2-2107710:  for direct to indirect path switch, the contents in RRC Reconfiguration message for Relay UE can include at least Uu and PC5 RLC configuration for relaying, and bearer mapping configuration.

S-measure:

Proposal-10:  S-measure criteria is not used by the Remote UE for direct-indirect path switch.

Agreements:

Proposal-11 (modified):  As a baseline, SL-RSRP of the serving relay is used as the SL measurement quantity for the case of path switch from indirect to direct path.

Proposal-12:  SD-RSRP is used as the SL measurement quantity for the case of path switch from direct to indirect path.

Stage 2 baseline:

Proposal-18: Use the procedure text and figures proposed at R2-2107046 for L2 Relay service continuity as the baseline to update the running stage 2 CR.

RAN2 #116

Agreements:

Proposal 1: Legacy Uu RRC measurement configuration and reporting signaling with extensions for relay case is used to configure Remote UE to perform Uu and SL measurements for direct-to-indirect and indirect-to-direct path switch.

Proposal 2 (modified): Legacy Uu measurement object (i.e. MeasObjectNR) is used to configure measurement on neighbor Uu frequencies for indirect-to-direct path switch, and legacy sidelink measurement object (i.e. SL-MeasObject) is used to configure measurement on candidate Relays for direct-to-indirect path switch.  Uu measurement operation according to legacy principles still applies for Uu frequencies.

Agreement:

Proposal 4: When SL-RSRP of the serving relay is not available, SD-RSRP is used as the SL measurement quantity.  FFS how to measure SD-RSRP and if there would be a separate threshold for this case.

Agreement:

Proposal 5: The following new events are to be defined:

‐	Event-X for indirect-to-direct path switch: serving relay becomes worse than threshold-X1 and neighbor Uu cell becomes better than threshold-X2.

‐	Event-Y for direct-to-indirect path switch: serving Uu cell becomes worse than threshold-Y1 and candidate relay becomes better than threshold-Y2.

This does not exclude the use of the legacy S2 event.

Agreements:

Proposal 18: RAN2 does not consider the sharing of unicast link between relay service and non-relay service in L2 relay, and the related descriptions are to be removed from stage 2 running CR.

Proposal 12: During indirect-to-direct path switch, Remote UE or Relay UE’s AS layer releases PC5-RRC connection and indicates upper layer to release PC5 unicast link after receiving RRC reconfiguration from gNB.

Proposal 13: The existing T304 is used for indirect-to-direct path switch.

Agreements:

Proposal 24 (modified): The legacy PDCP re-establishment or data recovery in UL should be performed by the Remote UE during path switch if gNB configures it.

Proposal 25: No spec impact is required for DL lossless transmission during path switch.