3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-2200418

Online, January 17 – January 25, 2022

Source:	CATT

Title:	Analysis on UE capability for RAN slicing enhancement

Agenda Item:	8.8.4

Document for:	Discussion and Decision

Introduction

In RAN2#116-e meeting, agreements on UE capability for RAN slicing were achieved:

- As baseline, consider the following capabilities. FFS on details, can consider changes in the next meeting. - #1: UE indicates its support of slice based cell reselection in the UE capability signalling with the following TS38.306 description. Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. sliceInfoforCellReselection-r17 Indicates whether the UE supports sliceInformation on RRC release for slice based cell reselection in RRC _IDLE and RRC INACTIVE as defined in TS 38.304 [21].; UE; No; No; No. - #2: Since slice based RACH is only applicable for UE in RRC IDLE and RRC INACTIVE, there is no need for explicit capability to inform network and should just be “Optional without UE capability” as follow under Section 5.4 Other features: Definitions for feature. Slice based random access It is optional for UE to support slice based random access as specified in TS 38.321 [8]..

In this document, we continue to discuss the issues for UE capability for RAN slicing enhancements.

Discussion

  - Slice specific cell reselection

It has been agreed in principle that UE will report capability to the network indicate whether it will support slice information on RRCRelease for slice based cell reselection in RRC_IDLE and RRC_INACTIVE. Regarding the UE feature without UE radio access capability parameters for slice specific cell reselection with slice specific frequency priority signalled in SIB, it can be supported in Clause 5.4 in 38.306 without too much complexity. And it can be captured as:

Slice specific cell reselection It is optional for UE that is configured to support slice specific cell reselection with slice specific frequency priority[8].

It is proposed that:

Proposal 1: Introducing one optional feature in Clause 5 for slice-specific cell reselection with the following TS38.306 description:

Slice specific cell reselection It is optional for UE that is configured to support slice specific cell reselection with slice specific frequency priority[8].

  - Slice-based RA prioritization

UE feature to support Random access prioritization for MPS and MCS has been supported in 38.306. And in the running CR 38.331 [1] and 38.321 [2], slice-based RA prioritization has been captured. Hence, a similar UE feature to support Random access prioritization for slice based RA prioritization should also be supported in 38.306. It can be captured as:

Random access prioritization for Slice It is optional for UE that is configured to support random access prioritization for slice as specified in TS 38.321 [8].

Proposal 2: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access prioritization for Slice It is optional for UE that is configured to support random access prioritization for slice as specified in TS 38.321 [8].

  - Slice-based RA prioritization

Regarding the UE capability for slice based RA prioritization, we think this can also be supported as one optional feature similarly. It can be one separate capability from slice based RA prioritization. And it can be captured as:

Random access partitioning for Slice It is optional for UE that is configured to support random access partitioning for slice as specified in TS 38.321 [8].

Proposal 3: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access partitioning for Slice It is optional for UE that is configured to support random access partitioning for slice as specified in TS 38.321 [8].

- Conclusion

In this document, we analyse the issues for UE capability for RAN slicing enhancement, and we propose that:

Proposal 1: Introducing one optional feature in Clause 5 for slice-specific cell reselection with the following TS38.306 description:

Slice specific cell reselection It is optional for UE that is configured to support slice specific cell reselection with slice specific frequency priority[8].

Proposal 2: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access prioritization for Slice It is optional for UE that is configured to support random access prioritization for slice as specified in TS 38.321 [8].

Proposal 3: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access partitioning for Slice It is optional for UE that is configured to support random access partitioning for slice as specified in TS 38.321 [8].

- Reference

- R2-2111444 , 38331 Running NR RRC CR for RAN slicing (Huawei)

- R2-2111435, 38.321 running CR for RAN Slicing;