3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-22xxxxx

Online, Jan. 17th – 25th, 2022

Agenda item:	8.1.3.1

Source	LG Electronics Inc.

Title	[Report of] e-mail discussion: [AT116bis-e][020][MBS] Multicast Start (LGE)

Document for	Discussion and Decision

# 1. Introduction

This document aims at gathering and summarizing companies’ views for the following e-mail discussion:

- [AT116bis-e][020][MBS] Multicast Start (LGE)

Scope: Address open issues related to Multicast start (ref green-marked Open issues R2-2200022), Group Notification - Applicability of PEI/WUS, applicability of short message. Connection establishment - Access Control and cause value

Intended outcome: Report

Deadline: Friday W1 for online CB.

# 2. Contact information

| Company | Name | Email |
| --- | --- | --- |
| Qualcomm | Prasad Kadiri | pkadiri@qti.qualcomm.com |
| Nokia | Jarkko Koskela | Jarkko.t.koskela@nokia.com |
| OPPO | Shukun Wang | wangshukun@oppo.com |
| Huawei, HiSilicon | Bin Xu | xubin10@huawei.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Futurewei | Jialin Zou | Jialinzou88@yahoo.com |
| Samsung | Vinay Kumar Shrivastava | shrivastava@samsung.com |
| Spreadtrum | Lifeng Han | Lifeng.han@unisoc.com |
| CATT | Rui Zhou | zhourui@catt.cn |
| Apple | Fangli XU | fangli_xu@apple.com |
| Sharp | Fangying Xiao | fangying.xiao@cn.sharp-world.com |
| vivo | Yitao Mo (Stephen) | yitao.mo@vivo.com |
| ITRI | Lin, Jung Mao | moumou3@itri.org.tw |
| Intel | Ziyi Li | ziyi.li@intel.com |
| ZTE | Tao QI | qi.tao3@zte.com.cn |

# 3. Discussion

## 3.1 Access Category for multicast

Upon initiation of the RRC connection establishment procedure, the UE performs the unified access control procedure using the Access Category and Access Identities provided by NAS. According to TS24.501, if the access attempt is for response to unicast paging, NAS sets the Access Category to ‘0’.

$5.3.3.2 in TS38.331 Upon initiation of the procedure, the UE shall: - if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection: 2> perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

Table 4.5.2.2 in TS24.501 Rule #; Type of access attempt; Requirements to be met; Access Category. 1; Response to paging or NOTIFICATION over non-3GPP access; 5GMM connection management procedure initiated for the purpose of transporting an LPP or location event report message without an ongoing 5GC-MO-LR procedure; Access attempt to handover of ongoing MMTEL voice call, MMTEL video call or SMSoIP from non-3GPP access; Access attempt is for MT access, or handover of ongoing MMTEL voice call, MMTEL video call or SMSoIP from non-3GPP access; 0 (= MT_acc).

In RRC connection resume case, if the resumption of the RRC connection is triggered by response to NG-RAN paging, AS layer autonomously selects the Access Category ‘0’.

$5.3.13.2 in TS38.331 Upon initiation of the procedure, the UE shall: - if the resumption of the RRC connection is triggered by response to NG-RAN paging: 2> select ‘0’ as the Access Category;

The first issue is whether Access Categoty ‘0’ can be re-used when RRC connection establishment is initiated in respons to the group paging, i.e. group notificaiton for multicast session activation. If the Access Category is ‘0, UE considers the access attempt as allowed.

$5.3.14.2 in TS38.331 Upon initiation of the procedure, the UE shall: (omitted) - else: 2> if the Access Category is ‘0’: 3> consider the access attempt as allowed;

The concern on the Access Catsgory ‘0’ raised in [1][3][5][8] is that upon receiving the group paging, all IDLE/INACTIVE UEs who have joined the multicast session will trigger the RACH almost at the same time. If no access control mechanism is applied in this case, a severe RACH congestion will occur. To mitigate the RACH congestion, [1][3][5][8] proposed to introduce new Access Category for group paging so that the access attempt initiated by the group paging can be under the unified access control. [2] proposed to introduce MBS spesific T390, but considering T390 is per Access Category, [2] also seems to support new Access Category.

On the contrary, [6][7] propose not to introduce MBS-specific UAC, i.e. to use Access Category ‘0’, for the following reasons: Firstly, the group notification is a kind of paging and intends to indicate the MT access for a multicast session to UE. Secondly, the paging for multicast activation notification is transmitted in the legacy Pos, not in MBS-specific PO, to avoid the PRACH capacity issue. Thirdly, for smart network implementation, the network can keep some Ues having the same legacy Pos in RRC_CONNECTED when the multicast session is deactivated to further relieve the PRACH capacity issue. Finally, RAN2 has agreed not to prioritize addressing of PRACH capacity issue due to group notification. Regarding this, companies are requested to answer the following questions.

#### Question 1. 	Which Access Category is suitable when the group paging is received?

- Option 1: 	Access Category ‘0’ [6, 7].

      - In RRC_IDLE, NAS provides Access Category ‘0’ to AS.

      - In RRC_INACTIVE, AS selects Acess Category ‘0’.

- Option 2: 	A new Access Category specific to MBS [1, 2, 3, 5, 8]. (whether to define one or multiple Access Categories for MBS will be discussed in Question 3.)

      - In RRC_IDLE, NAS provides a new Access Category to AS.

      - In RRC_INACTIVE, AS selects a new Access Category.

- Any other option?:

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 2 | Even though Multicast paging is sent in UE specific Pos, multiple Ues can have same PO and when multiple Ues respond to TMGI paging it can cause UL RACH overload and UL signaling overload. In order to randomize response time for Group paging, we believe it is better to specify new Access Category and specify Access delay timer to randomize group paging response. Note that idea is not to perform access barring. |
| Nokia | option 1 | This has been already agreed when RAN2 agreed no to optimize possible PRACH capacity issue. If we want to solve that we need to start looking for multitude of different possible alternatives and there is no time for that. |
| MediaTek | Option 1 | We did not see the benefit to introduce new Access Category for the response of group paging. |
| OPPO | Option 1 | For paging triggered RRC establishment, no need to specify new AC to perfrom UAC. |
| Huawei, HiSilicon | Option 1 | For the reasons rapporteur listed above, there is no need to introduce a new MBS specific Access Category. |
| Xiaomi | Option 2 | We think that it is important to avoid the collision of uplink transmissions when the same group paging received by many Ues at the same occasion. Using a new AC for MBS can reuse the framework of the legacy UAC. |
| Ericsson | Option 1 |  |
| Futurewei | Option 1 |  |
| Samsung | Option 2 | There is a need to distribute the access attempts and also differentiate the multiple multicast sessions with regard to their latency needs. |
| Spreadtrum | Option 2 | We think a new Access Category is benificial for load control in gNB. Because the gNB have no idea about how many Ues will be paged. |
| CATT | Option 2 | It is beneficial to enable gNB to control the access attempt for the multicast reception purpose. |
| Apple | Option 2 | The new access category can help NW to perform the different access controls for the multicast activation case and the MT case. |
| Sharp | Option 1 | We agree with the reasons listed above for the choice of option 1. |
| vivo | Option 1 | We agree with the reasons quoted from [6]. |
| ITRI | Option 1 | We have been agreed that no to optimize possible PRACH capacity issue. |
| Intel | Option 1 | There is no need to introduce new Access Categories for multicast |
| ZTE | Option 1 |  |

UE may initiate the RRC connection establishment for the purpose of receiving the multicast session even if the group paging is not received, e.g. when a multicast session of interest is about to start. Since this is a kind of MT call but not initiated by paging, all existing Access Categories seem not suitable for this case. Therefore, even though the MBS specific Access Category is not introduced, RAN2 needs to newly specify which Acess Category to use in this case. Therefore, companies are invited to answer the following question:

#### Question 2. 	Which Access Category is suitable when RRC connection establishment is initiated to receive the multicast session without receiving paging, e.g. when a multicast session of interest is about to start?

- Option 1: 	An existing Access Category

      - NAS provides one of the existing Access Categories to AS (if you support this option, please describe also which Access Category can be used in this case).

- Option 2: 	A New Access Category specific to MBS

      - NAS provides a new Access Category to AS. (whether to define one or multiple ACs for multicast will be discussed in Question 3.)

- Any other option?:

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 2 | In case of RAN congestion, it is possible for gNB to configure different access barring parameters for different access categories (device traffic type) and enable/disable access to different access identities (device type). Similarly, gNBs should have ability to control multicast UEs access attempts. gNB may allocate different priority for unicast vs multicast UE initiated access attempts. This is possible by introducing new access categories for multicast access attempts. |
| Nokia | Option 1 | MT access category (i.e. 0) seems good one here |
| MediaTek | Option 1 | Same view as Nokia |
| OPPO | Option 2 | It is benefit to speficy MBS specific AC. |
| Huawei, HiSilicon | Option 1 | If the UE already joined the multicast group, the UE is either kept in RRC-CONNECTED for multicast reception, or has retruned to IDLE or INACTIVE and is waiting for group paging. Not sure what scenairo is being discussed here. |
| Xiaomi | Option 2 | We think that using a new AC is simple way to mitigating the congestion issue for initial access. |
| Ericsson | Option 1 |  |
| Futurewei | Option 1 |  |
| Samsung | Option 2 |  |
| Spreadtrum | Option 2 | gNB may apply different access control policies for unicast and multicast. |
| CATT | Option 2 | Same comments as Q1 |
| Apple | Option 2 | gNB can give the different access control for the multicast and unicast. |
| Sharp | Option 1 |  |
| vivo | Comments | We are wondering whether the case illustrated by the moderator is valid or not. Based on SA2 spec TS23.247, there is no difference between the session start and session activation. So only two kinds of RRC connection establishment/resume are foreseen, one is multicast session join, the other is the multicast session activation making session transmit into the active state. The former one can be treated as MO-signalling and the latter one can be done based on Q1’s answer. |
| ITRI | Optino 1 |  |
| Intel | Opton 1 | It’s not clear why the UE will go to RRC_CONNECTED without receiving paging? We have agreed UE can only receive mutlicast service in RRC_CONNECTED mode, which is paged by group paging. |
| ZTE | Option 1 |  |

[3] says that the multicast session can be categorized into regular and high priority sessions, and two new access categories can be assigned to regular multicast services and high priority multicast services. With different multicast access categories, it gives flexibility for gNB to configure different access barring parameters and control access for UEs based on priority of different multicast services.

However, to enable NAS to assign different Access Categories depending on the priority of the multicast session, AS should inform NAS of the the priority of the multicast session when the group paging is received. Alternatively, NAS should know the priority for each multicat session, but we are not sure if it is possible. Furthermore, the multiple Access Categories for MBS would substantially add to CT/SA works. For these reasons, [1] propose to introduce a single MBS specific Access Category.

#### Question 3. 	If MBS specific Access Category is introduced, how many new Access Categories should be introduced?

- Option 1: 	A single Access Category specific MBS [1]

- Option 2: 	Two Access Categories, i.e. one for regular and the other for high priority multicast session [3]

- Any other option?:

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 2 | When UE initiates RRC connection for the purpose of joining Multicast session (not group paging response), NAS will provide new AC to RRC. NAS knows which multicast service UE wants to join and priority is detemined by NAS based on device configuration or upper layer indication (which is in the domain of CT1). So, we suggest to send LS to both CT1/SA1 and cc: SA2. |
| OPPO | Option 2 | The LS to SA1/CT1 is necessary to indicate RAN2’ understanding and suggestion. |
| Xiaomi | Either Option 1 or Option 2 |  |
| Samsung | Option 2 | We consider multicast services may differ in priority and criticality. As this needs to be determined at NAS layer, we support sending LS to CT1/SA2 |
| Spreadtrum | Option 1 or Option 2 | Should be decided by SA2. |
| CATT | Option 1 or Option 2 | Up to SA/CT1 |
| Apple | Option 1 or 2 | It can be decided in CT1/SA2. |

## 3.2 Interaction between AS and NAS

Upon receiving the Paging message, UE in RRC_IDLE forwards the ue-Identity and accessType (if present) to the upper layers.

- if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message: 2> if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers: 3> forward the ue-Identity and accessType (if present) to the upper layers;

If different handling for group paging is required in NAS, e.g. to assign MBS specific Access Category, AS should forward different information to NAS, e.g. multicast session ID.  However, as in option 1 in Q1, if NAS doesn’t need to differentiate the group paging from unicast paging, the same S behavior can be adopted, i.e. AS forwards the ue-Identity to NAS, though its ue-Identity is not indicated in the paging message. Therefore, compnies are required to answer the folloing question:

#### Question 4. 	Which information should be reported from AS to NAS upon receiving the group paging in RRC_IDLE?

- Option 1: 	ue-Identity

- Option 2: 	multicast session ID

- Any other option?:

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 2 | Based on AS reported TMGI, NAS makes decision. If both UE-Identity and TMGI are received in same page message then UE has to provide both of them to NAS. |
| Nokia | Not critical to specify – see the comment | SA/CT will request if we need to add some specific information to be sent to upper layers. In fact we could report everything and they will just use what they need. |
| MediaTek |  | This may be handled by UE implementation |
| OPPO | Option 2 if necessary | We are not sure whehter it is necessary to specify it in spec. |
| Huawei, HiSilicon | Option 2 | The multicast session ID might be needed even if no MBS specific AC is introduced. |
| Xiaomi | Option 2 |  |
| Ericsson |  | This should be based on what is requested, i.e. discussed elesewhere than RAN2 first. |
| Futurewei | Option 2 |  |
| Samsung | Option 2 |  |
| Spreadtrum | Option 2 | Multicast session ID is needed for NAS. |
| CATT | Option 2 |  |
| Apple | Option 2 |  |
| Sharp | Option 2 |  |
| vivo | Option 2 |  |
| ITRI | Option 2 |  |
| Intel | Option 2 |  |
| ZTE | Option 2 | If found in paging record. |

## 3.3 Establishment/Resume Cause for multicast

When UE sets the contents of RRCSetupRequest message, RRC layer sets the establishmentCause in accordance with the information received from upper layers. According to TS24.501, when Access category 0 is used, i.e. unicast paging is received, NAS sets the establishment casue to ‘mt-Access’.

$5.3.3.3 in TS38.331 - if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN): 2> set the establishmentCause to mps-PriorityAccess; - else: 2> set the establishmentCause in accordance with the information received from upper layers;

Table 4.5.6.1 in TS24.501 Rule #; Access identities; Access categories; RRC establishment cause is set to. 5; 0; 0 (= MT_acc); mt-Access.

Upon receiving the RAN paging message, the UE in RRC_INACTIVE sets the resume casue in accordance with Access Identity configured by upper layers.

$5.3.2.3 in TS38.331 - if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message: 2> if the ue-Identity included in the PagingRecord matches the UE’s stored fullI-RNTI: 3> if the UE is configured by upper layers with Access Identity 1: - initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess; 3> else if the UE is configured by upper layers with Access Identity 2: - initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess; 3> else if the UE is configured by upper layers with one or more Access Identities equal to 11-15: - initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess; 3> else: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

If gNB has different access polices for the MBS service and unicast service, the MBS specific establishment/resume casue would be beneficial for gNB to identify the purpose of UE’s access attempt to determine whether to accept or reject RRC setup/resume request. Therefore, [3] and [8] propose to introduce the MBS specific establishment/resume casue. [5] says that the MBS specific establishment/resume casue is also beneficial to early notify the network of the MBS interest of the UE.

According to [1][2][6], PTM transmission would be mainly used for multicast session and their access doesn’t not increase the RAN overload. Therefore, once UE has passed RACH procedure, there is no reason to reject the access for multicast reception based on the establishment cause, and ‘mt-Access’ that is used when unicast paging is received seems suitable also for access initiated by group paging.

On the other hand, [7][9] propose to follow the legacy UE behavior for resume case, i.e. set the resume casue in accordance with Access Identity configured by NAS. Companies are then requested to answer the following questions:

#### Question 5. 	Which Establishment Cause should be used when RRC connection establishment is initiated in response to the group paging?

- Option 1: 	NAS provides ‘mt-Access’ as a establishment cause to AS [1, 2, 6, 7, 9].

- Option 2:	NAS provides ‘new estalishment cause’ to AS [3, 5, 8].

- Any other option?

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 1 | New establishemnt causes are useful for UE initiated MBS session join instead of group paging response. [3] also mentioned same. |
| Nokia | Option 1 | We have not identified any need for new access causes |
| MediaTek | Option 1 |  |
| OPPO | Option 1 |  |
| Huawei, HiSilicon | Option 1 | Existing establishment/resume cause can be reused. The network might not be able to derive whether additional load is needed for the UE even if we introduce an MBS cause, for example. If the MBS service required by the UE is not being transmitted in the cell, the load will be increased anyway. |
| Xiaomi | Option 1 |  |
| Ericsson | Option 1 |  |
| Futurewei | Option 1 |  |
| Samsung | Option 2 | Useful for differential access treatment and suitable configuration by the gNB |
| Spreadtrum | Option 1 |  |
| CATT | Option 2 | For load balance, gNB may accept or reject RRC connection request based on the establishment cause in MSG3 from UE. Since multicast services could have different priorities compared to unicast services, it is beneficial to specify a new establishment cause for the purpose of multicast reception. |
| Apple | Option 2 | In RAN overload case, if gNB has different access polices for the MBS service and unicast service, gNB can decide to deprioritize or prioritize the MBS triggered initial access based on the resume/establishment cause. |
| Sharp | Option 1 |  |
| vivo | Option 1 | Same as the reasoning in Q1. When access category 0 is suitable for group paging, then the corresponding Establishment Cause is mt-Access. |
| ITRI | Option 1 |  |
| Intel | Option 1 | The primary motivation for establishment cause is to reject the access in case of RAN overload. In case of multicast access control, the same discussion regarding UAC applies since gNB has already allocated resource before UE initiates the state transition to RRC_CONNECTED. Given that paging is used for group notification, existing establishment cause mt-Access is sufficient. |
| ZTE | Option 1 |  |

#### Question 6. 	Which Resume Cause should be used when RRC connection resume is triggered by response to the group paging?

- Option 1: 	AS sets the resume casue to ‘mt-Access’ [1, 2, 6].

- Option 2: 	AS sets the resume casue in accordance with Access Identity configured by NAS [7, 9].

- Option 3: 	AS sets the resume casue to ‘new resume cause’ [3, 5, 8].

- Any other option?

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 1 |  |
| Nokia | Option 1 | We have not identified any need for new access causes |
| MediaTek | Option 1 |  |
| OPPO | Option 1 |  |
| Huawei, HiSilicon | Option 2 |  |
| Xiaomi | Option 1 |  |
| Ericsson | Option 1 |  |
| Futurewei | Option 1 |  |
| Samsung | Option 3 |  |
| Spreadtrum | Option 1 |  |
| CATT | Option 3 |  |
| Apple | Option 3 |  |
| Sharp | Option 1 |  |
| vivo | Option 1 | Same as the reasoning in Q1. When access category 0 is suitable for group paging, then the corresponding Establishment Cause is mt-Access. |
| ITRI | Option 1 |  |
| Intel | Option 1 |  |
| ZTE | Option 1 |  |

#### Question 7. 	Which Establishment Cause should be used when RRC connection establishment is initiated to receive the multicast session without receiving paging, e.g. when a multicast session of interest is about to start?

- Option 1: 	NAS provides ‘mt-Access’ as a establishment cause to AS.

- Option 2:	NAS provides ‘new estalishment cause’ to AS.

- Any other option?

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 2 | When a multicast UE is accessing gNB for multicast service joining purpose (not for group paging response), it is beneficial for gNB to identify the purpose of UE’s access attempt to determine whether to accept or reject RRC setup/resume request. This can be accomplished by specifying a new establishment cause and a resume cause for multicast service in both RRC setup request message and RRC resume request message. |
| Nokia | Option 1 | We have not identified any need for new access causes |
| MediaTek | Option 1 |  |
| OPPO | Option 2 | New case can be used for gNB to decide to reject or not due to CN congestion. |
| Huawei, HiSilicon |  | If the UE already joined the multicast group, the UE is either kept in RRC-CONNECTED for multicast reception, or has retruned to IDLE or INACTIVE and is waiting for group paging. Not sure what sceniro is being discussed here. |
| Xiaomi | Option 1 |  |
| Ericsson | Option 1 | Agree w Huawei that the case here is not clear. |
| Futurewei | Option 1 |  |
| Samsung | Option 2 | Agree with Qualcomm. This also facilitates gNB to early know UE’s interest in MBS and provide appropriate BWP/CFR configuration. |
| Spreadtrum | Option 1 |  |
| CATT | Option 2 | Agree with Qualcomm |
| Apple | Option 2 | Agree with Qualcomm |
| Sharp | Option 1 |  |
| vivo | Comments | Please see our comments for Q2. |
| ITRI | Option 1 |  |
| Intel | Option 1 |  |
| ZTE |  | Not sure what scenario is being discussed here. Confused too. |

Since new Access Category or new Establishment cause  has impact on other working groups, e.g. CT1, SA1 and SA2, if it is agreed to introduce the new Access Category or new Establishment cause, RAN2 may need to send an LS to inform other working groups of the RAN2 agreements on that.

#### Question 8.	Do you agree that if RAN2 agree to introduce new Access Category or new Establishment cause, RAN2 needs to send an LS to inform CT1/SA1/SA2 of the RAN2 agreements on that?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes, but | We’t better avoid that by not introducing any new Access Category or new establishment cause. |
| Xiaomi | Maybe | We could use some operator-reserved AC value (i.e. 32-63). |
| Futurewei | Yes, but | LS may not be needed as we don’t see new AC is required for MBS. |
| Samsung | Yes |  |
| Spreadtrum | Yes |  |
| CATT | Yes |  |
| Apple | Yes |  |

## 3.4 Group paging indication for UE power saving

In R17 power saving WI, the paging subgroup has been introduced and UE determines whether to monitor upcoming paging occasion based on PEI. I.e. If UE’s subgroup ID is indicated in PEI, UE monitors the associated paging occasion. UE doesn’t monitor the associated paging occasion if the subgroup ID is not indicated in the PEI. Since the paging subgroup is not applied to the group paging, when a group paging is transmitted, the network doesn’t indicate the subgroup that the UE which has joined the multicast session belongs to, if there is no unicast paging for the paging subgroup.

The first issue raised by [1][7] is that UE I wants to receive the group paging should monitor every paging occasion to check whether the group notification is included in the paging message, even though it supports the paging subgroup. Therefore, [1][7] propose to introduce a new indication in PEI to indicate whether the upcomming paging message includes the group notification so that the paging-subgroup capable UE can skip the paging occasion based on PEI though the UE supports the group paging.

Regading this, [5] says that it may be quite complex to include new notification, and proposes that MBS capable UE should monitor the paging occasions though its subgroup ID is not indicated in the PEI.

#### Question 9. 	Do you agree to solve the problem that MBS capable UE should monitor the paging occasion even though its paging subgroup ID is not indicated in the PEI?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| Qualcomm | Yes | PDCCH based PEI design involves RAN1 work as well. We need to consult RAN1. |
| Nokia | No | Unnecessary optimization at this point of time. |
| MediaTek | No | Not sure we have enough time to discuss this issue when approaching the end of the release |
| OPPO | Yes | In R17 power saving work item, the WUS for paging is introduced for UE power saving purpose. The PEI before PO will indicate whether there is paging for Ues in one UE Group. If no, the UE will skip the PDCCH monitoring for paging reception. Case 1: For Ues in a PO has deactivated MBS session, if the PEI only consider UE specific ID, i.e. not consider TMGI, and indicates there is no paging in the following PO. There are two UE actions. UE behaviour 1: The UE will miss the paging for MBS activation. UE behaviour 2: The UE with deactivated MBS session will ignore the PEI and always monitor PO. It means the paging WUS does not work for UE with deactivated MBS session even if there is paging WUS configuration. Case 2: No impact. Case 3: The UE will wake up to receive paging message and found no paging for this UE due to MBS paging. For case 1 Ues, the PEI will only indicate whether there is UE specific paging followed in PO, not indicate whether there is MBS paging or not. Consider the UE behaviour ½, the PEI should be extended to consider MBS paging case. |
| Huawei, HiSilicon | Yes | Otherwise it will add unnecessary compleity to CN handling and will add extra UE power consumtion. |
| Xiaomi | No | RAN1 MBS work has completed. We would quite doublt about the possiblity of introducing new features in RAN1. |
| Ericsson | No | Can be discussed in future rel. |
| Futurewei | Yes/Maybe | There is power saving with this feature. If time allows in R17, we could work with RAN1 on this. |
| Samsung | Yes | There is an implementation complexity vs power saving gain issue for multicast Ues |
| Spreadtrum | Yes | It is beneficial for UE power saving. |
| CATT | No | Can be considered in later release. |
| Apple | Yes | It may need to work with RAN1. |
| Sharp | No | Not sure if we have sufficient time to have this optimisation. |
| vivo | No | Parallel feature combination is not expected in this Release. |
| ITRI | No | This feature could be discussed in later release. |
| Intel | No | For PEI solution, RAN1 agreed that there’s only 8 subgroups supported for PEI, which is based on the assumption that all UEs are receiving unicast traffic. It is unclear whether the performance will be impacted by using 1 subgroup for multicast or not. Considering the remaining time of Rel-17 is not enough for RAN1 to evaluate the corresponding performance (i.e. one subgroup used for multicast, 7 subgroup used for UEs’ unicast traffic), it is proposed to not to use PEI as multicast activation notification power saving indication |
| ZTE | No | Maybe in future releases. |

#### Question 10. If you agree to solve the problem in Q9, which option do you prefer?

- Option 1: 	Include a group paging specific indicator in PEI [1,7]

- Option 2: 	Any other option?

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 1 |  |
| OPPO | Option 1 | Option 1: Configure one group ID for each TMGI. Option 2: Include indication in PEI to indicate whether there is TMGI record list in following paging message. |
| Huawei, HiSilicon | Option 1 | We think the group paging can be treated as a special RAN based group in PEI and thus RAN1 is not needed to be involved. |
| Futurewei | Option 1 |  |
| Samsung | Option 1 | Simpler approach like a configured paging sub-group for multicast group notification can be checked |
| Spreadtrum | Short messages based solution | There are another two options: Option2: Short Messages based solution Using a new bit in Short Message to indicate ‘group-only Paging is included’ is proposed in R2-2200640. Option3: Short Messages indicator based solution Using the reserved value “00” of “Short Messages Indicator” to indicate “Paging only contains multicast activation notification” is proposed in [10]. The option 1 only is applicable for WUS-enabled UEs. The option 3 will use up the only reserved code point in Short Message indicator and needs confirmation from RAN1. So we prefer the option2. |
| Apple | Option 1 |  |

Another issue raied in [7] is an impact of the group paging on legacy UE. If a paging message consits of group paging only, legacy UE doesn’t need to receive the paging occasion. So [7] proposes to redefine the state ‘10’ of short message indicator to also cover the case where the scheduling information for group-only paging present in the DCI. Then, legacy Ues will interpret “10” as “Only short message is present in the DCI” and would skip reading the Paging message which carries group paging only. For the same purpose, [10] proposes to use the reserved state ‘00’ of short message indicator.

Regarding this issue, [2][6] says that the group paging is not frequent, so the power saving gain of skipping the Paging message for MBS is quite marginal.

#### Question 11. 	Do you agree to solve the problem that legacy UE monitors the paging message even though the paging message consists of group notification only?

| Company | Yes/No | Comments / justification |
| --- | --- | --- |
| Qualcomm | No |  |
| Nokia | No | It was conscious decision which was highlighted by us when the decision was done. If people insist on reverting the decision of not optimizing this then we should just define new paging occasions for multicast. But like said there is not really need to do anything as multicast paging is not going to happen frequently and impact to legacy Ues in neglible. |
| MediaTek | No |  |
| OPPO | No |  |
| Huawei, HiSilicon | Yes | To avoid extra power consumption of legacy Ues. |
| Xiaomi | No |  |
| Ericsson | No |  |
| Futurewei | No |  |
| Samsung | No |  |
| Spreadtrum | Yes |  |
| CATT | Yes | It is necessary to avoid impact to legacy Ues and also the MBS capable Ues not interested in multicast services. |
| Apple | No |  |
| Sharp | No |  |
| vivo | No | We have agreed that Use of paging in all (legacy) PO with PRNTI is the baseline assumption. We fail to figure out any good solution to avoid negative impacts on legacy implemented UEs on the market. |
| ITRI | No |  |
| Intel | No |  |
| ZTE | No | It would be very rare cases! |

#### Question 12. If you agree to solve the problem in Q11, which option do you prefer?

- Option 1: 	Redefine the state ‘10’ of short message indicator to indicate also that the scheduling information for group-only paging is present in the DCI [7].

- Option 2: 	Use the reserved state ‘00’ of short message indicator to indiate that the scheduling information for group-only paging is present in the DCI [10].

- Option 3: 	Any other option?

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 1 or 2 |  |
| Huawei, HiSilicon | Option 1 | There is only one reserved value in the short message indicator and it is wiser to keep it for future use considering option 1 can already solve the problem without consuming the last reserved value. |
| Spreadtrum | Option 1 | Agree with HW. |
| CATT | Option 2 | Option 2 is simpler. Option 1 will impact the legacy UEs as it will treat “10” as “Only short message is present in the DCI” but this is a false alarm to the legacy Ues, so an unexpected behaviour of reading the short message will be performed by the legacy UE even if none of the legacy short message bit 1-3 is set; And compare with option 2 which only read the short message indicator, option 1 which needs UE to read the short message will introduce more complex. |

## 3.5 Conflict between group paging and unicast paging

UE may receive the group paging and the unicast paging in the same paging message, but UE may be required to take different action depending on the paging type, e.g. to use different establishment/resume case. Therefore, the UE behavior in this case should be clarified.

- Case1: UE in RRC_IDLE receives the group paging and CN paging in the same paging message

It is FFS whether to use the legacy establshment cause, i.e. ‘mt-Access’, or a new MBS specific establishment casue [Question 5] and which information AS forwards to NAS upon receiving the group paging [Question 4]. If RAN2 agree to use ‘mt-Access’ as a establishment cause and forward ue-Identity to NAS, there is no conflict. However, if not, we need to discuss how to handle the conflict between the group paging and unicast paging.

Regarding this, [11] proposes that RRC forwards both the unicast and multicast paging information to NAS, and the NAS layer decides which service to trigger the initial access. As an alternative, though no one proposes it, RRC may select one paging and forwards the paging information of the selected paging to NAS layer. [10] proposes to set the establishment/resume cause to the one with higher priority between unicast and multicast, but does not describe the detailed AS/NAS behavior.

#### Question 13. When UE in RRC_IDLE simultaneously receives the group paging and CN paging, which option do you prefer?

- Option 1: 	RRC forwards both the unicast and multicast paging information to NAS, and the NAS layer decides which service to trigger the initial access. [11].

- Option 2: 	RRC selects one based on the priority and forwards only the information of the selected paging to NAS.

- Option 3: 	Any other option? (Up to UE implementation)

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 1 |  |
| Nokia | option 3 | UE handles unicast paging first (if any) then multicast paging. This is already current CR – no need for changes. |
| MediaTek | Option 1 |  |
| OPPO | Option 1 |  |
| Huawei, HiSilicon | Option 1 |  |
| Xiaomi | Option 1 |  |
| Ericsson | - | This seems already handled in the base line endorsed CR. As we do not see any good motivation for anything but mt-Acess, there is not really a need for any additional enhancements. |
| Futurewei | Option 1 |  |
| Samsung | Option 1 |  |
| Spreadtrum | Option 1 |  |
| CATT | Option 1 |  |
| Apple | Option 1 |  |
| Sharp | Option 1 |  |
| vivo | Option 3 | Another possible solution can be UE implementation. We think the Access Category and the establishmentcause for group paging are 0 and mt-Access respectively, which is the same as the unicast paging. No matter what info is forwarded, the NAS layer will do the same behavior and so does the RRC layer. So, it can be up to UE implementation. |
| ITRI | Option 1 |  |
| Intel | Option 1 |  |
| ZTE | Option 1 |  |

[10]If CN paging and group paging are in the same Paging message, the establishment/resume cause should be set to the one with higher priority between unicast and multicast. [11] Upon receiving one paging message including both the UE specific paging and the group paging for the joint MBS session, the IDLE/INACTIVE UE RRC layer forwards both the unicast and multicast paging info to NAS layer, and the NAS layer decides which service to trigger the initial access. [5] Proposal 11: When UE in RRC_INACTIVE state receives both RAN paging and group notification in the same paging message, UE initiates RRC connection resumption procedure with resumeCause set to higher priority among resume causes pertaining to two pagings. [5] Proposal 10: When the UE in RRC_INACTIVE state receives both CN paging and group notification in the same Paging message, • Option a: UE does not transit to RRC_IDLE and resumes RRC connection for group notification (multicast session activation) with resumeCause set to mt-Access. • Option b: UE transits to RRC_IDLE and forwards the UE identity and accessType (if present) as well as forward TMGI to upper layers. Upper layers need to establish the RRC connection for dual purposes – CN paging response and multicast session activation

- Case2: UE in RRC_INACTIVE receives the group paging and RAN paging in the same paging message

If RAN2 agree to go with option 1 or 3 in Question 6, we need to discuss how to handle the conflict between the group paging and RAN paging in RRC_INACTIVE. [5][10] proposes to set the resume cause based on the priority, and the rapporteur cannot  alternatives

#### Question 14. Do companies agree that when UE in RRC_INACTIVE simultaneously receives the group paging and RAN paging, RRC layer sets the resume cause based on the priority? If not, please provide your suggested change.

| Company | Yes/No | Comments / Suggestion |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Nokia | No | UE handles unicast paging first (if any) then multicast paging. This is already current CR – no need for changes. |
| MediaTek | Yes |  |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Xiaomi | No strong view | If the current running already prioritizes the unicast paging, we may not need to change anything. |
| Ericsson | No | See erlier comment |
| Futurewei | Yes |  |
| Samsung | Yes | RRC layer to set the resume cause based on the priority |
| Spreadtrum | No strong view |  |
| CATT | Yes |  |
| Apple | See comments | The priority should be clarified first. |
| Sharp | Yes |  |
| vivo | No | We are fine with the current CR. |
| ITRI | Yes |  |
| Intel | Yes |  |
| ZTE | No |  |

- Case3: UE in RRC_INACTIVE receives the group paging and CN paging in the same paging message

RRC forwards the ue-identity to NAS layer and goes to RRC_IDLE upon receiving CN paging in RRC_INACTIVE, meanwhile RRC initiates RRC connection resume procedure upon receiving the group paging.

$5.3.2.3 in TS38.331 2> else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers: 3> forward the ue-Identity to upper layers and accessType (if present) to the upper layers; 3> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause ‘other’.

[5] proposes to select one of following options:

- •	Option a: UE does not transit to RRC_IDLE and resumes RRC connection for group notification (multicast session activation) with resumeCause set to mt-Access.

- •	Option b: UE transits to RRC_IDLE and forwards the UE identity and accessType (if present) as well as forward TMGI to upper layers.

On the contrary, [12] proposes the UE shall assume that the network has lost the UE context and move to RRC_IDLE. It seems a reasonable assumption that the RRC connection resume would fail if the RRC state mismatch has happened. The difference between  [12] and option b[5] is whether to forward TMGI to NAS layer. If TMGI is forwarded, NAS can initiate the RRC connection establishment for dual purposes, i.e. CN paging response and multicast session activation.

#### Question 15. When UE in RRC_INACTIVE simultaneously receives the group paging and CN paging, which option do you prefer?

- Option 1: 	UE does not transit to RRC_IDLE and resumes RRC connection for group notification [5].

- Option 2: 	UE transits to RRC_IDLE and forwards the UE identity and accessType (if present) as well as forward TMGI to upper layers [5].

- Option 3: 	UE acts as if only CN paging is received, i.e. go to IDLE [12].

- Option 4: 	Any other option?

| Company | Preferred option | Comments / justification |
| --- | --- | --- |
| Qualcomm | Option 2 |  |
| Nokia | Implicitly option 2 | UE handles unicast paging first (if any) then multicast paging. This is already current CR – no need for changes. |
| MediaTek | Option 2 |  |
| OPPO | Option 2 | Follow legcy behaviour. |
| Huawei, HiSilicon | Option 2 or 3 | When the UE receives both CN paging and group paging in the same Paging message. The UE will assume that the network has lost UE context. Correspondingly, a UE shall move to RRC_IDLE and respond to the CN paging. |
| Xiaomi | Option 2 |  |
| Ericsson | Option 2 |  |
| Futurewei | Option 2 |  |
| Samsung | Option 2 |  |
| Spreadtrum | Option 2 |  |
| CATT | Option 2 |  |
| Apple | Option 2 |  |
| Sharp | Option 2 |  |
| vivo | Option 2 or 3 | Firstly, we think in this case UE should transit to the RRC_IDLE state because the RRC state is mismatched. As for the information that AS layer forwards to the upper layers, we think it is up to UE implementation based on our comments for Q13. |
| ITRI | Option 2 |  |
| Intel | Option 2 |  |
| ZTE | Option 2 |  |

# 4. Conclusion

To be updated

# 5. Reference

[1] R2-2201292	Remaining issues on group notification for multicast session	LG Electronics

[2] R2-2201382	Remaining issues of the multicast notification	Xiaomi Communications	discussion

[3] R2-2200532	NR MBS control signaling aspects	Qualcomm Inc

[4] R2-2200385	Open issues multicast activation and service continuity of broadcast	OPPO

[5] R2-2200399	Discussion on MBS RRC issues	Samsung

[6] R2-2201259	Discussion on MBS Open Issues for RRC CR	vivo

[7] R2-2200816	MBS service continuity and notification for multicast	Huawei, HiSilicon

[8] R2-2201118	Control plane aspects of MBS	Apple

[9] R2-2200578	Discussion on L3 open questions for NR MBS	TD Tech, Chengdu TD Tech

[10] R2-2200236	Open Issues on Common RRC Aspects	CATT

[11] R2-2201119	Open issues for MBS RRC Running CR	Apple

[12] R2-2200818	Discussion on RRC parameters for MCCH and MTCH	Huawei, HiSilicon

[13] R2-2201175     Multicast service continuity and discussion on RAN3 LS

[14] R2-2200356	Miscellaneous MBS L3 open issues