3GPP TSG-RAN2 Meeting #116bis-e	R2-2201860

Online, 17 - 25 January, 2022

Agenda Item:	9.2.4

Source:		Huawei, HiSilicon

Title:		Report of [046][IoT-NTN] RRC Misc

Document for:	Discussion and decision

# Introduction

This is the summary of offline:

- [AT116bis-e][046][IoT-NTN] RRC Misc (Huawei)

Scope: Review of the last update IN R2-2201451 (including Latest L1 parameters). This phase of the discussion is offline only. If issues are found, capture as editors notes (or in an annex etc).

Intended outcome: Report

Deadline: Initial review during W1.

The document discusses how to capture the RAN1 parameters in [3] and some existing signalling FFS in the running CR [1].

# Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Ericsson | Jonas Sedin | Jonas.sedin@ericsson.com |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |
| OPPO | Haitao Li | lihaitao@oppo.com |
| Intel | Tangxun | xun.tang@intel.com |
| Nokia | Srinivasan Selvaganapathy | Srinivasan.selvaganapathy@nokia.com |
| Nordic Semiconductor | Jouni Korhonen | Jouni.korhonen@nordicsemi.no |

# Discussion

## Capturing RAN1 parameters

Ephemeris State Vectors parameters

In the spreadsheet, RAN1 has provided the range of the parameters but not the granularity.

However, RAN1#107 has agreed “Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network:

- Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network:

  - Position and velocity state vector ephemeris format is 17 bytes payload.

    - The field size for position (m) is 78 bits

      - Position range is driven by GEO : +/- 42 200 km

      - The quantization step is 1.3m for position

    - The field size for velocity (m/s) is 54 bits

      - Velocity range is driven by LEO@600 km: +/- 8000 m/s

      - The quantization step is 0.06 m/s for Velocity

Q1: The positionX, Y and Z parameters are defined as INTEGER (-33554432…33554431) by step of 1.3 meter. Actual value = IE value * 1.3.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | FFS | The proposed range translates to 43,620,761 .. 43,620,760 which exceeds the range [42,000,000 .. 42,000,000] indicated by RAN1. could be changed to INTEGER [-32461539 .. 32461539] |
| Qualcomm | FFS | We can check further or confirm the range or step size. |
| Intel | FFS | We could ask RAN1 to further check |
| Nokia | FFS | To be checked with RAN1 |
| Nordic | FFS |  |

Rapporteur’s summary

All companies agree that we should check with RAN1 the exact signalling (range and step size).

Proposal 1: Keep the current representation of positionX, Y and Z parameters and add an Editor’s note to check the exact signalling with RAN1

Q2: The velocityX, Y and Z parameters are defined as INTEGER (-131072…131071) by step of 0.06 m/s. Actual value = IE value * 0.06.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | FFS | The proposed range translates to -7864 .. 7863 which is shorter than the range [-8000 .. 8000] indicated by RAN1. |
| Intel | FFS | We could ask RAN1 to further check |
| Nokia | FFS | To be checked with RAN1 |
| Nordic | FFS |  |

Rapporteur’s summary

All companies agree that it is unclear if we should use the full signalling range, or restrict to the signalling range indicated by RAN1 or change the step size.

Proposal 2: Keep the current representation of velocityX, Y and Z parameters and add an Editor’s note to check with RAN1.

Orbital parameters

In the spreadsheet, RAN1 has provided the range of the parameters but not the granularity.

However, RAN1#107 has agreed “Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network:

- Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network:

  - Orbital parameter ephemeris format 18 byte payload

    - Semi-major axis α (m) is 33 bits

      - Range: [6500, 43000]km

    - Eccentricity e is 19 bits

      - Range: ≤ 0.015

    - Argument of periapsis ω (rad) is 24 bits

      - Range: [0, 2π]

    - Longitude of ascending node (Ω rad) is 21 bits

      - Range: [0, 2π]

    - Inclination i (rad) is 20 bits

      - Range: [- π/2 , + π/2]

    - Mean anomaly M (rad) at epoch time to is 24 bits

      - Range: [0, 2π]

Q3: The semiMajorAxis parameter is defined as INTEGER (0...8589934591) by step of 2-33. Actual value = (43000000 – 6500000) + IE value * 2-33.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | Inputting parameter value 0 yields value = (43000km – 6500km) + 0 * 2-33 = 36500 km, which is probably not the intention. It should probably be: Actual value = 6500000 + IE value * (43000000 – 6500000) * 2-33 |
| Huawei, HiSilicon | No | agree with E/// comment |
| Qualcomm |  | Agree with E///. |

Rapporteur’s summary

All companies agree that the actual value should be corrected.

Proposal 3: Change the description of the actual value of parameter semiMajorAxis to ‘6500000 + IE value * (43000000 – 6500000) * 2-33’

Q4: The eccentricity parameter is defined as INTEGER (0…524287) by step of 0.015 * 2-19. Actual value = IE value * (0.015 * 2-19).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | Minor typo: 2-19 should be 2-19 |

Rapporteur’s summary

One company indicates a typo, this will be corrected in the next updated of the running CR

Q5: The periapsis parameter is defined as INTEGER (0…16777215) by step of 2π * 2-24. Actual value = IE value * (2 * π * 2-24).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | Consider minor simplification: Actual value = IE value * (2 * π * 2-24) = IE value * (π * 2-23). Less numbers, less rooms for mistakes. |

Rapporteur’s summary

One company proposes to simplify the representation of the formula, this will be done in the next updated of the running CR

Q6: The longitude parameter is defined as INTEGER (0…2097151) by step of 2π * 2-21. Actual value = IE value * (2 * π * 2-21).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | Consider minor simplification: Actual value = IE value * (π * 2-20) |

Rapporteur’s summary

One company proposes to simplify the representation of the formula, this will be done in the next updated of the running CR

Q7: The inclination parameter is defined as INTEGER (-524288..524287) by step of π/2 * 2-19. Actual value = IE value * ((π/2) * 2-19).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | Consider minor simplification: Actual value = IE value * (π * 2-20) |

Rapporteur’s summary

One company proposes to simplify the representation of the formula, this will be done in the next updated of the running CR

Q8: The anomaly parameter is defined as INTEGER (0..16777215) by step of 2π * 2-24. Actual value = IE value * ( 2 * π * 2-24).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | Consider minor simplification: Actual value = IE value * (π * 2-23) |

Rapporteur’s summary

One company proposes to simplify the representation of the formula, this will be done in the next updated of the running CR

Proposal 4: For all ephemeris parameters, simplify the representation of the formulas.

TA common parameters and UL synchronisation validity duration

In the spreadsheet, RAN1 has provided the range and granularity of the parameters.

RAN1#107 has agreed:

- The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time.

- A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB.

- Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information.

Q9: TA common parameters, UL synchronisation validity duration and ephemeris epoch time are signalled in the NTN specific SIB (SIBXX)

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| Spreadtrum | Yes | The issue is also under discussion in NR_NTN. We will continue to maintain attention. |
| OPPO | Yes |  |
| Intel | Yes |  |
| Nokia | Yes |  |
| Nordic | Yes |  |

Rapporteur’s summary

All companies agree.

Proposal 5: TA common parameters, UL synchronisation validity duration and ephemeris epoch time are signalled in the NTN specific SIB (SIBXX).

Timing relationship enhancements parameters

In the spreadsheet, RAN1 has provided the range and granularity of the parameters.

RAN1#107 has agreed:

- For IoT NTN, signalling one value for cell-specific K_offset in system information is supported.

- For IoT NTN, the information of K_mac is carried in system information.

RAN1#107 did not indicate in which SIB the parameters are signalled. However, the parameters are NTN specific and common to all three technologies (i.e. independent of the actual cell resource configuration) thus it seems natural to signal them in the NTN specific SIB.

Q10: K_offset and K_mac parameters are signalled in the NTN specific SIB (SIBXX).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei, HiSilicon | yes | also agreed in NT NTN |
| Qualcomm |  | Generally, we would prefer to group the parameters that require SI change notification vs that do not. Also, we can wait if NR NTN decides any parameters that need more frequent update. |
| Spreadtrum | Yes |  |
| OPPO | Yes | We can following agreement in NR NTN. |
| Intel | Yes |  |
| Nokia | Yes |  |
| Nordic | Yes |  |

Rapporteur’s summary

Seven companies agree. One company indicates that they would prefer to group the parameters depending on whether they need change notification or not.

Proposal 6: K_offset and K_mac parameters are signalled in the NTN specific SIB (SIBXX).

UL transmission segment duration

In the spreadsheet, RAN1 has provided the range and granularity of the parameters.

RAN1 has indicated that the configuration of UL transmission segment for (N)PRACH is indicated in SIB and that the configuration of UL transmission segment for PUCCH and (N)PUSCH can be reconfigured by RRC signalling.

RAN1#107 did not indicate which SIB. However, based on the above and the fact that the parameters are dependent of the technologies and the actual configuration, it seems natural to signal them in SIB2.

Q11: UL (N)PRACH, (N)PUSCH and PUCCH transmission segment duration parameters are signalled in SIB2.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei, HiSilicon | yes |  |
| Spreadtrum | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Nokia | Yes | As new parameter in RadioResourceCommon |
| Nordic | Yes |  |

Rapporteur’s summary

All companies agree. One company clarifies this should be in RadioResourceCommon.

Proposal 7: UL (N)PRACH, (N)PUSCH and PUCCH transmission segment duration parameters are signalled in SIB2.

Sub-PRB in eMTC is only supported in RRC_Connected mode. Configuration of UL transmission segment for PUSCH for sub-PRB allocation is only signalled in dedicated signalling

Q12: Configuration of UL transmission segment for PUSCH for sub-PRB allocation is only signalled in dedicated signalling.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei, HiSilicon | yes |  |
| Spreadtrum | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Nokia | Yes |  |
| Nordic | Yes |  |

Rapporteur’s summary

All companies agree.

Proposal 8: Configuration of UL transmission segment for PUSCH for sub-PRB allocation is only signalled in dedicated signalling.

## FFS on signalling details

In the running CR there is an Editor’s Note as follows:

Editor’s Note: Agreement: The network may broadcast more than one TAC per PLMN in a cell, which is up to network implementation. Editor: Enhanced RAN sharing is not supported in NB-IoT, should that be introduced for NTN?

In R2-2201455, it is proposed that the list is common across all PLMN as in legacy.

Q13:  In NB-IoT, the list of TACs broadcast in the cell is common to all PLMNs same as in legacy, where all cell access information are common to all PLMNs.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | Seems reasonable. |
| Huawei, HiSilicon | yes |  |
| Qualcomm |  | This should be revisited if there is any concern from the operators. |
| Spreadtrum | Yes |  |
| OPPO | Yes |  |
| Intel | Yes | Since tracking area is geographically fixed, it could be common. |
| Nokia | Yes |  |
| Nordic | yes |  |

Rapporteur’s summary

All companies agree. One company indicates that this should be revisited if concern from operators.

Proposal 9: In NB-IoT, the list of TACs broadcast in the cell is common to all PLMNs same as in legacy, where all cell access information are common to all PLMNs.

In the running CR, maxTAC-r17 is FFS.

maxTAC-r17					INTEGER	::=	ffsValue	-- Maximum number of Tracking Area Codes

-- broadcast in a cell

Q14: Is the Maximum number of TACs that can be broadcast in a cell in IOT NTN the same as for NR NTN. If no, please suggest a value.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | 12 seems reasonable. |
| Huawei, HiSilicon | yes | 12 as baseline. Operators/ Satellite companies can comment if not sufficient |
| Qualcomm | Yes |  |
| Spreadtrum | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Nokia | Yes |  |
| Nordic | Yes |  |

Rapporteur’s summary

All companies agree. One company indicates that this should be revisited if concern from operators.

Proposal 10: The maximum number of TACs that can be broadcast in a cell in IOT NTN is 12, the same as NR NTN.

In the running CR, the flag to enable TA report during initial access is signalled in SIB2(-NB) and there is an Editor’s Note as follows:

]],

[[	ta-Report-r17						ENUMERATED {true}				OPTIONAL	-- Need OR

-- Editor’s Note: FFS on where this parameter is placed

]]

Q15: ta-Report-r17 is signaled at the top level of SIB2, i.e. outside of the radioResourceConfigCommon

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | FFS | We need it for handovers in eMTC, then it should probably be placed in radioResourceConfigCommon as this is signaled in mobilityControlInfo, but not sure if it is suitable given that this is mainly radio related configurations. Perhaps rach-config could be suitable. Alternatively maintain ta-Report in SIB2 and add it to mobilityConfolInfo. We leave it up to rapporteur. |
| Huawei. HiSilicon | FFS | It seems strange to add to radioResourceConfigCommon as this is not related to radio resource configuration, so SIB2 and adding to mobilityConfolInfo is preferred Another option is to signal in SIBXX as agreed in NR NTN . This also means adding to mobilityConfolInfo |
| Qualcomm | No | Ok in radioResourceConfigCommon as it has to be signaled in both SIB2 and mobilityConfolInfo. |
| Spreadtrum | No | Agree with QC. |
| OPPO | No | Agree with QC. |
| Intel | No | Can follow NR NTN, i.e., in SIBXX |
| Nokia | FFS | RACH-Resource configuration or radioResourcConfigCommon is suitable place. |
| Nordic | No | Can be in radioResourceConfigCommon or SIBXX, |

Rapporteur’s summary

Five companies indicates that it is needed in SIB2 and for handover so it is better signalled in radioResourceConfigCommon. Two companies propose SIBXX as in NR, this means also in mobilityConfolInfo. One company indicates that it can be in radioResourceConfigCommon or SIBXX.

Proposal 11: ta-Report-r17 is signaled in radioResourceConfigCommon.

## Other

Please indicate any other aspect not discussed in the above.

| Company | Comments |
| --- | --- |

# Conclusion

Eight companies provided comments. Based on the comments, the following proposals are made:

Proposal 1: Keep the current representation of positionX, Y and Z parameters and add an Editor’s note to check the exact signalling with RAN1.

Proposal 2: Keep the current representation of velocityX, Y and Z parameters and add an Editor’s note to check with RAN1.

Proposal 3: Change the description of the actual value of parameter semiMajorAxis to: 6500000 + IE value * (43000000 – 6500000) * 2-33

Proposal 4: For all ephemeris parameters, simplify the representation of the formulas.

Proposal 5: TA common parameters, UL synchronisation validity duration and ephemeris epoch time are signalled in the NTN specific SIB (SIBXX).

Proposal 6: K_offset and K_mac parameters are signalled in the NTN specific SIB (SIBXX).

Proposal 7: UL (N)PRACH, (N)PUSCH and PUCCH transmission segment duration parameters are signalled in SIB2.

Proposal 8: Configuration of UL transmission segment for PUSCH for sub-PRB allocation is only signalled in dedicated signalling.

Proposal 9: In NB-IoT, the list of TACs broadcast in the cell is common to all PLMNs same as in legacy, where all cell access information are common to all PLMNs.

Proposal 10: The maximum number of TACs that can be broadcast in a cell in IOT NTN is 12, the same as NR NTN.

Proposal 11: ta-Report-r17 is signaled in radioResourceConfigCommon.

The proposals, if agreed, will be captured in the next update of the running CR.

# Reference

- R2-2201451 Running CR - Support of Non-Terrestrial Network in NB-IoT and eMTC, Huawei, RAN2#116bis, January 2022

- R2-2200095 “LS on updated Rel-17 LTE and NR higher-layers parameter list”, RAN1#107-e, November 2021

- R1-2112975 “Consolidated high layers parameters list for Rel-17 LTE, RAN1#107-e, November 2021