3GPP TSG-RAN WG2 #116bis-e	R2-22xxxxx

E-meeting, 17 – 25 January 2022

Agenda item:	8.4.2.4

Source: 	Samsung (rapporteur)

Title: 	Summary of discussion [AT116bis-e][050][eIAB] MAC (Samsung)

Document for:	Discussion and decision

# Introduction

This document captures the following discussion:

- [AT116bis-e][050][eIAB] MAC (Samsung)

Scope: Review and Endorse MAC running in CR R2-2201527, Treat R2-2201353, R2-2200810, R2-2201298, R2-2201427, R2-2201526. Determine agreeable parts, Capture agreements, and update CR. Agree offline if possible

Intended outcome: Report, agreements Endorsed CR

Deadline: For potential CB Monday W2 (hopefully all offline).

Section 2 captures individual companies’ comments on rapporteur’s draft running CR for the MAC spec, as submitted to this meeting (R2-2201527). The goal is determine whether any changes are needed to the submitted version of the running CR, and to then endorse an initial version of the running CR as a starting point, as per Chair’s instructions:

R2-2201527	Running CR to 38.321 on Integrated Access and Backhaul for NR Rel-17	Samsung Electronics GmbH	CR	Rel-17	38.321	16.7.0	1171	-	B	NR_IAB_enh-Core	R2-2110453

- Samsung indicate that it covers all agreements up to now, but it was not endorsed.

- Endorse by email

Section 3 then covers new topics beyond this baseline running CR, based on submissions to this meeting (some of them also covered by Editor’s Notes in the running CR).

# Running MAC CR

Companies are invited to comment on R2-2201527 using the following table:

| Company | Section comment relates to (5.4.5, 6.1.3, 6.2.1) / General comment | Comment | Rapporteur’s response |
| --- | --- | --- | --- |
| Huawei, HiSilicon | 5.4.5 | For Regular and Periodic BSR, the MAC entity for which extendedBSR with value false is configured by upper layers shall: For Regular and Periodic BSR, the MAC entity for which extendedBSR with value true is configured by upper layers: There is no extendedBSR configuration in RRC. Instead, we use logicalChannelGroup-IABExt-r17. So, we can either say “if the LCG larger than 7 is configured” or say “if logicalChannelGroup-IABExt-r17 is configured”. This is the consensus when we endorsed R2-2111604 This comment will also impact the below sentence (should be deleted), and also to the paging BSR section: 2> if the maximum LCG ID among the configured LCGs is 7 or lower: 3> report Long BSR for all LCGs which have data available for transmission. |  |
| Huawei, HiSilicon | 6.1.3.1 | Figure 6.1.3.1-2: Extended Short BSR and Extended Short Truncated BSR MAC CE We need to add the new figure in the very end, to avoid the impact to the figure number of legacy format. |  |
| Huawei, HiSilicon | 6.1.3.1 | NOTE 2: Extended Short, Extended Short Truncated, Extended Long and Extended Long Truncated BSR formats need only be supported by an IAB-MT. NOTE 4: The Extended versions of the BSR formats may only be used by IAB nodes. Those two NOTE are redundant. We prefer the NOTE 4 only. |  |
| LGE | 6.1.3.1 | The following may be missed in section 6.1.3.1, i.e., “The extended BSR formats are identified by MAC subheaders with eLCIDs as specified in Table 6.2.1-2b” |  |

# Various issues submitted to ongoing meeting

## LCP: priority parameter range extension

In [2] it is argued that due to the agreed extension of the LCG space (to 256), the existing number of LCP priority levels may not be enough to differentiate (with substantial enough granularity) the resource allocation priorities of the BH RLC CHs belonging to these configured LCGs. In other words, [2] argues that differentiated resource allocation priority between different LCGs cannot always be achieved using existing LCP priority levels range.

Q1a. Do you support the LCP priority levels range extension, and if so, to what value?

| Company | Response |
| --- | --- |
| Huawei, HiSilicon | No. Maybe consider this in next release. |
| vivo | Yes, 256. |
| LGE | No. Logical channel priority is related to QoS requirement of DRB, not size of LCG space. Note that logical channel priority does not increase even when RAN2 introduce extended LCID space. No need to extend range of logical channel priority due to LCG extension as well. |

Q1b. If you answered yes to Q1a, do you think the extension should also apply to UEs other than IAB-MTs?

| Company | Response |
| --- | --- |
| vivo | No. The existing LCP range is enough for UE. |

## Extended BSR formats and padding BSR

[3] – [5] all treat the issue of the use of Extended BSR formats for padding BSR, although focus of the contributions is on slightly different aspects of padding BSR.

Both [3] and [5] propose use of legacy BSR formats when padding cannot accommodate Extended BSR formats. [5] has a more general approach (listing all possibilities including only using legacy formats, only using Extended formats, or agreeing on a rule for a mix of the two / leaving it to implementation), while [3] focuses on the specific case of using legacy Long BSR for padding BSR, in case the padding size is enough to send the legacy Long BSR but not the Extend Long BSR. [4] on the other hand assumes that Extended formats are always used for padding; however, due to the size of the LCGi information in Extended Long Truncated BSR, for certain padding sizes [4] argues that no information may be transmitted to the network.

The rapporteur proposes to agree on high-level principle first:

Q2a. Which of the following options do you prefer for padding BSR for IAB-MTs supporting Extended BSR formats:

1. Follow the legacy procedure i.e. do not use the Extended formats at all for padding;

2. Use exclusively the Extended formats by fully mirroring the legacy padding BSR procedure (use the Extended equivalents of all formats therein);

2a. Use exclusively the Extended formats but in an adaptive manner, by reporting Extended Short Truncated BSR instead of Extended Long Truncated BSR if the number of padding bits cannot include the fixed size of 256 LCGi plus subheader of the Extended Long Truncated BSR;

3. Allow mixing of legacy and Extended formats (through a normative solution) by e.g. using legacy formats when padding space is limited (i.e. when an equivalent Extended format could not be sent) and/or in an attempt to save on signaling (details TBC);

3a. Allow mixing of legacy and Extended formats (by leaving the details to implementation).

| Company | Response (1/2/2a/3/3a/other-please provide details) |
| --- | --- |
| Huawei, HiSilicon | Option 3. If extended BSR is configured, IAB-MT uses legacy Long BSR for padding BSR, in case the padding bits is larger than 2 bytes but smaller than 32 bytes (i.e. padding bit is enough to send the legacy BSR but not the extend BSR). Otherwise, extended BSR will be used. |
| vivo | Option 2 seems better if padding BSR is desired. If Option 2 is not acceptable, we can accept Option.1. Option 3 may cause ambiguity to the gNB if the LCG with non-zero buffer is not indicated by the padding BSR with legacy BSR formats. |
| LGE | Prefer option 2a. For option 1, considering the different size of LCG ID between legacy and Extended format, it is not clear how option 1 work because the higher LCG ID may not be included in the legacy format. For example, when there are no data available for LCG 1 through LCG 7 but LCG 9 has data available, the buffer size of LCG 9 cannot be transmitted by the legacy format due to lack of LCG ID size. As RAN2 already agree to support Extended Truncated BSR format in the last meeting, option 2 should be a baseline. However, when the number of padding bits is smaller than the fixed size of 256 LCGi (32bytes) plus subheader, if Extended Long Truncated BSR is transmitted by option 2, the fixed 32 bytes bitmap for 256 LCGi may not be included and the Extended Long Truncated BSR with no information may be transmitted to the network, i.e., only 3byts of bitmap for LCGi can be included in the Extended Long truncated BSR. We think that this is the problem of option 2 and, in this case, the Extended Short Truncated BSR is better than Extended Long Truncated BSR, i.e., one correct information is more helpful to the network than no information. The option 2a can avoid the problem of option 2 and this is simple and clear approach among all candidates above. For option 3, same problem as option 1 can occur because the higher LCG ID may not be included in the legacy format. To avoid this problem, more complex condition may need to be specified. In addition, unlink option 1/2/2a, many points are open and it is unclear what the IAB node behavior of option 3 is. Each company may have different design/procedure with option 3, so this may needs many discussion time to determine many open points. Therefore, considering this late stage of Rel-17, we prefer to have the simple and clear approach which is the option 2a. |

## RAN1 related MAC CEs

[1] and [3] discuss content and format of any new MAC CEs arising from RAN1 work on eIAB. Before discussing details and prioritizing certain MAC CEs over others (e.g. due to further RAN1/RAN4 progress being awaited), the discussion rapporteur proposes to confirm a comprehensive list first:

Q3a. Is the below in your view a comprehensive list (i.e. nothing is missing, and nothing is superfluous) of new MAC CEs that RAN2 need to design/modify existing ones, stemming from RAN1 work:

- Desired guard symbols

- Provided guard symbols

- Child IAB-DU restricted beam indication

- Timing case indication

- Case-7 timing offset

- Desired DL TX power adjustment

- DL TX power adjustment

- Desired IAB-MT PSD range

| Company | Response |
| --- | --- |
| Huawei, HiSilicon | Those are informed us in their LS. But, we understand there may be some more MAC CE to be informed us later. We can discuss those for now. |
| vivo | Agree with the comments from Huawei. According our knowledge, there are still some related FFSs in RAN1, and there may be probably new LSs from RAN1 regarding MAC CE later. |
| LGE | Yes so far. |

Q3b. Do you agree that RAN2 should focus in this meeting on 2 new timing modes (Case-6 timing and Case-7 timing) for Desired guard symbols and Provided guard symbols, as well as on the Case-7 timing offset (deprioritizing work on other MAC CEs until further input from RAN1/RAN4 is received)?

| Company | Response |
| --- | --- |
| Huawei, HiSilicon | Yes. Other MAC CEs are quite unclear for now. |
| vivo | Yes. Even for Desired guard symbols and Provided guard symbols for Case-6 timing and Case-7 timing, further conclusion from RAN4 is needed regarding the exact value ranges. |
| LGE | Fine. |

Q3c. Do you agree that Rel-16 Desired/Provided Guard Symbols MAC CE should be reused to indicate desired/provided number of symbols for the Case-6 and Case-7 timings (i.e. there is no need to define new MAC CE format), by allocating two additional eLCID values for the Desired/Provided Guard Symbols MAC CEs?

| Company | Response |
| --- | --- |
| Huawei, HiSilicon | No. prefer new MAC CEs. Why not to introduce two new MAC CEs, instead of changing the meaning of legacy MAC CE field. We anyway need to use the eLCID. The only impact is to give more figures on the new format. |
| vivo | No. Anyway, when an existing MAC CE is assigned with a new eLCID ID, it can be regarded as a new MAC CE. We can consider the new MAC CE only contain the new DU – MT operation switch cases or contain both the old DU – MT operation switch cases and the new ones. |
| LGE | No strong view. |

With regards to Case-7 timing offset MAC CE, RAN1 have agreed that the dynamic range of the MAC CE case #7 timing offset indication is 12 bits. It would therefore appear that the existing Timing Delta MAC CE cannot be reused as the length of the relevant field is 11 bits. However, the Absolute Timing Advance Command MAC CE also has a fixed size and consists of two octets including the 12-bit Timing Advance Command.

Q3d. Do you agree that the Absolute Timing Advance Command MAC CE should be reused to indicate the Case-7 Timing Offset (i.e. there is no need to define new MAC CE format), by allocating a new eLCID to indicate that the MAC CE is used to indicate the Case-7 Timing Offset?

| Company | Response |
| --- | --- |
| Huawei, HiSlicon | No. prefer to use the new MAC CE, which is clear. You anyway have to explain the R17 MAC CE is 12 bits TA. |
| vivo | No. New MAC CE is needed as 12-bit TA is used. |
| LGE | No strong view. |

## Any other issues

Q4a. Have you identified any issues not covered by the running MAC CR draft and the issues detailed above? If so, please provide details.

| Company | Response |
| --- | --- |

# Conclusions

…

# References

- R2-2201353	Discussion on MAC CEs for PHY layer support ZTE, Sanechips

- R2-2200810	Discussion on LCP Extension	vivo

- R2-2201298	LCG extension and R1 related MAC CE design	Huawei, HiSilicon

- R2-2201427	Remaining issues on LCG extension	LG Electronics Inc.

- R2-2201526	Extended BSR and padding	Samsung Electronics GmbH