- 3GPP TSG RAN WG2 Meeting #116bis-e      	               R2-2201925
E-Conference, 1st – 12th Nov. 2021

Agenda item: 	8.22.2

Source:	Intel Corporation

Title: 	[AT116bis-e][062][MGE] pre-configured measurement gap (Intel)

WID/SID:	NR_MG_enh-Core – Release 17

Document for:	Discussion and Decision

# Introduction

- Due to lack of time, pre-configured measurement gap was not discussed during online session and kick start easy agreements offline.

R2-2201687	Summary of 8.22.2 MGE: pre-configured measurement gap	Intel

- “Easy” agreements offline, discussion points for online CB (if possible)

- [AT116bis-e][062][MGE] pre-configured measurement gap (Intel)

Scope: Based on R2-2201687, attempt to agree offline “easy agreements”.

Intended outcome: Report

Deadline: 1/24/2022 10:00UTC

# Discussion

During email discussion [3], 11 companies support to use 1 bit indication to indicate pre-configured gap.

2 companies (also agree using 1 bit indication) think a separate IE for pre-configured gap can be introduced in case RAN4 reply simultaneous support of legacy gap and pre-configured gap.

1 company think that it is better to use a new IE for pre-configured gap for forward compatibility.

Rapporteur wants to note that even though MGE WI doesn’t discuss the join feature of pre-configured gap and concurrent gap, but other work item such as MUSIM, ePOS and NTN may end up using the same framework for join functionality. Especially majority of the companies agree to use 1 bit to indicate pre-configured gap.

Proposal 1: Add 1 bit indication in gapConfig to indicate pre-configured measurement gap.

Q1: Since majority of the companies support using 1 bit indication and we have very limited time for this work item, we would like to ask if proposal 1 is acceptable?

| Company | Support/ Acceptable/ unacceptable | Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Support |  |
| MediaTek | Support |  |
| Xiaomi | Support |  |
| Samsung | Support |  |
| DENSO | Support |  |
| Ericsson | Support |  |
| LGE | Support |  |
| Nokia | Acceptable | We prefer to configure Pre-MG with a dedicated Pre-MG IE which may be easier for extension to support co-existence between Pre-MG, ePOS and legacy MG. However, for the sake of progress, we are fine to accept this proposal. |
| ZTE | Support |  |
| Intel | support |  |
| Apple | Support |  |
| CATT | Support |  |

Summary Q1:

11 companies support and 1 company is acceptable of the proposal. Therefore, rapporteur proposal to conclude support of P1.

Proposal 1: Add 1 bit indication in gapConfig to indicate pre-configured measurement gap.

We understand there are discussion for simultaneous support of legacy gap and pre-configured gap, we are still waiting for RAN4 reply. What are companies view in case of RAN4 reply the simultaneous support of legacy gap and pre-configured gap. Some options are:

- Option 1: New IE to support pre-configure gap

- Option 2: Combine concurrent gap to indicate pre-configured gap if both are enabled

- Option3: ?

Q2: What is your view in case of simultaneous support of legacy gap and pre-configured gap?

| Companies | Option (feel free to add) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 | When legacy gap and pre-MG are supported simultaneously, the case is related to both pre-MG and con-MG features. The possible configuration can be as follows (as indicated in our paper R2-2200835): MeasGapConfig ::= SEQUENCE { gapFR2 SetupRelease { GapConfig } OPTIONAL, -- Need M …, [[ gapFR1 SetupRelease { GapConfig } OPTIONAL, -- Need M gapUE SetupRelease { GapConfig } OPTIONAL -- Need M ]], [[ gap2FR1-r17 SetupRelease { GapConfig } OPTIONAL, -- Need M gap2FR2-r17 SetupRelease { GapConfig } OPTIONAL, -- Need M gap2UE-r17 SetupRelease { GapConfig } OPTIONAL -- Need M ]] } GapConfig ::= SEQUENCE { gapOffset INTEGER (0..159), mgl ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6}, mgrp ENUMERATED {ms20, ms40, ms80, ms160}, mgta ENUMERATED {ms0, ms0dot25, ms0dot5}, …, [[ refServCellIndicator ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL -- Cond NEDCorNRDC ]], [[ refFR2ServCellAsyncCA-r16 ServCellIndex OPTIONAL, -- Cond AsyncCA mgl-r16 ENUMERATED {ms10, ms20} OPTIONAL -- Cond PRS ]], [[ preConfigInd-r17 ENUMERATED {true} OPTIONAL, -- Need R gapAssoication-r17 GapAssoication-r17 OPTIONAL, -- Need R measGapId-r17 MeasGapId-r17 OPTIONAL, -- Cond MultiMG nscgInd-r17 ENUMERATED {true} OPTIONAL -- Need R ]] |
| MediaTek | Option 2 | Similar view as Huawei. In addition, considering that pre-configured MG basically reuse the same gap pattern as legacy, we think option 2 is the easiest way. |
| Xiaomi | Option 2 |  |
| Samsung | Option 2 | Same view as other companies. |
| DENSO | Option 2 | The concurrent gap mechanism can be used to configure the legacy gap and preconfigure gap simultaneously. |
| Ericsson | Option 2 |  |
| LGE | Option 2 | Prefer to use only the new IE (for multiple measurement gap configuration) to simultaneously configure the legacy gap and pre-configured gap. To facilitate this, we prefer to use the list structure for the new IE so that the more than one gap can be configured for the same type, e.g. legacy FR1 gap + pre-configured FR1 gap in the new IE. |
| Nokia | No strong view. | In the Rel-17 ePOS WI, preconfigured gap should be configured for PRS and it should be indicated by a gap ID and activated by MAC CE. RAN2 is discussing whether ePOS gap could be considered as MGE pre-configured gap in AI 8.0.3. If yes, the ASN.1 design may need consider both MGE WI and Rel-17 positioning requirement as well as legacy MG co-existence. |
| ZTE | Option 2 | To be clear, for the following two cases, we think there is no need to combine concurrent gap with pre-configured gap. - legacy FR1 gap + pre-configured FR2 gap - legacy FR2 gap + pre-configured FR1 gap [Rapp]: not clear why the two cases above no need to combine concurrent and pre-configured gap. The discussion is when the above 2 cases supported by RAN4, then we combine concurrent and pre-configured gap in one structure. |
| Intel | Option 2 |  |
| Apple | Option 2 |  |
| CATT | Option 2 |  |

Summary Q2:

11 companies support option 2 and 1 company has no strong view. Company indicating support of ePOS also require the need for using gap ID to activate gap using MAC CE, therefore, ASN1 design considering co-existence of both gaps are needed. Therefore, rapporteur propose:

Proposal 2: In case of simultaneous support of legacy gap and pre-configured gap, it is agreed to support option 2: combine concurrent gap to indicate pre-configured gap if both are enabled.

During last meeting RAN2#116e, it is agreed:

- RAN2 hasn’t seen any usefulness of MAC-CE based activation/deactivation and prefers to not support it.

|  | Source | Related proposals |
| --- | --- | --- |
| Not support MAC CE activation | MT, R2-2201247 | Proposal 1: RAN2 confirms that MAC-CE based activation/deactivation for pre-configured MG is NOT supported. |

Media Tek proposes RAN2 to confirms not to support MAC-CE based activation/deactivation. Based on RAN4 LS [17], RAN4 has also decided MAC CE based activation/deactivation is not supported.

Regarding activation/deactivation of Pre-configured MG, RAN4 has reached the following conclusions:

- NW can control activation/deactivation of pre-configured MG for the specific BWP via RRC message ONLY.

- Additional activation/deactivation conditions are not considered in application to network-controlled pre-MG activation/deactivation. (i.e. MAC CE based activation/deactivation is not supported)

Even though it is unclear from the LS if this applies to all pre-configured gap or only network-controlled pre-configured gap. But Rapporteur thinks that majority of companies from email discussion are align not to support MAC CE activation/deactivation. We can make a propose to confirm that.

Proposal 3: RAN2 confirms that MAC-CE based activation/deactivation for pre-configured MG is NOT supported.

Q3: Can we confirm that RAN2 doesn’t support MAC-CE based activation/deactivation for pre-configured MG.

| Companies | Yes/ No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| MediaTek | Yes (Proponent) |  |
| Xiaomi | Yes | RAN4 also doesn’t support MAC-CE based activation/deactivation. |
| Samsung | Yes |  |
| DENSO | Yes |  |
| Ericsson | Yes |  |
| LGE | Yes |  |
| Nokia | No, but | MAC-CE based activation/deactivation for pre-configured MG is appliable to ePOS WI. RAN4 LS[17] is only valid for MGE (Pre-MG) WI. So, the proposal is fine if it is limited to MGE WI. [Rapp]: Yes, only apply to MGE WI. ePOS will have its own activation method. |
| ZTE | Yes | ePOS gaps can be discussed separately. |
| Apple | Yes |  |
| CATT | Yes |  |

Summary Q3:

11 companies support RAN2 to confirm MAC CE activation is not supported. 1 company concern about other work item (ePOS) has agree using MAC CE, therefore they are fine if proposal is only for MGE WI. Therefore, rapporteur propose to clarify the follow proposal:

Proposal 3: RAN2 confirms that MAC-CE based activation/deactivation for pre-configured MG is NOT supported in MGE WI.

During email discussion [3], some companies want to ask RAN4 for motivation and use case to support network-controlled activation/ deactivation pre-configured measurement gap.

Q4: Do you think it is needed to send LS to RAN4 to clarify motivation and use case of network-controlled activation/ deactivation pre-configuration measurement gap.

| Companies | Yes/ No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | In our understanding, Case 5 is to define a simple rule in the spec for the UE to follow, whereas Case 4 is to have a complex rule to guide the NW configuration and then UE follows the configuration. Since Case 5 is much simpler and already adopted, we want to figure out the real motivation of Case 4. To avoid receiving ambiguous reply which cannot address the real issue, we suggest the following wording: RAN2 cannot figure out the motivation and use case of network-controlled activation/ deactivation pre-configuration measurement gap (Case 4). Is there any use case in which Case 5 cannot work well and Case 4 has to be applied? |
| MediaTek | No, but please see comment | In our understanding, both case 4 and case 5 are already supported by RAN4. They are different controlling mechanism. The reason to have this per BWP indicator (case 4) is that some company think it would be simper than rule-based solution. With this clear RRC indicator, there is no room for misunderstanding between UE and NW on whether a gap is activated. Our preference is to support case 4. If there is really strong objection to have case 4, R2 should just inform R4 that R2 prefer NOT to support this mechanism in R17 with some clear reason and ask R4 if there is strong concern. Asking open question (like motivation) will just result in is not helpful to progress. R4 will repeat their original motivation and some company in R2 will not be convinced by the motivation. It consumes too much time on the LS exchange. We prefer to have a clear decision on this. |
| Xiaomi | See our comments | For activation/ deactivation of pre-configuration MG, RAN2 should follow RAN4’s agreement. So network-controlled activation/ deactivation pre-configuration MG should be supported by RAN2. But if majority think it is need to ask RAN4 for motivation and use case for better understanding, we are ok to send the LS and wait for RAN4 reply. |
| Samsung | Yes | We would like to have a clear understanding on what are the benefit/use case of network controlled activation/deactivation of pre-MG. |
| DENSO | Yes | Although we are O.K to introduce NW-controlled mechanism (Option 4), It would be better to learn why two options are needed and if so, how to use Option 4, in addition to 5. |
| Ericsson | See comment | We agree with the intention of asking this. On this matter, as like Huawei, we don’t see the need for a network-controlled approach either. However, we agree with MediaTek in the sense that a back-and-forth LS exchange with RAN4 wouldn’t be advisable at this point. Hence, we are OK including the text proposed by Huawei or, perhaps, the following: Considering the signaling requirements needed to enable pre-configured measurement gap, RAN2 will bound itself to the approach already agreed upon in this WG (i.e., “autonomous”/rule-based activation/deactivation), unless there is a clear motivation in RAN4 to have a network-controlled RRC-based method on top. |
| LGE | Yes | The NW-controlled activation/deactivation increases RAN2 specification work and appropriate RRC 6ignaling is affected by the use cases of the NW-controlled activation/deactivation, so RAN2 should ask RAN4 to clarify the necessity and the detailed use cases of the NW-Controlled activation/deactivation. |
| Nokia | No | It is clearly indicated in RAN4 LS that NW-Controlled (RRC-based) activation/deactivation mechanism should be supported and RAN2 is expected to design the RRC signaling. There is no need to ask RAN4 to double confirm. Furthermore, from RAN2 pov, we think the complexity of gaps in NR is exploding (e.g. gaps discussed in two RAN2-led Wis (MUSIM and NTN) and two RAN4-led Wis (MG enhancements, FR2 enhancements) and one RAN1-led ePos WI), we believe it is so complex that it is impossible to account all inter-actions between all configuration options now. Hence, NW controlled mechanism is always helpful for feature interactions and future feature design. |
| ZTE | Yes | We think companies (at least we) have doubt about case4 because it is unclear what rule the network should respect to when configures the activation/deactivation indicator. If the same rule in case 5 is applied, then explicit indication is redundant, and will become a burden to network (as network needs to update the field timely). |
| Intel | No | We share the same view as MT. But if companies want to send LS to get better understanding, we are ok as well. |
| Apple | No | We feel sending LS may not help at all in progress. In addition, we really don’t see the complexity of this 1-bit indication in dedicated signaling. And this field is optional, if NW does not want to enable this, they can just skip this field. With so many other important aspects to discuss in gap enhancement, do we really want to spend time in this matter? |
| CATT | No | It is supported by RAN4 LS, but we respect the majority view if it is needed. |

Summary Q4:

5 companies prefer to send LS to clarify motivation to support case 4. 1 company thinks that it is not needed but understanding not supporting case 4 companies would like to send and they are ok. All 6 companies are not in favor to introduce case 4.

6 companies do not think it is needed to send LS to RAN4 because case 4 is clear indicated in RAN4 LS to be supported. Companies also don’t think it is useful to ask motivation question where RAN4 will reply, and companies will still not be convinced.

Rapporteur thinks that there is no majority view to send the LS or not to RAN4 for motivation on case 4.

Proposal 4: Online discussion whether RAN2 send LS to clarify motivation to support of case 4.

# Conclusion

Easy agreement

Proposal 1: Add 1 bit indication in gapConfig to indicate pre-configured measurement gap.

Proposal 2: In case of simultaneous support of legacy gap and pre-configured gap, it is agreed to support option 2: combine concurrent gap to indicate pre-configured gap if both are enabled.

Proposal 3: RAN2 confirms that MAC-CE based activation/deactivation for pre-configured MG is NOT supported.

Online discussion

Proposal 4: Online discussion whether RAN2 send LS to clarify motivation to support of case 4. [maybe better discussion is way forward on support of case 4]

# References

[1] R2-2200497, Preconfigured measurement gap patterns, Samsung

[2] R2-2200219, Stage 3 detail for pre-configured gap, Intel Corporation

[3] R2-2200222, Draft running CR to 38331 for pre-configured measurement gap to support case 5, Intel Corporation

[4] R2-2200223, Draft running CR to 38331 for pre-configured measurement gap to support case 4 and 5, Intel

[5] R2-2200492, Discussion on Pre-configured MG, OPPO

[6] R2-2200499, Discussion on Pre-Configured MG, CATT

[7] R2-2200585, Discussion on per-configured measurement gap, vivo

[8] R2-2200606, Discussion on Pre-Configured MG, ZTE Corporation, Sanechips

[9] R2-2200832, Discussion on Pre-configured MG, Huawei, HiSilicon

[10] R2-2201011, Discussion on support of Pre-Configured Measurement Gap, Nokia, Nokia Shanghai Bell

[11] R2-2201247, Discussion on pre-configured MG, MediaTek Inc.

[12] R2-2201272, Discussion on Pre-configured MG, Xiaomi Communications

[13] R2-2201287, Pre-configured measurement gap, LG Electronics

[14] R2-2201288, Draft LS on NW-Controlled activation deactivation of pre-configured MG, LG Electronics

[15] R2-2201567, Pre-configured measurement gaps, Ericsson

[16] R2-2201107, RAN2 impact from Rel-17 Pre-MG, Apple

[17] R2-2200125, LS on R17 NR MG enhancements – Pre-configured MG, CATT, Intel

[18] R2-2201687, [Pre116bis][011][MGE] Summary of 8.22.2 Pre-configured MG patterns, Intel