3GPP TSG RAN WG2 #116bis-e	draft R2-2201756

Online, 17 - 25 Jan, 2022

Source: 	Huawei, HiSilicon

Title: 	Summary of [AT116bis-e][102][NTN] Idle/Inactive mode aspects – Third round (Huawei)

Agenda Item:	8.10.3.2

Document for:	Discussion and decision

# Introduction

This document serves as a summary of the following offline discussion:

- [AT116bis-e][102][NTN] Idle/Inactive mode aspects (Huawei)

Initial scope: Discuss idle/inactive mode aspects based on the summary in R2-2201731

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Tuesday 2022-01-18 0700 UTC

Initial deadline (for rapporteur's summary in R2-2201733): Tuesday 2022-01-18 0900 UTC

Updated scope: Continue the discussion on the remaining idle/inactive mode aspects including the content of possible LSs out regarding the decisions on the new NTN-specific SIBx and the support of Inactive mode.

Updated intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Updated deadline (for companies' feedback): Thursday 2022-01-20 2200 UTC

Updated deadline (for rapporteur's summary in R2-2201745): Friday 2022-01-21 0200 UTC

Final scope: Continue the discussion on the remaining idle/inactive mode aspects and draft LS to RAN1 asking to check the agreements on SIBx.

Final intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Final deadline (for companies' feedback): Monday 2022-01-24 1500 UTC

Final deadline (for rapporteur's summary in R2-2201756 and draft LS in R2-2201757): Monday 2022-01-24 1700 UTC

Proposals marked "for agreement" in R2-2201756 not challenged until Tuesday 2022-01-25 0800 UTC will be declared as agreed via email by the session chair (for the rest the discussion might continue during the GTW session on Tuesday).

# Contact Information

To make it easier to find the contact delegate for potential follow-up questions, delegates are encouraged to provide their contact information in the following table:

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Lili Zheng | zhenglili4@huawei.com |
| OPPO | Haitao Li | lihaitao@oppo.com |
| Apple | Pavan Nuggehalli | pnuggehalli@apple.com |
| MediaTek | Abhishek Roy | Abhishek.Roy@mediatek.com |
| Lenovo, Motorola Mobility | Min Xu | xumin13@lenovo.com |
| Intel | Tangxun | xun.tang@intel.com |
| Xiaomi | Xiaolong Li | lixiaolong1@xiaomi.com |
| LG | Oanyong Lee | aioy.lee@lge.com |
| vivo | Xiao XIAO | xiao.xiao@vivo.com |
| CATT | Xiangdong Zhang | zhangxiangdong@catt.cn |
| Turkcell | Izzet Sağlam | Izzet.saglam@turkcell.com.tr |
| ZTE | Yuan Gao | gao.yuan66@zte.com.cn |
| Ericsson | Helka-Liina Määttänen | Helka-liina.maattanen@ericsson.com |
| Transsino | Junwei Huang | junwei.huang@transsion.com |
| Spreadtrum | Qufang Huang | Qufang.huang@unisoc.com |
| Samsung | Shiyang Leng | shiyang.leng@samsung.com |
| Sequans | Olivier Marco | omarco@sequans.com |

# Discussion – First round

## System Information

### Serving cell information

In RAN2 #106-e, it was agreed for IoT NTN that serving cell ephemeris is signalled in a new SIB, and the update to serving cell ephemeris does not affect the SI value tag and will not trigger SI modification procedure either. For NR NTN there is no such agreement, and this aspect should be discussed.

The first issue is whether NTN-related information (such as cell stop time, cell reference location, ephemeris, common TA) should be in a new SIB. Contributions [4][6][14][19][21][24] show support for a new SIB.

Q1: Do you agree to introduce a new NTN-specific SIB (SIBx)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | These information (especially ephemeris) will take up much signalling and should not be carried in SIB1, and some other existing SIBs are also for certain purposes unrelated to NTN. |
| Thales | Yes | Then to be discussed whether this new SIB should be considered as a Minimum SI (MSI). |
| OPPO | Yes |  |
| Google | Yes | The NTN-related information could be huge and hence SIB1 may not have sufficient capacity to accommodate these information. Other on-demand SIBs are not appropriate as some of these NTN-related information are needed for initial access. |
| Nokia | Yes | Agree with Huawei, there are too many NTN-specific parameters to insert them into other existing SIBs (in particular SIB1). |
| Sony | Yes |  |
| Apple | Yes | We think it makes sense to define a separate SIB for NTN; We agree that SIB1 is inappropriate to carry this information. At the very least non-NTN UEs do not have to acquire this information. Whether this SIB should be considered part of MSI or allowed to be on-demand is FFS. |
| MediaTek | Yes |  |
| Lenovo, Motorola Mobility | Yes | This aligns with the agreement made for IoT NTN. |
| Intel | Yes |  |
| LG | Yes | NTN-specific SIB should be introduced to provide ephemeris information of NTN cells. If not, the ephemeris information of the neighbour cells should be contained in the SIB4 which will increase too much signalling overhead. |
| Xiaomi | Yes | The NTN specific parameters should be included in the new SIB since the parameters may be changed frequently. |
| Samsung | Yes |  |
| vivo | Yes | Considering that the satellite ephemeris information will require a lot of bits, it is technically infeasible to embed it into the existing SIB due to the SI size limitation, so to introduce NTN specific SIB to carry satellite ephemeris is necessary. |
| CATT | Yes |  |
| NEC | Yes |  |
| Turkcell | Yes |  |
| ZTE | Yes | Considering the limited size of SIB1 and the different update frequency of the NTN specific parameters, e.g. the ephemeris and the common TA parameters may update more frequently than the existing access information in SIB1, we prefer to introduce a new SIB for NTN. |
| Ericsson | Yes, but | I think we should reformulate the proposal to say NTN specific SI is not all included in SIB1. Then, if we make “NTN SIB” or add NTN info in other SIBs should be further considered. We should start by checking what is MSI for NTN, how often UE needs it/fresh status of it and how to efficiently deliver it. The rest SI can be in other SIBs, NTN SIB or existing SIBs. |

Summary: 19 companies expressed their views, and all of them support introducing a new SIB for NTN.

Proposal 1: (19/19) A new NTN-specific SIB is introduced (SIBx).

Regarding which information to broadcast, candidates include:

- Serving cell ephemeris [4][14][15][19][21][24][27]

- Common TA [14][19]

- Validity duration [14]

- The timing information on when the serving cell is going to stop serving the area (t-Service) [4][6]

- Cell reference location [4][6][8][22]

- Cell type indication (NGSO or GSO) [7]

- Centre of the cell on earth and potentially the radius, plus a time stamp [20]

- A list of cells served by the satellite (along with the ephemeris) [27]

Apart from identifying the necessary information, RAN2 can further discuss whether all the NTN-related information is in the new SIB or some of them might be in other SIBs (e.g., cell type indication can be in SIB1). Proposals related to adding the NTN information in legacy SIBs include:

- The serving cell stop serving time is broadcast in SIB2. [13]

- Add (indexes of) serving/neighbour cell’s ephemeris information or presence of ephemeris to SIB1~5 (for detailed solution, please refer to the corresponding paper). [15]

- UL synchronization assistance information (i.e. common TA parameters and serving satellite ephemeris) is in new SIB, other parameters for initial access are in SIB1. [19]

Q2: Please indicate whether you agree to add the following serving cell information to SIB and which SIB it should be:

Note 1: please use “SIBx” for the potential NTN-specific new SIB, for example: Y/N, SIB1/SIBx

Note 2: for 5), if you think a time stamp is needed, please indicate in the “Comments” column

1) ephemeris;

2) common TA parameters;

3) validity duration for UL sync information;

4) t-Service;

5) cell reference location (FFS plus a time stamp);

6) cell type indication (NGSO/GSO);

7) list of cells served by the satellite.

8) EpochTime

(as per RAN1#107-e Agreement

When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.

Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted.

When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.)

| Company | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Huawei, HiSilicon | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIB2 or SIBx | Y, SIBx | N | FFS | Y, SIBx | For 1~3, UL synchronisation related parameters can be put together in SIBx. For 4, it is already in SIB2 in the running CR, we are also ok with having it in SIBx. For 5, the reference location is needed for location-based reselection. It is FFS now whether location-based reselection will be supported for moving cell scenarios, if supported, since the prediction of reference location will need ephemeris as well, it is better to put reference location together with ephemeris (i.e., in SIBx). We don’t think the time stamp is needed. According to RAN1 agreements, the epoch time of ephemeris and common TA can be known implicitly (i.e., the starting time of a DL sub-frame). The reference location can also use the same epoch time. For 6, we think it can be deduced implicitly (e.g., by ephemeris). For 7, the motivation is not clear enough to us. For cell reselection in fixed cell scenario, the UE does not require ephemeris information of neighbour cells, t-Service or reference location is enough. Since it is FFS whether to support moving cell scenarios and FFS the specific method for UE autonomously adjusting SMTCs in Idle/Inactive mode, we think it is unclear for the moment whether this information (list of cells served by the satellite) is needed. |
| Thales | Y/SIBx | Y/SIBx | Y/SIBx | SIB2 | SIB2 | SIB2 | SIB2 | Y/SIBx | We have assumed that SIBx is handled as MSI |
| OPPO | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | N | FFS | Y, SIBx | Same views as Huawei except for t-Service. We think it is not urgent to send t-Service in SIB2 and it should be provided in SIBx. |
| Google | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIB2 | Y, SIBx | N | N | Y, SIBx | For 4), we prefer to keep it in SIB2 as in the running CR, since it indeed serves for the cell reselection purpose. For 5), we think the time stamp associated with the reference location might be needed only in the earth-moving cell scenario. For 6), it can be deduced from other information or parameters in SIB1/SIBx For 7) The motivation is not clear to us. |
| Nokia | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx or SIB2 | Y, SIBx | N | Y, SIBx | Y | For 4) SIB2 may be also OK. For 5) if moving cell scenario is also supported for the cell reference location then a timestamp shall be added. We are fine to support it, assuming the UE will be able to perform necessary calculations. For 7) we think it is important to indicate which cells are provided by the same satellite, as this may simplify some aspects (e.g. SMTC measurements, timing/sync issues, etc.). Overall, we think 4) and 5) should be kept in the same SIB. Both are mobility related and may not change very often. First three could be prone to frequent updating… |
| Sony | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIB2 or SIBx | Y, SIBx | N | N | Y, SIBx | For serving cell type and cell list, we don’t understand the motivation |
| Apple | Y, SIBx | Y, SIBx | Y, SIBX | Y, SIB2 | Y, SIB3/4 | N | N | Y, SIBX | We think that the parameters need to be grouped based on their periodicity requirement and purpose (cell reselection, timing advance calculation etc.). For this reason, we think SIBx needs to be divided into at least two SIBs (one for slow changing data, and the other for more rapidly changing data). Our understanding is that ephemeris data (1) changes quite infrequently (typically hours). Validity duration (3) is associated with ephemeris and should be sent on the same SIB as ephemeris. Common TA parameters (2) can change rapidly. t-service (4) and cell-reference location (5) are used for cell reselection purposes. So we think SIB2 is appropriate for t-service and SIB3/4 should be used for cell reference location. We have an agreement that cell type should be known by the time SIB1 is acquired. So it could be implicitly derived by knowing SIBx scheduling information contained in SIB1. Epoch time for ephemeris and TA parameters may be different, and may need to be signalled separately. |
| Qualcomm | Y | Y | Y | Y | Y | N | N | Y | Cell type indication should be in SIB1. For 7, it is sufficient to associate the satellite and inter frequency list or neighbor cell list. |
| MediaTek | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIB2 | Y, SIBx | N | Y | FFS |  |
| Lenovo, Motorola Mobility | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIB2 or SIBx | Y, SIBx | N | N | Y, SIBX | From our understanding the presence of SIBx can indicate an NTN cell and the contents of SIBx can indicate cell type. |
| Intel | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | N | Y, SIBx | Y, SIBx | For 5), a time stamp is not needed as the validity duration can be reused. For 6), the satellite type can be derived from ephemeris data. |
| LG | Y, SIBX | Y, SIBX | Y, SIBX | Y, SIBX | Y, SIBX | Y, SIBX | Y, SIBX | See comments | For 6, we think NGSO/GSO type of neighbour cells should be indicated. For 7, we think it is not clear what it means. We think the container satellite is not really important and we just need to provide PCI list of neighbour NTN cells. |
| Xiaomi | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | N | N | Y SIBx | Scheduling information in SIB1 can indicate the network type, and then the ephemeris data in SIBx can indicate NGSO or GSO. |
| Samsung | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | N | FFS, SIBx | Y, SIBx | For 5 epoch time can be used as time stamp of cell reference location. For 6, cell type can be implicitly indicated by ephemeris. |
| vivo | Y SIBx | Y SIBx | Y SIBx | Y SIBx | Y SIBx | N | N | Y SIBx | For 6), UE can know the cell type (NGSO/GSO) implicitly by ephemeris. Also, there were discussions on whether an explicit NW type indication is needed or not, but no consensus has ever reached. For 7), we think the existing neighboring cell broadcast mechanism is adequate for cell reselection. But we can follow the majority view if other companies think this information should be introduced for other motivations (e.g., UE adjusting SMTCs in idle/inactive mode). |
| CATT | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIB2 or SIBx | Y, SIB2 or SIBx | N | N | Y, SIBX | For 1~3, this two information are satellite characteristics, and used for UL sync, these information are large that can be put into SIBx. For 4~5, this two information are cell characteristics and used for cell reselection. Since these two information could not occupy very little space, they can be placed in SIB2, which is a dedicated SIB for cell reselection. But we think they can be also placed in SIBx. For 5, in earth moving scenario, a time stamp is needed together with location information. For 7, it is unnecessary. |
| NEC | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | N | FFS | Y, SIBx | #1-#4 and #8have been agreed to have, only open issue is which SIB is used to broadcasted, new SIB if agreed should be used in our view #5 we support to broadcasted, of course it depends on 1) if location-based cell reselection/measurement rule will be agreed or not, 2) for collected mode CHO and RRM, a common reference point in SIB would be simpler for UE/gNB implementation #6: agree with Huawei, #7: we need to first discuss whether neighbouring satellite/cell information is needed and for what, if only serving cell relevant, it seems not needed |
| Turkcell | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | Y, SIBx | N | Y, SIBx | Y | The cell type can be derived from ephemeris data. |
| ZTE | Y, SIBX | Y, SIBX | Y, SIBX | Y, SIB2 or SIBx | Y, SIB2 or SIBx | Y, SIBX or SIB1 | FFS | Y, SIBX | For 1-3, the UL synchronisation related parameters can be put together in SIBx and the valid time applies. For 4) and 5) used in cell reselection, it can either be put in the SIBX or SIB2. For 6) about the cell type, if UE vendors understand such information would be useful for UE to select a certain NW type to access, we are also fine to provide such information via SIBX or SIB1. For 7), we understand the association between the satellite ephemeris and the neighbour cell.frequency can be provided as assistance information for UE in IDLE and INACTIVE mode to adjust SMTC while the signalling structure is FFS. |
| ER | SIB1/ MSI | SIB1/ MSI | SIB1/ MSI | SIBx/SIB2 | SIBx/SIB2 | SIB1/ MSI | SIBx/SIB2 | SIB1/ MSI | Our understanding is that broadcast parameters in RAN1 excel belong to MSI camp. |

Summary: 20 companies joined the discussion. And the results are listed below:

|  | Yes/No | Which SIB |
| --- | --- | --- |
| 1) Ephemeris | Yes:20 No:0 | SIBx:18 SIB1/MSI:1 No preference expressed:1 |
| 2) common TA parameters | Yes:20 No:0 | SIBx:18 SIB1/MSI:1 No preference expressed:1 |
| 3) validity duration for UL sync information | Yes:20 No:0 | SIBx:18 SIB1/MSI:1 No preference expressed:1 |
| 4) t-Service | Yes:20 No:0 | SIBx:8 SIB2:4 SIB2 or SIBx:7 No preference expressed:1 |
| 5) cell reference location | Yes:20 No:0 Time stamp: 3 think it’s needed for moving cell, 4 think it’s not needed | SIBx:14 SIB2:1 SIB3/4:1 SIB2 or SIBx:3 No preference expressed:1 |
| 6) cell type indication (NGSO/GSO) | Yes:4 No:16 | SIB2:1 SIBx or SIB1:1 SIBx:1 SIB1/MSI:1 |
| 7) list of cells served by the satellite | Yes:7 No:8 FFS:5 | SIBx:5 SIB2:1 SIB2 or SIBx:1 |
| 8) EpochTime | Yes:18 FFS:1 No view:1 | SIBx:14 SIB1/MSI:1 |

Proposal 2: Introduce the following serving cell information to the corresponding SIB:

1) (20/20) Ephemeris to (18/20) SIBx;

2) (20/20) common TA parameters to (18/20) SIBx;

3) (20/20) validity duration for UL sync information to (18/20) SIBx;

4) (20/20) t-Service to (15/20) SIBx;

5) (20/20) cell reference location to (17/20) SIBx;

6) (18/20) Epoch time to (14/20) SIBx. FFS the details of Epoch time.

Another issue is whether the update of NTN-related information will trigger SI modification procedure, considering ephemeris and common TA may be updated frequently.

Companies’ inputs include the following:

- Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure. [3][4][14]

- If ntnUlSyncValidityDuration is provided in the new SIBX, it applies to the whole SIBX. UE acquires the updated SIBX when the timer expires. An alternative (when NW cannot provide ntnUlSyncValidityDuration) is to introduce an indication in Short Message for re-acquisition of SIBX. [19]

- Only changes to certain “important” fields trigger SI modification procedures on UE. [24]

- SIB ephemeris update can be triggered with defined fields in state vector and/or orbital parameters of ephemeris although, a change on these parameters does not always imply a SIB update. [29]

Q3: Which of the following options do you prefer?

- Option 1: Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.

- Option 2: The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires.

- Option 3: When NW cannot provide ntnUlSyncValidityDuration, introduce an indication in Short Message for re-acquisition of SIBX.

- Option 4: Only changes to certain “important” fields trigger SI modification procedures on UE.

- Option 5: SIB ephemeris update can be triggered with defined fields in state vector and/or orbital parameters of ephemeris although, a change on these parameters does not always imply a SIB update.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 and 2 | Option 1 and 2 can be combined. There is no need to trigger the system update procedure whenever ephemeris changes, and the UE can require the information when the timer expires. |
| Thales | Option 1 and 2 |  |
| OPPO | Option 1, FFS for option 2 | For option 2, we understand ntnUlSyncValidityDuration can be used by UE to acquire the updated SIBx, however UE behaviour on how to acquire the updated SIBx should be further discussed, e.g. it may not be exactly when the timer expires and it could also be before the timer expires, or the exact timing can be left to UE implementation. |
| Google | Option 1 | Option 1 is the baseline and Option 2 can be considered as an enhancement based on Option 1 if time allows. |
| Nokia | Option 1 and/or Option 2 | It is desirable to avoid frequent SIBx reacquisition, but on the other hand, another mechanism controlling the validity of SIB or just some selected elements of SIBx seems to be introducing unnecessary complexity. Maybe Option 2 is not that bad, from all those listed. |
| Sony | Option 1 and/or option 2 |  |
| Apple | Option 4 | Ephemeris change should trigger SI modification but change to TA parameters should not. |
| Qualcomm | Option 1 + Option 2 |  |
| MediaTek | Option 1 and 2 |  |
| Lenovo, Motorola Mobility | Option 1 FFS Option 2 | For Option 2 we agree that a timer-based solution can be considered, and we think whether we use ntnUlSyncValidityDuration or another timer can be further discussed. |
| Intel | Option 1 and 4 | If cell stop time changes, it should trigger SI modification procedure. |
| LG | Option 1 and 2 | Same view with Huawei. |
| Xiaomi | Option 1 and Option 2 | Even if ntnUlSyncValidityDuration only used for validity duration, when timer is expired, UE should reacquire the SIBx when the validity duration in the SIBx, option 2 is feasible. |
| Samsung | Option 1, 2 |  |
| vivo | Option 1 and option 2 | Since RAN1 has agreed that “NTN ephemeris validity timer should be started/restarted with configured timer validity duration at the epoch time of the assistance information”, a timer-based solution can be introduced for ephemeris update accordingly. |
| CATT | Option 1 and Option 2 |  |
| NEC | Option 1 and 2 |  |
| Turkcell | Option 5 |  |
| ZTE | Option 2 and Option 3 | - If the ntnUlSyncValidityDuration is provided, UE can rely on such information to decide when to re-acquire the updated SIBX. - For the case when ntnUlSyncValidityDuration is not provided, introduce an indication in Short Message for update of SIBX so that UE can be informed of the change and acquire the updated SIBX immediately. Value tag is not suitable for this SIBX as the UL synchronisation related parameters would update quite frequently, i.e. more than 32 times within 3 hours. We can either introduce an indication in Short Message as we did for CMAS/ETWS SIBs or extend the value tag for SIBX to cover the frequent update of the content. |
| ER | Option 1 Option 2 Option 4 | If cell stop time changes it needs SI mod procedure |

Summary: 20 companies joined the discussion. Option 1 gained the support of 17 companies, and 13 companies prefer “Option 1 and Option 2”. Option 3 is supported by 1 company, Option 4 is supported by 3 companies, and Option 5 is supported by 1 company.

Proposal 3: Regarding the update of UL synchronisation information, Option 1 is supported (17/20), FFS for Option 2 (13/20):

- Option 1: Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.

- Option 2: The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires.

On serving cell ephemeris, it was agreed in RAN2 #113-e that:

Agreements:

- Consider pre-configuration in uSIM, NAS, SIB and RRC signalling for providing the NTN ephemeris. Further discussion depends on the agreed ephemeris contents.

In [27], it is proposed that RAN2 confirms that possible pre-configuration is not further discussed in release 17

Since this is related to a previous agreement, it would be better to have a clarification.

Q4: Do you agree that “Provision of ephemeris is only in SIB, and possible pre-configuration is not further discussed in Rel-17.”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Thales | Yes | We agree that this applies to serving satellite ephemeris. However nothing should prevent the network to provide serving satellite ephemeris in RRC message (e.g. HO command |
| OPPO | See comments | “only in SIB” is not accurate as we need to cover the handover case where ephemeris is signalled in handover command. Suggest to revise as: Provision of ephemeris is only in SIB and RRC signalling in Rel-17. |
| Google | No | This is too restricted. |
| Nokia |  | In our opinion it should be first decided what are the exact contents of the ephemeris. Then bit consumption shall be calculated and static/dynamic components of the ephemeris shall be identified. On this basis RAN2 shall decide if everything can fit into the single SI block or some pre-configuration is to be used either. |
| Sony |  | Agree with Nokia |
| Apple | No | Agree with Nokia. A more careful analysis of ephemeris information is needed. We have used the term ephemeris quite loosely. Some components of the ephemeris that are static are best sent over NAS or pre-configured. In any case, we need to first agree on what we mean by ephemeris first. |
| Qualcomm | No | For now lets keep the agreement as it is. |
| MediaTek | No |  |
| Lenovo, Motorola Mobility | No | We would like to keep it as it is (open) for now. |
| Intel | FFS | This issue can be postponed for now. |
| LG | Yes |  |
| Xiaomi | FFS |  |
| Samsung | No |  |
| vivo | No | We prefer to postpone this discussion until the ephemeris information has been fully discussed. |
| CATT | FFS | Suggest to be postponed. |
| NEC | Yes with comment | If it is only taking about serving cell relevant ones. |
| Turkcell | No | Agree with Nokia |
| ZTE | Yes |  |
| ER | Yes | Technically we agree with Nokia but on practical side it may be too late for Rel-17. We are fine to keep this open as well. |

Summary: 20 companies expressed their views, only 6 companies said “Yes”. 8 companies prefer to postpone the discussion (wait until the exact contents of ephemeris are decided).

Since there is not enough support, no proposal is formulated now.

There is another detailed proposal on ephemeris:

- For a serving satellite, NW is allowed to provide either the state vector or the orbital elements or both of them. If both are provided, the valid time applies to both of them. [19]

And some other detailed proposals on t-Service:

- t-Service is defined as an absolute time value. [4][6]

- t-Service should be UTC time in 10ms units which is earlier than or equal to the time when the cell is going to stop serving the area; When t-Service is broadcast, SIB9 should also be broadcast; UE can acquire the actual UTC time by compensating for the signal propagation delay in the UTC time broadcast in SIB9 using the information of UE location and satellite ephemeris. [4]

The rapporteur’s view is that these can be addressed during the running CR drafting.

### Neighbour cell information

RAN4 sent an LS (R4-2120309) on NTN neighbour cell and satellite information, in which RAN4 lists several parameters and asks RAN2 to identify whether these parameters are relevant to UE measurement mobility.

For NTN UE measurements, e.g. neighbor cell measurement within- or inter-satellite: (A1) Neighbor cell Ephemeris information and the format, e.g. PVT format or Keplarian format (A2) Common TA (A3) Validity timer information for neighbor cell measurements, e.g. if it is different from that for serving cell open loop TA control - Would the timer length, if provided, be different from that for serving cell? For example, a required accuracy of service and/or feeder link delay information for neighbor cell measurement may not need to be as accurate as that for serving cell open loop TA control. (A4) The amount of frequency compensation, if DL frequency compensation for the service link Doppler is applied (A5) DL Polarization information For NTN UE mobility, e.g. target cell measurement, synchronization, and (conditional) handover within- or inter-satellite: (B1) Target cell Ephemeris information and the format, e.g. PVT format or Keplarian format (B2) Common TA (B3) Validity timer information for target cell mobility, e.g. if it is different from that for serving cell open loop TA control (B4) The amount of frequency compensation, if DL frequency compensation for the service link Doppler is applied (B5) DL and UL Polarization information (B6) K_offset

The detailed reply to this LS may not be in the scope of this summary, but some parameters may need to be broadcasted in system information. In [3], it is proposed that (A1)(A3)(A5) plus an extra (A6) Neighbour cell’s feeder link delay should be broadcasted for Idle/Inactive UEs.

Other proposals regarding neighbour cell information include:

- Network type (TN/NTN) and the PLMN ID of neighbour cell are broadcasted. [5]

- Neighbour cell ephemeris can be broadcast as delta information to that of the serving cell. [8][15]

- An indication is provided in the inter frequency list in SIB4 to associate the frequency with the corresponding satellite in the neighbour satellite list. [8]

- Neighbour cell’s ephemeris and the value tag of neighbour cell’s ephemeris are indicated to UE via SIB. [10]

- UE can skip obtaining or decoding the same ephemeris of a satellite/HAP constellation if there is no update. [15]

- Centre of the cell on earth and potentially the radius, plus a time stamp (only those neighbouring cells where UEs can handover to should be included) [20]

- For earth moving cell, the reference location of the cell (serving cell or the neighbour cells) is broadcast in system information. [22]

- Enhance SIB4 by geographic tags, with each tag corresponding to a set of (legacy) cell reselection information [23]. There are similar proposals (frequencies are related to certain geographical regions) in [26].

Q5: Please indicate whether you agree to add the following neighbour cell information to SIB and which SIB it should be:

Note 1: please use “SIBx” for the potential NTN-specific new SIB, for example: Y/N, SIB1/SIBx

Note 2: for 1), if you think delta information should be used, or if you think a value tag is needed, please indicate in the “Comments” column

Note 3: for 6), if you think a time stamp is needed, please indicate in the “Comments” column

1) ephemeris (FFS if in delta information to ephemeris of serving cell, FFS with a value tag);

2) validity timer for neighbour cells measurements (A3 in RAN4’s LS);

3) DL polarization (A5 in RAN4 LS);

4) neighbour cell’s feeder link delay + drift rate;

5) network type (TN/NTN) and the PLMN ID;

6) reference location (FFS plus a time stamp);

7) an indication associating the frequency with satellites;

8) geographic tags which are associated with part of the reselection information;

| Company | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Thales | N | Y, SIBx | Y, SIBx | Y, SIBx | N | N | N | FFS |  |
| Huawei, HiSilicon | FFS | N | Y, SIBx | N | N | Y, SIBx | N | Y, FFS which SIB | For 1, in fixed cell scenario, the UE does not require ephemeris information of neighbour cells to perform cell reselection, t-Service or reference location is enough. Since it is FFS whether to support moving cell scenarios and FFS the specific method for UE autonomously adjusting SMTCs in Idle/Inactive mode, we think it is unclear for the moment whether this information (neighbour cell ephemeris) is needed. For 2, we think the validity timer for serving cell ephemeris can be used, no need for an extra timer. For 3, it is already included in the parameter list from RAN1 (R1-2112976). For 4~5, we don’t see the necessity. For 6, the reference location is needed for location-based reselection. But we don’t think the time stamp is needed. According to RAN1 agreements, the epoch time of ephemeris and common TA can be known implicitly (i.e., the starting time of a DL sub-frame). The reference location can also use the same epoch time. For 7, it is not needed for reselection in fixed cell scenarios. For 8, we think it is useful considering NTN cells have a large coverage. |
| OPPO | Y, SIBx | FFS | Y, SIBx | Y, SIBx | N | Y, SIBx | N | N | For 4, we think it is needed for UE-based SMTC adjustment as UE needs to know both service link delay and feeder link delay to calculate the delay difference and to shift the SMTC for neighbour cells measurements. |
| Google | FFS | Y, SIB2/4 | Y, SIBx | N | FFS | N | N | FFS | For 1), neighbour ephemeris might be required if we want the UE to rely completely on itself to adjust the SMTC. For 2), The validity timer will be beneficial in terms of UE power saving, if it is associated with the SMTC(s) in SIB2/4. For 5), the network type indication (TN/NTN) might be beneficial for prioritizing TN over NTN in the cell reselection procedure. However, this topic should be addressed only if time allows. For 8), as this enhancement is for prioritizing TN over NTN in the cell reselection procedure, it should be addressed only if time allows. |
| Nokia | Y | N | Y, SIBx | N | N | Y | N | N | For 1) we agree that serving cell’s ephemeris is more important. But we also believe neighbour’s ephemeris (especially inter-satellite) can be provided as a delta over serving. For 6), if we support location-based reselection then the reference location for the neighbours shall be also supported. |
| Sony | Y, SIBx | N | Y, SIBx | N | Y, SIBx | Yes, SIBx | N | N | For 5, it’s necessary to allow a UE to know if neighbour cell is TN or NTN so that different reselection rules can be applied. |
| Apple | FFS | FFS | Y, SIBX | N | N | Y, SIB3/4 | N | Y | For (6), we think this information should be contained in cell reselection specific SIBs. We don’t think (8) is for prioritizing TN over NTN, but rather accounts for the fact that NTN cells have large coverage. |
| Qualcomm | Y | N | Y | Y | N | N | Y | N | 1, 3, 4, and 7 are least required parameters for UE to perform IDLE mode measurements. The ephemeris is the baseline for UE to save power in IDLE mode measurement. UE should also be made aware whether a cell or frequency belongs to which satellite. Otherwise, the UE will drain its battery in time tracking of SSBs of neighbor cells. |
| MediaTek | FFS | FFS | Y, SIBX | N | N | Y, SIB3/4 | N | FFS |  |
| Lenovo, Motorola Mobility | Y, SIBx | FFS | Y, SIBx | N | N | Y, SIBx | N | N | For 1) neighbour ephemeris is necessary and we can provide delta values to minimize its size. We also think it would be helpful to have a tag or timer to indicate update, and this depends on the conclusion of Q3. For 2) we can discuss whether the validity timer for serving ephemeris can be used. For 5) we think network type (TN/NTN) can be indicated by presence/absence of SIBx. |
| Intel | Y, SIBx | Y, SIBx | FFS | FFS | N | N | Y, SIBx | N | For 1), RAN2 already agreed “The NTN ephemeris is divided into serving cell’s ephemeris and neighbour’s ephemeris.” For 5), TN/NTN type can be derived from ephemeris data. |
| LG | Y | Y | Y | Y | Y | Y | FFS | FFS |  |
| Xiaomi | Y SIBx | N | Y SIBx | N | N | Y SIBx | N | N | In last meeting, RAN2 agreed that RAN2 assumes FL delay is known to and compensated by the network and RAN2 also assumes the UE needs to have neighbour cell ephemeris for the propagation delay estimation. So ephemeris is essential. |
| Samsung | Y, SIB2/3/4 or SIBx | Y, SIB2/3/4 | Y, SIB2/3/4 | Y | Y, SIB2/3/4 | Y, SIB2/3/4 | Y | FFS | For 1, dependent on delta is used or not |
| vivo | Y SIBx | N | Y SIBx | N | N | Y SIBx | N | N | For 1), neighbour ephemeris may be required for SMTC adjustment in IDLE/INACTIVE. For 2), from our perspective, the gain of knowing the starting/stopping time information of the neighbour cells (s) is limited. Since even if the UE knows the time when the new cell starts its service in advance, the UE cannot avoid performing measurements of a frequency, unless all cells on that frequency have not illuminated the area. For 6), knowing the reference location for neighbour cell(s) can help UE to decide the distance from the target cells. |
| CATT | FFS | FFS | Y | FFS | N | N | N | N | For 5, if TN and NTN are deployed in different frequency, they can be implicit distinguished. For 6, we don’t support that the reference location of neighbour cells should be introduced in cell reselection criteria. For 8, in case of NTN cell has numerous of neighbour TN cell, NTN can solve this problem by broadcasting the frequencies instead of cell ID. |
| NEC | FFS | N | Y, SIBX | FFS | N | N | N | Y | 1),4) depends on SMTC in idle mode discussion 5) may not needed if not NTN/TN are not deployed on same frequency 8) is needed consider the large coverage of NTN cell. |
| Turkcell | Y | FSS | Y | N | N | Y | N | N |  |
| ZTE | Yes, SIBX or SIB3/4 | N | Y for serving cell N for neighbour cell, SIBX or SIB1 | N | Y for NW type, SIBX or SIB3/4 | Y, SIBX or SIB3/4 | Y | N | For 1), the satellite ephemeris of neighbour cells/frequencies would be useful for UE in IDLE to adjust SMTCs. For 3), the DL/UL polarization of the serving cell is included in RAN1 parameter list R1-2112976 while the DL polarization info for neighbour cell is not included. For 5), UE should be aware of the neighbour cell NW type so that certain NW can be prioritized during reselection. For 6), the reference location of neighbour cell is needed for UE to evaluate the distance to it. For 7), we understand the association between the satellite ephemeris and the neighbour cell.frequency can be provided as assistance information for UE in IDLE and INACTIVE mode to adjust SMTC while the signalling structure is FFS. |
| ER | FFS, if cell broadcast the PCI list that helps here. | FFS | N for neighbor cell | N | Y | As optional ok | FFS | FFS |  |

Summary: 20 companies joined the discussion. And the results are listed below:

|  | Yes/No | Which SIB |
| --- | --- | --- |
| 1) ephemeris | Yes:12 No:1 FFS:7 Support of delta information:3 Support of value tag:1 | SIBx:6 SIB2/3/4 or SIBx:1 SIB3/4 or SIBx:1 |
| 2) validity timer for neighbour cells measurements | Yes:5 No:8 FFS:7 | SIBx:2 SIB2/4:1 SIB2/3/4:1 |
| 3) DL polarization | Yes:17 No:2 FFS:1 2 companies said “Y for serving cell N for neighbour cell”. Considering this is in neighbour cell information discussion, the rapporteur marked the answers as “No” | SIBx:12 SIB2/3/4:1 SIBx or SIB1:1 |
| 4) neighbour cell’s feeder link delay + drift rate | Yes:5 No:12 FFS:3 | SIBx:2 |
| 5) network type (TN/NTN) and the PLMN ID | Network type: Yes:5 No:14 FFS:1 PLMN ID: Yes:4 No:15 FFS:1 | SIBx:1 SIB2/3/4:1 (for network type) SIBx or SIB3/4:1 |
| 6) reference location | Yes:14 No:6 Support of time stamp:0 | SIBx:6 SIB3/4:2 SIB2/3/4:1 SIBx or SIB3/4:1 |
| 7) an indication associating the frequency with satellites | Yes:4 No:14 FFS:2 | SIBx:1 |
| 8) geographic tags which are associated with part of the reselection information | Yes:3 No:11 FFS:6 |  |

Proposal 4: Introduce the following neighbour cell information to the corresponding SIB:

1) (17/20) DL polarization to (12/20) SIBx;

2) (14/20) reference location to (6/20) SIBx;

3) (12/20) ephemeris to (6/20) SIBx.

### Upcoming cell

In RAN2 #116-e, there was discussion on whether to provide information on the next candidate cell but no conclusion was made.

Based on companies’ related to this [1][6][8][13][28], RAN2 can further discuss

- Whether the information of upcoming cell is provided

- If provided, determine the exact assistance information:

  - Frequency and PCI

[28] further proposes that UE may skip the measurements on intra-frequency and inter-frequency neighbouring cells except the configured incoming neighbouring cell. The rapporteur’s view is that this proposal can wait until there is conclusion on whether the information of upcoming cell is provided.

Q6: Do you agree to broadcast the information of the upcoming cell (e.g., frequency and PCI)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | It is useful for fixed cell scenarios. |
| Thales | Yes | It is useful for quasi Earth fixed cell scenarios. |
| OPPO | No | Existing neighbour cell list may suffice. Nothing more optimization and spec impact should be introduced. |
| Google | Yes | Since the location-assisted cell reselection has been agreed in RAN2#116-e, the UE may need to know the reference location of the neighbouring cell. Therefore we think at least the reference location of the upcoming cell can be provided. |
| Nokia | N | We do not think it is essential in IDLE mode. |
| Sony | Yes |  |
| Apple | No | Seems like an optimization. Existing neighbour cell information provided should be sufficient. |
| Qualcomm | Yes |  |
| MediaTek | Yes | It could be useful for cell reselection |
| Lenovo, Motorola Mobility | Yes | Only information that is identified as essential. |
| Intel | Yes |  |
| LG | Yes | As upcoming cells are predictable, if the serving cell provides in advance, the network does not need to update the neighbour cell list whenever the neighbour cell list changes. |
| Xiaomi | No | It will have impact on neighbour cell measurements procedure, but we already have some enhancements on neighour cell measurements, this optimization is not essential. |
| Samsung | Yes | We do think it provides much power saving gain especially considering power saving becomes more important due to GNSS in NTN. Otherwise the power consumption is bad for idle/inactive compared to TN. |
| vivo | No | The existing white cell list can be reused for such a purpose, i.e., the UE shall consider only the white listed cells, if configured, as candidates for cell reselection. |
| CATT | Yes, see the comment | We agree to broadcast the frequency and PCI of upcoming cell. But for the time information, since the actual network deployment is not clear, it cannot be assumed that the next candidate cell will always cover the exactly same area as the current serving cell. If the coverage scope of the incoming cell is not smaller than the coverage scope of the current serving cell, the time information may be useful. |
| NEC | No | We need to focus on essential aspect instead of optimization |
| Turkcell | No |  |
| ZTE | No | The existing neighbour cell information is sufficient. |
| ER | yes | For fixed cell |

Summary: 20 companies joined the discussion. 12 companies agree to broadcast the information of the upcoming cell (e.g., frequency and PCI).

Proposal 5: (12/20) The information of the upcoming cell (e.g., frequency and PCI) is broadcast.

## Location-based reselection

### Measurement

For time-based reselection in quasi-fixed cells, it was agreed in RAN2 #115-e that UE should start measurements on neighbour cells before the serving cell stops covering the current area.

However, there is no agreement so far regarding how the location information helps to determine when to measure neighbour cells. On this aspect, companies’ views are divergent.

No-optimisation camp:

- Location based neighbour cell measurement rule is not considered in R17. [1]

Yes-for-optimisation camp:

- UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold). [4] [11][13]

- Start measurements if the distance between UE and serving cell reference point is above a configured threshold [22]

- Two different threshold DIntraSearch and DnonIntraSearch are configured separately and used for UE to trigger measurement on intra and inter frequency cells (with lower or equal priority). A threshold DSeachTreshold is configured for “not at cell edge” criterion. [25]

[4][14][25] also mentions that the location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

Q7: Do you agree that location information is used to determine when to start measurement?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| OPPO | No | We think legacy RSRP-based approach is sufficient. |
| Google | Yes |  |
| Nokia | Yes | In our understanding the location information shall trigger the UE to perform reselection process. Whether the UE only then starts measuring or already has the measurements completed should be handled by the UE according to the requirements (e.g. from RAN4). |
| Sony | Yes |  |
| Apple | Yes | But we don’t think this needs to be over-specified, at least in RAN2 specs. Also, nothing should mandate UEs to acquire location information in idle/inactive state for cell reselection purposes. |
| Qualcomm | Yes |  |
| MediaTek | No | Agree with OPPO that legacy RSRP-based approach is sufficient. |
| Lenovo, Motorola Mobility | Yes |  |
| Intel | Yes |  |
| LG | Yes |  |
| Xiaomi | Yes |  |
| Samsung | Yes | Yes if location based cell reselection criteria is configured. |
| vivo | No | Since the radio link quality eventually decides whether the communication can really be performed or not, it makes no sense for the UE to keep staying on the serving cell if the radio measurement result is lower than the threshold configured by NW but the distance between UE and serving cell reference location is still shorter than a threshold. So, we think legacy RSRP-based measurement triggering, with time-based trigger (if configured), is sufficient. |
| CATT | Yes | The location information can help UE initiate measurement on neighbour cells timely, and avoid too early measurement to reduce power assumption. |
| NEC | Yes |  |
| Turkcell | Yes |  |
| ZTE | No | Agree with OPPO that legacy RSRP-based approach is sufficient. |
| ER | yes |  |

Summary: 19 companies joined the discussion. 15 companies said “Yes” and 4 companies said “No”.

Proposal 6: (15/19) Location information can be used to determine when to start measurement.

Q8: If your answer to Q7 is “Yes”, do you agree with the following?

UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Google | Yes |  |
| Nokia | Y |  |
| Sony | Y |  |
| Apple | Yes |  |
| Qualcomm | Yes |  |
| Lenovo, Motorola Mobility | Yes |  |
| Intel | Yes |  |
| LG | Yes |  |
| Xiaomi | Yes |  |
| Samsung | Yes |  |
| CATT | Yes |  |
| NEC | Yes |  |
| Turkcell | Yes |  |
| ER | yes | This only within NTN |

Summary: 15 companies expressed their views and all of them agree with the proposal.

Proposal 7: If proposal 6 is agreed, agree the following (15/15):

UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

Q9: If your answer to Q7 is “Yes”, do you agree with the following?

Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Google | Yes | The location-related parameter can be the reference location of the upcoming/neighbouring cell. The distance threshold can be also provided if it is configurable. |
| Nokia | Yes, but | What does it mean ‘by implementation the UE has location information’? That the UE does not measure it just for the purpose of reselection? |
| Sony | Yes |  |
| Apple |  | Provided UE does not have to acquire location just for cell reselection purposes. |
| Qualcomm | Yes |  |
| Lenovo, Motorola Mobility | Yes |  |
| Intel | Yes |  |
| LG | Yes |  |
| Xiaomi | Yes | When location related parameters are configured, whether perform location based cell reselection is based on UE implementation. If UE has available UE location, it can perform location based cell reselection, otherwise not. |
| Samsung | Yes |  |
| CATT | Yes |  |
| NEC | yes |  |
| Turkcell | Yes |  |
| ER | yes |  |

Summary:15 companies joined the discussion and all companies seem to accept the proposal.

Proposal 8: If proposal 6 is agreed, agree the following (15/15):

Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

[20] proposes that RAN2 will not specify how the UE performs location acquisition as long as the minimum accuracy, which is to be defined by RAN4, is fulfilled. The required minimum accuracy for location information for quasi Earth fixed cell can be lower in the cell centre then at the cell border.

The rapporteur’s view is that the requirement/accuracy issue is out of RAN2 scope, and no company is proposing to specify how the UE performs location acquisition, so no additional agreement seems necessary.

### Reselection rule

In RAN2 #116-e, it was agreed that:

Agreements:

- Location assisted cell reselection, with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account, is supported for quasi-earth fixed cell. FFS on how UE performs location acquisition.

The detailed solution is FFS. During Offline [AT116-e][102] (summary is in R2-211352), the following options are listed and companies showed a slight majority support of Option 1:

- Option 1: only neighbour cells with distance shorter than a threshold will be considered during cell reselection;

- Option 1b: exclude neighbour cells too far away i.e., distance longer than a threshold will not be considered during cell reselection

- Option 2: distance based ranking is used together with legacy R criteria.

Based on the contributions submitted to #116bis-e, most proposed options still fall into the above categories [1][4][12][14][19][22][25][28]. So the discussion can be continued based on the above options.

There are also proposals on using legacy mechanism:

- If there is no benefit brought by using the neighbour cells reference location in cell reselection criterion, the neighbour cells reference location should not be broadcast, and the legacy cell reselection criteria based on RRM measurement should be reused as baseline. [13]

- Distance-based criterion is only applied in neighbour cell measurement triggering. [16]

The rapporteur’s view is that, since it is already agreed in RAN2 #116-e that location assisted cell reselection is supported, RAN2 should try to converge to a solution.

Q10: Which option do you prefer?

-	Option 1: only neighbour cells with distance shorter than a threshold will be considered during cell reselection;

-	Option 1b: exclude neighbour cells too far away i.e., distance longer than a threshold will not be considered during cell reselection

-	Option 2: distance based ranking is used together with legacy R criteria.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 |  |
| Thales | Option 1 |  |
| OPPO | Option 2 | After legacy RSRP based ranking, the cell with the shortest distance to the satellite’s cell center is selected. This can benefit both link quality and subsequent transmission delay. |
| Google | Option 1 | Option 1 is simpler. |
| Nokia |  | Is there any firm difference between 1 and 1b? Both are OK, if indeed neighbour’s location shall be considered (i.e. not only the serving’s location). |
| Sony | Option 1 |  |
| Apple | Option 2 | Since we should not assume distance is known at the UE, it is best to enhance existing procedures with location information, if it is available at the UE. |
| Qualcomm | None | It is not guaranteed that network will provide reference location for each and every neighbor cell or in the neighbor cell list. |
| MediaTek | Option 1 |  |
| Lenovo, Motorola Mobility | None | We think location-based measurement triggering is sufficient. All options listed in this question requires broadcast of reference locations of multiple cells and calculations of distances based on that. |
| Intel | Option 1 |  |
| LG | Option 1 |  |
| Xiaomi |  | It is not clear how to determine the target cell for all options. For option 1, does it mean the cell with the highest rank among all neighbour cells with distance shorter than a threshold will be the target cell? |
| Samsung | Option 2 | Legacy R criteria should be used in any case. |
| vivo | Option 1 | Moreover, we think option 1 should be introduced for both inter-frequency and intra-frequency cell reselection criterion. |
| CATT | None | Distance information is only used for measurement triggering, and then legacy cell reselection criteria based on RSRP/RSRP is enough. And we also show the same view with Qualcomm. |
| NEC | Option 1b or nothing | Option 1b will be equal option1 only if we reword as below to cover the case that UE may not have satellite information of some detected neighbouring cells and hence cannot calculate the distance: “neighbour cells with distance shorter than a threshold or without distance information will be considered during cell reselection; |
| Turkcell | Option 1b |  |
| ZTE | Option 1b | Since it is not possible for NW to provide the reference location of all the neighbour cells, we understand it is better to use the distance threshold as an exclusion criteria so that neighbour cells not configured with reference location can also be considered by UE during reselection. |
| ER | Optin 1 or 1b | These are the same in practice? Also, when doing this and then making ranking, it can be understood as Option2 |

Summary: 20 companies joined the discussion. 10 companies support Option 1, 5 companies support Option 1b, 3 companies support Option 2, and 3 companies support none.

In the rapporteur’s understanding, the major difference between Option 1 and Option 1b is the treatment of neighbour cells without a reference location (as expressed by ZTE).

Proposal 9: Discuss which option to adopt for location-based reselection:

-	Option 1: only neighbour cells with distance shorter than a threshold will be considered during cell reselection; (10/20)

-	Option 1b: exclude neighbour cells too far away i.e., distance longer than a threshold will not be considered during cell reselection; (5/20)

-	Option 2: distance based ranking is used together with legacy R criteria. (3/20)

### Earth-moving cell

So far the time-based reselection and location-based reselection agreements are all confined to quasi-earth fixed scenarios.

In [6], it is proposed that “Broadcast of the reference location of the cell in SIB is not applicable to earth-moving cells. No further work in this release to address any moving cell specific details on supporting the location-based cell reselection.”

On the other hand, [11][12][13][20][22] propose to utilize location information, either in determining whether to start measurements, or in determining the target cell for reselection. [17] also voices that the reference location can be deduced by combing the moving trajectory and the coverage information. Note that the location-based CHO for CONNECTED mode UE does not exclude the moving cell scenarios.

Since there are contradicting views, it would be good to decide whether there will be enhancement for measurement/reselection based on time/location information for moving cell scenarios. If the conclusion is yes, RAN2 can further decide the details.

Q11: Do you think RAN2 should introduce enhancement for measurement/reselection based on time/location information for moving cell scenarios?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | There is not enough time for Rel-17, it could be postponed to future releases. |
| Thales | No | Agree with Huawei comment |
| OPPO | Yes | We can discuss this together with location-based CHO as in both cases neigbhor cells’ reference points need to be signalled to the UE. |
| Google | No | Since the time based cell reselection is only applicable for the quasi-earth fixed scenario, we suggest only focusing on this scenario. |
| Nokia | Yes | We agree this is not so straightforward as for Earth-fixed cells, but if we anyway introduce location-based mechanism for IDLE UEs then in our view, it is fine to support it also in EMC. |
| Sony | Yes | We also share the view that it can be extended to moving cells |
| Apple | No |  |
| Qualcomm | Yes | Just capture in Note this is up to UE how it determines the cell serving time based on ephemeris and beam information of the cell. |
| MediaTek | No | Agree with Huawei, Apple and Thales |
| Lenovo, Motorola Mobility | No | We would like to consider in further releases. |
| Intel | No | We may not have enough time to finish this enhancement in this release. |
| LG | Yes | We think we can just make simple approach for earth moving cell – Option 1 of Q10. |
| Xiaomi | No | It is more complicated for earth moving cell, we don’t have enough time in Rel-17. |
| Samsung | No | Not enough time for Rel-17 |
| vivo | No | Similar to the time-based cell reselection, the location-based cell reselection mechanism should only be supported for quasi-earth-fixed cells. |
| CATT | Yes | For earth moving cell, enhancement is also needed for UE to reselect a cell. Since the trajectory of cell center can be seen as a line in a period of time, with knowing the cell center position at a time and the moving speed vector, UE can calculate the real-time cell center roughly, and it is enough for UE to judge the cell edge. |
| NEC | Yes | But this only request very minor enhancement/additional specification: Same as quasi earth fixed cell, for earth moving cell, at certain time point, a cell is going to disappear e.g., feeder link change, hence time-based measurement/reselection is also useful for earth moving scenario, and we do not see any difference in term of specification work. Motivation of having location-based measurement/reselection is true for both quasi earth fixed and earth moving cell scenario. the only difference is the reference cell center of earth moving cell is moving (but information can be static, if refer to nadir of satellite), hence there would be sightly difference on providing reference cell center information, this would be the only specification difference. |
| Turkcell | Yes |  |
| ZTE | No |  |
| ER | No | Not in Rel-17 |

Summary: 20 companies joined the discussion, 8 said Yes and 12 said No.

Proposal 10: (12/20) No enhancement is introduced for measurement/reselection based on time/location information for moving cell scenarios in Rel-17.

## Time based cell reselection

### Measurement

In the last meeting, it is has been agreed by RAN2 that

Agreement: For quasi-earth fixed cell, same as legacy, UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the distance between UE and serving cell reference location.

The agreement only emphasizes the relationship between the higher priority measurements and the distance, not mentioning t-Service.

In [4], it is proposed that: For quasi-earth fixed cell, same as legacy, UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time.

It is also raised in [4] that, based on the above illustration, UE will start measurements during time interval 1 according to the agreement on t-Service (@ RAN2 #115-e: “For quasi-earth fixed cell, UE should start measurements on neighbour cells before the serving cell stops covering the current area.”) but the UE measurement behaviour in time interval 2 (before time interval 1) is unclear:

1)	Legacy behaviour (i.e., based on Srxlev/Squal), or

2)	Reduce measurements

The proposal in [4] is: Before the stop-time based measurements are triggered, the UE only performs measurements on higher priority frequencies.

The rapporteur’s view is that it would be good to discuss the above issues to achieve a better understanding of previous agreements on time-based measurements.

Q12: Do you agree with the following proposal?

For quasi-earth fixed cell, same as legacy, UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Thales | Yes |  |
| OPPO | Yes |  |
| Google | Yes |  |
| Nokia | Yes |  |
| Sony | Yes |  |
| Apple | Yes |  |
| Qualcomm | Yes |  |
| MediaTek | Yes |  |
| Lenovo, Motorola Mobility | Yes |  |
| Intel | Yes |  |
| LG | Yes |  |
| Xiaomi | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| CATT | Yes |  |
| NEC | Yes |  |
| Turkcell | Yes |  |
| ZTE | Yes |  |
| ER | Yes |  |

Summary: 20 companies joined the discussion and all of them agree with the proposal.

Proposal 11: (20/20) For quasi-earth fixed cell, same as legacy, UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time.

Q13: Before the stop-time based measurements are triggered (i.e., time interval 2 in the following picture),

which should be the intended UE behaviour?

- Option 1: Legacy behaviour (i.e., based on Srxlev/Squal)

- Option 2: Reduce measurements (e.g., only measure frequencies of higher priorities)

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 | The introduction of t-Service is to reduce measurements. If the UE performs measurements in both Interval 1 and Interval 2, the stop-time based measurements bring no gain. On the other hand, there is no urgent need for the UE to perform reselection since the current serving cell still has a considerable remaining serving time. Therefore, in Interval 2, the UE only needs to perform measurements on higher priority frequencies, not equal or lower priority. |
| Thales | Option 2 |  |
| OPPO | Option 1 | Stop-time scheme does not address the cell edge case. So UE should follow legacy behaviour just in case it is in the cell edge before the stop-time comes. |
| Google | Option 1 |  |
| Nokia | Option 2 | As we have commented in the past, by t-Service the UE should have completed measurements for reselection, as the current cell will disappear. So the UE can measure only higher priority cells as long as it can then complete all measurements for reselection by t-service expiry. |
| Sony | Option 2 |  |
| Apple | Option 1 | Agree with OPPO |
| Qualcomm | Option 1 | See in Q12, higher priority frequency search is regardless of this time. We do not see to add this complexity. This is just UE implementation how long is this interval. |
| MediiaTek | Option 1 |  |
| Lenovo, Motorola Mobility | Option 1 |  |
| Intel | Option 1 |  |
| LG | Option 1 | First of all, we are confused with the definition of the timer interval2. If we do not consider the legacy measurement rule, it should be “UE may perform measurements on neighbour cells) By the way, we do not think additional such measurement relaxation mechanism is not really needed. |
| Xiaomi | Option 1 |  |
| Samsung | Option 1 |  |
| vivo | Option 1 | UE should rely on legacy behaviour to trigger measurement, if UE moves to the cell edge. Otherwise the UE will miss some opportunities of cell reselection, resulting in loss of coverage. |
| CATT | Option 2, see the comment | Option 2 can help UE reduce energy consumption. For option 2, we want to further clarify that the time should be used together with RSRP. That is, if one of time condition and RSRP condition is satisfied, UE will start measurement on neighbour cells. |
| NEC | Option 1 | Legacy behaviour should be followed: UE at cell centre may not do measurement on equal or lower priority frequency, UE at the cell edge should still do normal measurement on all frequencies for possible cell reselection. |
| Turkcell | Option 1 |  |
| ZTE | Option 1 |  |
| ER | Option 1 |  |

Summary: 20 companies joined the discussion. 15 companies prefer Option 1 and 5 companies prefer Option 2.

Proposal 12: (15/20) Before the stop-time based measurements are triggered, the UE measurements follow Legacy behaviour (i.e., based on Srxlev/Squal) and there is no measurement relaxation.

### Reselection rule

In RAN2 #116-e, it was agreed that the cell stop time of neighbour cell is not broadcast, which implies that stop time will be not used for cell ranking:

(Proponents:17, Opponents:6) Proposal 4: RAN2 to discuss whether to support the following proposal:

For quasi-earth fixed cell, the cell stop time of neighbour cell(s) is NOT broadcast.

- Continue online

-	Huawei thinks if we agree on this then the cell stop time is not used for cell ranking but only to start measurements

-	Nokia is fine with p4

-	Oppo is fine not to use the cell stop time for cell ranking

-	Intel also supports p4

-	LG still thinks the stop time of neighbour cells is useful.

-	Ericsson thinks it would be useful to have this information to stop measurements but not for cell ranking

-	ZTE thinks this is useful to exclude some cells but can accept p4

- Agreed as “For quasi-earth fixed cell, the cell stop time of neighbour cell(s) is NOT broadcast”

However, there is no explicit agreement to exclude time-based ranking. And some proposals submitted to RAN2 #116bis-e are related to this aspect:

- For quasi-earth fixed cell, time based cell reselection criterion is not considered in R17. [13]

- RAN2 discuss how to apply t-service in the ranking rules for cell reselection. [27]

Q14: Do you think cell stop time should be applied to cell ranking?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Neutral | If applied, the cell stop time of neighbour cell(s) needs to be broadcast. |
| Thales | Yes | It may be used to prioritise the cell with the longest remaining serving time |
| OPPO | No |  |
| Google | No | We prefer to stick to the agreement RAN2 concluded previously. |
| Nokia | No | Enough only to provide it for the current serving. |
| Sony | No |  |
| Apple | No |  |
| Qualcomm | No |  |
| MediaTek | No |  |
| Lenovo, Motorola Mobility | No |  |
| Intel | no |  |
| LG | Yes | We still think stop time-based ranking is useful to avoid unnecessary cell reselections. |
| Xiaomi | No |  |
| Samsung | No | Time for cell’s incoming or disappearing are helpful to decide when the measurement needs to be performed and when the cell (re)selection performs, but not needed directly into cell reselection criteria itself. |
| vivo | No |  |
| CATT | No |  |
| NEC | No |  |
| Turkcell | No |  |
| ZTE | No |  |
| ER | yes | It would be useful but qwe are ok with majority view |

Summary: 20 companies joined the discussion, and 16 companies said “No”.

Proposal 13: (16/20) Cell stop time is not applied to cell ranking in determining the target cell for reselection.

## Joint working of time-based and location-based reselection

It is currently FFS whether time-based reselection and location-based reselection can be configured jointly.

It is proposed in [4] that: Time and location based measurements can be configured simultaneously by network, and UE can choose to apply one or both of them, which is determined by UE implementation. When UE chooses both of them, the measurement should be started if either time based condition or location based condition is satisfied.

Q15: Do you think time-based and location-based reselection can be configured simultaneously?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Thales | Yes |  |
| OPPO | No | It would be sufficient to configure either one but not both. |
| Google | No | This would increase the implementation complexity for the UE. |
| Nokia | No | There is no need to make it overly complex. |
| Sony | Yes | We think this option should be allowed. |
| Apple | No |  |
| Qualcomm | Yes |  |
| MediaTek | No |  |
| Lenovo, Motorola Mobility | Yes | We can keep it open |
| Intel | No |  |
| LG | Yes |  |
| Xiaomi | No |  |
| Samsung | No |  |
| vivo | No | Since time-based condition is only used to trigger measurement but location information is not used to determine when to start measurement, there is no need to discuss the issue. |
| CATT | Yes | But this can be left for network implementation. |
| NEC | yes | It is up to network configuration. No need to have restriction And they address difference cell reselection cases |
| Turkcell | Yes |  |
| ZTE | Yes | No need to have restriction. Up to NW implementation. |
| ER | FFS | We are open to it |

Summary: 20 companies joined the discussion, and 11 companies think time-based and location-based reselection can be configured simultaneously, others hold the opposite opinion.

Proposal 14: (11/20) Time-based and location-based reselection can be configured simultaneously. FFS UE behaviour when configured together.

## TN prioritization over NTN

In RAN2 #114-e Offline [104] (summary is in R2-2106526), most companies do not like a fixed priority for TN over NTN, and prefer to leave it to NW configuration. Therefore, the following was agreed:

Agreements online:

- For idle mode reselection, based on configuration NTN UE can prioritise TN over NTN. Configuration details FFS

Among the contributions submitted to RAN2 #116bis-e, some companies still hold that existing frequency priority framework is enough [9][13][16][17][20], whereas some other companies want enhancements on TN prioritization [1][12], the main argument is that NTN cells have a large coverage, UEs located in some parts of the coverage may be far away from TN neighbours while UEs of other locations may be in TN coverage:

The following options are proposed:

- If high priority TN cell cannot be detected for a period of time, then UE applies relaxed measurements to high priority TN frequency. [1]

- Network provides the coverage information of TN area to UE, e.g., area centre and radius, UE only starts measurements on TN frequencies when it is near or within TN area. [1]

- Provide neighbour cell type (TN/NTN) cell in SIB [1][19]. An alternative is implicit indicating by broadcasting NTN specific reselection parameters in the new SIB [19].

- UE increases frequency priority of TN frequencies when the UE enters a specific location area. The network configures increased frequency priority of TN neighbour frequencies mapped to the location area. [12]

- Add an offset to R value of TN, or add a priority offset to TN frequency priorities. The R value offset or reselection priority offset is configured by the NW to prioritize a certain NW type, e.g. TN. [19]

Q16: Regarding TN prioritization over NTN, which of the following do you prefer?

- Option 1: left to NW implementation in Rel-17

- Option 2: additional enhancement is introduced

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 | We don’t think RAN2 can converge to a specific solution given the limited time left in Rel-17. |
| Thales | Option 1 |  |
| OPPO | Option 1 |  |
| Google | Neutral |  |
| Nokia | Option 1 | Legacy prioritization (carrier frequency based) shall be enough in Rel-17. Rel-18 can further consider more advanced solutions in this area. |
| Sony |  | We prefer option 2 but ok to accept option 1 for rel-17 |
| Apple | Option 1 | At this point it is not clear that TN and NTN cells will use the same band. So prefer not to be develop a solution till there is a real need. |
| Qualcomm | - | As RAN4 has already clarified the NTN bands n255 and n256 overlaps with TN bands. It is not clear how overlapping of frequency will be avoided specially in cell edge. For these overlapping band frequencies, the offset based mechanism should be used and it would be cleaner to introduce a new one. |
| MediaTek | Option 1 | Rel-16 absolute priorities can be used by the network to configure this. |
| Lenovo, Motorola Mobility | Option 1 |  |
| Intel | Option 1 |  |
| LG | Option 2 | We already agreed to introduce location-based measurement rule/cell reselection, so it would not bring much specification issues if we introduce such additional enhancement. |
| Xiaomi | Option 2 | We prefer option 2, but we are also fine to discuss this in Rel-18 due to time limitation. |
| Samsung | Option 1 |  |
| vivo | Option 1 | The existing frequency prioritization method is sufficient in Rel-17, since the typical case would be the TN and NTN deployed on different bands in this release. |
| CATT | Option 1 |  |
| NEC | Option 2 | there is significant power issue without any enhancement. But if majority would like to delay the discussion to Rel-18, we can accept it |
| Turkcell | Option 2 |  |
| ZTE | Option 2 | At least UE should be aware of the neighbour cells’ NW type, i.e. TN or NTN, based on the serving cell’s system information. |
| ER | Option 2 | Due to limited time we are also ok with option 1 |

Summary: 20 companies joined the discussion, 11 companies prefer Option 1 and 6 companies prefer Option2, and 2 companies can accept both.

Proposal 15: (13/20) TN prioritization over NTN is left to NW implementation in Rel-17.

## RRC_INACTIVE mode

In it not decided yet whether RRC_INACTIVE mode is supported for NTN. [12] suggests RAN2 discuss this and send an LS to RAN3 if needed.

The rapporteur’s view it that so far the discussion has not excluded RRC_INACTIVE, but it would be good to make it clear.

Q17: Do you agree that RRC_INACTIVE mode is supported for NTN?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | But no enhancement specific to RRC_INACTIVE is introduced. |
| Thales | Yes |  |
| OPPO | Yes | For now we don’t see any showstopper for RRC_INACTIVE to be supported. |
| Google | Yes |  |
| Nokia | Yes | Agree with Huawei. |
| Sony | Yes |  |
| Apple | Yes |  |
| Qualcomm | Yes | But how RNA is configured and how RAN paging and CN paging would work should be addressed. |
| MediaTek | Yes |  |
| Lenovo, Motorola Mobility | Yes |  |
| Intel | Yes |  |
| LG | Yes | We agree with Huawei that no enhancement should be introduced. |
| Xiaomi | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| CATT | Yes |  |
| NEC | Yes |  |
| Turkcell | Yes |  |
| ZTE | Yes | Support with no enhancement in this release. |
| ER | yes | With no enhancements |

Summary: 20 companies expressed their views and all of them agree that RRC_INACTIVE is supported.

Proposal 16: (20/20) RRC_INACTIVE mode is supported for NTN.

Q18: Do you agree to send an LS to RAN3 about the conclusion of Q17?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Not needed | It is agreed in RAN3 #113-e that: Apart from RAN Paging, RRC_INACTIVE is kept unspecified from a RAN3 perspective, i.e. no further RRC_INACTIVE related specification text is added, neither on stage 2 nor stage 3 level. In our understanding, RAN3 already supports RRC_INACTIVE, so no LS is needed. |
| Thales | No |  |
| OPPO | Not needed |  |
| Google | No | There should be no impact to RAN3. |
| Nokia | No | We have the same understanding as Huawei. |
| Sony | No |  |
| Apple | No |  |
| Qualcomm | Yes | See our response in Q17, so far it was not clear whether RAN2 already agreed to support RRC INACTIVE. If we make this clear, then we should inform them on how to configure RNA and how RAN paging works specially for moving cell case. |
| MediaTek | No |  |
| Lenovo, Motorola Mobility | No | No need. |
| Intel | No |  |
| LG | Yes | It seems the RAN3 agreement is not really clear whether RAN3 already supports inactive state. It would be better to confirm it. |
| Xiaomi | No |  |
| Samsung | No |  |
| vivo | No |  |
| CATT | No |  |
| NEC | No |  |
| Turkcell | No |  |
| ZTE | No |  |
| ER | No |  |

Summary: 20 companies joined the discussion and only 2 companies prefer to send an LS to RAN3.

No LS should be sent and no proposal is formulated.

## Preventing non-NTN capable UEs from accessing an NTN cell

In the previous meeting, RAN4 has agreed to introduce new frequency bands for NR NTN due to the different frequency allocations for different services based on the ITU radio regulation. However, there may be frequency overlapping between TN and NTN bands.

Several contributions mention preventing non-NTN capable UEs from accessing an NTN cell [4][7].

First, RAN2 needs to decide there is need to figure out a solution or it can be guaranteed by implementation. It is mentioned in [4] that: If it can be guaranteed that NTN will always be deployed in NTN specific bands and that no TN band will ever be defined/ signaled as overlapping band (i.e., in frequencyBandList), then the freqBandIndicatorNR in SIB1 is enough to prevent a TN only UE to try and access a NTN cell. Otherwise a mechanism is needed.

If RAN2 identifies a need for solution, please note that legacy UEs cannot understand the new IEs introduced in Rel-17, so backward compatibility need to be ensured. Candidate solutions include:

- For non-NTN capable UEs, cellReservedForOtherUse IE and cellReservedForFutureUse-r16 IE in SIB1 can be set true. For NTN capable UEs, cellReservedForOtherUse IE and cellReservedForFutureUse-r16 IE should be ignored, and a new IE should be introduced in SIB1, e.g., cellReservedForFutureUse-r17. [4]

Q19: Do you think a mechanism is needed to prevent non-NTN capable UEs from accessing an NTN cell?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Ask RAN4 | If it can be guaranteed that NTN will always be deployed in NTN specific bands and that no TN band will ever be defined/ signaled as overlapping band (i.e., in frequencyBandList), then the freqBandIndicatorNR in SIB1 is enough to prevent a TN only UE to try and access a NTN cell. Otherwise a mechanism is needed. Therefore, we should ask RAN4 whether it can be guaranteed that no TN band will ever be defined/ signaled as overlapping band with NTN bands. |
| Thales | Ask RAN4 |  |
| OPPO |  | Agree with Huawei this is related to NTN operating in dedicated bands. Better to clarify in RAN4 first. |
| Google | FFS | We can come back to this issue when the situation becomes clearer in RAN4. |
| Nokia | No | At least not in Rel-17. We assume the UE can determine whether a cell is NTN or TN (e.g. by the existence of NTN SIBx), so the UEs which are not capable of NTN, shall not be admitted by the NW to access such cells. |
| Sony | No | We think similar LS might be discussed for IoT NTN |
| Apple | No | We assume UE is aware whether a cell is NTN cell or not (based on carrier freq., SIB information etc.). |
| Qualcomm | Yes | RAN4 already agreed the NTN bands overlap with TN band in the frequency range. So who is responsible the satellite frequency moving all around the world will not be overlapped with TN network operators on each country? We do not think it is RAN4 to confirm regulation of TN and NTN bands deployment. Instead of running out of time, it is about a just one single bit in SIB1 which will resolve issue. We think it is not too much work to have one barring bit in SIB1 for new UEs. |
| MediaTek | No | Not needed in Rel-17. |
| Lenovo, Motorola Mobility | Ask RAN4 |  |
| Intel | Yes | For a NTN cell, cellBarred in MIB should be used to block TN UEs, but NTN UE should ignore this indication. |
| Xiaomi | No | We think UE can recognize the network type based on frequency or SIB1, and non-NTN capable UEs can’t access the NTN network since it can’t acquire the NTN specific SIB for UL synchronization. |
| Samsung | No | Network implementation/configuration can guarantee non-NTN capable UEs will not accessing an NTN cell. |
| vivo | Maybe Yes | If a mechanism is not introduced to prevent non-NTN capable UEs from accessing an NTN cell, for the UE with only TN capability, it may try to camp on the cell of the NTN and try to obtain normal service. But in this case, the normal registration of the UE will be rejected by the network, and the UE needs to reselect to another cell to camp and try to register again, resulting in much delay to obtain normal service. |
| CATT |  | Agree with Huawei |
| Turkcell | Ask RAN4 |  |
| ZTE | Yes |  |
| ER | yes | Barring bit in SIB1 would only help with new UEs, right? Legacy UEs need MIB barring which NTN UEs can ignore. |

Summary:18 companies joined the discussion. 7 companies prefer to ask RAN4, 6 companies think no mechanism is needed and 4 companies think a mechanism to prevent legacy UEs is needed.

Proposal 17: (7/18) Send an LS to ask RAN4 whether it can be guaranteed that no TN band will ever be defined/ signaled as overlapping band with NTN bands.

## RRC_IDLE/RRC_INACTIVE SMTC

In RAN2 #116-e, it was agreed that UE-based solution for SMTC adjustments in NTN is supported for IDLE/INACTIVE UEs. FFS how does the UE perform the necessary shifts in SMTC.

Regarding how the UE performs the necessary shifts in SMTC, contributions of RAN2 #116bis-e provide the following candidates:

- RRC_IDLE/RRC_INACTIVE UEs can measure SSBs of neighbouring cells by adjusting the SMTC configuration in system information based on its location and satellite ephemeris. [4]

- Network assisted solution, can be 1) The network associates each SMTC in the SIB with the parameter(s) to assist UEs to apply/adjust/shift the SMTC. The parameter(s) can be drifting rate, validity timer associated with SMTC, start/end time of SMTC etc. 2) The network may provide a reference time in the system information to indicate from which time point the associated SMTC started taking effect. [18]

- UE chooses to monitor only the most relevant/suitable SMTCs based on e.g. satellite positions in relation to the UE; another possibility is UE uses timesharing between monitoring of different subsets of the broadcast SMTCs (e.g. monitor SMTC1 and SMTC2 during a first time period, then switch to SMTC3 and SMTC4 during the subsequent time period, and then switch back to SMTC1 and SMTC2). [27]

Q20: Regarding UE-based solution for SMTC adjustments, which of the following do you prefer?

- Option 1: UE autonomously adjust the SMTCs based on location and ephemeris;

- Option 2: UE uses timesharing between monitoring of different subsets of the broadcast SMTCs;

- Option 3: Assistance information is broadcast by the network.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 |  |
| Thales | Option 1 |  |
| OPPO | Option 1 + option 3 | Location and ephemeris may not be sufficient. Network still needs to provide neigbhor cell’s feeder link delay so that UE can not how to adjust the SMTCs. |
| Google | Option 3 | Completely relying on UE itself to adjust the SMTC setting is not always feasible for idle/inactive UEs (as UE may not always have the ephemeris information regarding the neighbouring cells), and may consume UE’s power in a way that is not desirable for idle/inactive UEs. A timer or time stamp associated with the SMTC can also prevent the UE from performing the measurement based on inappropriate SMTC settings. A drifting rate that is used to adjust/shift the time offset of SMTC based on the elapsed time can be also broadcasted by the network. |
| Nokia | Option 3 | NW provides additional information on the time shifts/threshold the UE can apply autonomously to the SMTC time window it measures. |
| Sony | Option 1/3 |  |
| Apple | Option 1 + 3 |  |
| Qualcomm | Option 1 + 3 | Assistance information on time drifting rate parameters are also needed. |
| MediaTek | Option 1 + Option 3 |  |
| Lenovo, Motorola Mobility | Option 1 |  |
| Intel | Option 1 |  |
| LG | No | We think such SMTC adjustment is not needed in idle/inactive because the propagation delay between UEs at cell center and cell edge do not different much. So if the network provides long enough SMTC, the UE does not needed to adjust the SMTC. Even it will increase the UE power consumption. |
| Xiaomi | Option 1 |  |
| Samsung | Option 1, 3 |  |
| vivo | Option 1+option 3 | UE can automatically adjust SMTC according to the delay difference between satellites. Furthermore, neighbour cell’s feeder link delay is needed to calculate the delay difference. |
| CATT | Option 1 |  |
| NEC | No strong opinion between option1 and 3 | Depends on the overhead of assistance information |
| Turkcell | Option 1, Option 3 |  |
| ZTE | Option 1 |  |
| ER |  | If option 1 it cannot be applied to gaps. SMTC is assistance info, question is does UE need more info than SMTC and if UE is required to measure outside on its own. |

Summary: 20 companies joined the discussion. 16 companies support Option 1, and 11 companies support Option 3. No support for Option2.

Proposal 18: (16/20) Regarding UE-based solution for SMTC adjustments, UE autonomously adjust the SMTCs based on location and ephemeris.

It is also raised in [18] that the previous agreements regarding the maximum number of SMTCs supported per frequency for RRC_CONNECTED UEs are also applicable to RRC_IDLE/RRC_INACTIVE Ues (i.e., there can be up to four SMTCs configured in one SIB).

Q21: How many SMTCs can be broadcast per frequency at the most?

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No strong view, 4 is ok |  |
| Thales | A priori 4 is ok (being also discussed at RAN4) |  |
| OPPO | Same as legacy | For UE-based SMTC adjustment, NW does not need to broadcast more SMTCs than legacy. |
| Google | 4 | To align the agreements made for connected Ues. |
| Nokia | 4 is OK, but 2 should be enough |  |
| Sony | 4 |  |
| Apple | 2 | Network cannot assume that a UE in idle mode supports 4 SMTCs in RRC connected |
| Qualcomm |  | If ephemeris and assistance information is provided, it is not necessary for IDLE mode. |
| MediaTek | 2 | Already agreed as 2 in Connected mode (RAN2 #116-e). Idle mode should not support more than be more than 2. |
| Lenovo, Motorola Mobility | 4 |  |
| Intel | 4 | Same as connected mode |
| Xiaomi | 2 |  |
| Samsung |  | RAN2-116e agreement can be applied to RRC_IDLE/RRC_INACTIVE that the network can configure up to 2 SMTCs in parallel and the UE uses all of them, FFS whether this (UE support for 2 SMTCs) requires a UE capability, a UE can optionally indicate support for 4 SMTCs. |
| vivo |  | We have a similar understanding as OPPO. If RAN2 agrees that UE autonomously adjusts the SMTCs based on location and ephemeris in RRC_IDLE/INACTIVE, only one SMTC per frequency is already sufficient, and the rest is left to the autonomous adjustment by the UE itself. |
| CATT | 4 | Same with the connected mode. |
| NEC | No strong opinion , 4 is ok |  |
| Turkcell | 4 |  |
| ZTE | Same as legacy | Since the intention of SIB2/3/4 is to provide cell reselection information while UE would continuously searching for the strongest cell via reselection progress, there seems to be no need to provide a full picture of the all the neighbour satellite or cells via the system information of one cell. UE reselect from cell to cell, acquires system information from each and would draw a picture by itself. Thus we understand one SMTC per frequency would be sufficient for IDLE/INACTIVE measurements. |

Summary: 18 companies joined the discussion, 11 companies think 4 SMTCs are ok.

Proposal 19: (11/18) At most 4 SMTCs can be broadcast per frequency.

## Other

[16] mentions the support of discontinuous coverage. The rapporteur’s view is that discontinuous coverage is not in the scope of this WI, so the discussion can be postponed to future releases.

[23] proposes “Assuming TN and NTN carriers are distinct, legacy cell selection/reselection principles, including cell reselection priority configuration, is baseline for NTN-TN idle mode mobility.” The rapporteur’s view is that in RAN2 #111-e it was agreed that “The existing cell reselection priority configuration can be taken as a baseline in NTN”, therefore no further clarification is needed.

For the following miscellaneous proposals, due to lack of inputs from other companies, the rapporteur’s view is that the they can be postponed to future releases:

- [14] proposes that the distance between UE and different satellite should be uniformed, for instance, dived the distance between UE and satellite into the satellite coverage radius, and the results can be defined as the distance for cell reselection.

- [24] proposes that RAN2 can assume the entire ephemeris is always available on the UE. Besides, the paper brings some enhancements to ephemeris provisioning (e.g. a SIBfast and a SIBslow; NAS signalling for slowly changing ephemeris and RRC signalling for rapidly changing ephemeris).

- [25] proposes to introduce a E speed threshold SPUE and time duration Tlowmobility are configured for “low mobility” criterion.

- [26] proposes that RAN2 discusses enhancements to signalling of TN neighbouring frequencies/cells in an NTN cell, and that gNB provides information with which UEs can identify they are in an NTN-only zone.

- [27] proposes that RAN2 discusses whether current size of RMSI is still ok or if scheduling of RMSI should be done such that it provides updated info while reducing total overhead.

- [29] proposes that state vector and orbital parameters of ephemeris can be individually configured to be enabled or disabled, and that we should use different SIB trigger duration for slowly and quickly varying parameters of position and velocity state vectors or orbital ones.

# Conclusion – First round

For easy agreement

Proposal 1: (19/19) A new NTN-specific SIB is introduced (SIBx).

Proposal 2: Introduce the following serving cell information to the corresponding SIB:

1) (20/20) Ephemeris to (18/20) SIBx;

2) (20/20) common TA parameters to (18/20) SIBx;

3) (20/20) validity duration for UL sync information to (18/20) SIBx;

4) (20/20) t-Service to (15/20) SIBx;

5) (20/20) cell reference location to (17/20) SIBx;

6) (18/20) Epoch time to (14/20) SIBx. FFS the details of Epoch time.

Proposal 11: (20/20) For quasi-earth fixed cell, same as legacy, UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time.

Proposal 16: (20/20) RRC_INACTIVE mode is supported for NTN.

For further discussion

Proposal 3: Regarding the update of UL synchronisation information, Option 1 is supported (17/20), FFS for Option 2 (13/20):

- Option 1: Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.

- Option 2: The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires.

Proposal 4: Introduce the following neighbour cell information to the corresponding SIB:

1) (17/20) DL polarization to (12/20) SIBx;

2) (14/20) reference location to (6/20) SIBx;

3) (12/20) ephemeris to (6/20) SIBx.

Proposal 5: (12/20) The information of the upcoming cell (e.g., frequency and PCI) is broadcast.

Proposal 6: (15/19) Location information can be used to determine when to start measurement.

Proposal 7: If proposal 6 is agreed, agree the following (15/15):

UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

Proposal 8: If proposal 6 is agreed, agree the following (15/15):

Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

Proposal 9: Discuss which option to adopt for location-based reselection:

-	Option 1: only neighbour cells with distance shorter than a threshold will be considered during cell reselection; (10/20)

-	Option 1b: exclude neighbour cells too far away i.e., distance longer than a threshold will not be considered during cell reselection; (5/20)

-	Option 2: distance based ranking is used together with legacy R criteria. (3/20)

Proposal 10: (12/20) No enhancement is introduced for measurement/reselection based on time/location information for moving cell scenarios in Rel-17.

Proposal 12: (15/20) Before the stop-time based measurements are triggered, the UE measurements follow Legacy behaviour (i.e., based on Srxlev/Squal) and there is no measurement relaxation.

Proposal 13: (16/20) Cell stop time is not applied to cell ranking in determining the target cell for reselection.

Proposal 14: (11/20) Time-based and location-based reselection can be configured simultaneously. FFS UE behaviour when configured together.

Proposal 15: (13/20) TN prioritization over NTN is left to NW implementation in Rel-17.

Proposal 17: (7/18) Send an LS to ask RAN4 whether it can be guaranteed that no TN band will ever be defined/ signaled as overlapping band with NTN bands.

Proposal 18: (16/20) Regarding UE-based solution for SMTC adjustments, UE autonomously adjust the SMTCs based on location and ephemeris.

Proposal 19: (11/18) At most 4 SMTCs can be broadcast per frequency.

# Discussion – Second round

## New issues from Tuesday online

In the Phase 1 discussion, the following proposals have achieved majority support (if not consensus), but some good discussion points were raised during online discussion on 18th Jan which have not been fully discussed in the offline. In the Phase 2 discussion, we will first try to solve the new issues from Tuesday online and then deal with the remaining proposals from first round.

Proposal 1: (19/19) A new NTN-specific SIB is introduced (SIBx), scheduled by SIB1.

- Agreed

Proposal 2: Introduce the following serving cell information to the corresponding SIBx:

1) (20/20) Ephemeris to (18/20) SIBx;

2) (20/20) common TA parameters to (18/20) SIBx;

3) (20/20) validity duration for UL sync information to (18/20) SIBx;

4) (20/20) t-Service to (15/20) SIBx;

5) (20/20) cell reference location to (17/20) SIBx;

6) (18/20) Epoch time to (14/20) SIBx. FFS the details of Epoch time.

-	Intel thinks we don't need the FFS on the Epoch time as it is already clear from RAN1

- Agreed in principle. However we will send a LS to RAN1 asking whether some parameters might be sent more frequently

Proposal 11: (20/20) For quasi-earth fixed cell, same as legacy, UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time.

- Agreed

Proposal 16: (20/20) RRC_INACTIVE mode is supported for NTN.

- Agreed.

- Continue the discussion on whether to inform other groups of this agreement

The new discussion points include:

1) Whether one NTN-specific SIB is enough. For instance, one NTN-specific SIB carrying the minimum information required for NTN access (NTN MSI), and another NTN-specific SIB for other NTN information; another grouping is based on periodicity requirements of the parameters.

2) Whether the NTN-specific information needs to be acquired soon (i.e., like SIB1 in TN network).

3) It is reminded that we had this agreement in RAN2 #113-e: “RAN2 thinks that a UE needs to know whether the network is a TN or NTN no later than SIB1 reception”, so it needs to be discussed how this is ensured.

The suggested way forward by the rapporteur is that,

1) Check if companies agree with the above intentions, if yes:

a) Among the information identified to be broadcast in 1st round offline, identify the MSI for NTN access;

b) Check whether we include it in legacy SIB1 or an NTN-specific SIB1 or it is acceptable to include it in the SIBx scheduled by SIB1 (may need RAN1 help);

2) Discuss how to guarantee the UE knows the NW type no later than SIB1.

Q1: The intention of the original P1 is to introduce a new NTN-specific SIB (SIBx), scheduled by SIB1. Combined with P2, the SIBx contains 1) Ephemeris, 2) common TA parameters, 3) validity duration for UL sync information, 4) t-Service, 5) cell reference location and 6) epoch time.

Concerning the above understanding, do you think there should be further enhancement?

Please indicate your preference among:

- Option 1: Stick to the original P1 and P2. The NTN information 1~6 as listed above are included in SIBx which is scheduled by SIB1

  - Option 1a: Some of the 1-6 NTN information are included in SIBX while some are included in the existing SIB.

- Option 2: Have more than one NTN specific SIBs for the NTN information 1~6

  - Option 2a: Group NTN information into MSI (minimum information for NTN access) and other SI;

  - Option 2b: Group NTN information into a SIBfast and a SIBslow. The 2 SIBs have different broadcasting/updating frequency.

Note that even if you choose Option 1, it does not mean we cannot add other information to SIB1 (e.g., 1-bit TN/NTN indicator). The above question is only for the information listed in the original P2.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option1 | We are not sure of the gain brought by separate SIBs. The most important information for NTN access is the information for UL sync, including ephemeris, common TA, and validity duration. These information need to be put in the same SIB (along with the epoch time), and take up the most overhead. Compared with the information for UL sync, other information like t-Service and reference location won’t take up much overhead, and we are not convinced whether there is significant gain of putting them into a separate SIB. |
| ZTE | Option 1 or option 1a | - From signalling overhead perspective, we have provided analysis on the required signalling overhead of all the NTN specific parameters, either introduced by RAN1 or RAN2, in R2-2201003 and found that 426~702 bits are required. It would be possible to include all these parameters into one single SIBX. This option 1 would be acceptable to us. - Another issue we need to consider is the update frequency of different NTN information. The ephemeris, common TA parameters and validity duration for UL sync information would update frequently for LEO case to meet the required accuracy of pre-compensation. The epoch time is an absolute time, it should seldom change actually but it is also useful for UL sync. The t-service and cell reference location would update less frequently as they are mainly used as assistance information for cell reselection and do not require high accuracy. From this perspective, we are thinking about option 1a by grouping 1)-6) into frequently update ones and less frequently update ones and have them in SIBX and the existing SIB1/2 separately. For example: SIBX (update more frequently): The ephemeris, common TA parameters, validity duration for UL sync information and epoch time for UL sync. SIB2: t-Service and cell reference location as these information are for cell reselection. |
| Ericsson | Option 2b |  |
| LG | Option 1 | We do not see any necessity to introduce separate NTN-specific SIBs. |
| Transsion | Option 1 | Those parameters are cell related common information that can be contained within a specific SIB and they are relative fixed for quasi-earth fixed cell. |
| MediaTek | Option 1 | Agreed with Huawei that the gain brought by separate SIBs is not sure. |
| Qualcomm | Option 1 | For now we can go with option 1. If RAN1 confirms some parameters are more frequently changed, then we can consider moving some to other SIBs. |
| Apple | Option 2b or Option 1a (less preferred) | We think there is benefit in grouping information based on how often they are broadcast. |
| Xiaomi | Option 1 | . |
| Google | Option 1a | One NTN-specific SIB is sufficient. For those parameters that are needed for accessing the cell (e.g., common TA and epoch time), we can consider to place them in SIB1 instead of in the NTN-specific SIB. |
| OPPO | Option 1 |  |
| CATT | Option1 or Option 1a | We think put these parameters into SIBx can work, and can clearly divide the system information of NTN and TN. But if the overhead of air interface is considered, we are also fine with put t-Service and reference location into SIB2, while other parameters in SIBx. |
| Spreadtrum | Option 1 | Same as Huawei. |
| vivo | Option 1 | The NTN information 1~6 as listed above can be divided into two categories: one includes cell reselection related information (i.e., 4) t-Service and 5) reference location); the other is related to UL synchronization related information (i.e., 1) Ephemeris, 2) common TA parameters, 3) validity duration for UL sync information, and 6) epoch time). For UL synchronization related information, since these are NTN-specific information and will take up a lot of bits, this information should be placed in the NTN-specific SIBx obviously. For cell reselection related information, t-Service is defined as an absolute time value that takes up 39 bits according to RRC running CR; both serving cell reference location and neighbour cell(s) reference location may be needed for location based cell reselection criteria. Considering the impact on the size of the existing SIBs, putting t-Service and reference location in NTN-specific SIBx is also preferred. Moreover, for simplicity, it is not necessary to distinguish NTN specific information by the updating frequency. |
| ITRI | Option 2b | For example, the validity period of an active ephemeris is considered to be much longer than that of a common TA parameter. It is not resource efficient to broadcast ephemeris as often as other frequently updated parameters. Therefore, we suggest to group NTN specific parameters regarding the updating frequency. |
| NEC | Option1 | It is not clear for us RAN2 which one is piratically more time critical or need broadcast more frequently and need to group into another SIB separately. |
| Nokia | Option 1 or Option 1a | We think SIB1 can schedule SIB_NTN. Some of the NTN parameters may not be broadcasted in SIB_NTN, but in the existing SIBs. Overall, no need to split information into multiple NTN related SIBs, as UEs doing new access would anyway need the full information. |
| Sony | Option 1a | We don’t have a strong preference but would like to avoid NTN UE access delay and any restriction on the scheduling of new SIB |
| Lenovo, Motorola Mobility | Option 1 | Agree with Huawei |
| Samsung | Option 1 | One NTN-specific SIBx is enough. |
| Turkcell | Option 2b |  |
| Sequans | Option 1 | Agree with Huawei. We are open to revisit if difference in broadcasting periodicity justifies using different SIBs. |

Summary: 22 companies expressed their views. 16 companies support Option 1 (including 3 companies who can accept both Option 1 and Option 1a). 4 companies prefer Option 2b and 2 companies want Option 1a only.

The rapporteur’s suggestion is that, the original P1 and P2 can be taken as a starting point.

Proposal 20: (16/22) Stick to the original P1 and P2. The NTN information listed in P2 are included in SIBx which is scheduled by SIB1.

Q2: If your answer to Q1 is Option 2 (either 2a or 2b), please indicate your preference among Option 1~4. If your answer to Q1 is Option 1, you can also comment this question by choosing Option 5.

- Option 1: legacy SIB1 + a new SIBx scheduled by legacy SIB1

- Option 2: an NTN specific SIB1 + a new SIBx scheduled by the NTN specific SIB1

- Option 3: two new NTN specific SIBs, namely SIBx and SIBy, both scheduled by legacy SIB1

- Option 4: send an LS to RAN1 (please indicate your suggestion of the question to RAN1)

- Option 5: none of the above.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 5 | Option 1 brings too much overhead to SIB1 (e.g., ephemeris can occupy 17/18 bytes). Option 2 brings considerable spec impact, and it’s better to be clarified by the proponent company which information in the legacy SIB1 is not needed in the NTN specific SIB1. Option 3: as indicated in Q1, the most important information for NTN access is the information for UL sync, including ephemeris, common TA, and validity duration. These information need to be put in the same SIB (along with the epoch time), and take up the most overhead. Compared with the information for UL sync, other information like t-Service and reference location won’t take up much overhead, and we are not convinced whether there is significant gain of putting them into a separate SIB. Option 4: regarding the frequency of the parameters update, we think it is controlled by the validity timer (at least from UE perspective, the UE only needs to check the SI when the timer expires). We don’t think there’s need to ask RAN1 to check the update frequency of each parameter. The only question we consider useful is to have RAN1 check if there’s any latency requirement, but even that is not necessary if RAN2 decides no enhancement to the original P1/P2. |
| Ericsson | Option4 | We should know which parameters are time critical and needed for initial access. E.g. UE could have obtained ephemeris from SIBx/y while camping but then gets paged or UE initiated call and should connect to network. How long it takes the UE to wait to receive SIBs that provide fresh enough info? Other option is of course not to pay attention to this in Rel-17 and just make simple design. |
| LG | Option 5 |  |
| Transsion | Option 5 |  |
| MediaTek | Option 5 |  |
| Qualcomm | Option 4 | Ok to ask RAN1 whether some parameters are more frequently changing, then we can discuss whether existing SIB2 to SIBX scheduling can meet the latency requirement. |
| Apple | Option 3 | Open to ask RAN1 |
| Xiaomi | Option 5 |  |
| Google | Option 5 |  |
| OPPO | Option 5 |  |
| Spreadtrum | Option 5 |  |
| ITRI | Option 4 | Agree to send LS to RAN1 for their view on how frequent the listed parameters would change, and whether the up-to-date value of each listed parameters is critical for initial access. |
| NEC | Option 5 but | Open to ask RAN1, or company can raise any concern in RAN1 directly. |
| Lenovo, Motorola Mobility | Option 5 |  |
| Samsung | Option 5 |  |
| Turkcell | Option 4 |  |
| Sequans | Option 4 | We are fine to ask. |

Summary: 17 companies expressed their views. 11 companies select Option 5 (none), 1 company prefers Option 3. 5 companies choose Option 4 (asking RAN1) but there are also 2 companies open to ask RAN1 even though they raised hands for other options.

There seems to be majority view for no action, but 7 companies are ok with asking RAN1 and it is not sure how many companies are strongly against asking RAN1.

The rapporteur’s suggestion is that, as a compromise, we can tell RAN1 that from RAN2 perspective an NTN-specific SIBx scheduled by SIB1 is enough (i.e., the content of P20), but we ask RAN1 if they foresee any problem with it.

Proposal 21: Ask RAN1 if they foresee any problem (e.g., due to latency requirement) with proposal 20.

In RAN2 #113-e it was agreed that:

Agreements:

- RAN2 thinks that a UE needs to know whether the network is a TN or NTN no later than SIB1 reception

Among the contributions submitted to RAN2 #116bis-e, there is one proposal on serving cell type and one proposal on neighbour cell type, but neither of them gained the majority support during 1st round offline:

- Cell type indication (NGSO or GSO) [7] (Offline result: Yes:4, No:16)

- Network type (TN/NTN) and the PLMN ID of neighbour cell are broadcasted. [5] (Offline result: Yes:5, No:14, FFS:1)

Companies are invited to provide views on how to guarantee the UE knows the NW type no later than SIB1 reception. In the rapporteur’s understanding, the “SIB1” in the agreement refers to the legacy SIB1.

Q3: how to make the UE knows the NW type no later than SIB1 reception:

- Option 1: Add NW type (TN/NTN) into legacy SIB1

- Option 2: Implicit indication

- Option 3: Other, please clarify the details.

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option2 | We think Option 2 is enough, the UE can determine the cell is an NTN cell through e.g., presence to TA report enable/disable indication, NTN specific bands, SIB1 scheduling SIBx. The SIB1 of the NTN cell will schedule the NTN-specific SIBx to facilitate UEs’ access, so UE can tell know it is an NTN cell no later than SIB1. |
| ZTE | Option 2 | Since we have agreed to have NTN specific SIBX, the scheduling info of SIBX would be available in SIB1 and UE would know the serving cell is a NTN cell. |
| ER | Option 2 | SIB1 should contain something that tells UE about cell being NTN. E.g. what is mentioned by HW. |
| LG | Option 2 | Based on the ephemeris information of neighbour cells, the UE can know which neighbour cells are NTN cells. By the way, then can UE assume that any detected neighbour cell is not NTN cell? The serving cell may not broadcast a neighbour NTN cell but the UE detects it. |
| Transsion | Option 2 | Option 2 should be sufficient, because NTN specific. As the agreement had been made on Tuesday’s online meeting “A new NTN-specific SIB is introduced (SIBx), scheduled by SIB1” which can implicit indicate this is NTN network. For NTN capable UE it can tell if this cell belong to NTN cell for this scheduling information. |
| MediaTek | Option 2 |  |
| Qualcomm | Option 1 | We think main question is how legacy UEs understand this is NTN cell? These UEs never know what is NTN. A new indication is needed but perhaps for barring purpose. What if TN cell also broadcasts NTN specific SIBX as assistance for TN vs NTN mobility? Is this already excluded now? |
| Apple | Option 2 | We don’t see an issue even if TN cell broadcasts an NTN specific SIBX since the UE should figure out which cell the ephemeris information corresponds to. Additionally TA enable/disable indication as noted by Huawei also provides cell type information. |
| Xiaomi | Option 2 | At least the scheduling information of SIBx in SIB1 can indicate the network type. |
| Google | Option 2 | If UE detects a certain NTN-specific parameter or the availability of the NTN-specific SIB (i.e., SIBx) in SIB1, it knows this cell is an NTN cell. |
| OPPO | Option 2 | It can be deduced from the presence/absence of SIBx’s scheduling information in SIB1. |
| CATT | Option2, with some comments | We agree with HW that TN and NTN can distinguish by the scheduling information of SIBx. However, for the cell type, we wonder if it is also necessary to distinguish between GSO and NGSO, earth moving cell and earth fixed cell, no later than SIB1. And these information may also have some impacts on the procedure of cell (re-)selection. So, can GSO/NGSO and earth moving/fixed cell be distinguished implicitly by the existing frequency or PLMN information in SIB1? |
| Spreadtrum | Option 2 | It can be deduced based on the presence of SIB x in scheduling information. |
| vivo | Option 1 | We prefer a clear NW type indication placed in SIB1. But we can follow the majority’s view on this issue. |
| ITRI | Option 2 | UE could determine NTN from TN by the scheduling information contained in SIB1 for NTN-specific SIBx has been agreed. |
| NEC | Option 2 | As of now it. implicate way is enough. |
| Nokia | Option 2 | SIB1 will schedule SIBx. This shall tell it is the NTN cell. |
| Sony | Option 2 |  |
| Lenovo, Motorola Mobility | Option 2 | If SIB1 schedules SIBx, the cell is an NTN cell. |
| Samsung | Option 2 |  |
| Turkcell | Option 2 |  |
| Sequans | Option 2 |  |

Summary: 22 companies expressed their views, 22 of them think implicit indication is enough (Option 2) and 2 companies prefer an explicit NW type indication (Option 1).

Regarding the question raise by Qualcomm (how the legacy UEs understand this is NTN cell), the rapporteur’s view is that this is related to another issue (barring legacy UEs). On the question raised by CATT (distinguishing between GSO and NGSO), this is included in Q2 of phase 1 discussion but there is not enough support.

Proposal 22: (20/22) UE can know the NW type implicitly no later than SIB1 reception, there is no explicit NW type indication in SIB1.

Another issue is whether to inform RAN3 about RAN2 decision on RRC_INACTIVE.

Proposal 16: (20/20) RRC_INACTIVE mode is supported for NTN.

- Agreed.

- Continue the discussion on whether to inform other groups of this agreement

It was already discussed in Q18 in phase 1, and 18 out of 20 companies think an LS is not needed.

LS not needed: HW, Thales, OPPO, Google, Nokia, Sony, Apple, MediaTek, Lenovo, Intel, Xiaomi, Samsung, vivo, CATT, NEC, Turkcell, ZTE, ER

LS is needed: QC, LG

Since there is still concern during the online discussion, the proponent company is expected to provide more details and try to convince other companies. The rapporteur’s view is that RAN3 already supports RRC_INACTVIE given the following agreement from RAN3 #113-e:

Apart from RAN Paging, RRC_INACTIVE is kept unspecified from a RAN3 perspective, i.e. no further RRC_INACTIVE related specification text is added, neither on stage 2 nor stage 3 level.

Whether to further specify RRC_INACTIVE related impacts depends on whether RAN3 wants enhancements to RRC_INACTIVE compared with TN. But apparently RAN3 does not exclude RRC_INACTIVE.

Q4: Do you agree to send an LS to RAN3 about the conclusion of Q17?

Please indicate below only if you think there is need to send an LS (if not indicated, the rapporteur assumes you hold the same view with phase 1 discussion).

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | No need to send LS just to inform our agreement. RAN3 colleagues are able to see it. |
| Ericsson | no | But if strong support we are ok to inform them shortly |
| LG | Yes | We do not think the RAN3’s agreement implies inactive state is accepted. Does “Apart from RAN Paging” and “no further RRC_INACTIVE related specification text” mean that inactive state is agreed? |
| Transsion | No |  |
| MediaTek | No |  |
| Qualcomm |  | Ok to go with majority. |
| Apple | No | No need for LS. |
| Xiaomi | No |  |
| Google | No | Same view as Rapporteur that RAN3 already stated in their agreement regarding the support of RRC_INACTIVE. |
| OPPO | No |  |
| CATT |  | We are ok to send LS to RAN3. But we also agree with the rapporteur’s view that RAN3 already supports RRC_INACTVIE. |
| Spreadtrum | No | No need for LS. |
| vivo | No | Agree with ZTE. |
| NEC | No | But no strong objection to send it neither. Since there is no harm to inform |
| Nokia | No | Why would the LS be needed, in the light of already quoted RAN3 agreement? |
| Sony | No |  |
| Lenovo, Motorola Mobility | No |  |
| Samsung | No | Also fine if majority think it’s better to send. |
| Turkcell | No |  |
| Sequans |  | No strong view, fine with the majority view. |

Summary: Only 1 company has strong concern of informing RAN3. Since the rapporteur assumes companies not replying this question hold the same view with phase 1 discussion (as in the question statement), the supporters of not sending the LS should be 18 (from phase 1) plus 3 (Transsion, Spreadtrum Sequans).

Proposal 23: (21/22) No LS is sent to RAN3 on the support of RRC_INACTIVE.

## Confirmation of first round proposals

For the rest of the proposals in phase 1 summary, the rapporteur does not expect repeated discussions given the limited time left. Please indicate bellow only if you have changed your mind or if you have not voiced in the phase 1 discussion.

| Company | Which Proposal | Preference in Phase 1 | New preference in Phase 2 | Comments |
| --- | --- | --- | --- | --- |
| Qualcomm | Proposal 4 |  | Add “Common TA parameter” | As per Proposal 18, it will be up to UE to adjust SMTC and for this common TA information of the neighbor would also be needed. |
| Google | Proposal 18 |  |  | Given that there are quite a number of companies (at least 10) supporting Option 3, we would like to ask the rapporteur to consider adding an FFS at the end of the proposal (i.e., Proposal 18: (16/20) Regarding UE-based solution for SMTC adjustments, UE autonomously adjust the SMTCs based on location and ephemeris. FFS on whether any NW assistance information is required for the UE-based solution for SMTC adjustments) |
| OPPO | Proposal 4 |  | Add “feeder link’s delay” | It is used for UE-based SMTC adjustment. |
| OPPO | Proposal 18 |  |  | Same view as Google. Propose to revise as: Regarding UE-based solution for SMTC adjustments, UE autonomously adjust the SMTCs based on at least UE location and ephemeris. FFS on other NW assistance information. |
| Samsung | Proposal 14 |  | FFS whether time-based and location-based reselection can be configured simultaneously | Since it’s nearly half vs half (11:9), we think it’s better to put FFS. |
| Samsung | Proposal 18 |  | Add FFS on other NW assistance information | Agree with Google and OPPO on Proposal 18. |

Summary: 2 companies wants to broadcast neighbour cell’s common TA parameters, which can be used for SMTC adjustment related to Proposal 18, the rapporteur thinks this discussion can be postponed until there is agreement on Proposal 18.

On Proposal 18, 3 companies want to add an FFS for NW assistance information, the rapporteur will modify Proposal 18 accordingly and see if companies can accept.

There is also a comment on putting Proposal 14 as an FFS. The rapporteur’s view is that having an FFS proposal will not make any progress (even if the proposal is agreed, it is still FFS), so it’s better to keep Proposal 14 as it is and see which way to go (either simultaneous configuration is allowed or not allowed).

Based on the above, the original proposal 18 is revised as follows (and the supporting ratio is removed). With this, the rapporteur assumes proponents for assistance information are happy and proposal 18 can be categorized as “for easy agreement”.

Proposal 18: Regarding UE-based solution for SMTC adjustments, UE autonomously adjust the SMTCs based on location and ephemeris. FFS whether NW assistance information is provided.

Proposals from Phase 1 conclusion

Proposal 3: Regarding the update of UL synchronisation information, Option 1 is supported (17/20), FFS for Option 2 (13/20):

- Option 1: Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.

- Option 2: The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires.

Proposal 4: Introduce the following neighbour cell information to the corresponding SIB:

1) (17/20) DL polarization to (12/20) SIBx;

2) (14/20) reference location to (6/20) SIBx;

3) (12/20) ephemeris to (6/20) SIBx.

Proposal 5: (12/20) The information of the upcoming cell (e.g., frequency and PCI) is broadcast.

Proposal 6: (15/19) Location information can be used to determine when to start measurement.

Proposal 7: If proposal 6 is agreed, agree the following (15/15):

UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

Proposal 8: If proposal 6 is agreed, agree the following (15/15):

Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

Proposal 9: Discuss which option to adopt for location-based reselection:

-	Option 1: only neighbour cells with distance shorter than a threshold will be considered during cell reselection; (10/20)

-	Option 1b: exclude neighbour cells too far away i.e., distance longer than a threshold will not be considered during cell reselection; (5/20)

-	Option 2: distance based ranking is used together with legacy R criteria. (3/20)

Proposal 10: (12/20) No enhancement is introduced for measurement/reselection based on time/location information for moving cell scenarios in Rel-17.

Proposal 12: (15/20) Before the stop-time based measurements are triggered, the UE measurements follow Legacy behaviour (i.e., based on Srxlev/Squal) and there is no measurement relaxation.

Proposal 13: (16/20) Cell stop time is not applied to cell ranking in determining the target cell for reselection.

Proposal 14: (11/20) Time-based and location-based reselection can be configured simultaneously. FFS UE behaviour when configured together.

Proposal 15: (13/20) TN prioritization over NTN is left to NW implementation in Rel-17.

Proposal 17: (7/18) Send an LS to ask RAN4 whether it can be guaranteed that no TN band will ever be defined/ signaled as overlapping band with NTN bands.

Proposal 18: (16/20) Regarding UE-based solution for SMTC adjustments, UE autonomously adjust the SMTCs based on location and ephemeris.

Proposal 19: (11/18) At most 4 SMTCs can be broadcast per frequency.

# Conclusion – Second round

For easy agreement

Proposal 20: (16/22) Stick to the original P1 and P2. The NTN information listed in P2 are included in SIBx which is scheduled by SIB1.

(revised) Proposal 18: Regarding UE-based solution for SMTC adjustments, UE autonomously adjust the SMTCs based on location and ephemeris. FFS whether NW assistance information is provided.

Proposal 22: (20/22) UE can know the NW type implicitly no later than SIB1 reception, there is no explicit NW type indication in SIB1.

Proposal 23: (21/22) No LS is sent to RAN3 on the support of RRC_INACTIVE.

For further discussion

Proposal 3: Regarding the update of UL synchronisation information, Option 1 is supported (17/20), FFS for Option 2 (13/20):

- Option 1: Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.

- Option 2: The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires.

Proposal 4: Introduce the following neighbour cell information to the corresponding SIB:

1) (17/20) DL polarization to (12/20) SIBx;

2) (14/20) reference location to (6/20) SIBx;

3) (12/20) ephemeris to (6/20) SIBx.

Proposal 5: (12/20) The information of the upcoming cell (e.g., frequency and PCI) is broadcast.

Proposal 6: (15/19) Location information can be used to determine when to start measurement.

Proposal 7: If proposal 6 is agreed, agree the following (15/15):

UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

Proposal 8: If proposal 6 is agreed, agree the following (15/15):

Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

Proposal 9: Discuss which option to adopt for location-based reselection:

-	Option 1: only neighbour cells with distance shorter than a threshold will be considered during cell reselection; (10/20)

-	Option 1b: exclude neighbour cells too far away i.e., distance longer than a threshold will not be considered during cell reselection; (5/20)

-	Option 2: distance based ranking is used together with legacy R criteria. (3/20)

Proposal 10: (12/20) No enhancement is introduced for measurement/reselection based on time/location information for moving cell scenarios in Rel-17.

Proposal 12: (15/20) Before the stop-time based measurements are triggered, the UE measurements follow Legacy behaviour (i.e., based on Srxlev/Squal) and there is no measurement relaxation.

Proposal 13: (16/20) Cell stop time is not applied to cell ranking in determining the target cell for reselection.

Proposal 14: (11/20) Time-based and location-based reselection can be configured simultaneously. FFS UE behaviour when configured together.

Proposal 15: (13/20) TN prioritization over NTN is left to NW implementation in Rel-17.

Proposal 17: (7/18) Send an LS to ask RAN4 whether it can be guaranteed that no TN band will ever be defined/ signaled as overlapping band with NTN bands.

Proposal 19: (11/18) At most 4 SMTCs can be broadcast per frequency.

Proposal 21: Ask RAN1 if they foresee any problem (e.g., due to latency requirement) with proposal 20.

# Discussion – Third round

As per the guidance from the chair, the scope of final round discussion is as follows:

Final scope: Continue the discussion on the remaining idle/inactive mode aspects and draft LS to RAN1 asking to check the agreements on SIBx.

## LS to RAN1

In the phase 2 discussion, according to companies’ feedback on Question 2, there seems to be majority view for no action, but 7 companies are ok with asking RAN1 and it is not sure how many companies are strongly against asking RAN1. The rapporteur’s suggestion was that, as a compromise, we can tell RAN1 that from RAN2 perspective an NTN-specific SIBx scheduled by SIB1 is enough (i.e., the content of P20), but we ask RAN1 if they foresee any problem with it. And Proposal 21 was drafted on this basis.

Proposal 21: Ask RAN1 if they foresee any problem (e.g., due to latency requirement) with proposal 20.

Since Proposal 20 is agreed via email, we can further progress on Proposal 21 and the LS to RAN1.

Q1: Do you agree with Proposal 21?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

A draft LS is provided in the “Phase 3 discussion” subfolder, with the following contents:

1. Overall Description: RAN2 has agreed to introduce a new NTN-specific SIB (SIBx) which is scheduled by SIB1. And at least the following serving cell information will be broadcast by SIBx: 1) Ephemeris; 2) Common TA parameters; 3) Validity duration for UL sync information; 4) t-Service (the timing information on when the serving cell is going to stop serving the area); 5) Cell reference location; 6) Epoch time. Note that 4) and 5) will only be broadcast by quasi-earth fixed cells not by moving cells, based on RAN2 agreements so far. Since some parameters also have RAN1 impact, RAN2 kindly asks if RAN1 foresees any problem (e.g., due to latency requirement) with the above agreements. 2. Actions: To RAN1 ACTION: RAN1 is kindly requested to take the above information into account and see if there is any problem with the RAN2 agreements.

Q2: Do you agree with the above contents in the LS to RAN1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

## Other issues

In this section, we attempt to achieve some more proposals for email agreements which have gained majority support in the first and second round of the offline discussion.

### SI update

In Phase 1 discussion, there was considerable support for Proposal 3 (at least for Option 1). Companies are invited to check if these two options can be agreed via email to spare some online time for other open issues.

Proposal 3: Regarding the update of UL synchronisation information, Option 1 is supported (17/20), FFS for Option 2 (13/20): - Option 1: Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure. - Option 2: The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires.

Q3: Do you agree with the following proposal?

Proposal 3 – Option 1: Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.

Note that the rapporteur assumes the supporters of Option 1 in Phase 1 discussion hold the same position by default.

Supporters for Option 1 (17): HW, Thales, Oppo, Google, Nokia, Sony, QC, MediaTek, Lenovo, Intel, LG, Xiaomi, Samsung, Vivo, CATT, NEC, ER

| Company | Yes/No | Comments |
| --- | --- | --- |

Q4: Do you agree with the following proposal?

Proposal 3 – Option 2: The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires.

Note that the rapporteur assumes the supporters of Option 2 in Phase 1 discussion hold the same position by default.

Supporters for Option 2 (14): HW, Thales, Nokia, Sony, QC, MediaTek, LG, Xiaomi, Samsung, Vivo, CATT, NEC, ZTE, ER

(Note that the original proposal 3 combines Option 1 and Option 2 together, and 13 companies support “Option 1 + Option 2”. Now the Q4 and Q3 are decoupled, and considering 1 company supports Option 2 but not Option 1 during Phase 1 discussion, the supporters of Option 2 rise from 13 to 14 here.)

| Company | Yes/No | Comments |
| --- | --- | --- |

### Location-based measurement

Proposal 6 of the phase 1 discussion gained majority support, and Proposal 7&8 can reach consensus if proposal 6 is agreed.

Proposal 6: (15/19) Location information can be used to determine when to start measurement. Proposal 7: If proposal 6 is agreed, agree the following (15/15): UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold). Proposal 8: If proposal 6 is agreed, agree the following (15/15): Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

Therefore, companies are invited to check if Proposal 6 can be accepted.

Q5: Do you agree with the following proposal?

Proposal 6: Location information can be used to determine when to start measurement.

Note that the rapporteur assumes the supporters of Proposal 6 in Phase 1 discussion hold the same position by default.

Supporters (15): HW, Google, Nokia, Sony, Apple, QC, Lenovo, Intel, LG, Xiaomi, Samsung, CATT, NEC, Turkcell, ER

| Company | Yes/No | Comments |
| --- | --- | --- |

Q6: Do you agree with the following proposals?

Proposal 7: If proposal 6 is agreed, agree the following: UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold). Proposal 8: If proposal 6 is agreed, agree the following: Location-based measurement initiation is only applied if the cell broadcasts location-related parameters (e.g. a threshold) and by implementation the UE has location information.

By Proposal 8, it means the UE is not required to acquire location information just for cell reselection purposes.

Note that the rapporteur assumes the supporters of Proposal 7 and Proposal 8 in Phase 1 discussion hold the same position by default.

Supporters (15): HW, Google, Nokia, Sony, Apple, QC, Lenovo, Intel, LG, Xiaomi, Samsung, CATT, NEC, Turkcell, ER

| Company | Yes/No | Comments |
| --- | --- | --- |

### Time-based measurement

Based on the above illustration, UE will start measurements during time interval 1 according to the agreement on t-Service (@ RAN2 #115-e: “For quasi-earth fixed cell, UE should start measurements on neighbour cells before the serving cell stops covering the current area.”), but the UE measurement behaviour in time interval 2 (before time interval 1) is unclear:

1)	Legacy behaviour (i.e., based on Srxlev/Squal), or

2)	Reduce measurements

During Offline discussion, most companies prefer Option 1, so the following Proposal was drafted:

Proposal 12: (15/20) Before the stop-time based measurements are triggered, the UE measurements follow Legacy behaviour (i.e., based on Srxlev/Squal) and there is no measurement relaxation.

The rapporteur’s understanding is that, the “start measurements” of the RAN2 #115-e agreement (as highlighted above) includes the measurements of all kinds of neighbour cells (i.e., intra/inter-frequency and inter-RAT neighbour cells).

Then, the motivation of the RAN2 #115-e agreement (as highlighted above) can be:

1) Make sure UE has found a target cell before the serving cell stops serving the area, i.e., in time interval 1, even if the serving cell has a good quality (Srxlev/Squal above a threshold), the UE still performs intra-frequency measurements and inter-frequency or inter-RAT measurements on equal/lower priority frequencies.

2) Save some UE power, i.e., when there is considerable remaining serving time, UE can reduce measurements (e.g., at least not measuring lower priority frequencies). If this is the correct understanding, UE should be able to reduce measurements in time Interval 2.

According to companies’ feedback (Question 13 in Phase 1), most companies think UE should follow legacy behaviour in time interval 2.

Therefore, to achieve a better understanding, the rapporteur recommends to revise the original Proposal 12 as follows and see if companies can accept it:

Q7: Do you agree with the following proposal?

(revised) Proposal 12: Before the stop-time based measurements are triggered, the UE measurements follow Legacy behaviour (i.e., based on Srxlev/Squal) and there is no measurement relaxation. The stop-time based measurement triggering aims to make sure UE finds a target cell before the serving cell stops serving the area, instead of saving UE power.

| Company | Yes/No | Comments |
| --- | --- | --- |

### Time-based reselection

In Phase 1 discussion, there was considerable support for Proposal 13. Companies are encouraged to check if this can be agreed without going online.

Proposal 13: (16/20) Cell stop time is not applied to cell ranking in determining the target cell for reselection.

Q8: Do you agree with the following proposal?

Proposal 13: Cell stop time is not applied to cell ranking in determining the target cell for reselection.

Note that the rapporteur assumes the supporters of Proposal 13 in Phase 1 discussion hold the same position by default.

Supporters (16): Oppo, Google, Nokia, Sony, Apple, QC, MediaTek, Lenovo, Intel, Xiaomi, Samsung, Vivo, CATT, NEC, Turkcell, ZTE

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | We think it is beneficial to apply cell stop time to cell ranking, so we stayed neutral in Phase 1 discussion. But we can accept the majority view. |

# Conclusion – Third round

To be completed

# Reference

- R2-2200215, Discussion on TN prioritization over NTN for idle mode, Intel Corporation

- R2-2200216, Discussion on enhancements to cell reselection, Intel Corporation

- R2-2200246, Discussion on NTN specific system information, OPPO

- R2-2200290, Discussion on idle mode aspects, Huawei, HiSilicon

- R2-2200342, System information to assist cell reselection, ITRI

- R2-2200378, Remaining issues on idle/inactive mode mobility, vivo

- R2-2200446, Cell type indication, Qualcomm Incorporated

- R2-2200447, IDLE mode measurements, Qualcomm Incorporated

- R2-2200621, Idle mode mobility for NTN-TN scenarios, MediaTek Inc.

- R2-2200630, Acquiring the ephemeris of neighbour cell, Spreadtrum Communications

- R2-2200650, Discussion on NTN Idle mode measurement and cell reselection	, Transsion Holdings

- R2-2200665, Remaining idle mode issues in NTN, LG Electronics Inc.

- R2-2200690, Further Discussion on the Leftover Issues of IDLE/INACTIVE, CATT

- R2-2200716, Discussion on RRC idle mode issues, Xiaomi

- R2-2200766, Ephemeris provision in system information for NTN, Lenovo, Motorola Mobility

- R2-2200767, Further discussion on idle mode mobility in NTN, Lenovo, Motorola Mobility

- R2-2200877, Further Considerations on Cell Re-selection, CMCC

- R2-2200933, SMTC Adjustment for Idle and Inactive UEs in NTN, Google Inc.

- R2-2201003, System information for NTN and idle mode mobility for intra-NTN and TN-NTN case	, ZTE corporation, Sanechips

- R2-2201079, On IDLE mode aspects in Rel-17 NTN, Nokia, Nokia Shanghai Bell

- R2-2201139, On Defining a New NTN-Specific SIB, MediaTek Inc.

- R2-2201165, Location-assisted cell reselection, InterDigital

- R2-2201179, NTN-TN idle mode mobility, Apple

- R2-2201180, NTN Ephemeris definition and signaling	Apple

- R2-2201195, Location-assisted cell reselection, NEC Telecom MODUS Ltd.

- R2-2201196, NTN to TN mobility in Idle or Inactive mode, NEC Telecom MODUS Ltd.

- R2-2201446, Idle mode aspects for NTN, Ericsson

- R2-2201580, Measurements and cell reselection, Samsung Research America

- R2-2201615, Discussion on system information enhancement for NR NTN, Turkcell, BT Plc, Deutsche Telekom, Aselsan