3GPP TSG-RAN WG2 Meeting #116is electronic	R2-2xxxxxx

Online Meeting, Jan 17th – 25th 2022

Agenda item:	8.2.3.1

Source:	CATT

Title:            Report of [AT116bis-e][224][DCCA] CPAC procedures from NW perspective (CATT)

WID/SID:	LTE_NR_DC_enh2-Core

Document for:	Discussion and Agreement

- 1	Introduction

This document is for summary of the following discussions:

- [AT116bis-e][224][DCCA] CPAC procedures from NW perspective (CATT)

Scope: Discuss the remaining details of CPAC procedures: 
A) For SN initiated CPC: 1) Is the indication of prepared PSCells always sent to S-SN, and in which procedure step? 2) What are the RAN2/RAN3 messages use for indicating a) accepted cells from MN to S-SN, b) updated configuration from S-SN to MN and c) RRCComplete from MN to S-SN 
B) For MN initiated CPAC: 1) Does MN provide separate list of proposed PSCells to T-SN? 2) Can T-SN pick different PSCells than those in the list?”

Intended outcome: Discussion summary in R2-2201704.

Deadline: Deadline 1

The document is structured as the following. Section 2 and 3 contain the discussions on some essential issues regarding SN-initiated CPC, and MN-initiated CPAC, respectively. Section 4 is for conclusions.

The participants are invited to leave their contact information in the following table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| J Huawei, HiSilicon | david.lecompte@huawei.com |
| CATT | E Erlin Zeng (erlin.zeng@catt.cn) |
| Lenovo, Motorola Mobility | Zhangcc16@lenovo.com |
| Google | Jing-Rong Hsieh (jinghsieh@google.com) |
| ddEEricsson | cicecilia.eklof@ericsson.com |
| F Futurewei | Jialinzou88@yahoo.com |
| cF |  |

- 2	SN-initiated CPC

- 2.1	Is the indication of prepared PSCells always sent to S-SN, and in which procedure step?

In the LS R2-2111323 [1], RAN2 has informed the following to RAN3

RAN2 has discussed SN initiated inter-SN CPC and has agreed on Solution 2, where the MN may inform the S-SN about the accepted/rejected candidate PSCell(s) and get a response from the S-SN including modifications of the UE configuration (e.g. measurement configuration) to be transmitted in the RRC Reconfiguration message including the CPC configurations to the UE. RAN2 assumes the MN decides, based on network implementation, whether to skip the second part of Solution 2 procedure. RAN2 has two understandings on the second part: a) MN not waiting for S-SN -> MN response or b) Both messages (i.e. MN-> S-SN and S->MN) being left out. RAN2 thinks MN can skip the second part of procedure in Solution 2 at least when T-SN acknowledges all candidate PSCells.

In order to conclude on this issue we first need to confirm companies’ views on the two understandings above. Email Rapporteur’s understanding is that

- with a) it means the indication of prepared PSCells is always sent to S-SN by the MN, but MN may not wait for S-SN -> MN response, before sending the RRC Reconfiguration message including the CPC configurations to the UE, while

- with b) it may be that MN does not send the indication of prepared PSCells to S-SN at all (which is b.1 in Q2), or MN skip such indication in the second part of the Solution 2 procedure but should still send the indication in some later steps in the procedure (which is b.2 in Q2).

With the above understanding, companies are invited to answer the following question firstly.

Question 1 Which understanding do you agree with, a) or b)?

| Company | Understanding a) or b) | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | b) (but if RAN3 prefers to add something, it is up to them) | We understand that: 1) the MN will always do the second part if one or more candidate target PSCells are not prepared by the source T-SN. 2) the MN will always send a message to the S-SN in order to confirm that the MN reconfigures the UE according to the initial S-SN request. If the S-SN receives 2) directly, it implies that all candidate PSCells were prepared by the T-SN, even if there is no explicit indication for that. Of course, RAN3 is free to add whatever explicit indication to RAN3 signalling, even though it is not necessary from RAN2 perspective. |
| Nokia | b) |  |
| CATT | a) | In RAN2#114e, the following working assumption is made. Working assumption (to clarify agreements 1-3 above) 4. S-SN is informed about which candidates were accepted/ rejected by T-SN.. Since anyway the S-SN should be informed of the candidate cells accepted by T-SN, we prefer MN always sends the indication of the prepared PSCells to S-SN before the CPC configuration sent to UE, which gives a simpler procedure, regardless of whether MN waits for the response from S-SN. |
| Lenovo, Motorola Mobility | b) | In our understanding, the 2nd step of solution 2 consists of SN modification request and SN modification request acknowledgement Xn messages. Even if both messages are skipped before sending the RRC reconfiguration message to UE, the MN will still inform the S-SN about the prepared PSCells by SN change confirm message after receiving the RRC reconfiguration complete message from UE. |
| Google | b) |  |
| Ericsson | b)-ish, but the information needs to be sent to the S-SN. | MN needs to send the information about prepared PSCells to S-SN, otherwise the S-SN cannot determine whether the SCG measConfig needs to be updated. That is the whole purpose of the procedure. In case all target cells were accepted and the UE was successfully configured, the information about prepared PSCells could possibly be omitted if it is described in the spec that this means that all PSCells were accepted. What is important is that the S-SN gets the information to be able to make the decision about possible updating of the measConfig. |
| Futurewei | a) | It is prudent that MN always notify the T-SN decision to the S-SN. As it is the S-SN initiated CPC and configures the measurement at the S-SN, in any situation, S-SN should know the final decision of T-SN. S-SN can decide whether to issue response to the MN and MN need not wait the S-SN response before issuing the CPC command. |

Then if you agree with understanding b), please further answer the following question.

Question 2 If you agree with understanding b), which option do you prefer,

- Option b.1 MN may not send the indication of prepared PSCells to S-SN, i.e.,

- Option b.2 MN should always send the indication of prepared PSCells to S-SN (i.e., if MN skips the indication to S-SN before sending the RRC Reconfiguration message including the CPC configurations to the UE, it should send the indication to S-SN in some later step in the procedure)

And if you agree with Option b.2 please clarify in this case in which step MN sends the indication to the S-SN.

| Company | Which option | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | b.1 or b.2 | b.1 but if RAN3 wants to have some indication in some message, it is up to RAN3. |
| Nokia | b.2 | In case MN decides to send CPAC configuration immediately to the UE, then MN informs the source SN about the list of PSCells that are prepared using SN Change Confirm message. |
| Lenovo, Motorola Mobility | b.2 | Even if both messages are skipped before sending the RRC reconfiguration message to UE, the MN will still inform the S-SN about the prepared PSCells by SN change confirm message after receiving the RRC reconfiguration complete message from UE. |
| Google | b.1 | Can leave it to RAN3 |
| Ericsson | b.2 | As mentioned above, the S-SN needs to have the information about prepared PSCells, otherwise it cannot decide whether to update the measConfig. The information about prepared PSCells can be sent in SN Change Confirm, both if the MN decided to configure the UE and if the MN decided not to configure the UE. |

Summary:

TBD

- 2.2	What are the RAN2/RAN3 messages used for the procedure

Generally, Email rapporteur’s understanding has been that the message interaction between MN, S-SN and T-SN used for the procedure of SN initiated inter-SN CPC is within the scope of the RAN3, and RAN3 is discussing on these messages currently. But there are some company contributions on the topic. More details can be found in the following.

### 2.2.1 Messages used for indicating the accepted cells from MN to S-SN

Based on the contribution of [10], SN change confirm message is used to provide the indication of accepted candidate PSCells to S-SN. Another option is to leave this to RAN3.

The following question is for the case if MN sends the indication to S-SN before sending the RRC Reconfiguration message including the CPC configurations to the UE.

Question 3: Which option do you prefer regarding the message used for indicating the accepted PSCells from MN to S-SN, before sending the RRC Reconfiguration message including the CPC configurations to the UE?

- Option 1: leave it to RAN3;

- Option 2: SN change confirm message is used to provide the accepted candidate PSCells to S-SN[10]

| Company | Which option | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | option 1 | RAN3 should choose what fits with RAN3 procedures. |
| Nokia | Option 1 | We are OK to leave it up to RAN3 to decide. We have a corresponding paper submitted to RAN3 (R3-220150), where we suggest to use optionally MN-initiated Modification procedure to contact S-SN . Then, SN Change Confirm is sent after receiving the complete message from the UE. |
| CATT | Option 1 | As far as we know, RAN3 is currently discussing on these messages. |
| Lenovo, Motorola Mobility | Option 1 | We understand RAN3 is discussing this issue this meeting. We can simply wait for RAN3’s conclusion. We prefer to use SN modification request message though. |
| Google | Option 1 |  |
| Ericsson | Option 2 | It is fine to leave it to RAN3, but good to send in SN Change Confirm in order to close the SN Change procedure and avoid nested procedures. The SN Change procedure may otherwise be open for a long time. If the SN Change Confirm is used, the accepted PSCells only needs to be added in one inter-node message (SN Change Confirm), otherwise it needs to be added in multiple messages (SN Change Confirm and possibly Modification Request or a new message). |
| Futurewei | Option 1 |  |

Summary:

TBD

The following question is for the case if MN sends the indication to S-SN after sending the RRC Reconfiguration message including the CPC configurations to the UE (i.e., as in option b.2 in section 2.1).

Question 4: Which option do you prefer regarding the message used for indicating the accepted PSCells from MN to S-SN, after sending the RRC Reconfiguration message including the CPC configurations to the UE (if option b.2 is supported)?

- Option 1: leave it to RAN3;

- Option 2: SN change confirm message is used to provide the accepted candidate PSCells to S-SN

| Company | Which option | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 |  |
| Nokia | Option 1 | But we think SN Change Confirm for this scenario is a good choice (which shall be made in RAN3). |
| Lenovo, Motorola Mobility | Option 1 | We understand RAN3 is discussing this issue this meeting. We can simply wait for RAN3’s conclusion. We prefer to use SN change confirm message though. |
| Google | Option 1 |  |
| Ericsson | Option 2 | SN Change Confirm, but we could let RAN3 decide. |
| Futurewei | Option 1 |  |

Summary:

TBD

### 2.2.2 Message used for S-SN to provide the updated configuration

Similarly there are possible options and views are invited on the message used for S-SN to provide the updated configuration.

Question 5: Which option do you prefer regarding the message used for S-SN to provide the updated configuration? Please specify the reasons or comments if any.

- Option 1: leave it to RAN3

- Option 2: SN initiated modification procedure is used for S-SN to provide the updated configuration [10];

| Company | Which option | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 |  |
| Nokia | Option 1 | But we think it should be MN initiated modification procedure (the response to request message sent by the MN). |
| CATT | Option 1 | As far as we know, RAN3 is currently discussing on these messages. |
| Lenovo, Motorola Mobility | Option 1 | We understand RAN3 is discussing this issue this meeting. We can simply wait for RAN3’s conclusion. We prefer to use SN modification request acknowledge message though. |
| Google | Option 1 |  |
| Ericsson | Option 2 | In this way nested procedures between MN and S-SN can be avoided. |
| Futurewei | Option 1 |  |

### 2.2.3 Message used for providing the RRCReconfigurationComplete message from MN to S-SN

Based on the stage 2 running CR [2], it is FFS how to provide the RRCReconfigurationComplete message to S-SN in step 8a, i.e., after the UE has sent the RRCReconfigurationComplete message to indicate the embedded RRCReconfiguration message of S-SN has been applied;

In legacy, the SN Reconfiguration Complete message is used by MN to provide the embedded RRCReconfigurationComplete message to S-SN. Thus, one option is to use the same message in CPAC. There are also other options as listed below.

Question 6: Which option do you prefer regarding the message used for providing the RRCReconfigurationComplete message from MN to S-SN?

- Option 1: leave it to RAN3

- Option 2: legacy SN Reconfiguration Complete message can be reused to provide the embedded RRCReconfigurationComplete message to S-SN;

- Option 3: to use the SN Change Confirm message to indicate the RRCReconfigurationComplete message in case understanding 2 of Question 1 is taken, and the MN decide to skip the second part of the Solution 2 procedure [10]

| Company | Which option | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 |  |
| Nokia | Option 1 (and Option 3) | RAN3 shall decide it is Option 3. |
| CATT | Option 1 or 2 | In legacy PSCell modification procedure, the SN Reconfiguration Complete message is used by the MN to indicate the embedded RRCReconfigurationComplete message received from UE to S-SN. We think it is straightforward to reuse the inter-node message for CPAC scenario. Furthermore, since the inter-node message is within the scope of RAN3, we are fine to leave it to RAN3. |
| Lenovo, Motorola Mobility | Option 1 | Since it has dependency on the question 1 to 5, we can wait for RAN3 conclusion first. We prefer option 2 though. |
| Google | Option 1 |  |
| Ericsson | Option 3 | SN Change Confirm, but fine to leave to RAN3. |
| Futurewei | Option 1 |  |

Summary:

TBD

- 3	MN-initiated CPAC

The following open issues on MN initiated CPAC should be addressed:

1) Does MN provide separate list of proposed PSCells to T-SN, and

2) Can T-SN pick different PSCells than those in the list?”

## 3.1 Does MN provide separate list of proposed PSCells to T-SN

On the issue of how to provide the candidate cells recommended by MN to T-SN, the following are proposed by [8] [10] [11] [13]:

- Option 1: Use legacy candidate cell information (candidateCellInfoListMN) to provide the candidate cells recommended by MN to T-SN [8] [10];

- Option 2: Introduce a new list to include the candidate cells recommended by MN for MN initiated CPAC [11];

- Option 3: Reuse the list of proposed PSCell candidates within CG-ConfigInfo introduced for SN initiated inter-SN CPC to provide the candidate cells recommended by MN to T-SN [13];

Question 7: Which option (i.e., Option 1/2/3) above do you prefer?

| Company | Which option | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 3 | We prefer to align with SN-initiated CPC, so that T-SN behaviour is the same. |
| Nokia | Option 1 or 3 | We think either the existing IE shall be used (Option 1) or it should be considered if the same IE as defined for SN-initiated CPC can be adopted (Option 3). |
| CATT | Option 3 | Currently, there is already an IE within CG-ConfigInfo, which is introduced to contain the recommended candidate cells information for SN initiated inter-SN CPC. Furthermore, the SN initiated inter-SN CPC and MN initiated inter-SN CPAC will not be configured simultaneously within one SN Addition Request message. Thus, we think it is straightforward to reuse the IE for MN initiated inter-SN CPC. |
| Lenovo, Motorola Mobility | Option 1 | In case of CPA and MN initiated CPC, it is possible to reuse the legacy candidateCellInfoListMN IE. |
| Google | Option 1 or 3 |  |
| Ericsson | Option 1 | We propose to reuse the legacy behaviour in this sense. Here it makes more sense that the T-SN decides on the candidates than the MN. |
| Futurewei | Option 3 | Align with CPC but differentiated from other purposes. |

Summary:

TBD

## 3.2 Can T-SN pick different PSCells than those in the list

On the issue of whether T-SN can pick target cells not proposed by the MN, the following are based on [8] [10][13]:

- Option 1: Only the cells within the list recommended by MN can be chosen by T-SN [8] [13];

- Option 2: Cells not belonging to the list recommended by MN can be chosen by T-SN [10];

Question 8: Which option (i.e., Option 1/2) above do you prefer?

| Company | Which option | Comments if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 | We prefer to align with SN-initiated CPC, in order to make specification maintenance easier. |
| Nokia | Option 2 | The same as we have argued for SN-initiated CPC: we see no reason why the ultimate list shall be prepared by the source/initiating node (S-SN or MN in this case). Ultimately it should be T-SN’s right to choose appropriate cells (if measurement results are provided) for CPAC. |
| CATT | Option 1 | For SN initiated inter-SN CPC, RAN2 has agreed that the T-SN cannot pick candidate PSCells outside the cells recommended by S-SN. The same principle can also apply to the case of MN initiated CPAC. Furthermore, if different principles apply for MN initiated CPAC and SN initiated CPAC, RAN2/RAN3 also need to further discuss how to indicate the type of CPAC to T-SN, since currently the T-SN has no idea whether the conditional reconfiguration is used for MN initiated CPAC or SN initiated CPC. So we do not see the benefit of Option 2, but it leads to more complexity. |
| Lenovo, Motorola Mobility | Option 2 | We don’t see the reason why not following the legacy principle. |
| Google | Option 1 | Prefer aligning with SN-initiated CPC |
| Ericsson | Option 2 | Agree with Nokia. The T-SN should have the right to decide as the cells belong to the T-SN. This is also inline with legacy behaviour. The MN may not have enough knowledge to take the decision. |
| Futurewei | Option 1 | We share the same view as Huawei and CATT. RAN2 already decided that the T-SN cannot pick candidate PSCells outside the cells recommended by S-SN. For the same reason, T-SN only has the local information to reject the candidates suggested by MN but T-SN don’t have sufficient global information to add good new candidates for MN. The efforts of option 2 is not worth the benefit. |

Summary:

TBD

- 4	Conclusion

TBD

- 5	References

- R2-2111323   LS on SN initiated inter-SN CPC

- R2-2201646	Introduction of CPA and inter-SN CPC	CATT	draftCR	Rel-17	37.340	16.8.0	B	LTE_NR_DC_enh2-Core	R2-2111640

- R2-2200361	Discussion on the CG-CandidateList	Google Inc.	discussion	LTE_NR_DC_enh2-Core

- R2-2200362	Support modification and cancellation of C-PSCells in the CG-CandidateList	Google Inc.	draftCR	Rel-17	38.331	16.7.0	B	LTE_NR_DC_enh2-Core

- R2-2200589	Discussion on CPAC procedures from NW perspective	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

- R2-2200613	Skip response from S-SN in SN-initiated CPC	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

- R2-2200773	Discussion on CPAC from NW perspective	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2200923	Remaining issues on CPAC procedure	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

- R2-2200924	Further consideration on CPAC procedure	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

- R2-2201000	CPAC network procedures	Ericsson	discussion	Rel-17	LTE_NR_DC_enh2-Core

- R2-2201072	CPAC procedures from network perspective	Qualcomm Incorporated	discussion	Rel-17

- R2-2201081	Solving open issues for Rel-17 CPAC	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

- R2-2201250	Discussion on CPAC from NW perspective	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

- R2-2201305	CPAC procedure for SCG update	Samsung R&D Institute UK	discussion