3GPP TSG-RAN WG2 Meeting #116bis-e                                 R2-22XXXXX

E-meeting, January 17th – 25th, 2022

Agenda item:	8.6.3

Source: 		Interdigital

Title: 	Summary of Rel-17 SDT contributions on Control Plane Common Aspects

Document for:	Discussion

# Introduction

This paper aims to provide a summary of the contributions to R2#116bis-e, AI 8.6.3 on control plane common aspects (contributions: see References section).

NOTE: Since the aim of this summary is to provide the chairman with a list of the topics that seems to have the support of the majority of the companies, some proposals for easy agreements and some important proposals for the sake of progress, aspects that are raised by only one or two companies are not included in this summary.

# Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| InterDigital (Rapporteur) | Keiichi Kubota | keiichi.kubota@interdigital.com |
| Samsung | Anil Agiwal | anilag@samsung.com |
| Apple | Fangli XU | fangli_xu@apple.com |
| Fujitsu | Ohta, Yoshiaki | ohta.yoshiaki@fujitsu.com |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |
| Sharp | Chongming Zhang | Chongming.zhang@cn.sharp-world.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |

# Non-SDT data arrival indication

The issues of the non-SDT data arrival indication were discussed in [1] [6] [8] [10] [14], [17] [23] [24] and [27].

[1] is comprehensively covering the open issues of the non-SDT data arrival indication and so the issues are not handled in this summary.

# SDT Failure Detection Timer handling

The issue of SDT failure detection timer (SFDT) handling was discussed in [4] [6] [9] [10] [12] [13] [14] [15] [19] [23] and [28].

There are 2 options.

Option 1: SFDT has an extended duration to accommodate subsequent SDT.

Option 2: SFDT is restarted upon (re)transmission or reception of small data.

In [4] [10] [13] [14] [19] [23] [28], Option 1 was proposed, while in [6] [12] [15], Option 2 was proposed.

In addition, [9] is the summary of email discussion [Post113-e][503], which discussed SFDT handling. At that time, 13 companies supported Option 1 and 12 companies supported Option 2.

Detailed comments for Option 1:

- Network can always fall back to CONN if UE timer running out

- Time bound nature avoids potential issues of long SDT duration

  - Smart gNB implementation an configure proper timer value

- SDT mechanism is short by definition, so one timer duration is sufficient

- A simple solution

Detailed comments for Option 2:

- Advantages:

  - Avoids SDT being time bound as timer for subsequent SDT is variable and cannot be predicted in advance

  - Provides flexibility

  - Allows failures to be detected more quickly

- Disadvantages:

  - Think new timer is maintained by RRC. If adopted frequent interaction between RRC and lower layers are required to (re)start timer

    - To avoid issue, timer could be in MAC layer

  - Timer may have issues for CG-SDT considering can have very long period

Summary: Option 1 has majority this time. Thus, it makes sense to go for the majority view (i.e. Option 1) for the sake of time.

Q1: Do you agree that the SDT failure detection timer has an extended duration to accommodate subsequent SDT (i.e. not restarted upon user activity)? If not, please provide the technical reasoning, which hasn’t yet been raised before.

| Answers to Q1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes | Our proposal in Tdoc is Option 2, but we are fine to go for majority’s preference option 1 and let RRC to control this SDT failure timer. |
| Fujitsu | Yes | As proposed in [4]. |
| ZTE | Yes | We think this is a simple solution that can work for both CG and RA-SDT and we can adopt this. |
| LGE | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |

# Paging

The issue of the paging was discussed in [2] [5] [6] [11] [14] [15] P12 [21] and [28].

There are two options.

Option 1: UE does not monitor any paging occasion for paging while in SDT operation.

Option 2: UE monitors paging while in SDT operation.

In [2] [6] [11] [14] [15] [21] [28], Option 1 is proposed.

In [5], Option 2 is proposed.

The proponents of Option 1 basically claim the following reasoning.

- Network is aware of UE and Network can resume connection if needed during an SDT session.

The proponent of Option 2 claims that UL procedure and DL procedure are separate procedure and so UE should monitor paging even if UE performs SDT operation.

Summary: Obviously majority support Option 1 and it technically makes sense that UE doesn’t monitor paging while UE is reachable from the network so we should go for Option 1.

Q2: Do you agree that UE doesn’t need to monitor paging occasion for paging while in SDT operation? If not, please state the technical argument, which hasn’t yet been raised before.

| Answers to Q2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments if the answer is “No” |
| Samsung | See comments | In our view, UE needs to monitor for paging until the response to initial UL transmission is received from network during the SDT procedure. During subsequent data transmission phase paging monitoring is not required. If UE does not monitor paging until the response to initial UL transmission is received from network during the SDT procedure, the issue is that - when the network sends the CN paging, UE will not be able to receive it until SDT procedure is completed (i.e. SDT timer expires). Network sending CN paging means that there is state mismatch and in this case network will not be able receive initial UL transmission in CG resource as it is not monitoring the CG resource. As a result network has to keep (re-)transmitting CN paging via one or more cell(s) unnecessarily and UE will keep transmitting in CG resource unnecessarily until SDT timer expires. |
| Apple | Yes | Paging reception during the initial SDT phase is up to UE implementation, since in legacy it’s up to UE implementation to receive paging during the RACH for IDLE/INACTIVE UE. Paging reception during the subsequent SDT phase is not needed, since NW is aware of the UE and can resume the connection if needed. |
| Fujitsu | Yes | As proposed in [4]. Our proposal is that UE behaviour can be same as legacy. |
| ZTE | Yes | We agree with the rapporteur. Since anyway the UE is initiating a resume procedure for SDT, monitoring the UE specific paging (which anyway would trigger resume) would not be needed. |
| LGE | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |

# SI update

The issue of SI update was discussed in [2] [3] [6] [11] [14] [19] [28].

SI change notification and emergency notification monitoring

In RAN2 #114, it was agreed that

- UE needs to at least monitor SI change notification and emergency notification during SDT procedure.

- FFS for other cases.

[2] tried to address the FFS.

In [2], the following proposals were proposed to clarify UE performs monitoring the notifications in the same way as the one in RRC_INACTIVE state.

Proposal 1: During the SDT procedure (i.e. while SDT timer is running), UE monitors SI change indication in any paging occasion at least once per modification period.

Proposal 2: During the SDT procedure (i.e. while SDT timer is running), ETWS or CMAS capable UEs monitors PWS notification in any paging occasion at least once every defaultPagingCycle.

[6] implicitly proposed the same by saying "UE behaves like for a legacy UE in RRC_INACTIVE when specification does not indicate different or SDT-specific operation (e.g. for measurements)".

Summary: Only one company tried to address the FFS but it looks reasonable to follow their proposals.

Q3: Do you agree the following proposals? If not, please provide the technical reasoning.

Proposal 1: During the SDT procedure (i.e. while SDT timer is running), UE monitors SI change indication in any paging occasion at least once per modification period.

Proposal 2: During the SDT procedure (i.e. while SDT timer is running), ETWS or CMAS capable UEs monitors PWS notification in any paging occasion at least once every defaultPagingCycle.

| Answers to Q3 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes | UE needs to at least monitor SI change notification and emergency notification during SDT procedure was already agreed. These proposals further clarify the POs/DRX cycles in which SI change notification and emergency notification are monitored during the SDT procedure. |
| Apple | No | The UE behavior can be same as legacy, i.e. monitor the indication/notification in its own paging occasion every DRX cycle. If the SDT is applicable for eDRX UE, UE’s own cycle may be longer than the default paging cycle. |
| Fujitsu | No | UE behaviour can be same as legacy. |
| ZTE | Yes | Even in connected mode UE monitors this. So, it seems we should not change this for SDT. |
| LGE | Yes |  |
| Sharp | Yes |  |
| Xiaomi | No | We can follow the legacy UE behavior for SI change notification and emergency notification monitoring. |

On-demand SI Request

In [14], it was proposed to select one of the following options:

- Option 1: Do not support on demand SI during SDT

- Option 2: Only support msg1 based on demand SI on the same carrier as the ongoing SDT

- Option 3: Use DedicatedSIBRequest for on demand SI (similar to RRC_CONNECTED)

[3] [19] proposed Option 1. The reasons are as follows:

- Before initiating the SDT procedure, UE would have acquired all essential SIBs needed for operating in a camped cell.

- The SI request may lead to UL carrier switching (UL carrier selected at the time of initiation of SDT can be different from UL carrier selected at the time of SI request) or BWP switching (in case SDT is performed on non-initial BWP) which interrupts the ongoing SDT procedure.

- for the approach of SI request for RRC_CONNECTED, it is not clear yet whether the DedicatedSIBRequest can be transmitted during SDT procedure. Moreover, it should also need to be discussed whether RRCReconfiguration message can be received in response to DedicatedSIBRequest during SDT procedure

[28] proposed either option 2 or 3 above.

[11] proposed Option 3 + a new DL RRC message, which carries the requested SIBs.

[11] justified the on-demand SI request during SDT procedure as follows:

The UE that is in the middle of an SDT session may need to request the network for delivery of on-demand SI. This can be useful in particular to the positioning UEs to be able to request posSIBs

Summary: There are split views. By considering the expected impacts mentioned above and the fact that SDT procedure won’t last long, it looks reasonable to go for Option 1: Do not support on demand SI during SDT.

Q4: Do you agree that on-demand SI request is not supported during SDT? If not, please provide the technical arguments.

| Answers to Q4 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes |  |
| Fujitsu | Yes or O2 | Option 2 can be acceptable as legacy. |
| ZTE | Yes | Considering that the SDT session is expected to be a short session, we think such optimization should not be considered. |
| LGE | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |

# RNA update

The issue of RNA update was discussed in [3] [6] [10] [11] [13] [14] [15] [18] and [28].

All the companies proposed not to perform RNA update during SDT procedure.

However, there are 2 different flavors how to avoid RNAU:

Option 1: T380 is stopped upon initiation of SDT procedure, T380 is restarted upon moving back to the legacy RRC_INACTIVE state i.e. upon reception of RRCRelease.

Option 2: T380 continues running upon SDT session initiation but RNA update is not initiated upon T380 expiry during SDT procedure.

In [3] [10] [13] [14] [15], Option 1 was proposed.

In [6] [11] [15] [28], Option 2 was proposed. Actually [6] had slightly different proposal from Option 2 but proposed:

Option 2’: T380 continues running upon SDT session initiation but RNA update is not initiated upon T380 expiry during SDT procedure unless SDT session initiation fails

The underlined text is the difference from Option 2.

In [19], it’s just proposed to avoid RNAU during SDT procedure, but it didn’t describe any further details.

Summary: All companies proposed to avoid RNA update during SDT procedure but there are 2 ways: a) [5/9] T380 is stopped upon initiation of SDT procedure, T380 is restarted upon moving back to the legacy RRC_INACTIVE state i.e. upon reception of RRCRelease, b) [4/9] T380 continues running upon SDT session initiation but RNA update is not initiated upon T380 expiry during SDT procedure.

Q5: Do you agree that T380 is stopped upon initiation of SDT procedure, T380 is restarted upon moving back to the legacy RRC_INACTIVE state? If not, please provide the technical argument.

| Answers to Q5 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes |  |
| Fujitsu | Yes |  |
| ZTE | Yes | For restarting the T380, we think there will be no change needed (i.e. it will be started upon receiving RRCRelease with suspendconfig same as today). Some companies were wondering if T380 should be restarted if SDT session fails. However, according to current agreements, if SDT session fails, UE moves to IDLE mode. So, then, there should be no T380 anyway in this case. So, stopping T380 upon initiating SDT seems necessary and sufficient. |
| LGE | No | If we go for Option 1, there may be other proposals to stop/restart the running timer in a future. We think Option 2 is much simpler. |
| Sharp | No | We think T380 could keep running until expiry. When it expires, if SDT failure detection timer is running, RNAU is not allowed to be triggered. |
| Xiaomi | Yes |  |

# RRC signalling issues

The issue of RRC signalling issue was discussed in [6] [10] [11] [15] [25] [26] and [28].

Delta signalling

There is one FFS in the running RRC regarding to the delta signalling.

In [6], it was proposed:

- Proposal 6. Delta signaling is supported for the SDT related configuration. This delta signaling applies across different SDT sessions and when resuming the RRC connection (i.e. SDT related configuration is released when UE enters RRC_IDLE or when the network explicitly releases the SDT configuration).

Companies are invited to provide their views on the delta signalling.

Q6: Do you agree the following delta signalling proposal?

Proposal: Delta signalling is supported for the SDT related configuration. This delta signaling applies across different SDT sessions and when resuming the RRC connection (i.e. SDT related configuration is released when UE enters RRC_IDLE or when the network explicitly releases the SDT configuration).

| Answers to Q6 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | - | No strong view |
| Apple |  | No strong view |
| Fujitsu | Yes | See no reason to prohibit the delta signalling. |
| ZTE | No | No strong view but we are okay with this if majority support it. It is an optimization in the end and can be supported if it is straight forward. However, one concern is the mismatch between UE and NW caused by the expiration of CG-validity timer. In case the timer expired, the CG resource will be released. However, considering the timer is maintained in RRC layer, and the delay in RLC/MAC/PHY, there maybe some case that the timer status is mismatch between UE and NW. To avoid such mismatch, one potential solution is to have the same behaviour as the expiration of TAT that we only release the CG resource in MAC but keep the CG resource in RRC. |
| LGE | Yes |  |
| Samsung | - | No strong view |
| Xiaomi | Yes |  |

Transmissible RRC messages during SDT procedure

In [15] [25], it was proposed to discuss what RRC and NAS messages should be allowed for the SDT procedure in case SRB1 and/or SRB2 is/are configured for SDT.

[25] specified "The message candidates are ULInformationTransfer (including NAS message), UEAssistanceInformation and SidelinkUEInformationNR."

[6] explained that RAN1 LS R1-2102125 informed us "It is feasible from RAN1 perspective to use either a new common search space or a UE-specific search space, thus it can be up to RAN2 to make the decision. The configuration of CORESET will be further discussed, basically following the same design logic for search space" and so gNB should be able to configure the UE specific serach space prior to SDT procedure. Then in [6], it was proposed

Proposal 9.	To confirm that for SDT procedure, a UE only gets SDT related configuration/parameters via broadcast signaling (e.g., common search space and CORESET) or via RRCRelease msg. I.e., RRCReconfiguration message is never used during an ongoing SDT session.

Proposal 20. RRCReconfiguration and RRCReconfigurationComplete are not supported during an SDT session.

It looks very reasonable proposal and companies may be fine to agree it.

Q7: Do you agree with the following proposal?

Proposal: RRCReconfiguration and RRCReconfigurationComplete are not supported during an SDT session.

| Answers to Q7 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes |  |
| Fujitsu | Yes |  |
| ZTE | Yes | We don’t see a need from SDT perspective. But if other Wis need this then it can be discussed and agreed |
| LGE | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |

In [25], it was proposed:

Proposal 20: The NW can configure whether UL NAS/RRC transmission is allowed over SRB1 using SDT procedure

It seems controversial issue, which we need to address, and companies are invited to provide their views on this.

Q8: Do you agree the following proposal?

Proposal: The NW can configure whether UL NAS/RRC transmission is allowed over SRB1 using SDT procedure

| Answers to Q8 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | No | If network want UL NAS message to trigger SDT, SRB2 can be configured as SDT RB. If network does not want UL NAS message to trigger SDT, SRB2 can be configured as non SDT RB. SRB1 is always resumed upon SDT procedure initiation. There is no need to restrict transmission of RRC/NAS message over SRB1 during SDT. Any message which is triggered during SDT procedure in RRC_INACTIVE can be transmitted over SRB1. |
| Apple | No | NW can configure the SRB2 as the SDT-RB for the UL NAS/RRC transmission during the SDT procedure. |
| Fujitsu | No | NAS/RRC message can be transmitted over SRB2. |
| ZTE | No | SRB1 should be resumed, but NAS messages cannot be transmitted over SRB1 since SRB2 is established (this legacy behaviour should not be changed). Otherwise this will lead to a lot of other issues (such as the higher priority radio messages being blocked by lower priority NAS messages etc). We don’t allow this today for this exact reason. |
| LGE | No |  |
| Sharp | No |  |
| Xiaomi | No |  |

Furthermore, there are some other proposals with regard to the transmissible RRC messages.

In [10], it was proposed to allow UE assistant information over SRB1 during SDT procedure.

In [28], it was proposed to transmit ULInformationTransfer containing CP small data in the first UL message if SRB2 is configured for SDT.

Summary: Companies expected to clarify what RRC messages can be sent during SDT procedure. There is no clear majority for each RRC message and so RAN2 should discuss what RRC messages should be allowed to transmit during the SDT procedure.

Q9: Do you agree with the following proposals?

ULInformationTransfer (including NAS message) over SRB2 configured for SDT, UEAssistanceInformation and SidelinkUEInformationNR can be allowed to send during SDT procedure

| Answers to Q9 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | See comments | UEAssistanceInformation and SidelinkUEInformationNR are not triggered during RRC_INACTIVE. Since the SDT procedure takes place in RRC_INACTIVE, we do not see need to trigger these messages during SDT procedure. |
| Apple | See comments | ULInformationTransfer (including NAS message) and UEAssistanceInformation should be allowed. For the SidelinkUEInformationNR, the scenario is not clear to us, and we have no strong view. |
| Fujitsu | Yes | What UL messages are transmitted can be up to UE. |
| ZTE | Yes for ULInformationTransfer | For other messages, we don’t see a need for supporting over SRB1 during SDT. |
| LGE | Yes for ULInformationTransfer |  |
| Sharp | Yes for ULInformationTransfer |  |
| Xiaomi | Yes for ULInformationTransfer |  |

Response RRC messages to SDT access attempt

In [11] [26], it was proposed to allow RRCSetup to be transmitted in response to SDT access attempt so that RRC connection can be re-established from scratch.

In [15] [26], it was proposed to allow responding with RRCReject to RRCResumeRequest for SDT.

In [26], it was proposed that Network can respond with RRCRelease w/wo suspendConfig to RRCResumeRequest for SDT so that Network can refuse the SDT procedure and let UE move back to RRC_INACTIVE or RRC_IDLE respectively

Summary: RRCReject part has already been covered by [1] and so it should be handled as part of [1]. The questions remain for RRCSetup and RRCRelease.

The legacy UE behaviour for each response message would be a baseline.

Q10: Do you agree with the following proposals? Please explain the expected UE behaviour upon reception of the response message if you think any special handling is required.

Proposal: Network can respond with RRCSetup to RRCResumeRequest for SDT.

| Answers to Q10 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes | UE behaviour upon receiving the RRCSetup message is same as legacy. |
| Fujitsu | Yes | UE behaviour can be same as legacy. |
| ZTE | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |

Q11: Do you agree with the following proposal? Please explain the expected UE behaviour upon reception of the response message if you think any special handling is required.

Proposal: Network can respond with RRCRelease with/without suspendConfig to RRCResumeRequest for SDT.

| Answers to Q11 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes | UE behaviour is same as legacy. |
| Fujitsu | Yes | UE behaviour can be same as legacy. |
| ZTE | Yes |  |
| Sharp | Yes |  |
| Xiaomi | Yes |  |

RRC response messages to the RRC message for non-SDT data arrival indication

[1] includes the discussion whether RRCReject can be sent as a response to the RRC message for non-SDT data arrival indication or not. Furthermore, in [1], some companies proposed to support the responses, RRCReject, RRCSetup and RRCRelease to the RRC message for non-SDT data arrival indication.

The legacy UE behaviour for each response message would be a baseline.

Q12: Do you agree with the following proposals? Please explain the expected UE behaviour upon reception of the response message if you think any special handling is required.

Proposal: Network can respond with RRCReject to the RRC message for non-SDT data arrival indication.

| Answers to Q12 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes | UE behaviour upon receiving RRCReject shall suspend all the resumed SDT-RBs. |
| Fujitsu | Yes | UE behaviour can be same as legacy. |
| ZTE | Unclear | There can be RRCReject, but the response message is for RRCResumeRequest sent by the UE not for the non-SDT data arrival indication. It is enough to capture that the network can respond with RRCReject to RRCResumeRequest for SDT (i.e. similar to the above proposals). We propose to capture a single agreement as follows: The network can respond with the following for RRCResumeRequest for SDT: - RRCResume - RRCSetup - RRCRelease with and without suspendConfig - RRCReject Questions 12 to 14 are not needed since the response is not for the non-SDT data arrival indication. Of course, CCCH camp think that the non-SDT arrival is another RRCResumeRequest, but the DCCH camp think that the non-SDT data arrival is just an indication from UE to the network and the network response is counted as the response to the original RRCResumeRequest message. #Since we have not concluded CCCH vs DCCH issue, we don’t need to discuss this secondary response here. |
| Sharp | Yes |  |
| Xiaomi |  | Agree with ZTE. |

Q13: Do you agree with the following proposals? Please explain the expected UE behaviour upon reception of the response message if you think any special handling is required.

Proposal: Network can respond with RRCSetup to the RRC message for non-SDT data arrival indication.

| Answers to Q13 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes |  |
| Fujitsu | Yes | It can be up to NW implementation. |
| ZTE | - | See above |
| Sharp | Yes |  |
| Xiaomi |  | See above |

Q14: Do you agree with the following proposals? Please explain the expected UE behaviour upon reception of the response message if you think any special handling is required.

Proposal: Network can respond with RRCRelease to the RRC message for non-SDT data arrival indication.

| Answers to Q14 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Samsung | Yes |  |
| Apple | Yes |  |
| Fujitsu | Yes | It can be up to NW implementation. |
| ZTE |  | See above |
| Sharp | Yes |  |
| Xiaomi |  | See above |

# References

| [1] | R2-2200026 | Report of [Post116-e][510][SDT] CCCH and DCCH (Nokia) | Nokia, Nokia Shanghai Bell |
| --- | --- | --- | --- |
| [2] | R2-2200201 | Paging Monitoring during SDT procedure | Samsung Electronics Co., Ltd |
| [3] | R2-2200202 | RNA update and SI request handling during SDT procedure | Samsung Electronics Co., Ltd |
| [4] | R2-2200312 | Handling of SDTF detection timer | Fujitsu |
| [5] | R2-2200313 | RAN paging reception and response during SDT | Fujitsu |
| [6] | R2-2200505 | Control Plane leftover issues on SDT procedure | Intel Corporation |
| [7] | R2-2200574 | Remaining control plane aspects of SDT | NEC |
| [8] | R2-2200663 | Emergency call in the middle of SDT operation | InterDigital, Europe, Ltd. Rakuten Mobile Inc. |
| [9] | R2-2200696 | Handling of SDT failure timer | InterDigital, Europe, Ltd. |
| [10] | R2-2200727 | Remaining issues on CP aspects of SDT | Qualcomm Incorporated |
| [11] | R2-2200811 | Control plane common aspects for SDT | Huawei, HiSilicon |
| [12] | R2-2200919 | Subsequent SDT failure detection timer | Sony |
| [13] | R2-2200986 | CP aspects for SDT | Ericsson |
| [14] | R2-2201029 | CP open issues for SDT | ZTE corporation, Sanechips |
| [15] | R2-2201125 | Control plane aspects of SDT | Apple |
| [16] | R2-2201126 | Power Saving for SDT | Apple |
| [17] | R2-2201174 | DCCH-based indication of non-SDT data arrival | Intel Corporation, ZTE Corporation, Sanechips, Samsung, Xiaomi, MediaTek, Radisys and Reliance JIO, Qualcomm, CMCC, OPPO, Lenovo, Sony, Apple |
| [18] | R2-2201217 | RNA Update during SDT | Sharp |
| [19] | R2-2201358 | Remaining issues on Control Plane Aspects for SDT | LG Electronics Inc. |
| [20] | R2-2201376 | Clarification on the area configured for ROHC continuity | Xiaomi Communications |
| [21] | R2-2201377 | Paging reception during SDT | Xiaomi Communications |
| [22] | R2-2201378 | RACH failure in subsequent data transmission phase | Xiaomi Communications |
| [23] | R2-2201440 | Remaining Issues on RRC-Controlled SDT procedure | vivo |
| [24] | R2-2201441 | Further Consideration on the Handling of non-SDT Data Arrival | vivo |
| [25] | R2-2201495 | SDT control plane aspects | Nokia, Nokia Shanghai Bell |
| [26] | R2-2201496 | RRC procedure for SDT | Nokia, Nokia Shanghai Bell |
| [27] | R2-2201535 | Remaining issues for non-SDT data arrival | China Telecommunications |
| [28] | R2-2201571 | Consideration on some CP issues | CATT |