- 3GPP TSG RAN WG2 Meeting #116bis-e      	               R2-220xxxx
E-Conference, 17th – 25th Jan. 2022

Agenda item:	8.7.1

Source:	Qualcomm Incorporated

Title:	Summary report of [AT116bis-e][607][Relay] Relay UE capabilities (Qualcomm)

WID/SID:	NR_SL_Relay-Core – Release 17

Document for:	Discussion and Decision

# Introduction

This is a summary report for below offline discussion:

- [AT116bis-e][607][Relay] Relay UE capabilities (Qualcomm)

Scope: Start discussion of UE capabilities for relaying, with R2-2200178 as an initial input, and attempt to conclude on a baseline set of capabilities for a draft CR to 38.306.

Intended outcome: Report to Tuesday CB session

Deadline:  Monday 2022-01-24 1800 UTC

From rapporteur respective, the targets of this email discussion are:

- The final target is to endorse running CR of TS 38.306 related to sidelink relay by RAN2#117-e.

- We hope that RAN2 can agree a set of baseline capabilities, where the baseline capability means that it is the starting point of the followed offline / online discussion

- We don’t need to endorse a draft CR during the meeting. But considering the short time after the meeting, it would be good to have a TP/draft CR available that people can see as a baseline.

# Discussion

We first provide a short review of LTE sidelink relay capability in Section 2.1.1 [2], and then Rel-16 NR V2X capability in Section 2.1.2 [3]. If you are familiar with them, you can skip them and jump to Section 2.2 to provide your inputs.

## Background of LTE relay capability and Rel-16 NR capability

### Review of LTE sidelink relay capabilities

All LTE discovery related capabilities are list in Table 1 with rapporteur’s analysis in 4th column

| Name of capability | Description of capability | Granularity of capability | Rapporteur’s view on whether to consider in NR sidelink relay? |
| --- | --- | --- | --- |
| discSupportedBands-r12 | The bands on which the UE supports sidelink discovery | Per UE | Yes (bands supporting discovery can be different from bands support NR SL comm.) |
| discScheduledResourceAlloc-r12 | Whether the UE supports transmission of discovery announcements based on network scheduled resource allocation | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| disc-UE-SelectedResourceAlloc-r12 | Whether the UE supports transmission of discovery announcements based on UE autonomous resource selection | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| disc-SLSS-r12 | Whether the UE supports SideLink Synchronization Signal (SLSS) transmission and reception for sidelink discovery | Per UE | No (SLSS related capability is not introduced in NR) |
| discSupportedProc-r12 | The number of processes supported by the UE for reception of sidelink discovery | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| discInterFreqTx-r13 | Whether the UE supports sidelink discovery announcements either a) on the primary frequency only or b) on other frequencies also, regardless of the UE configuration (e.g. CA, DC). | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| discPeriodicSLSS-r13 | Whether the UE supports periodic Sidelink Synchronization Signal (SLSS) transmission and reception for sidelink discovery | Per UE | No (SLSS related capability is not introduced in NR) |
| discSysInfoReporting-r13 | Whether the UE supports reporting of System Information for inter-frequency/PLMN sidelink discovery | Per UE | No (this feature is not introduced in NR) |
| Sidelink discovery gap (6.13.3) | It is optional for UE to support sidelink discovery gaps | Optional feature without UE radio access capability | No (this feature is not introduced in NR) |

Table.1 Summary and analysis of LTE discovery related capabilities

All LTE relay related capabilities are list in Table 2.

| Name of capability | Description of capability | Granularity of capability | Rapporteur’s view on whether to consider in NR sidelink relay? |
| --- | --- | --- | --- |
| Sidelink Relay UE operation (6.13.1) | It is optional for UE to support sidelink relay UE operation | Optional feature without UE radio access capability | Yes for L3 relay |
| Sidelink Remote UE operation (6.13.2) | It is optional for UE to support sidelink remote UE operation | Optional feature without UE radio access capability | Yes for L3 relay |

Table.2 Summary and analysis of LTE sidelink relay related capabilities

### Review of Rel-16 NR V2X capabilities

NR Rel-16 V2X capabilities are list in Table 3.

| Name of capability | Description of capability | Granularity of capability | Rapporteur’s comments |
| --- | --- | --- | --- |
| supportedBandCombinationListSidelink-r16 | The supported NR sidelink communication band combinations by the UE | Per UE | Band combinations of L2 relay and discovery may be different from it |
| supportedBandCombinationListSidelinkEUTRA-r16 | The supported V2X sidelink communication band combinations by the UE | Per UE | No cross-RAT control for relay |
| supportedBandCombinationListSidelinkEUTRA-NR-r16 | The supported joint NR sidelink and V2X sidelink communication band combinations by the UE. | Per UE |  |
| lcp-RestrictionSidelink-r16 | Whether UE supports the selection of logical channels for each SL grant based on RRC configured restriction | Per UE | They are applied to NR discovery and L2 relay because NR PSSCH is used to carry it |
| logicalChannelSR-DelayTimerSidelink-r16 | Whether the UE supports the logicalChannelSR-DelayTimer | Per UE |  |
| multipleSR-ConfigurationsSidelink-r16 | Whether the UE supports 8 SR configurations per PUCCH cell group | Per UE |  |
| multipleConfiguredGrantsSidelink-r16 | Whether UE supports 8 sidelink configured grant configurations (including both Type 1 and Type 2) in a resource pool. | Per UE |  |
| am-WithLongSN-Sidelink-r16 | Whether the UE supports AM DRB with 18 bit length of RLC sequence number for sidelink | Per UE |  |
| um-WithLongSN-Sidelink-r16 | Whether the UE supports UM DRB with 12 bit length of RLC sequence number for sidelink. | Per UE |  |
| outOfOrderDeliverySidelink-r16 | Whether UE supports out of order delivery of data to upper layers by PDCP | Per UE |  |
| accessStratumReleaseSidelink-r16 | The access stratum release for NR sidelink communication the UE supports | Per UE |  |

Table.3 Summary NR Rel-16 V2X related capabilities

## Discussion on NR sidelink relay capabilities

First, according to LTE relay capabilities [2], the capabilities on discovery (e.g., discSupportedBands-r12) are independent of capabilities on relay (e.g., Sidelink Relay UE operation). Rapporteur’s understanding is that this principle can be reused in NR, especially non-relay discovery being supported in this release. Thus, Rapporteur want to confirm whether this is RAN2 common understanding.

Q1) Do you agree that the capabilities on NR discovery are independent of capabilities on sidelink relay (including L2 and L3 relay), similar to LTE?

| Company | Response (Y/N) | Comments (if company answered No due to significant concerns) |
| --- | --- | --- |

### NR discovery related capabilities

In R2-2200178 [4], it proposed below baseline capability on NR discovery:

Proposal 1: As baseline UE capability, introduce below per-UE band combination list for NR discovery (e.g., with name supportedBandCombinationListDiscovery-r17), which is indicated to gNB.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportedBandCombinationListDiscovery-r17 Defines the supported band combinations of NR sidelink discovery message transmission and reception by the UE: - It is common to relay discovery and non-relay discovery - It is common to L2 and L3 relay. - it is common to remote UE and relay UE | UE | No | No | No |

| Sidelink Parameter | UECapabilityInformation | UECapabilityInformationSidelink |
| --- | --- | --- |
| supportedBandCombinationListDiscovery-r17 | X |  |

As you see, this proposal includes multiple aspects. Thus, rapporteur would like to get companies’ inputs on each detail.

First, it is whether we need separate capability on relay discovery and non-relay discovery. In LTE capability, it is common to relay and non-relay discovery.

Q2) On NR discovery capability related to relay/non-relay discovery, do you prefer:

- Option 1: common to relay discovery and non-relay discovery

- Option 2: separate capabilities for relay discovery and non-relay discovery

| Company | Response | Comments |
| --- | --- | --- |

Secondly, it is whether we need separate capability on transmission and reception of discovery. In LTE capability, it is common to transmission and reception.

Q3) On NR discovery capability related to transmission and reception of discovery message, do you prefer:

- Option 1: common to transmission and reception of discovery message

- Option 2: separate capabilities for transmission and reception of discovery message

| Company | Response | Comments |
| --- | --- | --- |

Next, Rapporteur would like to check whether we need separate capability on L2 and L3 relay. In LTE capability, these is no such issue because only L3 relay is supported.

Q4) On NR discovery capability related to relay type, do you prefer:

- Option 1: common to L2 and L3 relay

- Option 2: separate capabilities for L2 and L3 relay

| Company | Response | Comments |
| --- | --- | --- |

Next, Rapporteur would like to check whether we need separate capability on remote UE and relay UE. In LTE capability, it is common to relay UE and remote UE.

Q5) On NR discovery capability related to remote UE and relay UE, do you prefer:

- Option 1: common to remote UE and relay UE

- Option 2: separate capabilities for remote UE and relay UE

| Company | Response | Comments |
| --- | --- | --- |

Next, Rapporteur would like to discuss the signaling of NR discovery capability. In Rapporteur’s understanding, we have below options:

- Option 1: A list of band combination list, which is similar to Rel-16 sidelink communication band combination list (i.e., supportedBandCombinationListSidelink-r16) and can be a different list

- Option 2: A band list, same as LTE capability (i.e., discSupportedBands-r12)

- Option 3: A single bit on whether supporting NR discovery

Q6) On signaling of NR discovery capability, do you prefer:

- Option 1: A list of band combination list, which is similar to Rel-16 sidelink communication band combination list (i.e., supportedBandCombinationListSidelink-r16) and can be a different list

- Option 2: A band list, same as LTE capability (i.e., discSupportedBands-r12)

- Option 3: A single bit on whether supporting NR discovery

| Company | Response | Comments |
| --- | --- | --- |

Then, given your input on Q6, Rapporteur would like to check whether the signaling is indicated to gNB (i.e., in UECapabilityInformation), or peer UE (i.e., in UECapabilityInformationSidelink), or both.

Q7) On signaling of NR discovery capability, do you prefer:

- Option 1: Indicated to only gNB (i.e., only included in UECapabilityInformation)

- Option 2: Indicated to only peer UE (i.e., only included in UECapabilityInformationSidelink)

- Option 3: Indicated to both gNB and peer UE

| Company | Response | Comments |
| --- | --- | --- |

Finally, companies are welcome to provide inputs on other aspects on NR discovery capabilities.

Q8) Any other comments on NR discovery capabilities (e.g., new separate capability, missed aspects)?

| Company | Comments |
| --- | --- |

### NR L2 relay related capabilities

We discuss NR L2 relay related capability in this section.

In R2-2200178 [4], it proposed below baseline capability on L2 relay:

Proposal 2: As baseline UE capability, introduce 2 separate per-UE capability signaling for NR L2 relay UE operation and remote UE operation (e.g., with name L2RelayUEoperation-r17 and L2RemoteUEoperation-r17), which is indicated to gNB.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| L2RelayUEoperation-r17 Whether supporting NR L2 sidelink relay UE operation by the UE: - Support both Uu and PC5 SRAP - Support paging forwarding - Support SIB forwarding | UE | No | No | No |
| L2RemoteUEoperation-r17 Whether supporting NR L2 sidelink remote UE operation by the UE: - Support PC5 SRAP - Support RRC resume/reestablishment via relay | UE | No | No | No |

| Sidelink Parameter | UECapabilityInformation | UECapabilityInformationSidelink |
| --- | --- | --- |
| L2RelayUEoperation-r17 | X |  |
| L2RemoteUEoperation-r17 | X |  |

Proposal 3: The supported band combination list for L2 relay UE operation and remote UE operation is same as supportedBandCombinationListSidelink-r16

As you see, this proposal includes multiple aspects. Thus, rapporteur would like to get companies’ inputs on each detail.

First, we discuss the granularity of the capability signaling. In Rapporteur’s understanding, we have below options:

- Option 1: a common signaling to remote UE operation and relay UE operation

- Option 2: separate capability signaling for remote UE operation and relay UE operation

- Option 3: Multiple separate capability signaling for some L2 relay features (e.g., support of PC5 SRAP, support of paging forwarding, support of SIB forwarding).

Q9) On granularity of L2 relay capability, do you prefer:

- Option 1: a common signaling to remote UE operation and relay UE operation

- Option 2: separate capability signaling for remote UE operation and relay UE operation

- Option 3: Multiple separate capability signaling for some L2 relay features (details TBD)

| Company | Response | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |

If you prefer Option 3 of Q9, please provide your identified L2 relay features.

Q10) If you prefer Option 3 of Q9, please provide your identified L2 relay features which require separate capability signaling (e.g., support of PC5 SRAP, support of paging forwarding, support of SIB forwarding).

| Company | Identified L2 relay features which require separate capability signaling |
| --- | --- |
| Qualcomm |  |

Then, Rapporteur would like to discuss detailed signaling. In Rapporteur’s understanding, we have below options:

- Option 1: A list of band combination list which supports L2 relay functions / features (granularity depending on your answer to Q9)

- Option 2: A band list which supports L2 relay functions / features (depending on your answer to Q9)

- Option 3: A single bit on whether supporting L2 relay functions / features (depending on your answer to Q9). And the supported combination list is same as supportedBandCombinationListSidelink-r16

Q11) On signaling of L2 relay capability, do you prefer:

- Option 1: A list of band combination list which supports L2 relay functions / features (its granularity depending on your answer to Q9)

- Option 2: A band list which supports L2 relay functions / features (its granularity depending on your answer to Q9)

- Option 3: A single bit on whether supporting L2 relay functions / features (its granularity depending on your answer to Q9). And the supported combination list is same as supportedBandCombinationListSidelink-r16

| Company | Response | Comments |
| --- | --- | --- |

Finally, Rapporteur would like to check whether the signaling is indicated to gNB (i.e., in UECapabilityInformation), or peer UE (i.e., in UECapabilityInformationSidelink), or both.

Q12) On signaling of L2 relay capability, do you prefer:

- Option 1: Indicated to only gNB (i.e., only included in UECapabilityInformation)

- Option 2: Indicated to only peer UE (i.e., only included in UECapabilityInformationSidelink)

- Option 3: Indicated to both gNB and peer UE

| Company | Response | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |

Finally, companies are welcome to provide inputs on other aspects on L2 relay capabilities.

Q13) Any other comments on L2 relay capabilities (e.g., new separate capability, missed aspects)?

| Company | Comments |
| --- | --- |
| Qualcomm |  |

### NR L3 relay related capabilities

We discuss NR L3 relay capability in this section. Please note that LTE has provided similar capability (as illustrated in Table. 2). Rapporteur think it is straight forward to introduce similar capability for NR L3 relay, but need to get companies’ input.

Q14) For L3 relay, do you agree to introduce below capability similar to LTE?

- Introduce 2 separate optional UE feature without UE radio access capability parameters for NR L3 relay UE operation and remote UE operation (e.g., with name “L3 Sidelink Relay UE operation” and “L3 Sidelink Remote UE operation”):

Definitions for feature

L3 sidelink relay UE operation It is optional for UE to support L3 sidelink relay UE operation as specified in TS 38.331 [9].

L3 sidelink remote UE operation It is optional for UE to support L3 sidelink remote UE operation as specified in TS 38.331 [9].

| Company | Response (Y/N) | Comments (if company answered No due to significant concerns) |
| --- | --- | --- |

Finally, companies are welcome to provide inputs on other aspects on L3 relay capabilities.

Q15) Any other comments on L3 relay capabilities (e.g., new separate capability, missed aspects)?

| Company | Comments |
| --- | --- |

# Conclusion

TBD based on companies’ input

# References

[1] RP-210904, New WID on NR Sidelink Relay.

[2] TS 36.306 v16.3.0, User Equipment (UE) radio access capabilities (Release 16)

[3] TS 38.306 v16.3.0, User Equipment (UE) radio access capabilities (Release 16)

[4] R2-2200178, Initial consideration on UE capability of sidelink relay, Qualcomm Incorporated