- 3GPP TSG RAN WG2 Meeting #116bis-e      	               R2-220xxxx
E-Conference, 17th – 25th Jan. 2022

Agenda item:	8.7.1

Source:	Qualcomm Incorporated

Title:	Summary report of [AT116bis-e][607][Relay] Relay UE capabilities (Qualcomm)

WID/SID:	NR_SL_Relay-Core – Release 17

Document for:	Discussion and Decision

# Introduction

This is a summary report for below offline discussion:

- [AT116bis-e][607][Relay] Relay UE capabilities (Qualcomm)

Scope: Start discussion of UE capabilities for relaying, with R2-2200178 as an initial input, and attempt to conclude on a baseline set of capabilities for a draft CR to 38.306.

Intended outcome: Report to Tuesday CB session

Deadline:  Monday 2022-01-24 1800 UTC

From rapporteur respective, the targets of this email discussion are:

- The final target is to endorse running CR of TS 38.306 related to sidelink relay by RAN2#117-e.

- We hope that RAN2 can agree a set of baseline capabilities, where the baseline capability means that it is the starting point of the followed offline / online discussion

- We don’t need to endorse a draft CR during the meeting. But considering the short time after the meeting, it would be good to have a TP/draft CR available that people can see as a baseline.

# Discussion

We first provide a short review of LTE sidelink relay capability in Section 2.1.1 [2], and then Rel-16 NR V2X capability in Section 2.1.2 [3]. If you are familiar with them, you can skip them and jump to Section 2.2 to provide your inputs.

## Background of LTE relay capability and Rel-16 NR capability

### Review of LTE sidelink relay capabilities

All LTE discovery related capabilities are list in Table 1 with rapporteur’s analysis in 4th column

| Name of capability | Description of capability | Granularity of capability | Rapporteur’s view on whether to consider in NR sidelink relay? |
| --- | --- | --- | --- |
| discSupportedBands-r12 | The bands on which the UE supports sidelink discovery | Per UE | Yes (bands supporting discovery can be different from bands support NR SL comm.) |
| discScheduledResourceAlloc-r12 | Whether the UE supports transmission of discovery announcements based on network scheduled resource allocation | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| disc-UE-SelectedResourceAlloc-r12 | Whether the UE supports transmission of discovery announcements based on UE autonomous resource selection | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| disc-SLSS-r12 | Whether the UE supports SideLink Synchronization Signal (SLSS) transmission and reception for sidelink discovery | Per UE | No (SLSS related capability is not introduced in NR) |
| discSupportedProc-r12 | The number of processes supported by the UE for reception of sidelink discovery | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| discInterFreqTx-r13 | Whether the UE supports sidelink discovery announcements either a) on the primary frequency only or b) on other frequencies also, regardless of the UE configuration (e.g. CA, DC). | Per UE | No (can reuse NR SL comm capability because NR discovery is sent via PSSCH) |
| discPeriodicSLSS-r13 | Whether the UE supports periodic Sidelink Synchronization Signal (SLSS) transmission and reception for sidelink discovery | Per UE | No (SLSS related capability is not introduced in NR) |
| discSysInfoReporting-r13 | Whether the UE supports reporting of System Information for inter-frequency/PLMN sidelink discovery | Per UE | No (this feature is not introduced in NR) |
| Sidelink discovery gap (6.13.3) | It is optional for UE to support sidelink discovery gaps | Optional feature without UE radio access capability | No (this feature is not introduced in NR) |

Table.1 Summary and analysis of LTE discovery related capabilities

All LTE relay related capabilities are list in Table 2.

| Name of capability | Description of capability | Granularity of capability | Rapporteur’s view on whether to consider in NR sidelink relay? |
| --- | --- | --- | --- |
| Sidelink Relay UE operation (6.13.1) | It is optional for UE to support sidelink relay UE operation | Optional feature without UE radio access capability | Yes for L3 relay |
| Sidelink Remote UE operation (6.13.2) | It is optional for UE to support sidelink remote UE operation | Optional feature without UE radio access capability | Yes for L3 relay |

Table.2 Summary and analysis of LTE sidelink relay related capabilities

### Review of Rel-16 NR V2X capabilities

NR Rel-16 V2X capabilities are list in Table 3.

| Name of capability | Description of capability | Granularity of capability | Rapporteur’s comments |
| --- | --- | --- | --- |
| supportedBandCombinationListSidelink-r16 | The supported NR sidelink communication band combinations by the UE | Per UE | Band combinations of L2 relay and discovery may be different from it |
| supportedBandCombinationListSidelinkEUTRA-r16 | The supported V2X sidelink communication band combinations by the UE | Per UE | No cross-RAT control for relay |
| supportedBandCombinationListSidelinkEUTRA-NR-r16 | The supported joint NR sidelink and V2X sidelink communication band combinations by the UE. | Per UE |  |
| lcp-RestrictionSidelink-r16 | Whether UE supports the selection of logical channels for each SL grant based on RRC configured restriction | Per UE | They are applied to NR discovery and L2 relay because NR PSSCH is used to carry it |
| logicalChannelSR-DelayTimerSidelink-r16 | Whether the UE supports the logicalChannelSR-DelayTimer | Per UE |  |
| multipleSR-ConfigurationsSidelink-r16 | Whether the UE supports 8 SR configurations per PUCCH cell group | Per UE |  |
| multipleConfiguredGrantsSidelink-r16 | Whether UE supports 8 sidelink configured grant configurations (including both Type 1 and Type 2) in a resource pool. | Per UE |  |
| am-WithLongSN-Sidelink-r16 | Whether the UE supports AM DRB with 18 bit length of RLC sequence number for sidelink | Per UE |  |
| um-WithLongSN-Sidelink-r16 | Whether the UE supports UM DRB with 12 bit length of RLC sequence number for sidelink. | Per UE |  |
| outOfOrderDeliverySidelink-r16 | Whether UE supports out of order delivery of data to upper layers by PDCP | Per UE |  |
| accessStratumReleaseSidelink-r16 | The access stratum release for NR sidelink communication the UE supports | Per UE |  |

Table.3 Summary NR Rel-16 V2X related capabilities

## Discussion on NR sidelink relay capabilities

First, according to LTE relay capabilities [2], the capabilities on discovery (e.g., discSupportedBands-r12) are independent of capabilities on relay (e.g., Sidelink Relay UE operation). Rapporteur’s understanding is that this principle can be reused in NR, especially non-relay discovery being supported in this release. Thus, Rapporteur want to confirm whether this is RAN2 common understanding.

Q1) Do you agree that the capabilities on NR discovery are independent of capabilities on sidelink relay (including L2 and L3 relay), similar to LTE?

| Company | Response (Y/N) | Comments (if company answered No due to significant concerns) |
| --- | --- | --- |
| OPPO | Y |  |
| Xiaomi | Y | Agree with Rapp, since there is non-relay discovery which is independent of sidelink relay. |
| Qualcomm | Y with comments | Here, the meaning of “independent of” means RAN2 can have two separate capability signaling for discovery and L2/L3 relay function. And it is still possible to have coupled capability for some cases. For example, RAN2 can specify that if a UE indicates its support for L2 relay, it should also indicate its supports for discovery. |
| Ericsson | Y with comments | Our view is more aligned to what described by Qualcomm. Our understanding is that there are UE capabilities for relay and capabilities for discovery. We do not needs to differentiate discovery for relay and non-relay cases. |
| InterDigital | Y | This is preferred due to presence of non-relay discovery. |
| MediaTek | Y | We think NR sidelink discovery should be common capability that applies to both L2 relay and L3 relay |
| CATT | Y |  |
| Samsung | Y | Same view as Qualcomm |
| Nokia | Yes |  |
| vivo | Y | Agree with Rapporteur. |
| ZTE | Y | It’s reasonable for non-relay discovery. |
| Apple | Y | Same view as Qualcomm |
| LG | Y | Agree with Rapporteur |
| Huawei, HiSilicon | Y with comments | Discovery are independent from SL relay. However, L2/L3 sidelink relay capabilities are coupled with discovery, i.e. relay/remote needs to indicate support of discovery. |
| Philips | Y |  |
| Intel | Y |  |

### NR discovery related capabilities

In R2-2200178 [4], it proposed below baseline capability on NR discovery:

Proposal 1: As baseline UE capability, introduce below per-UE band combination list for NR discovery (e.g., with name supportedBandCombinationListDiscovery-r17), which is indicated to gNB.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportedBandCombinationListDiscovery-r17 Defines the supported band combinations of NR sidelink discovery message transmission and reception by the UE: - It is common to relay discovery and non-relay discovery - It is common to L2 and L3 relay. - it is common to remote UE and relay UE | UE | No | No | No |

| Sidelink Parameter | UECapabilityInformation | UECapabilityInformationSidelink |
| --- | --- | --- |
| supportedBandCombinationListDiscovery-r17 | X |  |

As you see, this proposal includes multiple aspects. Thus, rapporteur would like to get companies’ inputs on each detail.

First, it is whether we need separate capability on relay discovery and non-relay discovery. In LTE capability, it is common to relay and non-relay discovery.

Q2) On NR discovery capability related to relay/non-relay discovery, do you prefer:

- Option 1: common to relay discovery and non-relay discovery

- Option 2: separate capabilities for relay discovery and non-relay discovery

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 1 or 2 | We are open to both 1 and 2, yet considering for L2 / L3 relay, there is no solution without requiring discovery message, it seems more reasonable to handle relay discovery + communication as an integrated function / feature for capability discussion, i.e., logically, there should be just non-relay-discovery vs. relay functionality including both relay-discovery and relay-communication as a whole part. [OPPO2] after seeing the response by QC, we ACK that using prerequisite is also a feasible way-out, so in that way, 1 is also fine for us. Further clarification is that for L3-relay, it also require discovery, yet in case L3-relay is optional without capability bit, the handling may be different from L2-relay. |
| Xiaomi | Comments | This may depend on which capability rapp is referring to. If question aims at supported band for discovery, option 1 is preferred. For other aspects, separate capabilities may be preferred, for example, threshold based discovery initiation, which is only applicable for relay discovery. |
| Qualcomm | Option 1 | From UE implementation perspective, we don’t see technique difference between supporting relay discovery and non-relay discovery. Of course, we are happy to consider its separation if technique justification is valid. For OPPO’s comment on handling relay discovery + communication as an integrated function, we think it only related to L2 relay, right? Then, as we commented in Q1, RAN2 can specify that if a UE indicates its support for L2 relay, it should also indicate its supports for discovery. For Xiaomi’s question “which capability rapp is referring to”, although it is not crystal clear because it is our first discussion, our assumption is that it means a basic discovery capability (i.e., supported band or 1bit indication). We can further define capability for discovery enhancement features if necessary (you can provide such consideration in Q8) |
| Ericsson | Option 1 | We agree with Qualcomm. From a technical perspective we don’t see why a UE should support discovery for relay and not for non-relay cases (or vice versa). There is no technical difference and thus option 1 is simpler (and also it introduces less signalling overhead). |
| InterDigital | Option 1 | From a capabilities perspective, there should be no difference between relay and non-relay. |
| MediaTek | Option-1 |  |
| CATT | Option-1 | We share the same view as QC. |
| Samsung | Option 1 |  |
| Nokia | Option 1 |  |
| vivo | Option -1 | Common solution is preferred and we don’t see any motivation to differentiate relay discovery and non-relay discovery cases. |
| ZTE | Option 1 |  |
| Apple | Option 1 with comment | We think UE shall be allowed to support discovery w/o supporting relay. The capability design needs to be able to support such UE implementation. There is still some difference between non-relay discovery and relay discovery, and UE may not want to implement relay-related part, such as AS layer threshold-based triggering. |
| LG | Option-1 |  |
| Huawei, HiSilicon | Option 1 | We think it can be common to non-relay and L2/L3 relay. |
| Philips | Option 1 |  |
| Intel | Option 1 |  |

Secondly, it is whether we need separate capability on transmission and reception of discovery. In LTE capability, it is common to transmission and reception.

Q3) On NR discovery capability related to transmission and reception of discovery message, do you prefer:

- Option 1: common to transmission and reception of discovery message

- Option 2: separate capabilities for transmission and reception of discovery message

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 1 |  |
| Xiaomi | Option 1 |  |
| Qualcomm | Option 1 | Same as LTE, although the requirement on transmission and reception may be different. |
| Ericsson | Option 1 |  |
| InterDigital | Option 1 |  |
| MediaTek | Option-1 |  |
| CATT | Option-1 |  |
| Samsung | Option 1 |  |
| Nokia | Option 1 |  |
| vivo | Option 1 | Following the same principle as transmission and reception of communication. |
| ZTE | Option 1 |  |
| Apple | Option 1 |  |
| LG | Option 1 |  |
| Huawei, HiSilicon | Option 1 | LTE mechanism can be reused |
| Philips | Option 1 |  |
| Intel | Option 1 |  |

Next, Rapporteur would like to check whether we need separate capability on L2 and L3 relay. In LTE capability, these is no such issue because only L3 relay is supported.

Q4) On NR discovery capability related to relay type, do you prefer:

- Option 1: common to L2 and L3 relay

- Option 2: separate capabilities for L2 and L3 relay

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 1 | As replied to Q2, for the relay case, not sure if we need to further split discovery function out, since there is no solution for relaying requiring no discovery message. (we are fine to split L2 and L3 relay function as to be discussed in the following sections though). |
| Xiaomi | Option 1 | According the agreements, the transmission and reception of discovery message procedure is common to L2 and L3 relay. |
| Qualcomm | Option 1 | As commented in Q2, we prefer a common capability for relay and non-relay discovery. So, we prefer a common discovery capability to cover both L2 and L3 relay because we think the UE requirements on discovery is same between L2 and L3 relay. Of course, we are happy to consider its separation if technique justification is valid. Similarly, as we commented in Q1, RAN2 can specify that if a UE indicates its support for L2 relay, it should also indicate its supports for discovery. |
| Ericsson | Option 1 |  |
| InterDigital | Option 1 |  |
| MediaTek | Option-1 |  |
| CATT | Option-1 |  |
| Samsung | Option 1 |  |
| Nokia | Option 1 |  |
| vivo | Option 1 | Since RAN2 strives for common solution for L2 and L3 relay discovery, we can reuse LTE baseline as much as possible, i.e., no differentiation on the discovery UE capability for L2 and L3 relaying operation. |
| ZTE | Option 1 | We don’t see technical difference between them. So a common solution is enough. |
| Apple | Option 1 |  |
| LG | Option 1 |  |
| Huawei, HiSilicon | Option 1 | Common capability is ok. |
| Philips | Option 1 |  |
| Intel | Option 1 |  |

Next, Rapporteur would like to check whether we need separate capability on remote UE and relay UE. In LTE capability, it is common to relay UE and remote UE.

Q5) On NR discovery capability related to remote UE and relay UE, do you prefer:

- Option 1: common to remote UE and relay UE

- Option 2: separate capabilities for remote UE and relay UE

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 1 | As replied to Q2, for the relay case, not sure if we need to further split discovery function out, since there is no solution for relaying requiring no discovery message. (we are fine to split remote and relay UE related relay function as to be discussed in the following sections though). |
| Xiaomi | Option 1 | We don’t see the need to separate the capability in discovery. The UE type, i.e. relay or remote UE, could be differentiated by other capability indication. |
| Qualcomm | Option 1 | Now, we prefer a common discovery capability to cover both remote UE and relay UE because we think the UE requirements on discovery is same between remote UE and relay UE, irrespective of L2 or L3 relay. Of course, we are happy to consider its separation if technique justification is valid. Similarly, as we commented in Q1, RAN2 can specify that if a UE indicates its support for L2 relay, it should also indicate its supports for discovery. |
| Ericsson | Option 1 |  |
| InterDigital | Option 1 |  |
| MediaTek | Option-1 |  |
| CATT | Option 1 |  |
| Samsung | Option 1 |  |
| Nokia | Option 1 and 2 | We are fine with both option 1 and option 2. No strong preference and can agree to majority view. |
| vivo | Option-1 |  |
| ZTE | Option 1 |  |
| Apple | Option 1 |  |
| LG | Option-1 |  |
| Huawei, HiSilicon | Option 1 | Since both Model A and Model B are supported, and as discussed in Q3 that capability common to reception and transmission is preferred, we think this capability is also common to remote UE and relay UE. |
| Philips | Option 1 |  |
| Intel | Option 1 |  |

Next, Rapporteur would like to discuss the signaling of NR discovery capability. In Rapporteur’s understanding, we have below options:

- Option 1: A list of band combination list, which is similar to Rel-16 sidelink communication band combination list (i.e., supportedBandCombinationListSidelink-r16) and can be a different list

- Option 2: A band list, same as LTE capability (i.e., discSupportedBands-r12)

- Option 3: A single bit on whether supporting NR discovery

Q6) On signaling of NR discovery capability, do you prefer:

- Option 1: A list of band combination list, which is similar to Rel-16 sidelink communication band combination list (i.e., supportedBandCombinationListSidelink-r16) and can be a different list

- Option 2: A band list, same as LTE capability (i.e., discSupportedBands-r12)

- Option 3: A single bit on whether supporting NR discovery

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 3 | Although no strong view, we understand LTE adopt a per-band capability because discovery was carried via a dedicated PHY channel in LTE (i.e., PSDCH), yet considering now in LTE, discovery is carried using PSSCH as for communication, it is more a L2 capability, so seems more a per-UE than a per-band/BC capability. |
| Xiaomi | Option 1 | Option 1 is the most flexible and forward compatible way, which could also align with sidelink communication capability. |
| Qualcomm | Option 1 with comments | We agree with OPPO that PHY requirement seems to be same for discovery and Rel-16 sidelink comm, because PSSCH is used to carry discovery. However, we do see some difference in RLC/PDCP/MAC as RAN2 agreed before (e.g., AS security, whether to enable HARQ, RLC/PDCP handling and open-loop power control). Thus, we think option 1 is the most safe / flexible solution at this stage. If companies have concern on signaling overhead, we can use some way like “if the UE supports discovery but doesn’t provide band combination list, it means the list is same as Rel-16 sidelink communication”. |
| Ericsson | Option 3 | We agree with OPPO. We do not see any benefits to create a huge signalling overhead with Option 1 (unless justified). |
| InterDigital | Option 3 | For NR, we think this capability should be per UE and not per band combination. |
| MediaTek | Option-1 |  |
| CATT | Option 1 |  |
| Samsung | Option 1 |  |
| Nokia | Option 3 |  |
| vivo | Option 1 with comments | Regarding the capability granularity, we prefer to follow the UE Rel-16 sidelink communication i.e., per band combination instead of per band or per UE. However, we are not sure on the support of a different BC list for sidelink communication and discovery. Some further discussion and consideration may be needed. |
| ZTE | Option 1 | We prefer to keep align with R16 sidelink communication capability. |
| Apple | Option 3 | We share the same view as OPPO, there is no need for per-band capability for NR discovery. Fort AS layer, discovery is just a new SRB (SRB4), not a new radio. |
| LG | Option 1 |  |
| Huawei, HiSilicon | Option 3 | Since there is no different PHY requirement for SL discovery and SL communication, we think a single bit is enough. |
| Philips | Option 1 |  |
| Intel | Option 3 |  |

Then, given your input on Q6, Rapporteur would like to check whether the signaling is indicated to gNB (i.e., in UECapabilityInformation), or peer UE (i.e., in UECapabilityInformationSidelink), or both.

Q7) On signaling of NR discovery capability, do you prefer:

- Option 1: Indicated to only gNB (i.e., only included in UECapabilityInformation)

- Option 2: Indicated to only peer UE (i.e., only included in UECapabilityInformationSidelink)

- Option 3: Indicated to both gNB and peer UE

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 1 | Capability is based on GC/BC type (even for model-B, the first message is BC-based), so the capability signaling via PC5-RRC is not applicable. |
| Xiaomi | Option 1 | The discovery transmission and reception occurs before unicast connection establishment, so doesn’t rely on unicast connection. There seems to be unnecessary to indicate the discovery capability to peer UE. |
| Qualcomm | Option 1 | Discovery is before unicast PC5 link establishment. |
| Ericsson | Option 1 |  |
| InterDigital | Option 1 | This capability is not relevant to a unicast link, so PC5 RRC is not relevant. |
| MediaTek | Option-1 | We did not see the need to notify to the peer UE |
| CATT | Option 1 |  |
| Samsung | Option 1 |  |
| Nokia | Option 1 |  |
| vivo | Option 1 | We understand that peer UE capability exchange after PC5-S link establishment is not needed for discovery. |
| ZTE | Option 1 |  |
| Apple | Option 1 |  |
| LG | Option 1 |  |
| Huawei, HiSilicon | Option 1 | It is not unicast related capability. |
| Philips | Option 1 |  |
| Intel | Option 1 |  |

Finally, companies are welcome to provide inputs on other aspects on NR discovery capabilities.

Q8) Any other comments on NR discovery capabilities (e.g., new separate capability, missed aspects)?

| Company | Comments |
| --- | --- |
| Qualcomm | If per UE indication is agreed, RAN2 may need to further discuss whether TDD/FDD differ and whether FR1/FR2 diff. But it can be further discussed in RAN2#117-e. |

### NR L2 relay related capabilities

We discuss NR L2 relay related capability in this section.

In R2-2200178 [4], it proposed below baseline capability on L2 relay:

Proposal 2: As baseline UE capability, introduce 2 separate per-UE capability signaling for NR L2 relay UE operation and remote UE operation (e.g., with name L2RelayUEoperation-r17 and L2RemoteUEoperation-r17), which is indicated to gNB.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| L2RelayUEoperation-r17 Whether supporting NR L2 sidelink relay UE operation by the UE: - Support both Uu and PC5 SRAP - Support paging forwarding - Support SIB forwarding | UE | No | No | No |
| L2RemoteUEoperation-r17 Whether supporting NR L2 sidelink remote UE operation by the UE: - Support PC5 SRAP - Support RRC resume/reestablishment via relay | UE | No | No | No |

| Sidelink Parameter | UECapabilityInformation | UECapabilityInformationSidelink |
| --- | --- | --- |
| L2RelayUEoperation-r17 | X |  |
| L2RemoteUEoperation-r17 | X |  |

Proposal 3: The supported band combination list for L2 relay UE operation and remote UE operation is same as supportedBandCombinationListSidelink-r16

As you see, this proposal includes multiple aspects. Thus, rapporteur would like to get companies’ inputs on each detail.

First, we discuss the granularity of the capability signaling. In Rapporteur’s understanding, we have below options:

- Option 1: a common signaling to remote UE operation and relay UE operation

- Option 2: separate capability signaling for remote UE operation and relay UE operation

- Option 3: Multiple separate capability signaling for some L2 relay features (e.g., support of PC5 SRAP, support of paging forwarding, support of SIB forwarding).

Q9) On granularity of L2 relay capability, do you prefer:

- Option 1: a common signaling to remote UE operation and relay UE operation

- Option 2: separate capability signaling for remote UE operation and relay UE operation

- Option 3: Multiple separate capability signaling for some L2 relay features (details TBD)

| Company | Response | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| OPPO | At least 2 | We understand the capability for relay and remote UE seems differ in some aspect, so at least option-2 is more preferred? For option-3, we see some discussion on detailed aspect of relay-UE, e.g., in terms of SIB-forwarding, and paging monitoring, it would be good to have some discussion on the related aspects, and to conclude on that based on the discussion result |
| Xiaomi | Option 2 an Option 3 | We see the necessity to exchange SIB forwarding related capability between remote UE and relay UE to enable the SI request and forwarding. |
| Qualcomm | Option 2 as baseline | We think their supported features are different: - Relay UE operation: - Support both Uu and PC5 SRAP - Support paging forwarding - Support SIB forwarding - Remote UE operation: - Support at least PC5 SRAP - No need to support paging/SIB forwarding because the SIB/paging reception should be same as legacy - Support RRC resume/reestablishment via relay So, we prefer to have separate capability for remote UE and relay UE. In addition, we think it is better to have at least a capability of basic relay UE operation and remote UE operation because L2 relay can work only if the UE can support all of these basic features, e.g., a relay UE can work only if it can support Uu/PC5 SRAP, SIB and paging forwarding (i.e., it can’t report its support of relay UE operation if it supports SRAP but doesn’t support SIB forwarding) Finally, we can also define some extra capability for some enhancement features (e.g., if pre-emptive BSR can be agreed). That is why we say it is a baseline. |
| Ericsson | Option 2 as baseline | We think that Option 2 should be the baseline. However, about to have separate capabilities for different relay features can be discussed later on. Not is a bit premature to decide. |
| Lenovo | Option 2 as baseline | Apart from what QC has said above, we think being able to receive the Paging and SIB via Relay is also part of the remote UE’s capability. There’s something different here that a remote UE needs to do that it does not need to do in Uu case; e.g.,: - For Paging reception, it needs to provide parameters like Paging cycle, UE ID to relay and receive a confirmation on relay will monitor paging (for RRC Idle/ Inactive remote UE) and forward paging received dedicatedly from gNB (for RRC Connected remote UE) For SI Acquisition, the procedure for on-demand request will be different e.g., no Uu RACH for RRC Idle/ Inactive remote UE; requesting for SIBs that are provided on a regular basis from the gNB (i.e., not on on-demand basis) |
| InterDigital | Option 2 | For now, we should assume option 2 as a baseline, and discuss whether feature-based capabilities are needed or not. |
| MediaTek | Option-2 | We assume there are different UE types, where some UEs support Relay UE operation, some other UEs may only support Remote UE operation, and some other UEs may support both. |
| CATT | Option 2 | Relay and remote UE have different capabilities, hence Option 2 is preferred. |
| Samsung | Option 2 | Option 2 can be a baseline. |
| Nokia | Option 2 |  |
| vivo | Option 2 as baseline | Option 3 may be discussed after we have a clear picture on the basic functionality of remote UE and relay UE. For example, for optional feature like mode-1 support, different capabilities for remote UE and relay UE may be needed. |
| ZTE | Option 2 | The requirements for remote UE and relay UE are different, so separate capability signaling should be adopted. |
| Apple | Option 2/3 | We think option 3 may be needed after examining all the agreements by the time when R17 design is completed. |
| LG | Option 2 |  |
| Huawei, HiSilicon | Option 2 as baseline | Considering the requirements for remote UE and relay UE are totally different and UE may only be capable of one of the roles, separate capabilities for remote UE and relay UE are necessary. In addition, we are open to discuss the possibility of multiple separate capabilities for some features. |
| Philips | Option 2 | As a baseline. option 3 FFS |
| Intel | Option 2 |  |

If you prefer Option 3 of Q9, please provide your identified L2 relay features.

Q10) If you prefer Option 3 of Q9, please provide your identified L2 relay features which require separate capability signaling (e.g., support of PC5 SRAP, support of paging forwarding, support of SIB forwarding).

| Company | Identified L2 relay features which require separate capability signaling |
| --- | --- |
| Qualcomm | If per UE indication is agreed, RAN2 may need to further discuss whether TDD/FDD differ and whether FR1/FR2 diff. But it can be further discussed in RAN2#117-e. |
| Xiaomi | As we discussed in R2-2200796, Remote UE and relay UE may have different capabilities regarding SIB reception. It’s possible relay UE may not be able to forward certain requested SIBs from remote UE. In legacy, sidelink peer UEs would exchange capability via PC5 RRC to acknowledge each other’s capability, so to ensure subsequent reconfiguration doesn’t exceed peer UE’s capability. It’s straightforward to reuse the legacy procedure in relay, i.e. relay and remote UE exchanges each other’s capability via PC5 RRC. Therefore, remote UE would not request SIBs which is not supported by relay UE. Note the capability exchanged via PC5 RRC in R16 is only sidelink capability. Remote UE is not able to acknowledge the relay UE’s Uu capability. Therefore, relay UE should indicate which SIB could be forwarded to remote UE. Furthermore, the content of SIB may be extended in each release, due to new feature introduction. Therefore, it’s beneficial for relay UE to also indicate the release version. Relay UE indicates its SI forwarding capability to remote UE during capability exchange. The SI forwarding capability could include relay UE’s supported release version on Uu or SIBs which relay UE is capable to forward. Remote UE would only request the SIBs which relay UE is capable to forward, according to its SI forwarding capability. |
| Ericsson | We need a bit more time to check this |
| Lenovo (we did not indicate support for option 3 but still please read comments | We do not agree that relay UE can only provide SIBs that it supports. Relay UE implementation can be futuristic to provide a future SIB (SI-message) to a remote requesting for it. We do not see any technical impediments for this. |
| Nokia | Agree with Ericsson, the discussion on option 3 (multiple separate capabilities for L2 relay) is a bit premature and should be postponed. |
| Apple | As explained above, Option 3 may be needed after examining all the agreements by the time when R17 design is completed |

Then, Rapporteur would like to discuss detailed signaling. In Rapporteur’s understanding, we have below options:

- Option 1: A list of band combination list which supports L2 relay functions / features (granularity depending on your answer to Q9)

- Option 2: A band list which supports L2 relay functions / features (depending on your answer to Q9)

- Option 3: A single bit on whether supporting L2 relay functions / features (depending on your answer to Q9). And the supported combination list is same as supportedBandCombinationListSidelink-r16

Q11) On signaling of L2 relay capability, do you prefer:

- Option 1: A list of band combination list which supports L2 relay functions / features (its granularity depending on your answer to Q9)

- Option 2: A band list which supports L2 relay functions / features (its granularity depending on your answer to Q9)

- Option 3: A single bit on whether supporting L2 relay functions / features (its granularity depending on your answer to Q9). And the supported combination list is same as supportedBandCombinationListSidelink-r16

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 3 | Similar to the answer to Q6, we tend to see the function of L2 relay is more of L2 feature, instead of RAN1/4 feature, so seems sufficient as per-UE capability? |
| Xiaomi | Option 3 | L2 relay is higher layer function, which is independent of band. |
| Qualcomm | Option 3 | We think the L2 relay UE operation and remote UE operation use PSSCH and PC5-RRC signaling. Thus, we think the supported band combination list should be same as supportedBandCombinationListSidelink-r16. |
| Ericsson | Option 3 |  |
| InterDigital | Option 3 | This is similar to discovery. |
| MediaTek | Option-1 or op-3 |  |
| CATT | Option 3 | Per-UE capability is preferred since it is not band-related. |
| Samsung | Option 3 |  |
| Nokia | Option 3 |  |
| vivo | Option 3 |  |
| ZTE | Option 3 |  |
| Apple | Option 3 |  |
| LG | Option 3 |  |
| Huawei, HiSilicon | Option 3 | Same as the capability for discovery. |
| Philips | Option 1 or 3 |  |
| Intel | Option 3 |  |

Finally, Rapporteur would like to check whether the signaling is indicated to gNB (i.e., in UECapabilityInformation), or peer UE (i.e., in UECapabilityInformationSidelink), or both.

Q12) On signaling of L2 relay capability, do you prefer:

- Option 1: Indicated to only gNB (i.e., only included in UECapabilityInformation)

- Option 2: Indicated to only peer UE (i.e., only included in UECapabilityInformationSidelink)

- Option 3: Indicated to both gNB and peer UE

| Company | Response | Comments |
| --- | --- | --- |
| OPPO | 1 | We see it depends on the output of Q9, i.e., if there is no further differentiation on specific relay functionality for remote/relay UE, no need for PC5-RRC signaling, since relay/remote UE capability is already reflected since RSC-broadcasting by relay UE, and DCR transmission by remote UE. Otherwise, if there is further differentiation on specific relay functionality for remote/relay UE, some PC5-RRC signaling exchange would be helpful. |
| Xiaomi | Option 1 and 2 | gNB shall be indicated with relay capability to provide necessary configuration. As we responded in Q10, peer UE shall exchange their SIB forwarding capability. Therefore, both gNB and peer UE should be indicated, but the content may be different. |
| Qualcomm | Option 1 as baseline | Same view as OPPO. If we identify requirement of capability for enhancement features, we can also agree them to be included in UECapabilityInformationSidelink |
| Ericsson | Option 1 and maybe Option 2 | Option 1 seems straightforward. For Option 2 we see some benefits in doing it even if this may not be super essential. On the other side, if we decide this capability to be only 1 bit we don’t see any showstopper to include this also in the UECapabilityInformationSidelink. |
| Lenovo | Option 1 | Seems sufficient. |
| InterDigital | Option 1 | For now we can assume option 1 is the baseline and further discuss if option 2 is needed. |
| MediaTek | Option-1 |  |
| CATT | Option 1 and 2 | We reckon that the interaction in PC5 is needed. |
| Samsung | Option 1 | For U2N relay, capability information signaling with gNB seems sufficient. |
| Nokia | Option 1 and 2 | We are fine with both options. |
| vivo | Option 1 and Option 2 | Agree with Ericsson. Even if we don’t prefer to further differentiate on specific relay functionalities (e.g., SI delivery, PO monitoring) for remote/relay UE, the PC5-RRC signaling exchange would helpful for the subsequent L2 CP procedures and just 1-bit is enough. |
| ZTE | Option 1 | The indication to gNB is enough. It is not clear what the UE can do with the L2 relay capability indication received from peer UE. |
| Apple | Option 1. FFS option 2 | Option 2 may be needed if there are some addition L2 relay specific capabilities needs to be exchanged between UEs. |
| LG | Option 1 | We need further discussion for option 2. If L2/L3 relay support indication is included in the discovery message, option 2 may not necessary. |
| Huawei, HiSilicon | Option 1 as baseline | Same as the comments to Q9, we think Option 1 can be the baseline and if multiple separate capability for some features is justified, then we can check whether these capabilities should be exchanged between UEs. |
| Philips | Option 1 and maybe Option 2 | We agree with Ericsson |
| Intel | Option 1 and Option 2 |  |

Finally, companies are welcome to provide inputs on other aspects on L2 relay capabilities.

Q13) Any other comments on L2 relay capabilities (e.g., new separate capability, missed aspects)?

| Company | Comments |
| --- | --- |
| Qualcomm | If per UE indication is agreed, RAN2 may need to further discuss whether TDD/FDD differ and whether FR1/FR2 diff. But it can be further discussed in RAN2#117-e. |

### NR L3 relay related capabilities

We discuss NR L3 relay capability in this section. Please note that LTE has provided similar capability (as illustrated in Table. 2). Rapporteur think it is straight forward to introduce similar capability for NR L3 relay, but need to get companies’ input.

Q14) For L3 relay, do you agree to introduce below capability similar to LTE?

- Introduce 2 separate optional UE feature without UE radio access capability parameters for NR L3 relay UE operation and remote UE operation (e.g., with name “L3 Sidelink Relay UE operation” and “L3 Sidelink Remote UE operation”):

Definitions for feature

L3 sidelink relay UE operation It is optional for UE to support L3 sidelink relay UE operation as specified in TS 38.331 [9].

L3 sidelink remote UE operation It is optional for UE to support L3 sidelink remote UE operation as specified in TS 38.331 [9].

| Company | Response (Y/N) | Comments (if company answered No due to significant concerns) |
| --- | --- | --- |
| OPPO | Y |  |
| Xiaomi | Y |  |
| Qualcomm | Y | For Xiaomi’s comment, optional UE feature without UE radio access capability parameters is for the features which is optional for UE support but no need to be indicated to gNB (e.g., features only for IDLE/INACTIVE UE) |
| Ericsson | Y |  |
| InterDigital | Y |  |
| MediaTek | Y |  |
| CATT | Y |  |
| Samsung | Y |  |
| Nokia | Yes |  |
| vivo | Y |  |
| ZTE | Y |  |
| Apple | Yes |  |
| LG | Y |  |
| Huawei, HiSilicon | N | As agreed in previous RAN2 meeting, when in connected state, it is gNB to configure UE with dedicated configuration, including Uu RSRP threshold to be a relay UE, PC5 RSRP threshold for relay (re)selection. These configurations are applied to both L2 and L3 relay. In this cases, from NW perspective, we think it is necessary for gNB to know the UE capability of L3 remote UE and L3 relay UE. |
| Philips | Y |  |
| Intel | Y |  |

Finally, companies are welcome to provide inputs on other aspects on L3 relay capabilities.

Q15) Any other comments on L3 relay capabilities (e.g., new separate capability, missed aspects)?

| Company | Comments |
| --- | --- |

# Conclusion

TBD based on companies’ input

# References

[1] RP-210904, New WID on NR Sidelink Relay.

[2] TS 36.306 v16.3.0, User Equipment (UE) radio access capabilities (Release 16)

[3] TS 38.306 v16.3.0, User Equipment (UE) radio access capabilities (Release 16)

[4] R2-2200178, Initial consideration on UE capability of sidelink relay, Qualcomm Incorporated