3GPP TSG-RAN WG2 #116b-e											R2-22xxxxx

Online, 17 – 25 November 2021

Agenda item:	8.15.3

Source: 	LG Electronics Inc. (Rapporteur)

Title: 	Summary [AT116b-e][704][V2X/SL] Resource allocation enhancements

Document for:	Discussion and decision

# Introduction

This document summarizes the offline discussion as:

- [AT116bis-e][704][V2X/SL] Resource allocation enhancements (LG)

Scope: Identify real RAN2 scopes/issues to be discussed /decided in RAN2. Rapporteur should check each issue whether RAN2 should discuss/decide it or RAN2 leaves it to RAN1.

Intended outcome:  Summary discussion in R2-2201804

Deadline: 1/24 13:00 UTC

| Company | Name | E-mail |
| --- | --- | --- |
| LG | Giwon Park | giwon.park@lge.com |
| Ericsson | Min Wang | min.w.wang@ericsson.com |
| InterDigital | Martino Freda | martino.freda@gmail.com |

# The list of issues to be discussed

Table 1: The list of issues on resource allocation enhancements RAN2 scope

| Tdoc | Title | Company | Summary of proposals | Rapporteur’s view |
| --- | --- | --- | --- | --- |
| R2-2200263 | Discussion on inter-UE coordination | ZTE | Issues pointed out in the contribution: - Resource pool selection schemes for inter-UE coordination - Priority,HARQ attribute, HARQ retransmission number, PDB of MAC CE for inter-UE coordination information - LCP for inter-UE coordination MAC CE - Inter-UE Coordination MAC CE design (N combinations can be included in the MAC CE) | Inter-UE coordination MAC CE's RAN2 scope can be discussed. - LCP for Inter-UE Coordination MAC CE RAN2 can discuss MAC CE format of inter-UE coordination after RAN1 determine the details of the coordination information. For resource pool selection scheme, suggest waiting for more RAN1 progress. |
| R2-2200317 | Consideration on Resource Allocation Enhancements | CATT | Issues pointed out in the contribution: - Resource pool configuration for random resource selection and partial sensing - Introduce a new MAC CE to carry inter-UE coordination information | Proposals related to inter UE coordination MAC CE can be discussed together with proposals for RAN2 scope of inter-UE coordination MAC CE submitted by other companies. For resource pool configuration for random resource selection and partial sensing, suggest waiting for more RAN1 progress. |
| R2-220349 | Discussion on candidate resource selection with DRX and inter-UE coordination | NEC | Issues pointed out in the contribution: - Candidate resource considering RX UE’s active time - Recommended resource set of inter-UE coordination considering UE-A’s active time/inactive time | For candidate resource considering RX UE’s active time, RAN2 is discussing this issue in the SL DRX. - Recommended resource set of inter-UE coordination considering UE-A’s active time/inactive time can be discussed because it is a RAN2 specific issue. |
| R2-2200375 | Discussion on resource allocation enhancement | OPPO | Issues pointed out in the contribution: - Resource pool configuration for random resource selection and partial sensing - UE selection of resource allocation scheme - Alignment between partial sensing and SL DRX - Behaviour of UE-A for (non-)preferred resource set determination in scheme 1 - Scheme-1 IUC MAC CE design - Timer based approach for inter-UE coordination - Inter-UE Coordination Information Request - Only unicast can be supported for scheme 1 - MAC procedure using preferred resource set IUC report - Scheme 2 inter-UE coordination design | Following issues can be discussed: - Timer based approach for Inter-UE coordination - Cast type of Inter-UE coordination MAC CE - Scheme-1 IUC MAC CE design For the rest of the proposals, suggest waiting for more RAN1 progress. |
| R2-2200379 | RAN2 aspects on resource allocation enhancements for Rel-17 eSL | vivo | Issues pointed out in the contribution: - power-saving resource allocation (i.e. partial sensing and/or random selection) - Resource pool selection and resource allocation scheme selection - Separate pool configuration IE - report the type of the NR SL transmissions - CBR measurement - Random selection in exceptional pool | For power-saving resource allocation (i.e. partial sensing and/or random selection), suggest waiting for more RAN1 progress. |
| R2-2200485 | Consideration on resource allocation enhancement | Huawei | Issues pointed out in the contribution: - Priority value/order of inter-UE coordination MC CE - HARQ feedback option of inter-UE coordination MAC CE - Detailed contents of SL inter-UE coordination MAC CE - Inter-UE coordination in scheme 1 can be supported for unicast only. - Mode info (e.g., mode 1 or mode 2) negotiation for inter-UE coordination - Condition for the coordinating UE to transmit the inter-UE coordination information | Following RAN2 specific issues for inter-UE coordination MAC CE can be discussed with other companies’ proposals except for issues under discussion in RAN1: - Priority value/order of inter-UE coordination MC CE - HARQ feedback option of inter-UE coordination MAC CE - Inter-UE coordination in scheme 1 can be supported for unicast only. |
| R2-2200529 | On resource allocation and inter-UE coordination | Intel | Issues pointed out in the contribution: - power-saving resource allocation (i.e. partial sensing and/or random selection) - Resource re-evaluation and pre-emption for partial sensing or random resource selection. - design of CBR measurement and CR evaluation mechanism - Unicast inter-UE coordination - Event triggered reporting of inter-UE coordination information - Support SL inter-UE coordination MAC CE for scheme 1 - Contents of inter-UE coordination MAC CE | Following RAN2 specific issues for inter-UE coordination MAC CE can be discussed with other companies’ proposals except for issues under discussion in RAN1: - Support unicast inter-UE coordination - support SL inter-UE coordination MAC CE for scheme 1 |
| R2-2200537 | Discussion on Inter-UE coordination | LG | Issues pointed out in the contribution: - priority order of inter-UE coordination MAC CE - cast type of inter-UE coordination MAC CE - multiplexing the inter-UE coordination MAC CE with data - unicast inter-UE coordination MAC CE - “set of recommended resources of inter-UE coordination” considering UE-A’s DRX active time | Following RAN2 specific issues for inter-UE coordination MAC CE can be discussed with other companies’ proposals: - priority order of inter-UE coordination MAC CE - cast type of inter-UE coordination MAC CE - multiplexing the inter-UE coordination MAC CE with data - unicast inter-UE coordination MAC CE - “set of recommended resources of inter-UE coordination” considering UE-A’s DRX active time |
| R2-2200642 | Discussion on resource allocation enhancement for NR sidelink | Spreadtrum Communication | Issues pointed out in the contribution: - Cast types for inter-UE coordination - MAC CE format of SL inter-UE coordination MAC CE for scheme 1 | Cast types for inter-UE coordination can be discussed. RAN2 can discuss MAC CE format of inter-UE coordination after RAN1 determine the details of the coordination information. |
| R2-2200750 | Discussion on inter-UE coordination | AUSTek | Issues pointed out in the contribution: - Priority order of inter-UE coordination MAC CE - Feedback option of inter-UE coordination MAC CE | Following RAN2 specific issues can be discussed: - Priority order of inter-UE coordination MAC CE - Feedback option of inter-UE coordination MAC CE |
| R2-2200763 | RAN2 impacts on SL Resource allocation enhancements | Lenovo, Motorola Mobility | Issues pointed out in the contribution: - Signaling design and trigger conditions for the request from UE-B to UE-A - Periodic transmission and event-triggered transmission for inter-UE coordination - Resource selection triggering by considering the SL RX active time - signalling of preferred set of resources in the assistance information considering the common active period | Following RAN2 specific issues can be discussed except for issues under discussion in RAN1: - - Resource selection triggering by considering the SL RX active time - - signalling of preferred set of resources in the assistance information considering the common active period |
| R2-2200792 | Discussion on inter-UE coordination impact in RAN2 | Xiaomi | Issues pointed out in the contribution: - Priority order of inter-UE coordination MAC CE - Feedback option of inter-UE coordination MAC CE - Sending a zone ID to UE-B | Following basic issues of inter-UE coordination MAC CE first can be discussed: - Priority order of inter-UE coordination MAC CE - Feedback option of inter-UE coordination MAC CE |
| R2-2200799 | On Signalling for Inter UE Coordination | Nokia | Issues pointed out in the contribution: - Support SL inter-UE coordination MAC CE for scheme 1 - Inter-UE Coordination MAC CE design (flexible MAC CE or fixed size) - RAN1 related parameters for inter-UE coordination | Following RAN2 specific issues can be discussed except for issues under discussion in RAN1: - Support SL inter-UE coordination MAC CE for scheme 1 RAN2 can discuss MAC CE format of inter-UE coordination after RAN1 determine the details of the coordination information. |
| R2-2200939 | MAC CE design of inter-UE coordination | Ericsson | Issues pointed out in the contribution: - Triggering condition of request message in scheme 1 - Contents of request message in scheme 1 - Latency bound for inter-UE coordination information transmission - Priority order of inter-UE coordination MAC CE - Transmission of standalone inter-UE coordination MAC CE and multiplexing the inter-UE coordination MAC CE with data | Following RAN2 specific issues can be discussed except for issues under discussion in RAN1: - Latency bound for inter-UE coordination information transmission (can be discussed with timer based approach of R2-220375) - Priority order of inter-UE coordination MAC CE - Transmission of standalone inter-UE coordination MAC CE and multiplexing the inter-UE coordination MAC CE with data |
| R2-2201134 | Discussion on Inter-UE coordination | Apple | Issues pointed out in the contribution: - RRC (pre-)configurations for Inter-UE coordination per resource pool - IUC configuration in PC5-RRC - Triggering conditions for IUC schemes/options - PC5 RRC signaling (turn on/off IUC) - Cast types for inter-UE coordination | Following RAN2 specific issues can be discussed: - RRC (pre-)configurations for Inter-UE coordination per resource pool - IUC configuration in PC5-RRC - PC5 RRC signaling (turn on/off IUC) - Cast types for inter-UE coordination |
| R2-2201457 | Power reduction for Sidelink Mode 2 Resource Allocation | Fraunhofer | Issues pointed out in the contribution: - power-saving resource allocation (i.e. partial sensing and/or random selection) - dedicated resource pools or a set of dedicated radio resources within a resource pool for random resource selection - Random resource selection considering traffic load/CBR /priority - signaling to indicate the SL DRX active time of the receiver UE - Relay-based communication for battery-based UE - Inter-UE coordination to reduce power consumption - Location based power saving | For power-saving resource allocation (i.e. partial sensing and/or random selection), suggest waiting for more RAN1 progress. After discussing the RAN2 basic features for inter-UE coordination and power-saving resource allocation, it is suggested to discuss the issues below: - signaling to indicate the SL DRX active time of the receiver UE - Relay-based communication for battery-based UE - Inter-UE coordination to reduce power consumption - Location based power saving |
| R2-2201459 | Inter-UE Coordination for Sidelink Mode 2 Resource Allocation | Fraunhofer | Issues pointed out in the contribution: - details of the inter-UE coordination information - triggering condition for inter-UE coordination - The type of a UE (e.g. V-UE, RSU-type UE, battery-based UE) should be considered to determine UE-A | The issue under discussion in RAN1 below suggests waiting for the results of RAN1: - details of the inter-UE coordination information - triggering condition for inter-UE coordination After discussing the RAN2 basic features for inter-UE coordination, it is suggested to discuss the issues below: - The type of a UE (e.g. V-UE, RSU-type UE, battery-based UE) should be considered to determine UE-A |
| R2-2201479 | Interaction between partial sensing and DRX | Ericsson | Issues pointed out in the contribution: - Combining partial sensing and sidelink DRX - SL DRX considering the partial sensing | Sensing related issues are closely related to RAN1 and should be discussed first in RAN1. |
| R2-2201591 | Resource allocation enhancements | Samsung | Issues pointed out in the contribution: - power-saving resource allocation (i.e. partial sensing and/or random selection) - Resource pool configuration/resource pool selection | For power-saving resource allocation (i.e. partial sensing and/or random selection), suggest waiting for more RAN1 progress. |
| R2-2201625 | Discussion on Inter-UE coordination | Qualcomm | Issues pointed out in the contribution: - Multiplexing the inter-UE coordination MAC CE with data - Transmission of inter-UE coordination MAC CE on dedicated resource - Only unicast can be supported for request based IUC | Following RAN2 specific issues can be discussed first except for RAN1 related topic: - Multiplexing the inter-UE coordination MAC CE with data - Only unicast can be supported for request based IUC |

Rapporteur summary

The following issues were pointed out in the companies' contributions at this meeting. For each issue, the reporter makes the following suggestions:

Yellow: RAN2 specific issues

Green: RAN1 related issues

- Inter-UE coordination

  - Priority, HARQ attribute, HARQ retransmission number, PDB of MAC CE for inter-UE coordination information

  - LCP for inter-UE coordination MAC CE

  - Inter-UE Coordination MAC CE design (N combinations can be included in the MAC CE)

  - Introduce  a new MAC CE to carry inter-UE coordination information

    - Scheme-1 IUC MAC CE design

  - Recommended resource set of inter-UE coordination considering UE-A’s active time/inactive time

  - Behaviour of UE-A for (non-)preferred resource set determination in scheme 1

  - Timer (e.g., latencybound) based approach for inter-UE coordination

  - Inter-UE Coordination Information  Request

  - Only unicast can be supported for scheme 1

  - MAC procedure using preferred resource set IUC report

  - Scheme 2 inter-UE coordination design

  - Priority value/order of inter-UE coordination MC CE

  - HARQ feedback option of inter-UE coordination MAC CE

  - Detailed contents of SL inter-UE coordination MAC CE

  - Mode info (e.g., mode 1 or mode 2) negotiation for inter-UE coordination

  - Condition for the coordinating UE to transmit the inter-UE coordination information

  - Signaling design and trigger conditions for the request from UE-B to UE-A

  - Cast types(UC/GC/BC) of inter-UE coordination MAC CE

  - Transmission of standalone inter-UE coordination MAC CE and multiplexing the inter-UE coordination MAC CE

  - Periodic transmission and event-triggered transmission for inter-UE coordination

  - Transmission of inter-UE coordination MAC CE on dedicated resource

Rapporteur suggests:

- RAN1 related issues (green mark) of Inter-UE coordination wait for the discussion result of RAN1 before starting a discussion in the RAN2.

- starting a discussion in RAN2 first for the RAN2 specific issues marked in yellow above.

- Power-saving resource allocation (i.e., partial sensing and/or random selection)

  - Resource pool configuration for random resource selection and partial sensing

  - Resource pool selection and resource allocation scheme selection

  - Alignment between partial sensing and SL DRX

  - Selection of resource allocation scheme

  - Report the type of the NR SL transmissions

  - CBR measurement

  - Random selection in exceptional pool

  - Resource re-evaluation and pre-emption for partial sensing or random resource selection

  - SL DRX considering the partial sensing

The issues of Power-saving resource allocation are all related to RAN1 and are issues under discussion in RAN1. Therefore, rapporteur suggests waiting for the results of RAN1 before starting the RAN2 discussion.

# Conclusion

The following issues were pointed out in the companies' contributions at this meeting. For each issue, the reporter makes the following suggestions:

Yellow: RAN2 specific issues

Green: RAN1 related issues

- Inter-UE coordination

  - Priority, HARQ attribute, HARQ retransmission number, PDB of MAC CE for inter-UE coordination information

  - LCP for inter-UE coordination MAC CE

  - Inter-UE Coordination MAC CE design (N combinations can be included in the MAC CE)

  - Introduce  a new MAC CE to carry inter-UE coordination information

    - Scheme-1 IUC MAC CE design

  - Recommended resource set of inter-UE coordination considering UE-A’s active time/inactive time

  - Behaviour of UE-A for (non-)preferred resource set determination in scheme 1

  - Timer (e.g., latency bound) based approach for inter-UE coordination

  - Inter-UE Coordination Information  Request

  - Only unicast can be supported for scheme 1

  - MAC procedure using preferred resource set IUC report

  - Scheme 2 inter-UE coordination design

  - Priority value/order of inter-UE coordination MC CE

  - HARQ feedback option of inter-UE coordination MAC CE

  - Detailed contents of SL inter-UE coordination MAC CE

  - Mode info (e.g., mode 1 or mode 2) negotiation for inter-UE coordination

  - Condition for the coordinating UE to transmit the inter-UE coordination information

  - Signaling design and trigger conditions for the request from UE-B to UE-A

  - Cast types(UC/GC/BC) of inter-UE coordination MAC CE

  - Transmission of standalone inter-UE coordination MAC CE and multiplexing the inter-UE coordination MAC CE

  - Periodic transmission and event-triggered transmission for inter-UE coordination

  - Transmission of inter-UE coordination MAC CE on dedicated resource

- Power-saving resource allocation (i.e., partial sensing and/or random selection)

  - Resource pool configuration for random resource selection and partial sensing

  - Resource pool selection and resource allocation scheme selection

  - Alignment between partial sensing and SL DRX

  - Selection of resource allocation scheme

  - Report the type of the NR SL transmissions

  - CBR measurement

  - Random selection in exceptional pool

  - Resource re-evaluation and pre-emption for partial sensing or random resource selection

  - SL DRX considering the partial sensing

Proposal 1. For RAN1 related issues (green mark) of Inter-UE coordination, RAN2 waits for the discussion results of RAN1 before starting the discussion.

Proposal 2. RAN2 starts the discussion for the following RAN2 specific issues:

- LCP for inter-UE coordination MAC CE

- Introduce a new MAC CE to carry inter-UE coordination information

- Whether to support timer (e.g., latency bound) based approach for inter-UE coordination

- Cast type to be supported in IUC scheme 1

- Priority value/order of inter-UE coordination MAC CE

- HARQ feedback option of inter-UE coordination MAC CE

- RRC (pre-)configurations for Inter-UE coordination per resource pool

- IUC configuration in PC5-RRC

- Whether to support PC5 RRC signaling (turn on/off IUC)

Proposal 3. For issues of power-saving resource allocation, RAN2 waits for the discussion results of RAN1 before starting the discussion.