3GPP TSG-RAN2#117-e	Tdoc R2-2202264

Electronic meeting, 2022-02-17 - 2022-03-03

Agenda Item:	8.24.3 Other

Source:	Ericsson

Title:	Remaining issues for MINT

Document for:	Discussion, Decision

# 1	Introduction

For MINT, there are at least three remaining open issues:

- How a disaster roaming LTE UE handles a cell that is "reserved for operator use".

Note: This is already agreed that NR UEs will treat such a cell as barred.

- What the single bit approach means. As was indicated in an earlier LS from CT1, the network can broadcast either a list of PLMNs for which disaster roaming is offered, or a single bit. How the single bit approach should work is still open in CT1.

- Whether MINT is supported for PNI-NPNs

This contribution discusses bullet 1 above since the other two is pending CT1 input.

# 2	Discussion

## 2.1	Reserved for operator use and Access Identity 3 in LTE

The running 38.304 (NR) CR captures that a UE assigned to Access Identity 3 should behave as a cell is "barred" if the cell is "reserved for operator use. But this not captured in the 36.304 (LTE) CR because 36.304 does not capture any wording about Access Identities.

The wanted wording in 36.304 is this:

### 5.3.1 Cell status and cell reservations [...] When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN, - UEs assigned to Access Class 11 or 15 (or corresponding Access Identity) operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved". - UEs assigned to an Access Class in the range of 0 to 9 (or corresponding Access Identity 0), 12 to 14 (or corresponding Access Identity) or to Access Identity 1, 2 or 3 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN. NOTE 3: ACs 11, 15 (or corresponding Access Identity) are only valid for use in the HPLMN/ EHPLMN; ACs 12, 13, 14 (or corresponding Access Identity) are only valid for use in the home country TS 22.011 [4]. NOTE 4: Access Identities 1, 2 are valid in the PLMNs as specified in TS 22.261 [x]. NOTE 5: Access Identity 3 is only valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN as specified in TS 22.261 [x].

The CRs in R2-2202220 and R2-2202221 are proposing to add Access Identity-handling in Rel-15 and Rel-16 as corrections (i.e., those Access Identities that exist in Rel-15/Rel-16). The MINT CR should therefore only capture the handling of Access Identity 3, meaning the following wording. During CR-implementation will turn in to the wanted wording above:

### 5.3.1 Cell status and cell reservations [...] When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN, - UEs assigned to Access Class 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved". - UEs assigned to an Access Class in the range of 0 to 9, 12 to 14 or to Access Identity 3 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN. NOTE 3: ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN; ACs 12, 13, 14 are only valid for use in the home country TS 22.011 [4]. NOTE X: Access Identity 3 is only valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN as specified in TS 22.261 [x].

We propose:

Update the MINT CR for 36.304 to capture handling of Access Identity 3 for when a cell is "reserved for operator use" as the TP above.

# 3	Conclusion

Based on the discussion above we propose:

Proposal 1	Update the MINT CR for 36.304 to capture handling of Access Identity 3 for when a cell is "reserved for operator use" as the TP above.