3GPP TSG-RAN WG2 Meeting #117-e                                                    R2-2202358

Electronic, 21th Feb - 3rd Mar, 2022

Source:              CATT

Title:	             Impacts on RAN of AN Release of Relay UE

Agenda Item:	8.7.2.1

Document for:	Discussion and Decision

Introduction

In this contribution, one cross-layer issue for L2 U2N relay will be discussed that is the RAN impact of AN release of relay UE.

Discussion

According to the description in [1], the AN release procedure is as below:

Figure-1 AN Release procedure

The initiation of AN release may be due to:

- (R)AN-initiated with cause e.g. O&M Intervention, Unspecified Failure, (R)AN (e.g. Radio) Link Failure, User Inactivity, Inter-System Redirection, request for establishment of QoS Flow for IMS voice, Release due to UE generated signalling connection release, mobility restriction, Release Assistance Information (RAI) from the UE, etc.; or

- AMF-initiated with cause e.g. Unspecified Failure, etc.

Regarding to the (R)AN-initiated AN release, the corresponding description in [3] is as below:

8.3.2 UE Context Release Request (NG-RAN node initiated) 8.3.2.1 General The purpose of the UE Context Release Request procedure is to enable the NG-RAN node to request the AMF to release the UE-associated logical NG-connection due to NG-RAN node generated reasons. The procedure uses UE-associated signalling. 8.3.2.2 Successful Operation Figure 8.3.2.2-1: UE context release request The NG-RAN node controlling a UE-associated logical NG-connection initiates the procedure by sending a UE CONTEXT RELEASE REQUEST message towards the affected AMF. The UE CONTEXT RELEASE REQUEST message shall indicate the appropriate cause value, e.g., "TXnRELOCOverall Expiry", "Redirection", for the requested UE-associated logical NG-connection release. …… 9.2.2.4 UE CONTEXT RELEASE REQUEST This message is sent by the NG-RAN node to request the release of the UE-associated logical NG-connection over the NG interface. Direction: NG-RAN node AMF IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. PDU Session Resource List; 0..1; YES; reject. >PDU Session Resource Item; 1..<maxnoofPDUSessions>; -. >>PDU Session ID; M; 9.3.1.50; -. Cause; M; 9.3.1.2; YES; ignore. …… 9.3.1.2 Cause The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Cause Group; M. >Radio Network Layer. >>Radio Network Layer Cause; M; …… User inactivity, ……. User inactivity; The action is requested due to user inactivity on all PDU sessions, e.g., NG is requested to be released in order to optimise the radio resources.. 8.3.3.2 Successful Operation Figure 8.3.3.2-1: UE context release: successful operation …… Upon reception of the UE CONTEXT RELEASE COMMAND message, the NG-RAN node shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message.

Based on the above description in [3], it is obvious that:

Observation 1: For a UE, when there is no data transmission/reception requirement on all its PDU sessions, NG-RAN can initiate the UE context release procedure. Once receiving the UE context release command from AMF, NG-RAN will release the RRC connection of the UE.

For L2 U2N relay UE, the UP protocol stack is as below. According to the UP protocol, for L2 U2N relay, the relay UE’s PDU session(s) is different from the remote UE’s PDU session(s).

Figure-2 UP protocol stack of L2 U2N relay

Observation 2: For L2 U2N relay, the PDU sessions of relay UE and remote UE are different.

Based on the above observation 1 and observation 2, one issue raised. The issue is that if relay UE is only used for remote UE data forwarding and relay UE itself has no data transmission/reception requirement on its own all PDU sessions, according to the current TS 38.413, whether the (R)AN-initiated AN release should be initiated? Based on the definition of user inactivity in TS 38.413, it is unclear whether the all PDU sessions refers all PDU sessions of relay UE itself or all PDU sessions including relay UE and its connected remote UEs.

| User inactivity | The action is requested due to user inactivity on all PDU sessions, e.g., NG is requested to be released in order to optimise the radio resources. |
| --- | --- |

If “all the PDU session” refers to all the PDU sessions including relay UE itself and its connected remote UE, (R)AN-initiated AN release is not allowed to be initiated at least one remote UE has data transmission/reception requirement on any PDU session.

If “all the PDU session” refers to all the PDU sessions of relay UE itself, (R)AN-initiated AN release is allowed to be initiated if relay UE is only used for remote UE’s data forwarding. In this case, it should further discuss whether step 3 ((R)AN Connection Release) of Figure-1 should be performed in the AN release procedure. If step 3 of Figure-1 is performed, the RRC connection of relay UE will be lost which will impact the remote UE’s data forwarding.

Considering the UE CONTEXT RELEASE is discussed in RAN3, suggest RAN2 to send LS to RAN3 to check the (R)AN behavior. In our view, it had better clarify that in case of L2 U2N relay, all PDU sessions in the definition of user inactivity should include the PDU sessions of both relay UE and its served remote UE.

Proposal 1: Send LS to RAN3 to check if relay UE is only used for remote UE data forwarding and it has no data transmission/reception requirement on its own PDU sessions, whether the NG-RAN can initiate the UE CONTEXT RELEASE procedure, and if it is allowed, how to handle the relay UE’s RRC connection.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Send LS to RAN3 to check if relay UE is only used for remote UE data forwarding and it has no data transmission/reception requirement on its own PDU sessions, whether the NG-RAN can initiate the UE CONTEXT RELEASE procedure, and if it is allowed, how to handle the relay UE’s RRC connection.

Reference

- TS23.502  Procedures for the 5G System (5GS); Stage 2

- TS23.501  System Architecture for the 5G System; Stage 2

- TS38.413   NG-RAN; NG Application Protocol (NGAP)