3GPP TSG-RAN WG2 #117-e	R2-22xxxx

E-meeting, 21 February – 03 March 2022

Agenda item:	8.4.1.3

Source: 	Samsung (rapporteur)

Title: 	Summary of discussion [AT117-e][014][eIAB] MAC (Samsung)

Document for:	Discussion and decision

# Introduction

This document captures the following discussion:

- [AT117-e][014][eIAB] MAC (Samsung)

Scope: Wait for RAN1 LS, kick off discussion when received. Based RAN1 LS and R2-2203278, progress remaining proposals (on MAC CEs). Determine agreeable parts, points for discussion if needed, open issues if needed. Aim for offline agreement, if not possible then pave the way for efficient on-line. This discussion will continue as post meeting discussion for MAC CR, and updated MAC CR (taking into acc this meetings agreements) can also be reviewed as part of this discussion.

Intended outcome: Report (assume that CR revision is not needed for CB).

Deadline: In time for on-line CB W2 Wednesday

Section 2 captures individual companies’ responses to proposals put together by the rapporteur, based on the LS from RAN1 in R2-2204012. (References are occasionally made in Section 2 below to the draft running eIAB MAC CR in R2-2203276.) The questions are based around new MAC CEs. Section 3 is open-ended and asks companies to share any eIAB-related MAC issues not covered in Section 2 (or in the parallel discussion on the running CR). In Section 4 the rapporteur proposes a way forward for RAN2, to be discussed further at RAN2#117-e, and used to revise the running MAC CR during or immediately after the ongoing RAN2#117-e meeting.

# Open issues

## MAC CEs for Rel-17 Desired Guard Symbols and Rel-17 Provided Guard Symbols

The following is the description of the relevant MAC CE (for the Desired Guard Symbols) put together by RAN1 and shared in the above-quoted LS:

Number of symbols the IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot for the following transitions between the IAB node MT and DU per cell:

- Case #6 MT TX to/from Case #1 DU RX

- Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU RX

- Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU TX

- Case #6 MT TX to/from Case #1 DU TX

The value range for the number of guard symbols as agreed and communicated by RAN1 is 0-7 symbols. The ‘default value’ is left for RAN2 to fill.

The following is the description of the relevant MAC CE (for the Provided Guard Symbols) put together by RAN1 and shared in the above-quoted LS:

Number of symbols the IAB node uses at the edge (beginning or end) of a slot for the following transitions between the IAB node MT and DU at the child node per cell:

- Case #6 MT TX to/from Case #1 DU RX

- Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU RX

- Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU TX

- Case #6 MT TX to/from Case #1 DU TX

The value range for the number of guard symbols as agreed and communicated by RAN1 is 0-7 symbols. The ‘default value’ is left for RAN2 to fill.

The rapporteur proposes the following:

- Remove the EN in section 5.18.19, as RAN1 have now confirmed their Working Assumption on the support of the switching scenarios covering Case-6 MT Tx to/from Case-1 DU TX.

- Remove the EN in section 6.1.3.x, as RAN1 have now confirmed the guard symbols range.

- There is no need for RAN2 to specify any ‘default value’.

- No further additional work (apart from proposals immediately above) is required by RAN2 to implement MAC CEs for Rel-17 Desired Guard Symbols and Provided Guard Symbols.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

## MAC CEs for Child IAB-DU Restricted Beam Indication and IAB-MT Recommended Beam Indication

The following is the description of the Child IAB-DU Restricted Beam Indication MAC CE put together by RAN1 and shared in the above-quoted LS:

Signaling from an IAB-node/IAB-donor to a child node indicating beams of the child IAB-DU in the direction of which simultaneous operation is restricted.

SSB ID (and additionally STC index, if needed) and/or CSI-RS ID can be used to indicate child IAB-DU’s restricted beams.

The indication can optionally comprise some combination (one or multiple) of the following IAB node’s parameters, associated with the indicated beam restriction:

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI

- Slot index

The value range as agreed and communicated by RAN1 is as follows:

The maximum number of restricted beams per DU cell in a given indication (including all associated parameters/conditions) is 8.

List of slots indicated by “slot index” can have the following ranges for periodicity: {16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120} slots.

The ‘default value’ is marked as N/A.

The following is the description of the IAB-MT Recommended Beam Indication MAC CE put together by RAN1 and shared in the above-quoted LS:

Signaling from an IAB-node to its parent-node indicating the recommended beams of the IAB-MT for DL RX beams and/or UL TX beams.

For DL Rx beam(s) indication, DL TCI state ID and RS ID (SSB ID and/or CSI-RS ID) can be used.

For UL Tx beam(s) indication, SRI can be used.

The indication can optionally comprise some combination (one or multiple) of the following IAB node’s parameters, associated with the indicated beam restriction:

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {MT CC} if independent of DU cell(s)

- Slot index

The value range as agreed and communicated by RAN1 is as follows:

The maximum number of recommended beams per MT CC in a given indication (including all associated parameters/conditions) is 8.

List of slots indicated by “slot index” can have the following ranges for periodicity: {16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120} slots.

The ‘default value’ is left blank.

The rapporteur proposes the following:

- RAN2 will design MAC CE(s) that support all possible combinations of configurations listed in relevant RAN1 descriptions of the parameters. No further input is expected on these MAC CE(s) from RAN1 as there are no FFSs in the LS against these MAC CE(s).

- Two MAC CEs are required, one for downlink on restricted beam information (parentchild), and another one for uplink on recommended beam information (childparent).

- Variable-length MAC CEs are used i.e. total length will vary based on specific combination of configurations being signaled.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

- The Child IAB-DU Restricted Beam Indication MAC CE will support all following options of indicating child IAB-DU’s restricted beams:

- SSB ID

- SSB ID and STC index

- CSI-RS ID

- 2 bits are set aside to indicate which of the 3 options in Proposal 8 is being signaled in a specific Child IAB-DU Restricted Beam Indication MAC CE.

- The IAB-MT Recommended Beam Indication MAC CE will support all following options of indicating child IAB-DU’s restricted beams:

- DL TCI state ID and SSB ID

- DL TCI state ID and CSI-RS ID

- 1 bits is set aside to indicate which of the 2 options in Proposal 10 is being signaled in a specific Child IAB-DU Restricted Beam Indication MAC CE.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

- The IAB-MT Recommended Beam Indication MAC CE and The Child IAB-DU Restricted Beam Indication MAC CE will support both the following options

- {MT CC, DU cell} pair,

- only {DU cell} if independent of MT CC(s).

- 1 bits is set aside to indicate which of the 2 options in Proposal 12 is being signaled in a specific Child IAB-DU Restricted Beam Indication MAC CE and IAB-MT Recommended Beam Indication MAC CE.

- RAN2 assumes that the maximum number of MT CCs and DU cells are respectively 32 serving cells for the UE/MT and 512 cells per DU.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

## MAC CE for Timing Case Indication

The following is the description of the Timing Case Indication MAC CE put together by RAN1 and shared in the above-quoted LS:

The parent-node indicates to an IAB-node a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot).

The value range as agreed and communicated by RAN1 is as follows:

{Case 1, Case 6, Case 7} per slot, for a number of slots. The list of slots can have the following ranges for periodicity: {16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120} slots.

The ‘default value’ is marked as N/A.

- MAC CE for Timing Case Indication contains the following information:

- 2 bits to identify the transmission mode to which the information in the MAC CE applies (Case-1/Case-6/Case-7)

- 4 bits to indicate the specific periodicity value from the set agreed by RAN1

- An absolute slot index (starting position) to which the information applies

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

## MAC CE for Case7 Timing Offset

The following is the description of the Case7 Timing Offset MAC CE put together by RAN1 and shared in the above-quoted LS:

The parent-node indicates to an IAB-node an offset to be used by the IAB-MT to set its UL TX timing based on the legacy TA loop and the indicated offset.

The value range as agreed and communicated by RAN1 is as follows:

FFS endpoint values. The granularity is the same as the UL TA granularity. The range is 12 bits.

The ‘default value’ is left for RAN2 to fill.

The rapporteur proposes the following:

- Keep the EN in 6.1.3.y, in light of the above FFS on endpoint values.

- There is no need for RAN2 to specify any ‘default value’.

- No additional work (apart from proposals immediately above) is required by RAN2 to implement MAC CEs for Case7 Timing Offset.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

## MAC CEs for Desired DL TX Power Adjustment and DL TX Power Adjustment

The following is the description of the Desired DL TX Power Adjustment MAC CE put together by RAN1 and shared in the above-quoted LS:

The IAB-MT indicates to its parent-node, its desired DL TX power adjustment to assist with the parent-node’s DL TX power allocation. The indication can optionally comprise some combination (one or multiple) of the following IAB node’s parameters, associated with the indicated desired DL TX power adjustment:

- Multiplexing mode

- MT’s DL beam: (TCI state ID and RS ID (SSB ID and/or CSI-RS ID) is used to indicate IAB-MT’s DL beam. If no information about the associated IAB-MT’s DL beams is present, the adjustment is applied to all MT’s DL beams.)

- (MT CC, DU cell) pair

- DU resource configuration: an indication of whether a desired power adjustment is applied on FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain and/or on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain, for a given (MT CC, DU cell).

- Slot index

The value range as agreed and communicated by RAN1 is as follows:

FFS range of values for DL TX power. List of slots indicated by “slot index” can have the following ranges for periodicity: {16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120} slots.

The ‘default value’ is marked as N/A.

The following is the description of the DL TX Power Adjustment MAC CE put together by RAN1 and shared in the above-quoted LS:

The parent-node indicates to the IAB-node an adjustment to the parent-node’s DL TX power (e.g., in response to receiving Desired DL TX Power Adjustment from the IAB-node). The indication can optionally comprise some combination (one or multiple) of the following IAB node’s parameters, associated with the indicated DL TX power adjustment:

- Multiplexing mode

- MT’s DL beam: (TCI state ID and RS ID (SSB ID and/or CSI-RS ID) is used to indicate IAB-MT’s DL beam. If no information about the associated IAB-MT’s DL beams is present, the adjustment is applied to all MT’s DL beams.)

- (MT CC, DU cell) pair

- DU resource configuration: an indication of whether a provided power adjustment is applied on FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain and/or on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain, for a given (MT CC, DU cell).

- Slot index

The value range as agreed and communicated by RAN1 is as follows:

FFS range of values for DL TX power. List of slots indicated by “slot index” can have the following ranges for periodicity: {16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120} slots.

The ‘default value’ is marked as N/A.

The rapporteur proposes the following:

- RAN2 will design MAC CE(s) that support all possible combinations of configurations listed in relevant RAN1 agreements. No further input is expected on this MAC CE from RAN1 (apart from the range of values for DL TX power).

- Two MAC CEs are required, one for downlink on DL TX Power Adjustment (parentchild), and another one for uplink on Desired DL TX Power Adjustment (childparent).

- Variable-length MAC CEs are used i.e. total length will vary based on specific combination of configurations being signaled.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

- The Desired DL TX Power Adjustment MAC CE and DL TX Power Adjustment MAC CE will support all following options of indicating MT’s DL beam to which the information applies:

- TCI state ID and SSB ID

- TCI state ID and CSI-RS ID

- No information on MT’s DL beam

- 3 bits are set aside to indicate which of the 3 options in Proposal 22 is being signaled in a specific Desired DL TX Power Adjustment MAC CE and DL TX Power Adjustment MAC CE.

- The Desired DL TX Power Adjustment MAC CE and DL TX Power Adjustment MAC CE will support all following options of indicating the DU resource configuration for a given (MT CC, DU cell) pair:

- provided power adjustment is applied on FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain

- provided power adjustment is applied on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain

- provided power adjustment is applied on both FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain, and on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain

- 3 bits are set aside to indicate which of the 3 options in Proposal 24 is being signaled in a specific Desired DL TX Power Adjustment MAC CE and DL TX Power Adjustment MAC CE.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree if the following comments are clarified: On P23 and P25, 2 bits may be sufficient to indicate 3 options instead of 3 bits. |

## MAC CE for Desired IAB-MT PSD range

The following is the description of the Desired IAB-MT PSD range MAC CE put together by RAN1 and shared in the above-quoted LS:

The IAB-node indicates to its parent-node, its desired PSD range to help with its MT’s UL TX power control. The indication can optionally comprise some combination (one or multiple) of the following IAB node’s parameters, associated with the indicated desired PSD range:

- Multiplexing mode,

- MT’s UL beam (SRI id; if information about the associated IAB-MT’s UL beams is not present, the PSD range is applied to all MT’s UL beams.),

- (MT CC, DU cell) pair,

- DU resource configuration: an indication of whether a desired UL PSD range is applied on FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain and/or on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain, for a given (MT CC, DU cell).

- Slot index

The value range as agreed and communicated by RAN1 is as follows:

FFS

The ‘default value’ is marked as N/A.

The rapporteur proposes the following:

- RAN2 will design MAC CE(s) that support all possible combinations of configurations listed in relevant RAN1 agreements. No further input is expected on this MAC CE from RAN1 (apart from the range of values).

- Variable-length MAC CE is used i.e. total length will vary based on specific combination of configurations being signaled.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

- The Desired IAB-MT PSD range will support both following options of indicating MT’s UL beam to which the information applies:

- SRI id

- No information on MT’s UL beam

- 1 bit is set aside to indicate which of the 2 options in Proposal 28 is being signaled in a specific Desired IAB-MT PSD MAC CE.

- Desired IAB-MT PSD MAC CE will support all following options of indicating the DU resource configuration for a given (MT CC, DU cell) pair:

- desired UL PSD range is applied on FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain

- desired UL PSD range is applied on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain

- desired UL PSD range is applied on both FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain, and on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain

- 3 bits are set aside to indicate which of the 3 options in Proposal 30 is being signaled in a specific Desired IAB-MT PSD MAC CE.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree if the following comments are clarified: On P31, 2 bits may be sufficient to indicate 3 options instead of 3 bits. |

## MAC CE for Timing Delta MAC CE (existing)

The following is the description of the update to the existing Timing Delta MAC CE MAC CE put together by RAN1 and shared in the above-quoted LS:

The Timing Delta MAC CE carries Tdelta which is used to determine the timing adjustment.

Upon reception of a Timing Delta MAC CE the IAB node shall:

- apply the value of Tdelta as specified in TS 38.213.

The Tdelta range is updated to support Case 6 timing.

The value range as agreed and communicated by RAN1 is as follows:

FFS: updated range of Tdelta

The ‘default value’ is marked as N/A.

The rapporteur proposes the following:

- Wait for the updated range before updating the existing Timing Delta MAC CE to support Case-6 timing. No other work on the Timing Delta MAC CE is needed.

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

## MAC CE for Case6 timing required

The following is the description of the Case6 timing required MAC CE put together by RAN1 and shared in the above-quoted LS:

A child IAB-MT can inform a parent node whether Case 6 timing is required for simultaneous operation.

The value range as agreed and communicated by RAN1 is as follows:

Not specified

The ‘default value’ is not specified.

The rapporteur proposes the following:

- Information carried in the Case6 timing required MAC CE is a single-bit indication (Case 6 timing is required for simultaneous operation/Case 6 timing is not required for simultaneous operation).

| Company | Response (agree/do not agree) |
| --- | --- |
| LGE | Agree |

# Any other issues

If companies feel certain issues have been left out from Section 2 above, and the parallel, ongoing discussion on running MAC CR, please would you share the details in the following Table:

| Company | Comments/details |
| --- | --- |

# Proposals for a way forward

Based on the responses received, the rapporteur is proposing the following for further discussion and confirmation at RAN2#117-e:…