- 3GPP TSG RAN WG2 Meeting #117-e	      	               R2-220xxxx
E-Conference, 21st Feb – 3rd Mar, 2022

Agenda item: 	8.22.4

Source:	Intel Corporation

Title: 	[AT117-e][020][MGE] UE capabilities -phase2 (Intel)

WID/SID:	NR_MG_enh-Core – Release 17

Document for:	Discussion and Decision

# Introduction

During online, RAN2 has agreed the following:

P1, P3, P5 are agreed:

- introduce (A) UE capability to support NCSG as indicated in RAN4 feature list as a baseline. It can be removed if no longer needed after more input from RAN4 on (B) UE capability to support NCSG pattern.

- introduce 1 bit UE capability to support concurrent gap.

- introduce separate UE capability for pre-configured measurement gap as follow:

- Pre-configured measurement gap with network-controlled activation and deactivation mechanism

- Pre-configured measurement gap with UE autonomous activation and deactivation mechanism

P2, P4, P6, P7 are also agreed (but need to care to avoid double work with R4):

- Wait for more input from RAN4 on (B) UE capability to support NCSG pattern.

- FFS additional UE capability for support perUE concurrent gap for index 2 only in addition to concurrent gap.

- FFS on if CA and non-CA case should have separate UE capability.

- FFS pre-configured gap should be FR differentiated.

This is to start offline discussion below:

- [AT117-e][020][MGE] UE capabilites (Intel)

Scope: Ph1:Based on R2-2203522. Determine agreeable parts, points for discussion, open issues if needed. Converge as far as possible to reduce the need for on-line discussion.

Ph2: Treat R2-2202462 and R2-2202463, collect comments revise accordingly and endorse.

Intended outcome: Ph2: Endorsed CRs for merge

Deadline: Ph2: EOM (3/3 08:00UTC)

# Phase 2 Discussion

During phase 1, there are a few FFS discussion point, rapporteur would like to get initial input from companies before final proposals made on those FFS points.

QC proposed to have an additional UE capability to indicate support of index-2 only from the RAN4 table due to some UE limitation.

| Qualcomm incorporated | Agree … please see comment | We have a concern that the UE may support legacy per-FR gap (independentGapConfig) and concurrent gap capability, but UE does not support the concurrent per-FR gaps, i.e. device only supports 2 concurrent per UE gap. Therefore, we recommend to introduce 2 capabilities instead of 1 capability: - Concurrent_perFR  UE supports the entire table proposed by RAN4 - Concurrent_perUE  UE supports only index-2 from the table proposed by RAN4 |
| --- | --- | --- |

Q1: Do you support to introduce a separate UE capability for UE only support index-2 from the table for concurrent gap?

| Company | Support/ no support | Comments |
| --- | --- | --- |
| Samsung | No support | We think that a separate UE capability for only index 2 is not needed. Further RAN4 has not requested for this additional capability and we don’t see any specific challenge from RAN2 side for supporting all indices. |

Oppo wonder if we need capability bit to indicate CA and non-CA case.

| OPPO | Agree | I wonder whether we need one bit capability to say it is for CA case or non-CA case in pre-configured gap case. |
| --- | --- | --- |

Q2: Do you think we need to have separate bit to indicate support of CA and non-CA case for pre-configured gap?

| Company | Support/ no support | Comments |
| --- | --- | --- |
| Samsung | No support | For automatic gap activation case, we think that the changes will be in RAN4 spec, and RAN4 hasn’t requested for separate gaps for CA case and non-CA case. For network activated case, we think it is simpler to have a single capability for CA and non-CA, considering only single BWP switching at a time, and we don’t see any specific challenge from RAN2 side for supporting it. |

Lastly, QC proposed that pre-configured gap to be FR Differentiated for interoperability testing.

| Qualcomm incorporated | Agree … please see comment | We strongly recommend the capability to be FR Differentiated for practical deployment (interoperability testing). |
| --- | --- | --- |

Q3: Do you support to change the UE capability for pre-configured gap to be FR Differentiated?

| Company | Support/ no support | Comments |
| --- | --- | --- |
| Samsung | No | From RAN4 LS- R2-2203877, it is per UE without the need for FR differentiation, and we don’t see any specific challenge from RAN2 side for supporting this. |

Summary:

TBD

# Conclusion

TBD

# References

[1] R2-2202324, Discussion on capability for MG enhancement, vivo

[2] R2-2202462, UE capability for NR and MR-DC measurement gap enhancements, Intel

[3] R2-2202463, UE capability for NR and MR-DC measurement gap enhancements, Intel

[4] R2-2202879, Discussion on UE capabilities of MGE, MediaTek Inc.

[5] R2-2202892, Discussion on UE capability for MGE, Huawei, HiSilicon

[6] R2-2203065, Discussion on UE capabilities for gap enhancement, Xiaomi Communications

[7] R2-2203450, UE capabilities for MGE, Ericsson

[8] R2-2203522, [Pre117-e][020][MGE] AI summary of 8.22.4 UE capabilities (Intel), Intel

[9] R2-2202462, CR 38.306, UE capability for NR and MR-DC measurement gap enhancements, Intel

[10] R2-2202563, CR 38.331, UE capability for NR and MR-DC measurement gap enhancements, Intel

[11] R4-2206282, LS on Rel-17 RAN4 UE feature list for NR, RAN4