3GPP TSG RAN WG2 Meeting #117-e	                                        R2-220xxxx

- Electronic, 21st February – 3rd March 2022

Agenda item:		6.1.4.4

Source:			Lenovo

Title:		Report from email discussion [AT117-e][036][NR1516] Idle Inactive procedures (Lenovo)

Document for:	 	Discussion and decision

# Introduction

This report summarizes the email discussion below that took place during RAN2#117-e meeting:

- [AT117-e][036][NR1516] Idle Inactive procedures (Lenovo)

Scope: Treat R2-2202539, R2-2202220, R2-2202221. Ph1 Determine agreeable parts, Ph2 for agreeable parts, progress CRs.

Intended outcome: Report, Agreed CRs.

# Reference

The following documents are treated in this email discussion:

[1]	R2-2202539	Correction for cell reselection on CAG cells in white list	Apple	CR	Rel-16	38.304	16.7.0	0229	-	F	NG_RAN_PRN-Core

[2]	R2-2202220	Addition of missing description on handling of Access Identities when cell is reserved for operator use	Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-15	36.304	15.7.0	0837	-	F	LTE_5GCN_connect-Core

[3]	R2-2202221	Addition of missing description on handling of Access Identities when cell is reserved for operator use	Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell, Ericsson	CR	Rel-16	36.304	16.6.0	0838	-	A	LTE_5GCN_connect-Core

# Contact information

| Company | Contact Name, Email |
| --- | --- |
| Lenovo | Hyung-Nam Choi, hchoi5@lenovo.com |

# Discussion

## Cell reselection on CAG cells in white list

TS 38.331 includes “intraFreqCAG-CellList-r16” and “interFreqCAG-CellList-r16” per PLMN in SIB3/4. In the concerned field descriptions, a reference to TS 38.304 is provided but in TS 38.304 the description of how to use them is missing. Therefore, the CR [1] proposes to add the following requirement for cell reselection of CAG-only UEs in TS 38.304 R16, subclause 5.2.4.1 (Reselection priorities handling):

If the CAG-only indication in the UE is set, the UE shall consider only white listed cells from the cag-CellList, if configured, as candidates for cell reselection.

Rapporteur’s comment: As chairman commented the language in the CR may need to be aligned since the term “white list” is no longer allowed. During inclusive language discussion it was agreed to replace the term “whitelisted” by “allow-listed” and “CSG whitelist” by “Permitted CSG list”.

Question 1: Do companies agree on the proposed change to TS 38.304 R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | No | Prioritization of CAG cells during cell reselection by a CAG capable UE was discussed but not agreed. The following agreement was made in RAN2#109-e meeting (Feb/Mar 2020) during “offline discussion 117: [PRN] Cell Selection and selection - Intermediate status”, see R2-2001680: 10. CAG-capable UE is not allowed to reselect to a CAG member cell ignoring highest ranked cell or best cell acc. To absolute priority reselection rules The above agreement also applies in case the CAG-capable UE is configured with CAG-only indication. |

## Handling of Access Identities when cell is reserved for operator use

There are following issues in TS 36.304:

- The cell access in 5GC is based on Access Identities and a 5GC-capable UE can be allocated with one or multiple Access Identities which are specified in TS 22.261, subclause 6.22.2.2, Table 6.22.2.2-1. However, in the current specification of TS 36.304, subclause 5.3.1 any description on handling of Access Identities is missing.

- Acc. to TS 22.261 the Access Classes 0 to 9 are equivalent to Access Identity 0 and Access Classes 11 to 15 are equivalent to Access Identities 11 to 15.

- Acc. to current specification a 5GC-capable UE that is assigned with Access Identity 1 or 2 may treat a 5GC cell as candidate during the cell selection/reselection procedures even it is reserved for operator use. But this is not the intended behaviour. In NR it has been specified in TS 38.304, subclause 5.3.1 that UEs that are assigned with Access Identity 1 or 2 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN, see below.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN and not "true" for other use, - UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved". - UEs assigned to an Access Identity 0, 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN. NOTE 1: Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN; Access Identities 12, 13, 14 are only valid for use in the home country as specified in TS 22.261 [12].

To address the above issues, in the CRs [2], [3] the following changes are proposed to TS 36.304 for R15 and R16:

- In subclause 2 to add a new reference to TS 22.261.

- In subclause 5.3.1 to specify that UEs assigned with Access Identity 1 or 2 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN. Furthermore, to clarify that Access Classes 0 to 9 are equivalent to Access Identity 0 and Access Classes 11 to 15 are equivalent to Access Identities 11 to 15.

Question 2: Do companies agree on the proposed changes to TS 36.304 for R15 and R16?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Lenovo | Yes | Proponent |

# Conclusion

Based on company’s feedback the following proposals are made:

<To be updated>