GPP TSG-RAN WG2 #117	R2-220xxxx

- Online, February 21 – March 3, 2022

Agenda item:	8.6.3

Source:	Samsung

Title:	Summary of [AT117-e] [501] [ Sdata] CP additional open issues (Samsung)

Document for:	Discussion & Decision

# Introduction

This document summarizes the following email discussion.

- [AT117-e] [501] [Sdata] CP additional open issues (Samsung)

Remaining CP open issues

Deadline: Proposals by rapporteur by Friday (intermediary deadlines for comments to be set by rapporteur)

Deadline for companies’ inputs: Thursday, Feb 24th, 12:00 UTC

# Discussion on selected proposals from CP open issue list summary

## DVT Threshold configuration

P17: DataVolumeThreshold is configured only in SIB1

<Rapporteur Comments>: This seems to be the general preference and this is already implemented in the running CR. I think this may be easily agreeable.

Q1a: Do you agree that DataVolumeThreshold is configured only in SIB1?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Yes | It seems this is fine. We also now added the Txxx configuration also in SIB1 (similar to legacy where the error detection timers are configured in SIB1). Please see: R2-2203296. |
| Nokia | - Yes |  |
| Samsung | - Yes |  |
| LGE | - Yes |  |
| Sharp | - Yes |  |
| ASUSTeK | - Yes |  |

P16: The following is used for sdt-DataVolumeThreshold

ENUMERATED {byte10, byte14, byte20, byte28, byte38, byte53, byte74, byte102, byte142, byte198, byte276, byte384, byte535, byte745, byte1038, byte1446}

Q1b: Provide your comments on the preferred values (if any) in the table below?

| Company | Comments (if any) |
| --- | --- |
| ZTE | Traditional TCP IP header is 20 bytes and may be one value for this and considering we have ROHC option, one value below this value can also be considered. For the remaining values we don’t have a strong view. Please see a proposal below as starting point. ENUMERATED {byte10, byte14, byte20, byte28, byte38, byte53, byte74, byte102, byte142, byte198, byte276, byte384, byte535, byte745, byte1038, byte1446, byte2014} |
| Nokia | ENUMERATED {byte10, byte14, byte20, byte28, byte38, byte53, byte74, byte102, byte142, byte198, byte276, byte384, byte535, byte745, byte1038, byte1446} In case SDT resources are (temporarily) congested NW can signal 0 i.e. SDT is not temporarily allowed. Otherwise, NW need to disable whole SDT procedure. |
| Samsung | Ok with suggestion from ZTE |
| LGE | The data volume threshold is used for the UE to decide whether to trigger SDT procedure or transitions to RRC_CONNECTED. Thus, smaller values are not so useful in our view. In addition, considering that SDT procedure is typically used for messenger service, and the messenger service may transfer photo files, larger values may be needed. Thus, our suggestion is as follows. - for text messages: byte100, byte200, byte300, byte500 - for photo files: byte1M, byte2M, byte3M, byte5M The kbyte level threshold can be added if use case is identified. |
| Sharp | We are fine with suggestion from ZTE. |

## UE capabilities

P10: UE supporting CG-SDT shall also support 4-step RA-SDT (10/4)

Note clarification added to show it is 4-step RACH that is mandatory since 2-step RA is optiona1.

Q2: Do you agree that UE supporting CG-SDT shall also support 4-step RA-SDT?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Yes |  |
| Nokia | - Yes |  |
| Samsung | - Yes |  |
| LGE | - No | We prefer to remove dependencies between them. |
| Sharp | - Yes |  |
| ASUSTeK | - No | Same view with LGE. |

P11: RA-SDT is defined as an optional capability per UE without need for xDD and FRx differentiation

<Rapporteur Comments>: Per UE seems sufficient (per band seems not needed since there is no band specific impact on the overall procedure)?

Q3: Do you agree that RA-SDT is defined as an optional capability per UE without need for xDD and FRx differentiation?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Yes | Per band seems not needed since there is no band specific procedure involved in SDT. |
| Nokia | - Yes |  |
| Samsung | - Yes |  |
| LGE | - Yes |  |
| Sharp | - Yes |  |
| ASUSTeK | - Yes |  |

P12: CG-SDT is defined as an optional capability per UE without need for xDD and FRx differentiation

<Rapporteur Comments>: Per UE seems sufficient (per band seems not needed since there is no band specific impact on the overall procedure)?

Q4: Do you agree that CG-SDT is defined as an optional capability per UE without need for xDD and FRx differentiation?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Yes |  |
| Nokia | - Yes |  |
| Samsung | - Yes |  |
| LGE | - Yes |  |
| Sharp | - Yes |  |
| ASUSTeK | - Yes |  |

P13: Separate capability is needed for SRB (i.e. for NAS messages)

Q5: Do you agree that separate capability is needed for SRB (i.e. for NAS messages)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Yes | We can go with majority view and it seems chipset vendors prefer this. |
| Nokia | - No | SRB SDT is basic SDT functionality, and we see no any complication compared to DRB SDT. Additional capability would only complicate SDT operation from the network point of view. |
| Samsung |  | Not essential. Can go with majority. |
| LGE | - Yes |  |
| Sharp | - Yes |  |
| ASUSTeK | - No |  |

P14: Separate capability is not needed for multiple CG-SDT configured grants

<Rapporteur Comments>: Unclear why this is needed since UE can indicate the number of supported configured grants

Q6: Do you agree that separate capability is not needed for multiple CG-SDT configured grants?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Yes - (i.e. not needed) | It seems the UE can indicate the supported number of configured grants explicitly, this can be reused. |
| Nokia | - Yes - (i.e. not needed) |  |
| Samsung | - Yes - (i.e. not needed) |  |
| LGE | - Yes |  |
| Sharp | - Yes |  |
| ASUSTeK | - Yes |  |

## Error detection timer and CG-SDT periodicity

P5: use the following values for SDT error detection timer (discuss together with P18)

t3XX    ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms6000, ms10000, spare6, spare5, spare4, spare3, spare2, spare1}

P18: Implement longer CG-SDT periodicity values similar to PUR and send an LS to RAN1 to check if this is okay.

<Rapporteur Comments>: Companies think longer values are not good for SDT error detection. On the other hand, network vendors think longer periodicities are needed for CG-SDT. If long periodicities are agreed for CG and the error detection timer is too short, then it will expire before the CG occasion.

Q7: Do companies support long CG-SDT periodicities (if supported we need to send RAN1 LS – seems a straight forward change in RAN1 spec)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Probably No - (If we can agree solution for Q8 we are okay to consider these) | This is nice to have, but it will create problems for the error handling. We want to see if there is quick convergence on how to handle the error detection timer issue below if we allow larger values. On the one hand network vendors want the longer values but UE vendors prefer to keep the error detection timer value small. So, we propose to have a solution for both if we want to go this way. Otherwise, we should not include these longer values. |
| Nokia | - Yes | Due to subsequent transmissions CG-SDT periodicities needs to be longer compared to PUR |
| Samsung | - No |  |
| LGE |  | The value of t3XX indicated above seems ok. For the CG-SDT periodicities, it is not clear to us how long the periodicity is. We want to discuss it after the proposal becomes clear. However, our general view is that PUR and CG-SDT are different features, and the values in PUR may not be suitable for CG-SDT. Taking the above t3XX values as baseline, we think the maximum periodicity of CG-SDT should be less than ms1000. |
| Sharp | - No | Longer values should be avoided considering there is no RLM or BFD. If the quality of the link is deteriorated, longer value will cause unnecessary service delay. |
| ASUSTeK | - Yes |  |

Q8: Provide your comments on the preferred values of SDT error detection timer (if any) in the table below?

| Company | Comments (if any) |
| --- | --- |
| ZTE | On the timer values t3XX ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms6000, ms10000, spare6, spare5, spare4, spare3, spare2, spare1} Traditionally we have up to 2 sec. Clearly, we need some extension to this to cover the Xn round trip delay and application round trip time. So, we think a few more values could be added and we propose to add 3, 6 and 10 sec values. It should be noted that these are the maximum values and a typical network may configure lower values too. Further, the timer is there to cover the abnormal cases. In normal cases, the network will release the UE much earlier (once the data exchange ends). Long CG periods Firstly, we should be clear that such long CG periods will only be configured for devices that are known to never generate any time critical data (e.g. smart meters) – this should be known at service level to the network. Otherwise, there will be impact on the overall RRCResume duration for such device (i.e. the resume procedure can take in the worst case up to the longest CG period + error detection time value). It should also be noted that most likely there will be no chance for UE autonomous retransmission for the initial UL packet for such long periods (but may be this is not such a big issue since network can schedule dynamic grants in this case). Then, if we extend the CG periodicity to up to 20 sec, then we need to consider either extending this timer or delaying the start of this timer. Since simple extension of the timer may be problematic, we suggest that the start of the timer is delayed (i.e. start the Txxx after the MAC PDU containing the CCCH message has been transmitted by lower layers). With this approach we can avoid too much impacts to the specs and can include longer CG periods whilst keeping lower values for the error detection timer. So, we propose to handle it as follows: 1> if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled: 2> consider the resume procedure is initiated for SDT; 2> if CG-SDT is selected: 3> start the timer Txxx(NewSDTTimer) after the MAC PDU containing the CCCH message has been transmitted by lower layers; 2> else: 3> start timer Txxx(NewSDTTimer); |
| Samsung | Below set of values seems fine t3XX ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms6000, ms10000, spare6, spare5, spare4, spare3, spare2, spare1} |
| LGE | The value of t3XX indicated above seems ok. |

## Carrier selection

Proposal 7: When SDT is initiated, RRC will indicate the selected carrier to MAC

Note: MAC will still perform carrier selection for this and indicate this to RRC which will just be informed for the RACH selection purpose by RRC. As below:

Q9: Do you agree that when SDT is initiated, RRC will indicate the selected carrier to MAC?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZT | - Yes | We think this will simplify the modelling. Note that MAC will still do the carrier selection (as currently captured in MAC CR). It is only that this is informed back to RRC and will then be fixed for the remaining procedure. Other options of modelling are also possible, but we currently have such procedure already in RRC (e.g. in on demand SI) where RRC indicates the selected carrier to MAC. So, it seems this can be reused. |
| Nokia | - No | MAC will do the carrier selection and there is no need to do this kind of modeling |
| Samsung | - Yes |  |
| LGE | - Yes |  |
| Sharp | - Yes |  |
| ASUSTeK | - No | No need to specify in spec. |

## RACH failure handling

P8: RLC failure handling needs to be added in RRC but Max RACH preamble transmission indication from lower layers leads to no response in RRC (same as legacy).

Q10: Which of the following options do you agree for handling RACH failure (i.e. that Max RACH preamble transmission is reached) during SDT procedure?

Option 1: MAC indicates RACH problem indication to RRC. RRC does not any action for this indication similar to legacy operation in RRC_INACTIVE/RRC_IDLE. RA procedure is continued.

Option 2: SDT failure is triggered. Failure handling is same as in case of SDT timer expiry.

| Company | Option 1/2 | Comments (if any) |
| --- | --- | --- |
| ZTE | - Option 1 | We thought this was the majority view and is aligned with legacy procedure. In RRC, whilst T310 or T319 is running, RACH failure indication from lower layers is ignored. The same behaviour should be used also whilst Txxx (NewSDTTimer) is running. |
| Nokia | - Option 2 | With Option 1 and the extended values, UE may attempt RA for a very long time (even 10 seconds!) and generate interference to the SDT-RACH resources. |
| Samsung | - Option 1 | RACH is still performed in RRC_INACTIVE, so its ok to perform legacy procedure. |
| LGE | - Option 2 | Agree with Nokia. |
| Sharp | - Option 2 | Agree with Nokia. |
| ASUSTeK | - Option 2 | Agree with Nokia. Considering that the new T319 is extended to cover the subsequent transmission(s), the timer may keep running for a long time. Option 1 would make the UE to repeat the failed transmission before the timer expiry. |

## RRC Reject handling

P15: Discuss the following options for RRCReject

Option 1: No change (i.e. EDT behaviour is followed)

If option 1 is agreed, we can discuss whether we need a note that says: “UE shall avoid a consecutive SDT procedures with a different payload but same security key”

Option 2: RRCReject is not supported for SDT

Option 3: Release SDT configuration upon receiving RRCReject

Q11: Which of the following options do you agree for handling RRC Reject during SDT procedure?

| Company | Option 1/2/3 | Comments (if any) |
| --- | --- | --- |
| ZTE | - Option 1 | We prefer to stick with EDT behaviour. We can add a note to clarify UE behaviour and inform this to SA3 if companies prefer. Option 2 may be restrictive for network implementation but is acceptable from security perspective (we can accept this if this is the majority view) Option 3 is also okay, but we need to inform SA3 (we can check this along with moving to IDLE if companies prefer). But we think we should first try option 1 which is already accepted behaviour in EDT. |
| Nokia | - Option 1 |  |
| Samsung | - Option 1 |  |
| LGE | - Option 1 |  |
| Sharp | - Option 1 |  |
| ASUSTeK | - Option 1 |  |

Q12: if option 1 is agreed in Q11, do you agree to add a note that says: “UE shall avoid a consecutive SDT procedures with a different payload but same security key”?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| ZTE | - Okay | We think it is okay to capture a note (and we can also inform SA3). |
| Nokia | - No | Legacy behaviour for RRC Resume Request followed by RRC Reject can be followed. |
| Samsung | - - | No strong view |
| LGE | - No | Agree with Nokia. |
| Sharp | - No |  |
| ASUSTeK | - No |  |

## Non SDT Indication

Proposal x (new): For the non-SDT data arrival indication, whilst we wait for CT1 feedback, RAN2 can select between the following options for DCCH solution

Option 1: New message

Option 2: reuse UAI

Q13: Which of the following options do you agree for the non-SDT data arrival indication?

Option 1: New message

Option 2: reuse UAI

| Company | Option 1/2 | Comments (if any) |
| --- | --- | --- |
| ZTE | - Option 1 | It is a matter of taste and we can go with majority view since both options can be made to work. If UAI is adopted, we need to clarify which parts of UAI are allowed during SDT and which are not etc. We think it is cleaner to have a new message for this hence. |
| Nokia | - Option 2 | It seems easier to use existing procedure instead of defining completely new one. |
| Samsung | - Option 1 | Fine with either option. Option 1 is slightly preferred as it can have less overhead and is a clean approach. |
| LGE | - Neither | We prefer to reuse RRCResumeRequest message. |
| Sharp | - Option 2 |  |
| ASUSTeK |  | Agree with LGE. |

# Discussion on new issues submitted to 8.6.3

## Non-SDT data buffered at PDCP/RLC

| Source | Proposal |
| --- | --- |
| OPPO [1] | A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled: 1> the upper layers request resumption of RRC connection; and 1> the UE supports SDT; and 1> SIB1 includes sdt-ConfigCommon; and 1> all the pending data in UL is mapped to the radio bearers configured for SDT; and 1> lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled. It’s worth to confirm with companies how to understand ‘all the pending data in UL’, i.e., does it include the data buffered in PDCP/RLC. We provide our understanding as follows: - Option1: all the pending data including packets that have already buffered at PDCP/RLC. With this option, SDT cannot be triggered when there is non-SDT data buffered at AS. - Option2: all the pending data refers to new arriving data from upper layer, which means that SDT can be triggered even if there is non-SDT data available at PDCP/RLC. Then we would have to further address several issues, such as, how to restrict non-data transmission during SDT, whether UE needs to inform the network of the existing of non-SDT data. We think option1 is more reasonable and can make the procedure clear and simple. So we propose: - Proposal: SDT cannot be triggered if there is non-SDT data buffered at PDCP/RLC. |

Q14. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - Yes | - Agree | Sorry, misunderstood the original proposal! We agree with the proposal. But there should be no issue. The intention is as follows: - All RBs shall have the RLC bearers re-established upon RRCRelease if SDT is configured (this is currently missing in the RRC CR, but will be fixed) - PDCP suspend upon RRCRelease clears all buffered PDCP PDUs for DRBs (this is legacy behaviour and no changes needed) - SRB can use SDU discard procedure which also clears all old data (this ensures no pending data here too) – this is to be agreed yet and will be implemented once it is agreed. With this there will be no old RLC data or PDCP PDUs in non-SDT RBs. |
| Nokia | - Yes | - Agree |  |
| Samsung | - Yes | - Agree |  |
| LGE | - No | - Agree | If there is non-SDT data buffered at PDCP/RLC, the UE should trigger legacy RRCResume procedure instead of SDT procedure. But if there is non-SDT data buffered at PDCP/RLC, why did the network move the UE to RRC_INACTIVE? |
| Sharp | - No | - Agree | We share the same view with LGE. |
| ASUSTeK | - Yes | - Agree |  |

## SDT for SRB only

| Source | Proposal |
| --- | --- |
| Intel [4] | Proposal: To discuss whether SDT can be configured only for SRB (i.e. without resuming any DRB). If so, to define the sdt-DRB-List-r17 with the list starting in 0, or as SetupRelease type. |

<Rapporteur Views>: At least one of sdt-DRB-List-r17 and sdt-SRB2-Indication-r17 should be included in SDT-Config-r17. sdt-DRB-List-r17 can be absent instead of starting this list with zero.

Q15. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

Proposal: SDT can be configured only for SRB (i.e. without resuming any DRB)

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - Yes | - Agree | Seems no reason to restrict the network configuration to allow only SRB (e.g. this is useful for positioning applications). In this case, we are fine with the proposal to define the sdt-DRB-List-r17 with the list starting in 0, or as SetupRelease type. |
| Nokia | - Yes | - Agree | This is one of the work item objectives. |
| Samsung | - Yes | - Agree |  |
| LGE | - No | - Agree | It is up to network decision. |
| Sharp | - Yes | - Agree |  |
| ASUSTeK | - Yes | - Agree |  |

## NAS signalling transfer failure handling

| Source | Proposal |
| --- | --- |
| Huawei [11] | Proposal : If UE detects an SDT failure of ongoing SDT session for the transfer of NAS message, RRC informs NAS about the failure for NAS message transfer. |

Q16. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - Already clear in CR | - Already clear in CR | Whenever the resume procedure for SDT fails, then already according to the current CR, RRC will inform NAS about the cause 'RRC Resume failure'. NAS already is able to react to this failure cause and can initiate a new NAS procedure or initiate retransmission of pending data using existing NAS procedures. So, nothing new is needed. |
| Nokia |  |  |  |
| Samsung | - Already clear in CR | - Already clear in CR |  |
| Sharp | - Yes | - Agree |  |
| ASUSTeK | - Yes | - Agree |  |

## UAC for CG-SDT

| Source | Proposal |
| --- | --- |
| Lenovo [3] | Proposal: The access attempt is considered as allowed if the pre-configured CG resources are configured for SDT and the arrival data corresponds to the configured SDT DRB/SRB. |

<Rapporteur Views>: Optimisation, not essential.

Q17. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - No | - Disagree | Not essential. |
| Samsung | - No | - Disagree | Not essential |
| LGE | - No | - Disagree |  |
| Sharp | - No | - Disagree | Legacy UAC could be followed. Not sure why such an exception should be done |
| ASUSTeK | - No | - Agree |  |

## SDT and NR-U

| Source | Proposal |
| --- | --- |
| Intel [4] | - To agree that CG parameters specific to unlicensed spectrum are not applicable for SDT. If companies want to consider NR-U CG related parameters for SDT, RAN1 confirmation is required. |

Q18. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - No impact to CR? |  | We generally agree with the intention. But we are not sure if this will result in any changes to the CRs. It is not clear if this will have any impact to the CR. We can simply follow the WI description for unlicensed operation. We should keep in mind that there may be unlicensed bands which don’t need LBT with Rel-17 too. |
| Nokia | - No |  |  |
| Samsung | - No |  |  |
| LGE | - No | - Agree |  |
| Sharp | - No |  |  |
| ASUSTeK | - No | - Agree |  |

## I-RNTI in resume request

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal : Short version of the I-RNTI is used in RRC Resume Request for CG-SDT. |

<Rapporteur’s Views>: Optimisation, Not essential.

Q19. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - No | - Disagree | It is unclear why short I-RNTI only can be used since this is a network configuration (not just for SDT). |
| Nokia | - Yes | - Agree | Short version of the I-RNTI is sufficient for CG-SDT, because NW can identify easily identify where the UE context is, because CG-SDT is performed only on the same cell where it was configured. Actually, only the UE part would be needed for the identification. Bits can be saved for actual data transmission. |
| Samsung | - No | - Disagree | No need to have different behavior |
| LGE | - No | - Disagree | Optimization |
| Sharp | - No | - Disagree | Agree with rapporteur that it is an optimisation and not essential. |
| ASUSTeK | - No | - Disagree |  |

## NW RRC response during the SDT procedure

| Source | Proposal |
| --- | --- |
| Apple [2] | Proposal: During the SDT procedure, upon receiving RRCReject in response to the RRCResumeRequest, the UE shall suspend all the resumed SDT-RBs. |
| China Telecom [5] | Proposal 1: To keep the CG-SDT configuration still valid in the next RRC Resume procedure, the UE behavior upon reception of RRCReject should be modified. Proposal 2: To avoid the key stream reuse, UE can continue the PDCP COUNT value in the second RRC Resume procedure. |
| NEC [9] | Proposal: To avoid ciphering a different payload but same security key and same COUNT value after RRCReject reception during SDT, the UE goes to IDLE mode, and initiates RRC setup procedure |
| Huawei [14] | Proposal: When the UE receives RRCReject message in response to RRCResumeRequest message for SDT, the UE shall suspend all the RBs/PDCP entities that are configured for SDT and re-establish corresponding RLC entities |

<Rapporteur Views>: Handling of RRC Reject is covered by [POST116bis-e] [511] CP open issues list for SDT. No need to discuss here.

| Source | Proposal |
| --- | --- |
| Intel [4] | - Proposal: When UE initiates resume for SDT and network responds with RRCReject (for congestion), or RRCSetup (for fallback to setup), it is left up to UE the decision on how to handle any retransmission of the not acknowledged UL data that was included in the 1st UL SDT. Discuss whether TS needs to specify or clarify (e.g. with a NOTE) the related handling of the data for either scenario. |

Q20. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal(s)?

Proposal 1: When UE initiates resume for SDT and network responds with RRCReject (for congestion), or RRCSetup (for fallback to setup), it is left up to UE the decision on how to handle any retransmission of the not acknowledged UL data that was included in the 1st UL SDT.

Proposal 2: TS needs to specify or clarify (e.g. with a NOTE) the related handling of the data for either scenario.

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal(s) | Comments |
| --- | --- | --- | --- |
| ZTE | - No | - Agree | We don’t think it is essential but we are okay to capture the note if this is the majority view. |
| Nokia | - Yes | - Agree with proposal 2 | It needs to be specified how the data is handled. |
| Samsung | - No | - Agree | We don’t think it is essential but we are okay to capture the note if this is the majority view. |
| LGE | - Yes | - Disagree | The unacknowledged PDCP SDU will be retransmitted at initiation of next SDT procedure. No need for clarification. |
| Sharp | - Yes | - Agree | It should be clarified and a note could be helpful. |
| ASUSTeK | - No | - Disagree | No need for clarification in spec. |

## TA Validation

| Source | Proposal |
| --- | --- |
| Apple [2] | Proposal: For the RSRP change based TA validation mechanism, the RSRP result upon receiving the RRCRelease with CG-SDT configuration can be based on the broadcasted INACTIVE measurement configuration |

<Rapporteur Views>: This was discussed in [POST116bis-e][510][Sdata] UP open issues (Huawei). No need to discuss here.

| Source | Proposal |
| --- | --- |
| Apple [2] | In last RAN2 meeting it was agreed that the number of SSB for RSRP derivation is reused the nrofSS-BlocksToAverage configuration in SIB2. - Highest N SSBs of all SSBs actually transmitted as indicated in SIB is used for RSRP based TA validation. But in the RRC Running CR, the configuration is provided in the RRCRelease with SuspendConfig message, which means the configuration could be different from the value in SIB2. In our understanding, RAN2 previous agreement is to use the RSRP according to the legacy derivation method based on SIB2 configuration for the RSRP change based TA validation. It can simplify the UE operation for the cell level RSRP derivation in INACTIVE state. Therefore, the new configuration in the UE dedicated signaling is not needed. Proposal: The nrofSS-BlocksToAverage configuration in SIB2 is reused for the RSRP change based TA validation. <Rapporteur Views>: This was discussed in [POST116bis-e][510][Sdata] UP open issues (Huawei). No need to discuss here. |

Q21. Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

Proposal: The nrofSS-BlocksToAverage configuration in SIB2 is reused for the RSRP change based TA validation.

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - No | - Agree | We think the proposal is reasonable (we can make the IE optional with need S) in this case. But this is an optimisation. So, agree if there is consensus. |
| Nokia | - No | - Disagree |  |
| Samsung | - No | - Agree | In this case we do not need to include this in RRC Release |
| LGE | - No | - Agree |  |
| Sharp | - No | - Disagree |  |
| ASUSTeK | - Yes | - Disagree | It could be separately configured. |

## RAN-based Notification Area Update

| Source | Proposal |
| --- | --- |
| Lenovo [3] | Proposal: CG resource can be used for RRCResumeRequest transmission for RNAU purpose if there is not ongoing SDT. <Rapporteur Views>: Optimisation for RNAU, not essential. Proposal does not impact SDT procedure. |
| Qualcomm [8] | Proposal: T380 stops upon initiation of SDT procedure and starts when receiving the RRCRelease message as legacy. <Rapporteur Views>: RAN2 has agreed that “UE shall not perform any periodic RNA update during SDT procedure. The rapporteur will find a simple solution to capture this in the RRC, aiming to follow legacy behaviour (i.e. keep T380 running)”. Considering this agreement and RRC CR further discussion is not needed. |
| Huawei [11] | Proposal: If T380 expires and UE receives RRCReject during SDT procedure, the following options can be considered to keep the periodic RNAU: - Option 1: UE triggers the RNAU procedure directly; - Option 2: UE re-starts T380 and triggers periodic RNAU when T380 expires; - Option 3: UE sets the variable pendingRNA-Update to true. |

Q22: Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, which option do you prefer?

Proposal: If T380 expires and UE receives RRCReject during SDT procedure, the following options can be considered to keep the periodic RNAU:

- Option 1: UE triggers the RNAU procedure directly;

- Option 2: UE re-starts T380 and triggers periodic RNAU when T380 expires;

- Option 3: UE sets the variable pendingRNA-Update to true.

| Company | Essential Issue (Yes/No) | Preferred Option (option 1/2/3/other (if any) | Comments |
| --- | --- | --- | --- |
| ZTE | - Yes | - Option 3 | Something similar to option 3 is already used for legacy case here. So, we can do the same. Actually the change needed is fairly simple (in section 5.3.15.2): |
| Nokia | - Yes | - Option 3 |  |
| Samsung | - Yes | - Option 3 |  |
| Sharp | - Yes | - Option 3 |  |
| ASUSTeK | - Yes | - Option 3 |  |

## SDT terminology

| Source | Proposal |
| --- | --- |
| Intel [4] | - Proposal: Update SDT running CRs to include the term “session” when referring to the actual SDT (i.e. “SDT session”) that might be initiated, ongoing or terminated. For example, for initiating SDT session, or ongoing SDT session or terminating SDT session. |

<Rapporteur Views>: Not essential change. Current CR seems fine.

Q23: Do you think the issue is important to be addressed in rel-17 and why? If the issue is important to be addressed, do you agree with the proposal?

| Company | Essential Issue (Yes/No) | Agree/Disagree with proposal | Comments |
| --- | --- | --- | --- |
| ZTE | - No | - Disagree | Seems editorial. No strong view, but seems not really critical. |
| Nokia | - No |  | No strong view, but seems not critical |
| Samsung | - No | - Disagree | Not essential |
| LGE | - No | - Disagree |  |
| Sharp | - No | - Disagree |  |
| ASUSTeK | - No | - Disagree |  |

## UL signaling transmission over SRB1

RAN2#116bise Agreement

- The NW can NOT configure whether UL NAS transmission is allowed over SRB1 using SDT procedure

- ULInformationTransfer (including NAS message) over SRB2 configured for SDT can be sent during SDT procedure if configured.

| Source | Proposal |
| --- | --- |
| OPPO [1] | - Proposal: UL signalling over SRB1 can be always transmitted during SDT. FFS on removing the configuration of SRB1 for SDT from WID. |
| Nokia [12] | Proposal: Support configuring of SRB1 for SDT for carrying RRC messages Proposal: The UE is not allowed to transmit UL RRC messages over SRB1 in case SRB1 is not configured for SDT. Proposal: Network can configure which UL RRC messages are allowed over SRB1 using SDT procedure |

<Rapporteur Views>: These issues were discussed as part of email discussion [AT116bis-e] [502] in RAN2#116bise. SRB1 is always resumed upon SDT initiation. According to email discussion report R2-2201824, there was no support to have network control to allow/disallow UL NAS/RRC messages over SRB1. There was also no support to allow transmission of UEAssistanceInformation and SidelinkUEInformationNR during SDT procedure. There is no need to repeat the discussion.

<Nokia>: SRB1 usage for RRC message was not concluded in the previous meeting. In case this issue is not solved we assume that the UE shall not transmit any RRC messages over SRB1 using SDT procedure.

## Assistance Information

| Source | Proposal |
| --- | --- |
| Apple [2] | Proposal 1: The UE can indicate the SDT preference in UAI when the ReleasePreference is the INACTIVE state. Proposal 2: The UE can indicate the ReleasePreference in UAI during the SDT procedure, to stop the ongoing SDT procedure. - Proposal 3: The UE can indicate the preferred CG-SDT pattern (i.e. periodicity, offset, duration) to NW |
| Qulacomm [8] | Proposal 1: UE is allowed to send UE assistance information during RACH procedure to assist network for configuring CG resource in RRC Release message. Proposal 2: The UE assistance information contains at least the current buffer status, traffic pattern indication with one-shot or multi-shot, periodicity of traffic and estimated amount data of each shot. Proposal 3: The UE assistance information is allowed to be sent over SRB1 during SDT. |
| Ericsson [10] | Proposal 1: The values in UEAssistanceInformation are remapped and used for RAI in SDT. RAI can also be used by the UE to end the current data activity and initiate an early release of unused resources, particularly in the case of CG-SDT. For instance, in CG-SDT, the pre-configured PUSCH resources are reserved until the Timing Advance Timer expires. However, if the UE realizes that it does not need to transmit and wants to release the PUSCH resources, then the UE needs to wait until the current configuration becomes invalid due to timer expiry or TA failure. This leads to the PUSCH resources being held up unnecessarily. Such scenarios can be avoided if one would allow an early resource release for CG-SDT. Proposal 2: The UE can request release of CG-SDT configuration. |

<Rapporteur Views>: RAI and CG resource request is discussed in [POST116bis-e] [511] CP open issues list for SDT. No need to discuss here.

<Nokia >: UAI over SDT procedure seems not to be discussed in [POST116bis-e] [511]. We think that NW should be able to allow the UE to transmit UAI (e.g. RRC state preference or non-SDT data indication) using SDT procedure.

## RRM Measurement

| Source | Proposal |
| --- | --- |
| Apple [2] | Proposal: The RRM measurement during the SDT subsequent transmission phase follows IDLE/INACTIVE measurement framework. |

<Rapporteur Views>: This is discussed in [POST116bis-e] [511] CP open issues list for SDT. No need to discuss here.

## DVT

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal : RAN2 to decide whether UL data volume threshold is applicable to SDT initiation or for the whole SDT procedure. |

<Rapporteur Views>: No need to discuss. It’s already clear from current CR as data volume threshold is applied to entire data in buffer.

<Nokia >: Yes, but what happens if new SDT data becomes available during SDT procedure? Is the UE allowed to transmit any amount of data during SDT procedure? UEs could easily misuse the SDT procedure by starting always with SDT and then continue the data transmission in the subsequent data transmission phase. This would not be acceptable.

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal : NW can configure minimum and maximum UL data volume threshold which are used by UE to decide whether to do SDT or not |

<Rapporteur Views>: No need to discuss. We have already agreed to configure a single threshold

<Nokia >: Yes, but additional minimum threshold has not been discussed.

## ASN.1 Related

| Source | Proposal |
| --- | --- |
| Intel [4] | - Proposal : To discuss whether legacy CG-SDT configuration (labelled as SDT-MACPHY-CG-Config in current TP) is option (a) defined as a new octet string (which would require RAN3 to define a new mechanism to transfer this new container), or option (b) defined within legacy CellGroupConfig IE. |

<Rapporteur’s views> Discussed with RRC Rapporteur. RAN3 has already agreed to include the MACPhy configuration for CG in a new container. They need to refer to the correct RRC IE after we finalize the ASN.1.

| Source | Proposal |
| --- | --- |
| Intel [4] | - Proposal : The following fields need to be defined as “Need M”: sdt-Config-r17, sdt-DRB-List-r17, sdt-MAC-PHY-CG-Config-r17, sdt-DataVolumeThreshold-r17, txx-r17, cg-SDT-timeAlignmentTimer-r17, cg-SDT-Configuration-r17, and sdt-SSB-PerCG-PUSCH-r17. |

<Rapporteur’s views> Discussed with RRC Rapporteur.

- sdt-Config-r17 – set to need R because legacy gNB not supporting SDT (e.g. after RNAU) may implicitly release the SDT configuration if it is need R. Otherwise, it has to be explicit release using setupRelease structure.

- sdt-DRB-List-r17 – can start from size zero in this case and made need M (as proposed and discussed in section 2.3.)

- sdt-MAC-PHY-CG-Config-r17 is need M now in new CR

- sdt-DataVolumeThreshold-r17, txx-r17 – these two have been moved into SIB1 based on the comments received.

- cg-SDT-timeAlignmentTimer-r17, and sdt-SSB-PerCG-PUSCH-r17: They are all now need M.

- For cg-SDT-Configuration-r17: This can be converted to Need M.

| Source | Proposal |
| --- | --- |
| Intel [4] | - Proposal : cg-SDT-TA-ValidityThresholdSSB-r17 and cg-SDT-nrofSS-BlocksToAverage-r17 are defined as “NEED R”; RAN2 needs to discuss how would CG-SDT would work if any of this fields were not provided. |

<Rapporteur’s views> Discussed with RRC Rapporteur. Field description is updated in latest CR to clarify this. Please check.

| Source | Proposal |
| --- | --- |
| Intel [4] | Proposal 4: The following fields need to be defined with SetupRelease type: SDT-Config-r17, SDT-MACPHY-CG-Config (with a new parent/child structure is proposed), and CG-SDT-Configuration-r17 |

<Rapporteur’s views> Discussed with RRC Rapporteur. This has already been fixed (except for CG-SDT-Configuration-r17). This can also be fixed.

## PUCCH for SDT

| Source | Proposal |
| --- | --- |
| Intel [4] | Update CG-SDT configuration provided via RRCRelease message not to include PUCCH-Config to align this with RAN1 related agreement. |

<Rapporteur’s Views>: Discussed with RRC Rapporteur. It will be fixed in revision of CR. No need to discuss here.

## Redcap aspects

| Source | Proposal |
| --- | --- |
| Qualcomm [8] | Proposal : If a RedCap UE is configured in RedCap-specific initial UL BWP and the associated RedCap-specific initial DL BWP does not include the CD-SSB and CORESET #0, UE does not need to monitor SI change and PWS notification during SDT procedure. |

<Rapporteur’s Views>: Redcap UE monitors PO only in initial BWP (default or RedCap specific) associated with CD-SSB. So nothing specific to SDT needs to be separately agreed.

## Others

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal : The network can configure the payload size for CCCH message for both MSGA and MSG3. |

<Rapporteur’s Views>: RACH configuration specific to SDT is configured by network and network can configure the size. So no need to discuss this proposal.

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal : Both dedicated and broadcast RACH configuration is supported for SDT. |

<Rapporteur’s Views>: Dedicated RACH configuration specific to SDT was discussed previously and not agreed. So no need to discuss this proposal.

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal : Network can temporarily suspend the use of SDT with broadcast signalling. |

<Rapporteur’s Views>: Optimisation. Discussed previously. So no need to discuss this proposal.

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal: Small data transmission can be further configured by the network on a per QoS flow ID (QFI) basis. |

<Rapporteur’s Views>: Optimisation. We have already agreed to configure on a per DRB basis. So no need to discuss this proposal.

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal: “Not at cell edge” threshold specified for REL16 UE power saving is re-used for SDT purpose i.e. the UE is not allowed to use SDT at cell edge |

<Rapporteur’s Views>: We have already agreed to configure an RSRP threshold for SDT/Non SDT selection. So no need to discuss this proposal.

| Source | Proposal |
| --- | --- |
| Nokia [12] | Proposal: Msg4 / MsgB can multiplex a ciphered downlink data with the RRC Release message. |

<Rapporteur’s Views>: No special handling is needed. It is not prohibited and can be done if network wants.

| Source | Proposal |
| --- | --- |
| Nokia [12] | Subsequent data transmission would be straight forward to perform using early contention resolution where contention resolution is completed on MAC level without RRC message. Subsequent UL/DL data could be scheduled using C-RNTI and after the data transfer is complete the UE can be sent back to RRC_INACTIVE with RRC Release message. Proposal: Subsequent UL/DL data transfer can be completed before the network responses with RRC message to RRC Resume Request for SDT |

<Rapporteur’s Views>: No need to discuss. This is already supported in the CRs.

## DL non-SDT during SDT without Anchor Relocation

| Source | Proposal |
| --- | --- |
| CATT [6] | Proposal 1: Introduce a new specific cause value or indication in RRCRelease message for DL non-SDT arrival during SDT without anchor relocation. Proposal 2: Upon receiving the new specific cause value or indication in RRCRelease message, the UE behaves as when responding to RAN paging and triggers a follow-up resume procedure. Proposal 3: Send the reply LS to RAN3 on the agreements of handling of DL non-SDT during SDT without anchor relocation. |
| RAN3 LS R2-2202144 | RAN3 discussed how to handle the DL non-SDT data/signalling arrival during SDT procedure. Based on the discussion, it’s agreed that the anchor gNB could move the UE back to RRC Inactive by using RRCRelease message during SDT without anchor relocation. Then, the UE should re-initiate a new RRC Resume procedure (i.e. UE will be resumed to RRC_CONNECTED) for follow-up data transmission. On how to trigger UE to re-initiate another RRC Resume procedure, two possible options were discussed in RAN3: Option 1: Use RAN paging to trigger the following-up RRC resume procedure after UE is moved to Inactive state. Option 2: Add specific cause value or Indication in RRCRelease message to indicate UE to trigger the follow-up resume procedure. |

<Rapporteur Views>: This was discussed in [POST116bis-e] [511] CP open issues list for SDT. No consensus on any enhancement.

Q24: Which of the following option do you agree to handle the DL non-SDT data/signalling arrival during SDT procedure

Option 1: Use RAN paging to trigger the following-up RRC resume procedure after UE is moved to Inactive state.

Option 2: Add specific cause value or Indication in RRCRelease message to indicate UE to trigger the follow-up resume procedure.

| Company | Option 1/Option 2 | Comments |
| --- | --- | --- |
| Samsung | - Option 1 | In our view option 1 is sufficient |
| LGE | - Option 1 |  |
| Sharp | - Option 1 |  |
| ASUSTeK |  | NW could send RRCResume to transit the UE to connected state to receive the DL non-SDT data. |
| ZTE | - Option 1 | Option 1 is enough in our view too. By the way, we could also use release with redirection (which is part of option 1 in our view). We want to also note that as long as there is any pending UL data/NAS procedure, UE will immediately initiate new resume procedure (without waiting for paging) even with option 1. |

# Conclusion

In summary, the following are proposed:

TBD

# Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Samsung | Anil Agiwal | anilag@samsung.com |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |
| ASUSTeK | Erica Huang | Erica_Huang@asus.com |

# References

[1] R2-2202275	Discussion on control plane issues of SDT	OPPO

[2] R2-2202556	Control plane aspects of SDT	Apple

[3] R2-2202590	Analysis on CP open issue of SDT	Lenovo, Motorola Mobility

[4] R2-2202674	Additional discussion on identified open CP issues	Intel Corporation

[5] R2-2202736	Remaining issues of control plane aspects of SDT	China Telecom

[6] R2-2202805	Handling of DL non-SDT during SDT	CATT

[7] R2-2202846	Remaining issue on CS-RNTI configuration for CG-SDT	ASUSTeK

[8] R2-2202960	Remaining issues on CP aspects of SDT	Qualcomm Incorporated

[9] R2-2203009	Remaining control plane aspects of SDT	NEC

[10] R2-2203155	CP aspects for SDT	Ericsson

[11] R2-2203337	Control plane common aspects for SDT	Huawei, HiSilicon

[12] R2-2203353	SDT control plane aspects	Nokia, Nokia Shanghai Bell

[13] R2-2202982	Further Consideration on the Handling of non-SDT Data Arrival	vivo

[14] R2-2202610, Discussion on remaining issues for SDT user plane, Huawei, HiSilicon