3GPP TSG-RAN2 Meeting #117-e	R2-220xxxx

Online, 21 February – 03 March 2022

Agenda Item:	9.2.3

Source:		Huawei, HiSilicon

Title:		[Pre117-e][012][IOT-NTN] Control Plane Open Issues (Huawei)

Document for:	Discussion and decision

# Introduction

This is the report of the email discussion on IOT NTN control plane open issues:

- [Pre117-e][012][IOT-NTN] Control Plane Open Issues (Huawei)

Scope: Collect companies views on Control Plane Open Issues

Deadline for comments: Monday 14th February, 23:53 UTC

The document collects companies views on the control plane open issues identified for pre117-e discussion.

# Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |
| Interdigital | Brian Martin | brian.martin@interdigital.com |
| MediaTek | Abhishek Roy | Abhishek.Roy@mediatek.com |
| Lenovo, Motorola Mobility | Min Xu | xumin13@lenovo.com |

# Discussion

## OI 2.1: Define a new barring bit for NTN UEs barring

RAN2#116bis-e has agreed:

- We will have the barring bit to prevent terrestrial UEs to use NTN. FFS if we define a new barring bit for NTN UEs barring.

At least, the following points need to be addressed:

- option 1: introduce a new barring bit: which name, how does it work, anything else needed

- option 2: reuse an existing bit: which one, how does it work, anything else needed

Discussion point 1: please provide your views on the above issue

| Company | Option 1/ Option 2 | Comments |
| --- | --- | --- |
| InterDigital | Option1 | Existing barring bits have their own purpose. The usual way to do this is that the UE ignores the existing barring bit, and checks the new barring bit instead. This allows NW to bar legacy UEs and control the new UEs independently using the new barring bit. As for naming, no strong view but something like “cellBarred-NTN-r17” would be aligned with previous new barring bits. |
| MediaTek | Option 1 | A new barring bit will be better and easier to manage. |
| Lenovo, Motorola Mobility | Option 1 |  |

## OI 2.6 If some mechanism is needed to trigger the UE to reacquire the NTN specific SIB in RRC_IDLE

RAN2#116bis-e has agreed:

- Update to serving cell ephemeris information does not affect the system information value tag and does not trigger System information modification procedure. How to trigger re-read of this information is FFS. FFS if the UE shall reacquire the new SIB when SI update is triggered.

- Updates to serving cell ephemeris information are not bound to the BCCH modification period.

- When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-aquires the required SI, and then comes back. FFS whether anything additional is needed.

- UE acquires the NTN specific SIB before accessing the cell.

However, handling of the NTN SIB in RRC_IDLE has not been discussed yet

At least, the following points need to be addressed:

- is SIBXX an essential SIB?

- is the UE required to acquire SIBXX in RRC_IDLE? what are the triggers?

- any other aspects

Discussion point 2: please provide your views on the above issue

| Company | Comments |
| --- | --- |
| InterDigital | - it is essential, without having a valid version UE cannot perform e.g. pre-compensation. - UE is required to have a valid version of the SIB before access, therefore acquiring before access this seems to be the only requirement. - How to ensure UE can re-acquire the SIB in time before responding to paging? Does UE therefore need to acquire the SIB in every (e)DRX cycle or can we rely on the validity timer (and UE only needs to acquire before access if the validity timer expires)? |
| MediaTek | - SIBXX is an essential SIB - UE required to acquire SIBXX in RRC_IDLE - The UE can rely on the validity timer and acquire the SIB when the validity timer expires. |
| Lenovo, Motorola Mobility | - SIBXX is essential, at least for TA pre-compensation. - UE needs to acquire SIBXX in IDLE, and at least the validity timer expiration can trigger re-acquiring. - An indication to trigger re-acquiring can be considered, which is controlled by network whenever re-acquiring is necessary. |

## OI 2.7 If anything additional is needed on expiry of the UL synchronisation timer

RAN2#116bis-e has agreed:

- When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-acquires the required SI, and then comes back. FFS whether anything additional is needed.

In RAN2#116bis-e contributions, there were proposals to have a guard timer for the acquisition of the new SIB and trigger RLF at expiry.

At least, the following points need to be addressed:

- do we need a ‘guard’ mechanism in case the UE does not manage to reacquire the SIB quick enough

- any other aspects

Discussion point 3: please provide your views on the above issue

| Company | Comments |
| --- | --- |
| InterDigital | We don’t think further optimisation is needed at this stage |
| MediaTek | Further optimizations are not needed at this stage. |
| Lenovo, Motorola Mobility | Not needed in this release. |

## OI 2.x: Provision of SIBxx in dedicated signalling at HO

Aspects related to handover have not been discussed yet in RAN2.

It has only been agreed ‘ta-Report-r17 is signaled in radioResourceConfigCommon’ so that it is included in mobilityControlInfo.

Other NTN specific parameters may need to be provided to the eMTC UE at handover.

At least, the following points need to be addressed:

- which NTN specific information is needed in mobilityControlInfo

- which NTN specific SIB, if any, should be provided in RRCReconfiguration for handover

- any other aspects

Discussion point 4: please provide your views on the above issue

| Company | Comments |
| --- | --- |
| InterDigital | At least ephemeris information used for pre-compensation would be needed |
| MediaTek | The ephemeris information, required for L1 pre-compensation, is definitely needed for eMTC. |
| Lenovo, Motorola Mobility | The ephemeris of the target satellite is needed. Besides, like the serving satellite, the epoch time and validity time associated to the ephemeris of the target satellite are also needed. |

# Conclusion

# Reference

- R2-2202053 Summary of [Post116bis-e][089][IoT-NTN] Open Issues (Mediatek), RAN2#116bis-e, January 2022