3GPP TSG-RAN WG2 Meeting #117-e		R2-220xxxx

Electronic, 21st Feb. – 3rd Mar. 2022

Source:	vivo

Title:	Summary of [Pre117-e][106][RedCap] MAC open issues

Agenda Item:	8.12.3.1.1

Document for:	Discussion and Decision

# Introduction

The document summarizes the following pre-meeting offline discussion:

[Pre117-e][106][RedCap] MAC open issues (vivo) Regarding the MAC mode open issues listed in R2-2201891: - Issues "1-3", "2-3" and "2-4" will be handled in offline discussion [Pre117-e][106][RedCap] MAC open issues - Issue "1-1" can be discussed as part of the RACH indication and partitioning discussion (AI 8.18) - Issue "1-2" will be handled in offline discussion [Pre117-e][107][RedCap] - Issues "2-1" and "2-2" can be handled via company contributions in AI 8.12.4 Other MAC issues can be handled via company contributions in AI 8.12.3.1.2

In order for rapporteur to have sufficient time to provide the summary, your comments before Monday 2022-2-14 23:59 UTC is appreciated.

- Contact information

| Company | Name and email address |
| --- | --- |
| vivo | Chenli (chenli5g@vivo.com) |
| Huawei, HiSilicon | Yulong (shiyulong5@huawei.com) |
| Samsung | Jaehyuk JANG (jack.jang@samsung.com) |
| LGE | SunYoung LEE (ssunyoung.lee@lge.com) |
| Nokia, Nokia Shanghai Bell | Samuli Turtinen (samuli.turtinen@nokia.com) |

# Discussion

The open issue list on MAC aspects for RedCap was discussed and summarized in [1] with the following open issues:

Regarding early identification:

| OI Index | Open issue | Rapporteur comment | OI type |
| --- | --- | --- | --- |
| 1-1 | How to apply the dedicated RACH resource for RedCap or how to define the applicability priority (e.g. whether RedCap type is prioritized) | This OI will be handled in RACH partitioning session. | Type 2 |
| 1-2 | Confirm Working assumption or not on: Working assumption: - Msg3 early identification is mandatorily supported by RedCap UE | This OI will be handled in RAN2 also considering MsgA early identification. | Type 1 |
| 1-3 | Whether LCID for RedCap is always indicated when MsgA CCCH is sent by a RedCap UE (i.e. no other precondition) | This OI will be handled in RAN2 | Type 1 |

Regarding NCD-SSB:

| OI Index | Open issue | Rapporteur comment | OI type |
| --- | --- | --- | --- |
| 2-1 | Whether/how NCD-SSB could be applied for Non-RedCap UEs | This OI will be handled by both RAN1 and RAN2 | Type 2 or Type 1? |
| 2-2 | Any other impacts on BWP operation in RRC_CONNECTED for the behavior for NCD-SSB, e.g. RRM, RLM, etc. | This OI will be handled by RAN2 | Type 2 or Type 1? |
| 2-3 | How to implement the redcap specific initial BWP in MAC, including RACH procedure and BWP operation | This OI will be handled by RAN2 | Type 3 |
| 2-4 | FFS whether to have CD-SSB/NCD-SSB concept in MAC specification |  | Type 3 |

Per Chair guidance, we only need to discuss these issues:

Issues "1-3", "2-3" and "2-4" will be handled in offline discussion [Pre117-e][106][RedCap] MAC open issues

## Open issue for early identification

In RAN2#116bis-e, it was agreed that:

Agreements online: 1. In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, when the Msg3 includes the CCCH data (no other precondition) 2. Also when msg1 early identification is configured, new dedicated LCID is used for CCCH identification

For 2-step RACH, Rapporteur thinks similar conclusions should be made accordingly.

- Companies are invited to show your views on whether LCID for RedCap is always indicated when MsgA CCCH is sent by a RedCap UE (i.e. no other precondition). Otherwise, please provide your reason.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Same as Msg3. |
| Samsung | Yes | To add precondition would only add unnecessary complexity to the specification/implementation. |
| LGE | Yes |  |
| Nokia | Yes | No precondition needed. |

## Whether/how to capture separate initial BWP in MAC

In RAN2#116bis-e, it was agreed that:

1. If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, measurements are based on CD-SSB for initial RACH resource selection. 3. From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

Rapporteur thinks this is related to RACH procedure in idle/inactive mode, and it could be captured in MAC specification as a Note. During the running MAC CR discussion, some companies provided some rewording suggestions as below [2]:

NOTE X1: If a RedCap UE in RRC_IDLE or RRC_INACTIVE mode is configured with a separate initial BWP this is not associated with any SSB for RACH, measurements are based on CD-SSB for initial RACH resource selection, while it is up to UE implementation whether to perform new RSRP measurements in the DL BWP associated with CD-SSB for Random Access Preamble retransmission.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusions:

- Option 1: Agree.

- Option 2: Not agree, please provide your suggestions.

- Option 3: No need to capture this in MAC specification.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 3 for agreement 1; FFS/Wait for agreement 3; | There is no differentiation between CD-SSB and NCD-SSB in MAC specification for RA procedure. It can be clarified in RRC that the SSB to be used is NCD-SSB. The agreement 3, “it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission”, we can wait for the R1/4 discussion and see how they capture/clarify this in their specification. |
| Samsung | Option 1 | - |
| LGE | Option 2 for Agreement 1; Option 3 for Agreement 3; | The intention of the agreement is to allow using normal initial BWP for a RedCap UE if there is no SSB on the RedCap specific initial BWP. There is no need to differentiate CD-SSB or NCD-SSB in MAC. More importantly, if we introduce CD-SSB/NCD-SSB, it becomes unclear what the legacy SSB in MAC specification means. Thus, we have a concern on having CD/NCD-SSB in MAC specification only for RedCap. The only one thing we need in MAC is that, for RedCap UE, there is a RedCap specific initial BWP. In this regards, our suggestion is: NOTE X1: If a RedCap UE is configured with an [initial BWP for RedCap] not associated with any SSB, SS-RSRP is measured for the SSB associated with initial BWP. |
| Nokia | No strong view | It would need to be captured somewhere the CD-SSB is used. |

In RAN2#116bis-e, it was agreed that:

2. If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

Rapporteur thinks this conclusion has impacts on MAC specification on BWP switching (i.e. clause 5.15).

In idle/inactive mode, there was no BWP switching before, as there is only initial BWP, on which UE performs SI/Paging reception, RACH, etc. For RedCap, if a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH. In this way, the corresponding UE behaviour should be captured. During the running MAC CR discussion, some companies provided some rewording suggestions as below [2]:

[A RedCap UE in RRC_IDLE or RRC_INACTIVE mode may be configured with a RedCap-specific initial UL BWP, as specified in TS 38.213 [6]. If the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: 1> if the RedCap-specific initial UL BWP is configured with RACH: 2> perform RACH procedure as specified in clause 5.1 by using the RedCap-specific initial UL BWP; 3> if the RedCap-specific initial DL BWP is configured: 4> monitor the PDCCH on the RedCap-specific initial DL BWP.] Editor’s NOTE: FFS any other impacts on BWP operation in RRC_CONNECTED for the behavior for NCD-SSB, e.g. RRM, RLM, etc. Editor’s NOTE: How separate initial UL/DL BWP impacts MAC specification will be discussed and determined further. Editor’s NOTE: The behaviour on RedCap specific initial BWP need to be updated based on further progress.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusions:

- Option 1: Agree.

- Option 2: Not agree, please provide your suggestions.

- Option 3: No need to capture this in MAC specification.

- Option 4: Others, e.g., wait for more progress on separate initial BWP.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 | The above procedure text is only to clarify one thing that the RACH is on RedCap-specific initial UL BWP. Also, There is no definition in MAC on “RedCap-specific initial UL BWP”. We suggest to just capture one NOTE in the beginning of 5.1.1 to clarify all those parameters for RedCap: “NOTE: If configured, RedCap UE should use the configurations provided in [RedCap-specific initial UL BWP configuration], as specified in TS 38.331[x].” |
| Samsung | Option 1 | 3> and 4> can move one level up (i.e., to 2> and 3> respectively). |
| LGE | Option 4 | As indicated by EN, this may be impacted by the discussion under RACH partitioning, e.g., feature priority based BWP selection. Therefore, we propose to wait for more progress. |
| Nokia | Option 4 | Let’s capture once we have all the details how things should work. |

During the running MAC CR discussion, some companies commented that connected mode should be also considered. Then, the corresponding TP was provided in MAC running CR as below [2]:

[1> if for a RedCap UE, PRACH occasions for the Redcap UE are not configured for the active UL BWP, and if the RedCap-specific initial UL BWP is configured with RACH: 2> switch the active UL BWP to the RedCap-specific initial UL BWP; 2> if the RedCap-specific initial DL BWP is configured: 3> switch the active DL BWP to the RedCap-specific initial DL BWP; 2> else: 3> switch the active DL BWP to BWP indicated by initialDownlinkBWP. 1> else if, for a RedCap UE, the RedCap-specific initial UL BWP is not configured with RACH: or,] 1> [except for RedCap UEs,] if PRACH occasions are not configured for the active UL BWP: 2> switch the active UL BWP to BWP indicated by initialUplinkBWP; 2> if the Serving Cell is an SpCell: 3> switch the active DL BWP to BWP indicated by initialDownlinkBWP.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusions:

- Option 1: Agree.

- Option 2: Not agree, please provide your suggestions.

- Option 3: No need to capture this in MAC specification.

- Option 4: Others, e.g., wait for more progress on separate initial BWP.

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 | See our comment in DP3, and our suggestion can cover both idle/inactive and connected. Also, existing spec already supports the BWP switching to the one with RACH resource. We only need to clarify in RRC what’s the RedCap-specific initial BWP. |
| Samsung | Option 1 | The text change can be slightly updated (merely for better readability) as follows: 1> if PRACH occasions are not configured for the active UL BWP: 2> if UE is a RedCap UE and if initialUplinkBWPRedcap is configured: 3> switch the active UL BWP to BWP indicated by initialUplinkBWPRedcap; 2> else: 3> switch the active UL BWP to BWP indicated by initialUplinkBWP; 2> if the Serving Cell is an SpCell: 3> if UE is RedCap and if initialDownlinkBWPRedcap is configured: 4> switch the active DL BWP to BWP indicated by initialDownlinkBWPRedcap; 3> else: 4> switch the active DL BWP to BWP indicated by initialDownlinkBWP. |
| LGE | Option 4 | As indicated by EN, this may be impacted by the discussion under RACH partitioning, e.g., considering CE. Therefore, we propose to wait for more progress. |
| Nokia | Option 4 | Agree with the intention but let’s wait for progress before capturing. |

In RAN2#116bis-e, it was agreed that:

8. For connected mode operation if NCD-SSB is configured in a dedicated DL BWP whose paired UL BWP is configured with RACH-ConfigDedicated, RACH-ConfigCommon or BeamFailureRecovery Config, SSB in that RACH configuration (e.g., in CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE) refers to the NCD-SSB configured in that DL BWP.

This conclusion concern RACH procedure in connected mode. Some companies think there is no need to capture this conclusion in RACH procedure or no need to capture it in MAC specification.

- Companies are invited to show your preference among the following options on whether/how to capture the above conclusion:

- Option 1: No need in MAC, clarification in other specification (e.g. in RRC) is enough.

- Option 2: Yes in MAC, please provide where/how to capture it (e.g. as a Note).

| Company’s name | Option | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Option 1 | Apparently, the SSB clarification on CFRA-SSB-Resource IE or in PRACH-ResourceDedicatedBFR IE should be captured in RRC. |
| Samsung | Option 1 | - |
| LGE | Option 1 |  |
| Nokia | Option 1 |  |

## Others

- Companies are invited to provide your views on any other open issues not included above which has impacts on MAC specification:

| Company’s name | Comments, if any |
| --- | --- |

# Conclusion

Based on companies’ input,

# References

- R2-2201891, Open issue list on MAC for RedCap, vivo

- R2-2201890, Running MAC CR for RedCap, vivo