3GPP TSG-RAN2 Meeting #117-e	R2-220xxxx

Online, 21 February – 03 March 2022

Agenda Item:	9.1.2

Source:		Huawei, HiSilicon

Title:		[Pre117-e][302][NBIOT/eMTC R17] Capabilities open issues (Huawei)

Document for:	Discussion and decision

# Introduction

This is the report of the email discussion on capabilities open issues :

- [Pre117-e][302][NBIOT/eMTC R17] Capabilities open issues (Huawei)

Scope: Collect companies views on UE capabilities open issues

Deadline for comments: Monday 14th February, 23:53 UTC

The document collects companies views on the capabilities open issues identified for pre117-e discussion in [1].

# Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Odile Rollinger | odile.rollinger@huawei.com |
| Qualcomm | Mungal Dhanda | mdhanda@qti.qualcomm.com |
| ZTE | Ting Lu | lu.ting@zte.com.cn |

# Discussion

Open Issue 2: Scope of connected mode measurements for RLF capability, i.e. per UE w/o FDD/TDD differentiation, or per UE with FDD/TDD differentiation.

Discussion point 1: please provide your views on the above issue

| Company | Comments |
| --- | --- |
| Qualcomm | Propose to have both FDD/TDD differentiation and inter/intra-frequency differentiation (i.e., four capabilities). |
| Huawei, HiSilicon | Propose Per UE w/o TDD/FDD differentiation. (Note that RAN4 is not making any differentiation between TDD and FDD in their discussions and is defining core requirements applicable to both. In that case, there is no justification for RAN2 to introduce FDD/TDD differentiation) |
| ZTE | Agree with Huawei that a capability per UE without TDD/FDD differentiation. |

# Conclusion

# Reference

- R2-2201796, [AT116bis-e][302][NBIOT/eMTC R17] Capabilities Open issues (Huawei), Huawei, RAN2#116bis-e, January 2022