3GPP TSG RAN WG2#117-e	R2-2202128

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221243

Online, January 17th – 26th 2022

Title:	LS on QoE Measurement Session Start and Measurement Session End Indication from the UE

Response to:	-

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN3

To:	RAN2

Cc:	SA5

Contact Person:

Name: 		Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

To enable time alignment of the QoE and Immediate MDT measurements, RAN3 has agreed that an NG-RAN node is required to receive from the UE a QoE measurement session start indication and a session end indication from the application and via RRC signalling.

In the case that MDT and QoE are configured simultaneously to NG-RAN, the NG-RAN node can configure the UE with an immediate MDT configuration upon receiving the QoE measurement session start indication, to enable the MDT and QoE measurements to run in parallel. Before receiving the measurement session start indication, the corresponding immediate MDT configuration is not configured to UE.

Further, the session start/end indication can enable the NG-RAN node to add the session start time and session end time in the QoE report sent to MCE, for the alignment of an ongoing Immediate MDT and a QoE measurement started later.

The relevant agreements from RAN3#114-e and RAN3#114bis-e meetings:

To enable time alignment between an already ongoing Immediate MDT and a QoE measurement started later, the start time and end time of the QoE measurement, in addition to the Trace Reference and Trace Recording Session ID, needs to be added to the QoE measurement report at the NG-RAN node

UE assisted solution can be used for MDT-QoE alignment. UE can indicate to NG-RAN via a flag whether a QoE measurement session started/ended. If the NG-RAN knows there is an MDT configuration associated with a QoE configuration (e.g., upon receiving NG-RAN Trace ID in the QoE configuration from OAM),

- NG-RAN can configure the UE with that associated MDT configuration upon receiving the QoE measurement session start indication from the UE

- NG-RAN can deactivate the associated MDT configuration upon receiving the QoE measurement session end indication from the UE

2. Actions:

To 3GPP RAN2

ACTION:

- RAN3 respectfully asks RAN2 to take the above information into consideration and specify in RRC the QoE measurement session start/end indication from the UE to the NG-RAN node.

3. Date of next TSG RAN WG3 meetings:

RAN3#115-e	February 21st  - March 3rd  2022	Online