3GPP TSG-RAN WG2 Meeting #118-e	R2-2204957

Online, 9th May – 20th May 2022

Agenda item:	   6.2.3

Source:		Lenovo

Title:		Miscellaneous corrections to 37.340 CPAC

Document for:		Discussion and Decision

# 1	Introduction

In this paper, text is proposed to correct miscellaneous issues to 37.340 CPAC related paragraph, to better align the wording crossing different subclauses and reduce the ambiguity.

# 2 Text Proposal for 37.340

--------------------------------------------------- Change Starts -----------------------------------------------------------------

## 10.2	Secondary Node Addition

### 10.2.1	EN-DC

The Secondary Node Addition procedure is initiated by the MN and is used to establish a UE context at the SN to provide resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the first cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). In case of CPA, the Conditional Secondary Node Addition procedure can be used for CPA configuration and CPA execution.

Text Omitted Without Change

Conditional Secondary Node Addition

Figure 10.2.1-2 shows the Conditional Secondary Node Addition procedure.

Figure 10.2.1-2: Conditional Secondary Node Addition procedure

1.	The MN decides to configure CPA for the UE and requests the candidate SN to allocate resources for a specific E-RAB, indicating E-RAB characteristics (E-RAB parameters, TNL address information corresponding to bearer type), provides the upper limit for the number of PSCells to prepare. In addition, for the bearers requiring SCG radio resources, the MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the candidate cells recommended by MN via the latest measurement results for the SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. The MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision. In case of bearer options that require X2-U resources between the MN and the SN, the MN provides X2-U TNL address information for the respective E-RAB, X2-U DL TNL address information for SN terminated bearers, X2-U UL TNL address information for MN terminated bearers. In case of SN terminated split bearers the MN provides the maximum QoS level that it can support. The SN may reject the request.

NOTE 6:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective E-RAB is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MNs decision is reflected in step 1 by the E-RAB parameters signalled to the SN, which may differ from E-RAB parameters received over S1.

NOTE 7:	For a specific E-RAB, the MN may request the direct establishment of an SCG or a split bearer, i.e., without first having to establish an MCG bearer. It is also allowed that all E-RABs can be configured as SN terminated bearers, i.e. there is no E-RAB established as an MN terminated bearer.

2.	If the RRM entity in the SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer option, respective transport network resources, and provides the prepared PSCell ID(s) to the MN. For bearers requiring SCG radio resources, the SN performs Random Access at the CPA execution so that synchronisation of the SN radio resource configuration can be performed. Within the list of cells as indicated within the measurement results indicated by the MN, the SN decides the list of PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***) contained in the SgNB Addition Request Acknowledge message. The target SN can either accept or reject each of the candidate cells suggested by the MN, i.e. it cannot come up with any alternative candidates. In case of bearer options that require X2-U resources between the MN and the SN, the SN provides X2-U TNL address information for the respective E-RAB, X2-U UL TNL address information for SN terminated bearers, X2-U DL TNL address information for MN terminated bearers. For SN terminated bearers, the SN provides the S1-U DL TNL address information for the respective E-RAB and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided.

NOTE 8:	For the SN terminated split bearer option, the SN may either decide to request resources from the MN of such an amount, that the QoS for the respective E-RAB is guaranteed by the exact sum of resources provided by the MN and the SN together, or even more. The SNs decision is reflected in step 2 by the E-RAB parameters signalled to the MN, which may differ from E-RAB parameters received in step 1. The QoS level requested from the MN shall not exceed the level that the MN offered when setting up the split bearer in step 1.

NOTE 9:	In case of SN terminated bearers and MN terminated split/SCG bearers, early data forwarding may take place after step 2. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN. For the early data forwarding of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN. The MN sends the Early Status Transfer message to the SN.

3.	The MN sends to the UE an RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) including the CPA configuration, (i.e. a list of RRCConnectionReconfiguration** messages) and associated execution conditions, in which a RRCConnectionReconfiguration** message contains a RRCReconfiguration*** received from the candidate SN and possibly an MCG configuration. Besides, the RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements.

4.	The UE applies the RRC configuration (in RRCConnectionReconfiguration*) excluding the CPA configuration, stores the CPA configuration and replies to the MN with an RRCConnectionReconfigurationComplete message (RCConnectionReconfigurationComplete*) without any NR RRC response message. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration* message, it performs the reconfiguration failure procedure.

4a.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration message (RRCConnectionReconfiguration**) corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete message (RRCConnectionReconfigurationComplete**), including an NR RRC message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN.

5.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully via SgNB ReconfigurationComplete message, including the RRCReconfigurationComplete*** message. The MN sends the SgNB Release Request message(s) to cancels CPA in the other target candidate SN(s), if configured.

6.	The UE performs synchronisation towards the selected PSCell indicated in RRCConnectionReconfiguration** message. The order the UE sends the RRCConnectionReconfigurationComplete** message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

7.	If PDCP termination point is changed to the SN for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status Transfer.

8.	For SN terminated bearers moved from the MN, dependent on the bearer characteristics of the respective E-RAB, the MN may take actions to minimise service interruption due to activation of EN-DC (Data forwarding).

9-12.	If applicable, the update of the UP path towards the EPC is performed.

### 10.2.2	MR-DC with 5GC

The Secondary Node (SN) Addition procedure is initiated by the MN and is used to establish a UE context at the SN in order to provide resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the initial SCG serving cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). In case of CPA, the Conditional Secondary Node Addition procedure can be used for CPA configuration and CPA execution.

Secondary Node Addition

Figure 10.2.2-1 shows the SN Addition procedure.

Figure 10.2.2-1: SN Addition procedure

1.	The MN decides to request the target SN to allocate resources for one or more specific PDU Sessions/QoS Flows, indicating QoS Flows characteristics (QoS Flow Level QoS parameters, PDU session level TNL address information, and PDU session level Network Slice info). In addition, for bearers requiring SCG radio resources, MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the latest measurement results for SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. In NGEN-DC and NR-DC, the MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision. The MN may request the SCG to be activated or deactivated.

For MN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides Xn-U UL TNL address information. For SN terminated bearers, the MN provides a list of available DRB IDs. The S-NG-RAN node shall store this information and use it when establishing SN terminated bearers. The SN may reject the request.

For SN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides in step 1 a list of QoS flows per PDU Sessions for which SCG resources are requested to be setup upon which the SN decides how to map QoS flows to DRB.

NOTE 1:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective QoS Flow is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MN decision is reflected in step 1 by the QoS Flow parameters signalled to the SN, which may differ from QoS Flow parameters received over NG.

NOTE 2:	For a specific QoS flow, the MN may request the direct establishment of SCG and/or split bearers, i.e. without first having to establish MCG bearers. It is also allowed that all QoS flows can be mapped to SN terminated bearers, i.e. there is no QoS flow mapped to an MN terminated bearer.

2.	If the RRM entity in the SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer type options, respective transport network resources. For bearers requiring SCG radio resources the SN triggers UE Random Access so that synchronisation of the SN radio resource configuration can be performed. The SN decides for the PSCell and other SCG SCells and provides the new SCG radio resource configuration to the MN within an SN RRC configuration message contained in the SN Addition Request Acknowledge message. If the MN requested the SCG to be deactivated, the SN may keep the SCG activated. If the MN requests the SCG to be activated, the SN shall keep the SCG activated. In case of bearer options that require Xn-U resources between the MN and the SN, the SN provides Xn-U TNL address information for the respective DRB, Xn-U UL TNL address information for SN terminated bearers, Xn-U DL TNL address information for MN terminated bearers. For SN terminated bearers, the SN provides the NG-U DL TNL address information for the respective PDU Session and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided.

NOTE 3:	In case of MN terminated bearers, transmission of user plane data may take place after step 2.

NOTE 4:	In case of SN terminated bearers, data forwarding and the SN Status Transfer may take place after step 2.

NOTE 5:	For MN terminated bearers for which PDCP duplication with CA is configured in NR SCG side, the MN allocates up to 4 separate Xn-U bearers and the SN provides a logical channel ID for primary or split secondary path to the MN.

For SN terminated bearers for which PDCP duplication with CA is configured in NR MCG side, the SN allocates up to 4 separate Xn-U bearers and the MN provides a logical channel ID for primary or split secondary path to the SN via an additional MN-initiated SN modification procedure.

2a.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

3.	The MN sends the MN RRC reconfiguration message to the UE including the SN RRC configuration message, without modifying it. Within the MN RRC reconfiguration message, the MN can indicate the SCG is deactivated.

4.	The UE applies the new configuration and replies to MN with MN RRC reconfiguration complete message, including an SN RRC response message for SN, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

5.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRC response message, if received from the UE.

6.	If configured with bearers requiring SCG radio resources and the SCG is not deactivated, the UE performs synchronisation towards the PSCell configured by the SN. The order the UE sends the MN RRC reconfiguration complete message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

7.	If PDCP termination point is changed to the SN for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status Transfer.

8.	For SN terminated bearers or QoS flows moved from the MN, dependent on the characteristics of the respective bearer or QoS flow, the MN may take actions to minimise service interruption due to activation of MR-DC (Data forwarding).

9-12.	If applicable, the update of the UP path towards the 5GC is performed via a PDU Session Path Update procedure.

Conditional Secondary Node Addition

Figure 10.2.2-2 shows the Conditional SN Addition procedure.

Figure 10.2.2-2: Conditional Secondary Node Addition procedure

1.	The MN decides to configure CPA for the UE. The MN requests the target candidate SN to allocate resources for one or more specific PDU Sessions/QoS Flows, indicating QoS Flows characteristics (QoS Flow Level QoS parameters, PDU session level TNL address information, and PDU session level Network Slice info), provides the upper limit for the number of PSCells to prepare. In addition, for bearers requiring SCG radio resources, the MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the candidate cells recommended by MN via the latest measurement results for the SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. In NR-DC, the MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision.

For MN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides Xn-U UL TNL address information. For SN terminated bearers, the MN provides a list of available DRB IDs. The S-NG-RAN node shall store this information and use it when establishing SN terminated bearers. The SN may reject the request.

For SN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides in step 1 a list of QoS flows per PDU Sessions for which SCG resources are requested to be setup upon which the SN decides how to map QoS flows to DRB.

NOTE 6:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective QoS Flow is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MN decision is reflected in step 1 by the QoS Flow parameters signalled to the SN, which may differ from QoS Flow parameters received over NG.

NOTE 7:	For a specific QoS flow, the MN may request the direct establishment of SCG and/or split bearers, i.e. without first having to establish MCG bearers. It is also allowed that all QoS flows can be mapped to SN terminated bearers, i.e. there is no QoS flow mapped to an MN terminated bearer.

2.	If the RRM entity in the SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer type options, respective transport network resources, and provides the prepared PSCell ID(s) to the MN. For bearers requiring SCG radio resources the SN triggers UE Random Access so that synchronisation of the SN radio resource configuration can be performed. Within the list of cells as indicated within the measurement results indicated by the MN, the SN decides the list of PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***), contained in the SN Addition Request Acknowledge message. The target SN can either accept or reject each of the candidate cells suggested by the MN, i.e. it cannot come up with any alternative candidates. In case of bearer options that require Xn-U resources between the MN and the SN, the SN provides Xn-U TNL address information for the respective DRB, Xn-U UL TNL address information for SN terminated bearers, Xn-U DL TNL address information for MN terminated bearers. For SN terminated bearers, the SN provides the NG-U DL TNL address information for the respective PDU Session and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided.

NOTE 8:	For MN terminated bearers for which PDCP duplication with CA is configured in NR SCG side, the MN allocates up to 4 separate Xn-U bearers and the SN provides a logical channel ID for primary or split secondary path to the MN.

For SN terminated bearers for which PDCP duplication with CA is configured in NR MCG side, the SN allocates up to 4 separate Xn-U bearers and the MN provides a logical channel ID for primary or split secondary path to the SN via an additional MN-initiated SN modification procedure.

NOTE 9:	In case of SN terminated bearers and MN terminated split/SCG bearers, early data forwarding may take place after step 2. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN. For the early data forwarding of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN.

2a.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message. In case of early data forwarding in CPA, the MN sends the Early Status Transfer message to the SN.

3.	The MN sends to the UE an RRC reconfiguration message (RRC reconfiguration*) including the CPA configuration, (i.e. a list of RRC reconfiguration** messages) and associated execution conditions, in which a RRC reconfiguration** message contains a RRCReconfiguration*** received from the candidate SN and possibly an MCG configuration. Besides, the RRC reconfiguration message (RRC reconfiguration*) can also include the current MCG updated configuration. e.g. to configure the required conditional measurements.

4.	The UE applies the RRC configuration (in RRC reconfiguration*) excluding the CPA configuration, stores the CPA configuration and replies to the MN with an RRC reconfiguration complete message (RRC reconfiguration complete*) without any NR SN RRC response message). In case the UE is unable to comply with (part of) the configuration included in the RRC reconfiguration* message, it performs the reconfiguration failure procedure.

4a.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRC reconfiguration message (RRC reconfiguration**) corresponding to the selected candidate PSCell, and sends an MN RRC reconfiguration complete message (RRC reconfiguration complete**), including an NR RRC reconfiguration complete message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN.

5.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete*** response message. The MN sends the SN Release Request message(s) to cancels CPA in the other target candidate SN(s), if configured.

6.	The UE performs synchronisation towards the selected PSCell indicated in RRC reconfiguration** message. The order the UE sends the MN RRC reconfiguration complete** message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

7.	If PDCP termination point is changed to the SN for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status Transfer.

8.	For SN terminated bearers or QoS flows moved from the MN, dependent on the characteristics of the respective bearer or QoS flow, the MN may take actions to minimise service interruption due to activation of MR-DC (Data forwarding).

9-12.	If applicable, the update of the UP path towards the 5GC is performed via a PDU Session Path Update procedure.

--------------------------------------------------- Next Change -----------------------------------------------------------------

## 10.3	Secondary Node Modification (MN/SN initiated)

### 10.3.1	EN-DC

The Secondary Node Modification procedure may be initiated either by the MN or by the SN and be used to modify, establish or release bearer contexts, to transfer bearer contexts to and from the SN or to modify other properties of the UE context within the same SN. It may also be used to transfer an NR RRC message from the SN to the UE via the MN and the response from the UE via MN to the SN (e.g. when SRB3 is not used). In case of CPA or CPC, this procedure is used to configure or modify CPA or CPC configuration within the same SN. In case of CPA or CPC, this procedure may also be triggered by the target SN to add some prepared PSCells from the suggested list or cancel part of the prepared PSCells. This procedure may be initiated by the MN or SN to request the SN or MN to deactivate or activate the SCG.

Text Omitted Without Change

--------------------------------------------------- Next Change -----------------------------------------------------------------

### 10.3.2	MR-DC with 5GC

The SN Modification procedure may be initiated either by the MN or by the SN and be used to modify the current user plane resource configuration (e.g. related to PDU session, QoS flow or DRB) or to modify other properties of the UE context within the same SN. It may also be used to transfer an RRC message from the SN to the UE via the MN and the response from the UE via MN to the SN (e.g. when SRB3 is not used). In NGEN-DC and NR-DC, the RRC message is an NR message (i.e., RRCReconfiguration) whereas in NE-DC it is an E-UTRA message (i.e., RRCConnectionReconfiguration). In case of CPA or CPC, this procedure is used to configure or modify CPC configuration within the same SN. In case of CPA or CPC, this procedure may also be triggered by the target SN to add some prepared PSCells from the suggested list or cancel part of the prepared PSCells. The CPC configuration cannot be used to configure target PSCell in NE-DC or in NGEN-DC. This procedure may be initiated by the MN or SN to request the SN or MN to activate or deactivate the SCG.

Text Omitted Without Change

--------------------------------------------------- Next Change -----------------------------------------------------------------

## 10.5	Secondary Node Change (MN/SN initiated)

### 10.5.1	EN-DC

The Secondary Node Change procedure is initiated either by MN or SN and used to transfer a UE context from a source SN to a target SN and to change the SCG configuration in UE from one SN to another. In case of CPC, the Conditional Secondary Node Change procedure initiated either by the MN or SN is also used for CPC configuration and CPC execution.

Text Omitted Without Change

MN initiated conditional SN Change

The MN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

Figure 10.5.1-3: Conditional SN Change – MN initiated

Figure 10.5.1-3 shows an example signalling flow for the MN initiated Conditional Secondary Node Change:

1/2.	The MN initiates the conditional SN change by requesting the target candidate SN to allocate resources for the UE by means of the SgNB Addition procedure. The MN also provides the candidate cells recommended by MN via the latest measurement results for the SN to choose and configure the SCG cell(s), and provides the upper limit for the number of PSCells to prepare. Within the list of cells as indicated within the measurement results indicated by the MN, the SN decides the list of PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***) contained in the SgNB Addition Request Acknowledge message with the prepared PSCell ID(s). If forwarding is needed, the target SN provides forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration. The target SN can either accept or reject each of the candidate cells suggested by the MN, i.e. it cannot come up with any alternative candidates.

NOTE 5:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration before step 1.

3.	The MN sends to the UE an RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) including the CPC configuration, (i.e. a list of RRCConnectionReconfiguration** messages) and associated execution conditions, in which a RRCConnectionReconfiguration** message contains a RRCReconfiguration*** received from the candidate SN and possibly an MCG configuration. Besides, the RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements.

4.	The UE applies the RRC configuration (in RRCConnectionReconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message (RCConnectionReconfigurationComplete*) without any NR RRC response message. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration* message, it performs the reconfiguration failure procedure.

4a.	Upon receiving the RRCConnectionReconfigurationComplete message from the UE, the MN triggers the Data Forwarding Address Indication procedure to the source SN to inform that the CPC has been triggered, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding.

5.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration message (RRCConnectionReconfiguration**) corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete message (RRCConnectionReconfigurationComplete**), including an NR RRC message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN.

6a-6b.	The MN triggers the MeNB initiated SgNB Release procedure to inform the source SN to stop providing user data to the UE, and the address of the selected SN and if applicable, to start data forwarding.

7.	If the RRC connection reconfiguration procedure was successful, the MN informs the selected target SN via SgNBReconfigurationComplete message, including the RRCReconfigurationComplete*** message for the target SN. The MN sends the SgNB Release Request messages to cancel CPC in the other target candidate SNs, if configured.

8.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the target SN indicated in RRCConnectionReconfiguration**.

9.	For SN terminated bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends to the target selected SN, if needed.

10.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding address in step 3.

11.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 6:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped.

12-16.	If applicable, a path update is triggered by the MN.

17.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the target SN by cancellation of all the prepared PSCells at the target SN and releasing the CPC related UE context at the target SN.

Figure 10.5.1-4: Conditional SN Change – SN initiated

Figure 10.5.1-4 shows an example signalling flow for the Conditional Secondary Node Change initiated by the SN:

1.	The source SN initiates the conditional SN change procedure by sending SgNB Change Required message which contains a CPC initiation indication. The message also contains target SN ID(s) information and may include the SCG configuration (to support delta configuration), and contains the measurement results related to the target candidate SN(s). The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells, and may also include the SCG measurement configurations for CPC (e.g. measId(s) to be used for CPC).

2/3.	The MN requests the target candidate SN(s) to allocate resources for the UE by means of the SgNB Addition procedure(s), including a CPC initiation indication, and the measurements results related to the target candidate SN and indicate the list of proposed PSCell candidates received from the source SN, but not including execution conditions. Within the list of PSCells, the SN decides the list of PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration****) contained in the SgNB Addition Request Acknowledge message. If forwarding is needed, the target SN provides forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The target SN can either accept or reject each of the candidate cells suggested by the source SN, i.e. it cannot come up with any alternative candidates.

4.	The MN may indicate the candidate PSCells accepted by the target SN to the source SN via SgNB Modification Request message, if needed, e.g., when T-SN does not acknowledge all candidate PSCells, otherwise step 4 and 5 are skipped, e.g. when T-SN accepts all candidate PScells.

5.	The source SN may provide the updated measurement configurations and the execution conditions for CPC to the MN via SgNB Modification Request Acknowledge message.

6.	The MN sends to the UE an RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) including the CPC configuration, (i.e. a list of RRCConnectionReconfiguration*** messages) and associated execution conditions, in which a RRCConnectionReconfiguration*** message contains a RRCReconfiguration**** received from the candidate SN and possibly an MCG configuration. Besides, the RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements, as well as the NR RRC configuration message (RRCReconfigutation**) generated by the source-SN.

7.	The UE applies the RRC configuration (in RRCConnectionReconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message (RCConnectionReconfigurationComplete*), which can include an NR RRC response message (RRCReconfigurationComplete**). In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration* message, it performs the reconfiguration failure procedure.

8a.	If an NR RRC response message is included, the MN informs the source SN with the NR RRC response message (RRCReconfigutationComplete**) for the source SN via SgNB Change Confirm message. If step 4 and 5 are skipped, the MN will indicate the candidate PSCells accepted by the target SN(s) to the source SN in the SgNB Change Confirm message.

The MN sends the SgNB Change Confirm message towards the Source SN to indicate that CPC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the target SN, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding. In case multiple Target SNs are prepared, the MN includes a list of Target SgNB ID and list of data forwarding addresses to the source SN.

9a-9d.	The source SN may send the SgNB Modification Required message to trigger an update of CPC execution condition or corresponding SCG measConfig for CPC for the UE if any. In such case in step 9b and 9c, the MN reconfigures the UE as in step 6 and 7.

10.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration message (RRCConnectionReconfiguration***) corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete message (RRCConnectionReconfigurationComplete***), including an NR RRC message (RRCReconfigurationComplete****) for the selected candidate PSCell, and the selected PSCell information to the MN.

11a-11b.	The MN triggers the MeNB initiated SgNB Release procedure to inform source SN to stop providing user data to the UE, and provide the address of the selected target SN and if applicable, start late data forwarding.

12.	If the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete**** response message for the target SN. The MN sends the SgNB Release Request messages to cancel CPC in the other target candidate SNs, if configured.

13.	The UE synchronizes to the target SN indicated in RRCConnectionReconfiguration***.

14.	For SN terminated bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

15.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding message from the MN.

16.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 7:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SgNB may send the report when the transmission of the related bearer is stopped.

17-21.	If applicable, a path update is triggered by the MN.

22.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

--------------------------------------------------- Next Change -----------------------------------------------------------------

### 10.5.2	MR-DC with 5GC

Text Omitted Without Change

MN initiated conditional SN Change

The Conditional Secondary Node Change procedure is initiated by the MN for CPC configuration and CPC execution.

Figure 10.5.2-3: Conditional SN change procedure - MN initiated

Figure 10.5.2-3 shows an example signalling flow for the conditional SN Change initiated by the MN:

1/2.	The MN initiates the conditional SN change by requesting the target candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides the candidate cells recommended by MN via the latest measurement results for the SN to choose and configure the SCG cell(s), provides the upper limit for the number of PSCells to prepare. Within the list of cells as indicated within the measurement results indicated by the MN, the SN decides the list of PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***) contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration. The target SN can either accept or reject each of the candidate cells suggested by the MN, i.e. it cannot come up with any alternative candidates.

NOTE 4:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1.

3.	The MN sends to the UE an RRC reconfiguration message (RRC reconfiguration*) including the CPC configuration, (i.e. a list of RRC reconfiguration** messages) and associated execution conditions, in which a RRC reconfiguration** message contains a RRCReconfiguration*** received from the candidate SN and possibly an MCG configuration. Besides, the RRC reconfiguration message (RRC reconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements.

4.	The UE applies the RRC configuration (in RRC reconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRC reconfiguration complete message (RRC reconfiguration complete*) without any NR SN RRC response messag). In case the UE is unable to comply with (part of) the configuration included in the RRC reconfiguration* message, it performs the reconfiguration failure procedure.

4a.	Upon receiving the MN RRC reconfiguration complete message from the UE, the MN informs the SN that the CPC has been triggered via Xn-U Address Indication procedure, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding.

5.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRC reconfiguration message (RRC reconfiguration**) corresponding to the selected candidate PSCell, and sends an MN RRC reconfiguration complete message (RRC reconfiguration complete**), including an NR RRC reconfiguration complete message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN.

Editor's Note: Whether a message is needed and which message is used to inform source SN to stop providing user data to the UE, and the address of the selected target SN and if applicable, to start late data forwarding are FFS.

6a-6c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the source SN the address of the selected target SN and if applicable, to start late data forwarding.

7.	If the RRC connection reconfiguration procedure was successful, the MN informs the selected target SN via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete*** response message for the target SN. The MN sends the SN Release Request messages to cancel CPC in the other target candidate SNs, if configured.

8.	If configured with bearers requiring SCG radio resources the UE synchronizes to the selected SN indicated in RRC reconfiguration**.

9.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target selected SN, if needed.

10.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding address in step 3.

11.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 5:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

12-16.	If applicable, a PDU Session path update procedure is triggered by the MN.

17.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the target SN by cancellation of all the prepared PSCells at the target SN and releasing the CPC related UE context at the target SN.

Figure 10.5.2-4: Conditional SN change procedure - SN initiated

Figure 10.5.2-4 shows an example signalling flow for the conditional SN Change initiated by the SN:

1.	The source SN initiates the conditional SN change procedure by sending the SN Change Required message, with a CPC initiation indication. The message also contains candidate target node ID(s) and may include the SCG configuration (to support delta configuration), and contains the measurements results which may include cells that are not CPC candidates. The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells to prepare, and may also include the SCG measurement configurations for CPC (e.g. measId(s)to be used for CPC).

2/3.	The MN requests the target SN(s) to allocate resources for the UE by means of the SN Addition procedure(s), including a CPC initiation indication, and the measurements results which may include cells that are not CPC candidates received from the source SN to the target SN, and indicates the list of proposed PSCell candidates received from the source SN, but not including execution conditions. Within the list of PSCells, the SN decides the list of PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration****) contained in the SgNB Addition Request Acknowledge message. If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The target-SN can either accept or reject each of the candidate cells suggested by the Source-SN, i.e., it cannot come up with any alternative candidates.

4.	The MN may indicate the candidate PSCells accepted by the target SN to the source SN via SN Modification Request message, if needed, e.g., when T-SN does not acknowledge all candidate PSCells, otherwise step 4 and 5 are skipped, e.g. when T-SN accepts all candidate PScells.

5.	The source SN may provide the updated measurement configurations and the execution conditions to the MN via SN Modification Request Acknowledge message.

6.	The MN sends to the UE an RRC reconfiguration message (RRC reconfiguration*) including the CPC configuration, (i.e. a list of RRC reconfiguration*** messages) and associated execution conditions, in which a RRC reconfiguration*** message contains a RRCReconfiguration**** received from the candidate SN and possibly an MCG configuration. Besides, the RRC reconfiguration message (RRC reconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements, as well as the NR RRC configuration message (RRCReconfiguration**) generated by the source-SN.

7.	The UE applies the RRC configuration (in RRC reconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRC reconfiguration complete message (RRC reconfiguration complete*), which can include an NR RRC response message (RRCReconfigutationComplete**). In case the UE is unable to comply with (part of) the configuration included in the RRC reconfiguration* message, it performs the reconfiguration failure procedure.

8a.	If an SN RRC response message is included, the MN informs the source SN with the SN RRC response message (RRCReconfigutationComplete**) for the source SN via SN Change Confirm message. If step 4 and 5 are skipped, the MN will indicate the candidate PSCells accepted by the target SN(s) to the source SN in the SgNB Change Confirm message.

The MN sends the SN Change Confirm message towards the Source SN to indicate that CPC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the target SN, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding. In case multiple Target SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN.

9a-9d.	The source SN may send the SN Modification Required message to trigger an update of CPC execution condition or corresponding SCG measConfig for CPC for the UE if any. In such case in step 9b and 9c, the MN reconfigures the UE as in step 6 and 7.

10.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRC reconfiguration message (RRC reconfiguration***) corresponding to the selected candidate PSCell, and sends an RRC reconfiguration complete message (RRC reconfiguration complete***), including an NR RRC message (RRCReconfigurationComplete****) for the selected candidate PSCell, and the selected PSCell information to the MN.

11a-11c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the source SN the address of the selected target SN and if applicable, starts late data forwarding.

12.	If the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete**** response message for the target SN. The MN sends the SN Release Request messages to cancel CPC in the other target candidate SNs, if configured.

13.	The UE synchronizes to the target SN indicated in RRCConnectionReconfiguration***.

14.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

15.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the data forwarding address related information from the MN.

16.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 6:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

17-21.	If applicable, a PDU Session path update procedure is triggered by the MN.

22.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

--------------------------------------------------- Next Change -----------------------------------------------------------------

## 10.6	PSCell change

In MR-DC, a PSCell change does not always require a security key change.

If a security key change is required, this is performed through a synchronous SCG reconfiguration procedure towards the UE involving random access on PSCell and a security key change, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For SN terminated bearers, PDCP is re-established. In all MR-DC options, to perform this procedure within the same SN, the SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a S-KgNB (for EN-DC, NGEN-DC and NR-DC) or S-KeNB (for NE-DC) update is required when the procedure is initiated by the SN or including the SgNB Security Key / SN Security Key when the procedure is initiated by the MN. In all MR-DC options, to perform a PSCell change between different SN nodes, the SN Change procedure as described in clause 10.5 is used.

If a security key change is not required (only possible in EN-DC, NGEN-DC and NR-DC), this is performed through a synchronous SCG reconfiguration procedure without security key change towards the UE involving random access on PSCell, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For DRBs using RLC AM mode PDCP data recovery applies, and for DRBs using RLC UM no action is performed in PDCP. For SRB3 PDCP may discard all stored SDUs and PDUs. Unless MN terminated SCG or split bearers are configured, this does not require MN involvement. In this case, if location information was requested for the UE, the SN informs the MN about the PSCell change (as part of location information) using the SN initiated SN modification procedure independently from the reconfiguration of the UE. In case of MN terminated SCG or split bearers, the SN initiated SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a PDCP data recovery is required. If the MN subscribes to PSCell changes to retrieve the SCG UE history information, the SN informs the MN about the SCG UE history information using the SN initiated SN modification procedure.

A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change is triggered. Intra-SN CPC without MN involvement, inter-SN CPC initiated either by MN or SN are supported.

The following principles apply to CPC:

-	The CPC configuration contains the configuration of CPC candidate PSCell(s) and execution condition(s) and may contain the MCG configuration for inter-SN CPC.

-	An execution condition may consist of one or two trigger condition(s) (CPC events A3/A5, as defined in TS 38.331 [4]). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evalution of CPC execution condition of a single candidate PSCell.

-	Before any CPC execution condition is satisfied, upon reception of PSCell change command or PCell change command, the UE executes the PSCell change procedure as described in clause 10.3 and 10.5 or the PCell change procedure as described in clause 9.2.3.2 in TS 38.300[3] or clause 10.1.2.1 in TS 36.300 [2], regardless of any previously received CPC configuration. Upon the successful completion of PSCell change procedure or PCell change procedure, the UE releases all stored CPC configurations.

-	While executing CPC, the UE is not required to continue evaluating the execution condition of other candidate PSCell(s).

-	Once the CPC procedure is executed successfully, the UE releases all stored CPC configurations.

-	Upon the release of SCG, the UE releases the stored CPC configurations.

CPC configuration in HO command, PSCell change command or conditional reconfiguration is not supported.

--------------------------------------------------- Change Ends -----------------------------------------------------------------