3GPP TSG RAN2 # 118e								R2-2206683

Electronic Meeting, 9-19 May 2022

Agenda Item:		6.0.1

Source:			Qualcomm Inc.

Title:		Implementing agreement on [Q302] from ASN.1 ad hoc

WI:   				NR_MBS-Core, NR_ext_to_71GHz-Core

Document for:		Discussion and decision

# Introduction

This document aims to provide a baseline for how to implement the changes agreed for [Q302] during ASN.1 ad hoc in the respective WI RRC CRs.

## 1.1 Relevant agreement from ASN.1 Ad-hoc (highlight added)

PDSCH-TimeDomainResourceAllocationList merging issue

R2-2204301	PDSCH-TimeDomainResourceAllocationList merging issue [Q300] [Q301] [Q302]	Qualcomm Incorporated	discussion	Rel-17	NR_ext_to_71GHz-Core, NR_MBS-Core

Noted

P1

- Ericsson prefer to keep the current design.

- Huawei think we attempt to do non-critical extension. Nokia agrees. Intel MTK agrees.

[Q300] Extend k0-r16 instead of introducing PDSCH-TimeDomainResourceAllocation-r17 Adopt changes shown in section 3.2.

[Q302] Remove last sentence in repetitionNumber field description and update the conditional presence table, as shown in section 3.1.

P2 no change needed (r16 version intended).

Relevant section 3.1 from R2-2204301 is copied below for quick reference.

-- Copy start--

#### 3.1	[Q302] Changes in conditional presence of repetitionNumber

Regardless of outcome of P1 and P2, this change is common.

PDSCH-TimeDomainResourceAllocation field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1). For k0-r17, only values {0..32} are applicable for PDSCH SCS of 120 kHz. When the field is absent the UE applies the value 0.

mappingType PDSCH mapping type (see TS 38.214 [19], clause 5.3).

repetitionNumber Indicates the number of PDSCH transmission occasions for slot-based repetition scheme in IE RepetitionSchemeConfig. The parameter is used as specified in 38.214 [19].

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary (see TS 38.214 [19], clause 5.1.2.1).

MultiPDSCH-TimeDomainResourceAllocation field descriptions

pdsch-AllocationList One or multiple PDSCHs which can be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 5.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Formats1-0and1-1 | In pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH, and SIB20, this field is absent. Otherwise, in pdsch-TimeDomainResourceAllocationList-r16 and pdsch-TimeDomainResourceAllocationList-r17, this field is optionally present, Need R. |

-- Copy End--

# Suggested split of changes on how to implement these agreements in respective WI CRs

## 2.1 For 71GHz WI CR for 38.331

PDSCH-TimeDomainResourceAllocation field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1). For k0-r17, only values {0..32} are applicable for PDSCH SCS of 120 kHz. When the field is absent the UE applies the value 0.

mappingType PDSCH mapping type (see TS 38.214 [19], clause 5.3).

repetitionNumber Indicates the number of PDSCH transmission occasions for slot-based repetition scheme in IE RepetitionSchemeConfig. The parameter is used as specified in 38.214 [19]. This field is not present when PDSCH-TimeDomainResourceAllocation-r16 is included in SIB20.

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary (see TS 38.214 [19], clause 5.1.2.1).

MultiPDSCH-TimeDomainResourceAllocation field descriptions

pdsch-AllocationList One or multiple PDSCHs which can be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 5.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Formats1-0and1-1 | In pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH, this field is absent. Otherwise, in pdsch-TimeDomainResourceAllocationList-r16, this field is optionally present, Need R. In pdsch-TimeDomainResourceAllocationList-r17, this field is optionally present, Need R. |

## 2.2 For MBS WI CR for 38.331

PDSCH-TimeDomainResourceAllocation field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1). For k0-r17, only values {0..32} are applicable for PDSCH SCS of 120 kHz. When the field is absent the UE applies the value 0.

mappingType PDSCH mapping type (see TS 38.214 [19], clause 5.3).

repetitionNumber Indicates the number of PDSCH transmission occasions for slot-based repetition scheme in IE RepetitionSchemeConfig. The parameter is used as specified in 38.214 [19].

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary (see TS 38.214 [19], clause 5.1.2.1).

MultiPDSCH-TimeDomainResourceAllocation field descriptions

pdsch-AllocationList One or multiple PDSCHs which can be in consecutive or non-consecutive slots (see TS 38.214 [19], clause 5.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Formats1-0and1-1 | In pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH, and SIB20, this field is absent. Otherwise, in pdsch-TimeDomainResourceAllocationList-r16 and pdsch-TimeDomainResourceAllocationList-r17, this field is optionally present, Need R. |

# Summary

With the above changes in respective CR, the final outcome will be as shown in 3.1 in R2-2204301 (also shown at the top of the document) i.e. as agreed during the ASN.1 ad hoc.

# Proposal

Proposal: To implement agreements for [Q302] from the ASN.1 ad-hoc, take the suggested split from section 2 as baseline in respective WI CRs.