3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208528

E-Meeting, 17th – 26th Aug., 2022

Agenda item:	8.4.2.1

Source:	Samsung

Title:	Scenario and Target Performance Enhancements for L1/L2 mobility

Document for:	Discussion & Decision

# Introduction

According to WID on Further NR mobility enhancements [1], the detail objectives to support L1/L2 inter-cell mobility is provided as below:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In this contribution, we provide the main scenario which meets the above objectives and the target performance enhancements in terms of latency reduction during the mobility.

# Discussion

## Scenario of L1/L2 Inter-cell Mobility

In Rel-17 feMIMO WI, inter-cell beam management (ICBM) using unified TCI framework was introduced. Main functionality of the inter-cell beam management using unified TCI framework is that:

- Both UL/DL channels use the unified TCI state which can be configured from inter-cell (i.e. each TCI state indicates the PCI info which the beam is originated from).

- UE can receive or transmit UE dedicated channels/signals via a TRP associated with a PCI different from the serving cell.

- Non-UE-dedicated channels/signals can only be received via a TRP associated with a PCI of the serving cell.

- Support a measurement configuration includes SSB resources associated with PCIs different from the PCI of a serving cell.

However, the L1/L2 based inter-cell mobility is not supported even though L1 measurement/report and L1/L2 signaling for inter-cell beam management are supported. The main functionality between Rel-17 ICBM and Rel-18 L1/L2 inter-cell mobility is described in Fig 1.

Fig 1: Scenario for Rel-17 inter-cell beam management and Rel-18 L1/L2 inter-cell mobility

The expected procedure on Rel-18 L1/L2 inter-cell mobility is summarized as below:

- UE receives from serving cell, configurations (L1 measurement/report configuration and pre-configuration of the cell applicable after inter-cell mobility) for the cells having different PCI from the serving cell.

- UE performs beam measurement and report L1 measurement to serving cell.

- Based on the L1 measurement reports, TCI state of the cell having different PCI from the serving cell is activated from the serving cell (by L1/L2 signaling).

- UE applies the inter-cell mobility (i.e. HO) and beam management simultaneously.

  - UE applies the pre-configured cell configuration

  - If needed (i.e. UL Timing advance is required), random access to the indicated cell is performed

  - UE starts receiving UE-dedicated PDSCH, PDCCH from target cell

  - UE starts transmitting UE-dedicated PUSCH, and PUCCH to target cell

Proposal 1: RAN2 adopt the basic scenario of L1/L2 inter-cell mobility using unified TCI framework.

- UE receives from serving cell, configurations (L1 measurement/report configuration and pre-configuration of the cell applicable after inter-cell mobility) for the cells having different PCI from the serving cell.

- UE performs beam measurement and report L1 measurement to serving cell.

- Based on the L1 measurement reports, TCI state of the cell having different PCI from the serving cell is activated from the serving cell (by L1/L2 signaling).

- UE applies the inter-cell mobility (i.e. HO) and beam management simultaneously.

  - UE applies the pre-configured cell configuration

  - If needed (i.e. UL Timing advance is required), random access to the indicated cell is performed

  - UE starts receiving UE-dedicated PDSCH, PDCCH from target cell

  - UE starts transmitting UE-dedicated PUSCH, and PUCCH to target cell

## Target Performance Enhancements

As shown in Fig 1, the HO procedure with beam management is significantly reduced i.e. latency reduction of HO + ICBM by L1/L2 inter-cell mobility.

- Legacy procedure (HO + BM)

- Step 1: RRC reconfiguration (L3 measurement/report, TCI state, etc.)

- Step 2: L3 measurement/report

- Step 3: L3 HO command

- Step 4: RRC reconfiguration (L1 measurement/report, TCI state, etc.)

- Step 5: L1 measurement/report

- Step 6: Beam management (i.e. TCI state indication)

- Rel-17 inter-cell BM + legacy HO

- Step 1: RRC reconfiguration (L1 measurement/report, TCI state, etc.)

- Step 2: L1 measurement/report

- Step 3: L1/L2 signaling for ICBM

- Step 4: L3 measurement/report

- Step 5: L3 HO command

- L1/L2 inter-cell mobility

- Step 1: RRC reconfiguration (L1 measurement/report, TCI state, pre-configuration for the cells having PCI different from the serving cell)

- Step 2: L1 measurement/report

- Step 3: L1/L2 signaling for HO with ICBM

We also think this L1/L2 inter-cell mobility can be subsequently applied after HO completion so it also reduce all inter-cell mobility procedures in terms of RRC configuration and measurement/report procedures.

From our understanding, the latency reduction factors and steps which can be obtained from L1/L2 inter-cell mobility compared with the legacy HO+BM model is that:

- L1 measurement based HO decision: Do not require the L3 measurement/report and HO indication by RRC

- HO indication with the optimal beam indication: Do not require the additional RRC reconfiguration and L1 measurement/report after HO completion

- Subsequent L1/L2 inter-cell mobility

Observation 1: L1/L2 inter-cell mobility provides the latency reduction upon handover with beam management.

- Do not require the L3 measurement/report and HO indication by RRC

- Do not require the additional RRC reconfiguration and L1 measurement/report after HO completion

- Support the subsequent L1/L2 inter-cell mobility after HO completion

## Applicable Deployment Scenario

First, we think all described scenarios in [1] can be feasible scenario so all below cases should be studied to support L1/L2 inter-cell mobility.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

In Fig 1, only intra-DU case is explained as an example and we think this deployment scenario is the default for the starting point because it is simple compared with other deployment scenario (i.e. intra-CU inter-DU case). However, inter CU case should be deprioritized because it requires more work on RAN2/3 i.e. signaling between CUs (e.g. XnAp) is also required.

For supporting L1/L2 inter-cell mobility on intra-CU inter-DU case, the assumption that source and target cells are not synchronized is needed. In this case, random access procedure toward the target cell after L1/L2 signaling for L1/L2 inter-cell mobility may be required. However, this random access procedure reduces the advantage of L1/L2 inter-cell mobility in terms of latency reduction during the inter-cell mobility.

Proposal 2: RAN2 need to support L1/L2 inter-cell mobility in case of intra-DU case and intra-CU inter-DU case (i.e. deprioritized the inter-CU inter-DU case).

Observation 2: Intra-CU inter-DU case may requires the random access procedure after L1/L2 signaling for L1/L2 inter-cell mobility.

Proposal 3: Timing advance management at least for intra-CU inter-DU case should be required and RAN2 need to enhance the procedures for latency reduction.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Observation 1: L1/L2 inter-cell mobility provides the latency reduction upon handover with beam management.

- Do not require the L3 measurement/report and HO indication by RRC

- Do not require the additional RRC reconfiguration and L1 measurement/report after HO completion

- Support the subsequent L1/L2 inter-cell mobility after HO completion

Observation 2: Intra-CU inter-DU case may requires the random access procedure after L1/L2 signaling for L1/L2 inter-cell mobility.

Proposal 1: RAN2 adopt the basic scenario of L1/L2 inter-cell mobility using unified TCI framework.

- UE receives from serving cell, configurations (L1 measurement/report configuration and pre-configuration of the cell applicable after inter-cell mobility) for the cells having different PCI from the serving cell.

- UE performs beam measurement and report L1 measurement to serving cell.

- Based on the L1 measurement reports, TCI state of the cell having different PCI from the serving cell is activated from the serving cell (by L1/L2 signaling).

- UE applies the inter-cell mobility (i.e. HO) and beam management simultaneously.

  - UE applies the pre-configured cell configuration

  - If needed (i.e. UL Timing advance is required), random access to the indicated cell is performed

  - UE starts receiving UE-dedicated PDSCH, PDCCH from target cell

  - UE starts transmitting UE-dedicated PUSCH, and PUCCH to target cell

Proposal 2: RAN2 need to support L1/L2 inter-cell mobility in case of intra-DU case and intra-CU inter-DU case (i.e. deprioritized the inter-CU inter-DU case).

Proposal 3: Timing advance management at least for intra-CU inter-DU case should be required and RAN2 need to enhance the procedures for latency reduction.

# References

RP-213565	New WID on Further NR mobility enhancements	MediaTek.