3GPP TSG-RAN WG2 #119-e	R2-22xxxxx

E-meeting, 17 – 26 August 2022

Agenda item:	6.4.3

Source: 	Samsung (rapporteur)

Title: 	Summary of discussion [AT119-e][018][IAB17] MAC (Samsung)

Document for:	Discussion and decision

# Introduction

This document captures the following discussion:

- [AT119-e][018][IAB17] MAC (Samsung)

Scope: Await online, Take into account online progress. Treat remaining parts R2-2208907, R2-2207188, R2-2207625, R2-2207782, R2-2208100, R2-2208102,

Determine agreeable parts, For agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

Section 2 captures rapporteur’s and individual companies’ comments on changes in [1] – [5], more specifically those changes NOT covered or related to the discussion in R2-2208907.

Section 3 then covers topics covered in R2-2208907 (for which initial discussion was held online during the ongoing meeting) with a view to progressing it further by collecting companies’ input.

# Minor / editorial / ’straightforward’ changes

(Please refer to each individual CV’s cover sheet for motivation/background for proposed change. Also note that only one change from [1] is included in the Table below – the remaining changes from [1] all appear to pertain to discussion in Section 3, and are predicated on answers to questions posed therein, and will therefore be treated – or not – depending on outcome of Section 3 discussions.)

| Source | Section proposed change relates to | Short description of proposed change | Rapporteur’s view, followed by individual companies view [Company name] [Comment] |
| --- | --- | --- | --- |
| [2] | 5.4.5 | Add the missing RRC parameter (logicalChannelGroup-IAB-Ext) to the list at the start of the clause. | [Rapporteur] Agree (from rapporteur CR). |
| [2] | 5.4.7 | Make the following change: IAB-MT may report Extended Pre-emptive BSR (as defined in clause 6.1.3.1) if the MAC entity of the IAB-MT is configured with logicalChannelGroup-IAB-Ext by upper layers. Otherwise IAB-MT may report Pre-emptive BSR (as defined in clause 6.1.3.1). | [Rapporteur] Agree (from rapporteur CR). [Huawei]: We think the current spec is also fine, which is more like IAB implementation. Fine to go with majority |
| [2] (similar change also in [3]) |  | Make the following change: A MAC PDU shall contain at most one Pre-emptive BSR MAC CE or Extended Pre-emptive BSR MAC CE, even when multiple events have triggered a Pre-emptive BSR. | [Rapporteur] Agree (from rapporteur CR). [Huawei]: [3] also propose another change in the next paragraph. “All triggered Pre-emptive BSR(s) shall be cancelled when a MAC PDU is transmitted and this PDU includes the corresponding Extended Pre-emptive BSR or Pre-emptive BSR MAC CE.” |
| [3] | 5.18.19 | Make the following change: The MAC entity may: 1> if a Desired Guard Symbol query has not been triggered: 2> trigger a Desired Guard Symbol query for this Serving Cell for the timing case(s) configured by Timing Case Indication MAC CE. | [Rapporteur] Not sure this is needed (it should be clear from description of Timing Case Indication MAC CE). [Huawei]: In R16, there is only one type of Guard Symbol query for this Serving Cell. In R17, we need to clarify that MT can generate more types based on the configured timing case. |
| [5] | 5.18.27 6.1.3.63 | Make the following changes (plus a handful of other related changes): The provided DL TX Power Adjustment MAC CE is used by an IAB-node to indicate to its child node spatial and frequency resources where the DL TX power adjustment contained in the MAC CE applies. Desired DL TX Power Adjustment MAC CE is used by an IAB-node to indicate to its parent node recommendations for such a restriction. Time resources where these restrictions/recommendations apply are indicated via RRC. Upon reception of a provided DL TX Power Adjustment MAC CE the IAB-node shall: … #### 6.1.3.63 DL TX Power Adjustment MAC CEs The DL TX Power Adjustment MAC CEs (i.e. Provided DL TX Power Adjustment MAC CE and Desired DL TX Power Adjustment MAC CE) are identified by MAC subheaders with eLCIDs as specified in Table 6.2.1-1b and Table 6.2.1-1b, respectively. Otherwise the format of the two MAC CEs is identical. Each has a variable size with following fields (Figure 6.1.3.63-1): | [Rapporteur] Not needed as the name of the MAC CE is “DL TX Power Adjustment CE”, and not “Provided DL TX Power Adjustment CE”. ([4] makes a proposal for a related change to name of MAC CE.) Rationale in [4] is to align this with Guard Symbols MAC CEs – but rapporteur proposes we stick with existing naming matching the naming of parameters and coming from RAN1. [Huawei]: Seems not essential to make the change. |
| [2], [5] | 6.1.3.37 | Correct the typo in the two Figures. | [Rapporteur] Agree (from rapporteur CR). |
| [5] | 6.1.3.62 | Make the following change: - Cell info type IDij: indicates the type of information included in the Cell info IDij field. If the value of the field is set to zero, the combination of {MT CC, DU cell} is included in the Cell info IDij field, by including the IAB-DU cell in the first nine bits of the field, the IAB-MT Serving Cell index into the next 5 bits of the field, and by setting the remaining bit to zero. If the value of the field is set to one, only MT CC information is included in the Cell info IDij field, by including the IAB-MT Serving Cell index in the first 5 bits of the field, and by setting the remaining ten bits to zero. The length of the field is 1 bit; | [Rapporteur] The text does not appear to be incorrect as it currently stands, so the change is not necessary. [Huawei]: Changes are not essential. |
| [5] | 6.1.3.63 | Make the following change: DL Tx Power adjustment: For the case of the Desired DL TX Power Adjustment MAC CE, this field contains desired DL TX power adjustment sent by an IAB-MT to assist with the parent node's DL TX power allocation. For the case of the DL TX Power Adjustment MAC CE, this fields contains adjustment to a node's DL TX power sent by its parent node. The value of the power adjustment is in both cases contained in the 5 right-most bits of the field, while the remaining 3 bits are set to zero. The length of the field is 8 bits; | [Rapporteur] Agree (from rapporteur CR). |
| [5] | 6.1.3.64 | Make the following change: - PSD range: This field contains the desired PSD range a node sends to its parent node to help with the sending node's MT UL TX power control. The 4 left-most bits of the field indicate the offset, while the next 7 left-most bits indicate the Pmax range. The remaining 5 bits are set to zero. The length of the field is 16 bits; | [Rapporteur] Agree (from rapporteur CR). |
| [2] | 6.1.3.65 | Make the following change: - Timing mode IDi: when this field is set to 00, the timing mode applied to the ith slot in the list of slots slotList provided via RRC in IAB-ResourceConfig is Case-1; when this field is set to 01, the timing mode applied to the ith slot in the list of slots slotList provided via RRC in IAB-ResourceConfig is Case-6; when this field is set to 10, the timing mode applied to the ith slot in the list of slots slotList provided via RRC in IAB-ResourceConfig is Case-7. | [Rapporteur] Agree (from rapporteur CR). |
| [2] | 6.1.3.65 | Make the following change: The total number N of Timing mode IDi fields corresponds to the size of slotList provided via RRC in IAB-ResourceConfig. If number N is not divisible by 4, (4 - N mod 4) of the final Timing mode IDi fields are disregarded by the receiving IAB node. The length of the field is 2 bits. | [Rapporteur] Agree (from rapporteur CR). |
| [5] | 6.1.3.61 6.1.3.62 6.1.3.63 6.1.3.64 6.1.3.65 | Change IAB-ResourceConfigID to iab-ResourceConfigID | [Rapporteur] Agree, to align with RRC spec. |
| [1] | 6.1.3.61 6.1.3.62 6.1.3.63 | Change CSI-RS ID to CSI-RS index | [Rapporteur] Agree. |
| [2] | 6.1.3.62 6.1.3.63 6.1.3.64 6.1.3.65 | Make the following change: - Resource Configuration ID: This field indicates the RRC configuration which applies to this MAC CE and corresponds to IAB-ResourceConfigID parameter in IAB-ResourceConfig as specified in TS 38.331 [5]. The length of the field is 16 bits; | [Rapporteur] Agree (from rapporteur CR). |

# Continuation of online discussion: changes discussed in R2-2208907

Based on R2-2208907 (please refer to it for background/motivation), and the online discussion captured in Chair’s notes, the following is proposed (closely following R2-2208907, since no decisions were made in the online session, while R2-2208907 was found to capture key issues well), input is kindly asked from contributing companies:

- Q1. Should we keep the existing (potentially more flexible) design, or to adopt the change from [1] (or similar change) ensuring that recommended/restricted beam can only be associated with one single IAB-MT/child IAB-DU cell?

| - Company | - Answer (NO - adopt change from [1] / YES - keep spec as is) |
| --- | --- |
| - Qualcomm | - No, adopt change from [1]. Based on feedback from our RAN1 team, CR proposed in [1] fixes suboptimalities of MAC CE. We are willing to compromise to Yes in case there is a large number of companies that do not like the change. |
| - Huawei, HiSilicon | - No strong view. - If majority wants to go with “ensuring that recommended/restricted beam can only be associated with one single IAB-MT/child IAB-DU cell”, we should add one sentence to clarify this rather than changing the MAC CE format. |
| - Ericsson | - Yes, keep spec as is - We think the current design reflects the RAN1 agreement - Can indicate any {MT CC, DU cell} pair. If RAN2 does not implement {MT CC, DU cell} indication, it should inform RAN1 and get a RAN1 approval - All 4 types association (slot format, multiplexing info, {MT CC, DU cell} info, IAB-MT recommended beams) can be indicated independently - The current design can also provide the same indication as the modified version. The current design can provide {MT CC, --} or {--, DU cell} indication |

- Q2. Should each IAB-MT beam associated with a restricted beam be indicated per single cell pairing in Child IAB-DU Restricted Beam Indication MAC CE?

| - Company | - Answer (YES - adopt change from [1] / NO - keep spec as is) |
| --- | --- |
| - Qualcomm | - Yes |
| - Huawei, HiSilicon | - Just to clarify: it seems the current spec already support this. - Cell info type IDij: indicates the type of information included in the Cell info IDij field. If the value of the field is set to zero, the combination of {MT CC, DU cell} is included in the Cell info IDij field, by including the IAB-DU cell in the first nine bits of the field, the Child IAB-MT Serving Cell index into the next 5 bits of the field, |
| - Ericsson | - We do not understand why this is an error. We think the current implementation is correct. - RAN1 has no agreement on how many associated IAB-MT recommended beams can be indicated to one child IAB-DU restricted beam. Consider 32 IAB-MT CCs and each one can have maximum 8 recommended beams, it could be 32x8=256 IAB-MT recommended beams associated to one child IAB-DU restricted beams. |

- Q3. Should a new field be added in DL TX Power Adjustment and Desired DL TX Power Adjustment MAC CEs, to indicate the index of reference CSI-RS used for (desired) DL Tx Power adjustment?

| - Company | - Answer (YES - adopt change from [1] / NO - keep spec as is) |
| --- | --- |
| - Qualcomm | - Yes |
| - Huawei, HiSilicon | - Seems Yes |
| - Ericsson | - There is a mismatch between RAN1 agreement and RAN1 spec which should be sorted out by RAN1 first. According to RAN1 agreement, the MAC CE provides a relative offset to CSI-RS TX power Agreement(RAN#107) - The indicated desired/provided DL TX power adjustment is in terms of a relative offset to the PDSCH a CSI-RS TX power that is RRC configured. - But the RAN1 spec 38.213 states that MAC CE provides a PDSCH power offset - The PDSCH EPRE can be derived from a downlink CSI-RS EPRE as described in [6, TS 38.214] and a PDSCH power offset provided by powerControlOffsetIAB as described in [11, TS 38.321]. |

- Q4. Is indicating whether multiplexing mode information is applicable to non-overlapping frequency resources (already supported in the current spec) equivalent to indicating requirement (or lack thereof) for FDM?

| - Company | - Answer (NO - adopt change from [4] / YES - keep spec as is) |
| --- | --- |
| - Qualcomm | - Yes, keep spec as is. |
| - Ericsson | - According to F1AP, multiplexing mode is provided per {MT CC, DU cell} pair. Rel-16 multiplexing mode does not differentiate FDM and SDM. Rel-17 multiplexing info has included an indication for FDM - For child IAB-DU restricted beam, the following association requires an indication of FDM resource. With rel-16 multiplexing info it is not clear simultaneous operation is based on FDM or SDM. - For 6.1.3.61, there is an additional association with FDM-required or FDM-not-required: DU resource configuration ID: when this field is set to 00, the provided power adjustment is applied on FDM resources where the simultaneous MT and DU signals are non-overlapping in the frequency-domain; when this field is set to 01, the provided power adjustment is applied on non-FDM resources where the simultaneous MT and DU signals may overlap in the frequency-domain for a given (MT CC, DU cell); when this field is set to 10, the provided power adjustment is applied on FDM resources where the simultaneous MT and DU signals are non-overlapping in the frequency-domain, and on non-FDM resources where the simultaneous MT and DU signals may overlap in the frequency-domain for a given (MT CC, DU cell). The length of the field is 2 bits; - For the other MAC CEs (e.g., 6.1.3.38, 6.1.3.39, 6.1.3.62, 6.1.3.63, 6.1.3.64, …) can also be indicated only apply to FDM or both FDM and SDM. |

- Q5. Should multiplexing mode info be indicated per cell pairing for the case of Child IAB-DU Restricted Beam Indication/IAB-MT Recommended Beam Indication MAC CE?

| - Company | - Answer (YES - adopt change from [1] / NO - keep spec as is) |
| --- | --- |
| - Qualcomm | - Yes |
| - Huawei, HiSilicon | - Similar to Q2 |
| - Ericsson | - Yes |

# References

- R2-2207188	Corrections on IAB related MAC CEs	ZTE, Sanechips

- R2-2207625	Miscellaneous corrections to 38.321 on Integrated Access and Backhaul for NR Rel-17	Samsung R&D Institute UK

- R2-2207782	Corrections on the Desired Guard Symbol query and extended pre-BSR	Huawei, HiSilicon

- R2-2208100	Corrections to multiplexing mode info definition	Ericsson

- R2-2208102	Miscellaneous MAC Corrections	Ericsson