3GPP TSG RAN WG2 Meeting #119bis-e                                                       R2-2209583

# Electronic meeting, October 10th – 19th, 2022

Agenda item:	8.9.2

Source:		Intel Corporation

Title:		Discovery and reselection with UE-to-UE relaying

Document for:	Discussion

Introduction

As per the new WI on Sidelink Relaying enhancements [1] we have an objective to specify L2 and L3 based UE-to-UE relay. We started initial discussions during RAN2#119e and made the following agreements related to scenarios, assumptions and some requirements to be met to support U2U relaying discovery and (re)selection.

Agreement:

RAN2 confirm that the Scenario, Assumption and Requirement in section 5.1 of TR 38.836 apply for UE-to-UE relay support, with below clarifications:

-	For cast type on UE-to-UE communication, only unicast is considered

-	FFS if coverage and RRC state aspects need to be revisited in light of the existing U2N support.

-	RAN2 will follow SA2 decision on the discovery model including cast type.

gNB will not configure a Uu RSRP threshold to be used by U2U Relay or Remote UE to determine whether to transmit U2U discovery signalling.  FFS what conditions would govern transmission of the discovery signalling.

In this contribution, we aim to provide further details of these aspects considering the study item in Rel-17 and work item specification of L2/L3 U2N relay in Rel-17. The corresponding conclusions from these previous discussions are provided in the annex for reference.

Discussion

SA2 TR 23.700-33 lays out the key issue#1 as support of UE-to-UE Relay and one of the first aspects to study is the following: “- How to discover UE-to-UE Relay(s) and (re)-select a UE-to-UE Relay UE in proximity.” There are several solutions that are being considered for discovery as per the TR and we have summarized them below for quick reference.

Table 1. Summary of SA2 solutions for U2U Discovery

| Solution# | Details |
| --- | --- |
| Sol# 1 | No relay discovery - Integrate discovery into unicast establishment - Integrate discovery into model B |
| Sol#2, sol# 9 | Announcement based U2U relay discovery for L3 U2U relaying, Relay UE announces the U2U relaying capability with target UE list |
| Sol# 3 | Group member discovery with model A and model B where each UE discovers the other UEs (including Relay) via group member discovery procedure |
| Consolidated sol# 10 | Relay UE announces the U2U relaying capability with target UE list [U2U Relay has previous discovery and has existing or prior connection with the target UE) Both model A and model B can be used. In model B, source UE sends solicitation with target UE info to nearby Relay UEs and the Relay UE which has information of the target Relay UE sends response) |
| Sol# 12, sol# 30, sol# 33 | Model A and model B based discovery where the Relay UE discovers other UEs in proximity and sends response to the source UE |
| Sol# 34 | Model A and model B route discovery where the announcement/solicitation message from the source UE is also relayed by the nearby Relay UEs with E2E discovery information until it reaches the intended destination through some Relay UE. |

  - Discovery

During the last meeting, as shown above, we have agreed that RAN2 is to rely on SA2 for the UE-to-UE relay discovery message definition and the L2 ID design. Irrespective of the solution for discovery mechanism to be defined in SA2, we can consider that many of the agreements from previous discussions on U2N Relay are still quite relevant and applicable for U2U Relay and we cover them in this section. To begin with, the protocol stack for the discovery message is considered to be as shown in figure 1 (similar to the control plane stack for SCCH for PC5-S in TS 38.300.

Figure 1. Protocol stack of discovery message for UE-to-UE Relay

Unlike UE-to-NW Relay case where the Relay UE is allowed to trigger relay discovery transmission only when it fails to meet a certain Uu link quality threshold, we think that this is not necessary  for UE-to-UE relaying case and leave it to upper layer as was the study item conclusion as well. This is assuming that the UEs involved in U2U relaying have corresponding authorization from the network.

The discovery message could be defined by the SA2 in a way that the Relay UE can advertise itself as a Relay only when it has reachability to some proximity UEs (e.g. it has a target UE list) and it is triggered by the upper layer to act as a U2U Relay UE. At the same time, since we already agreed that gNB will not configure any Uu RSRP thresholds for the Remote/Relay UEs, we can agree that the discovery transmission is primarily triggered by the upper layer. We can further discuss whether the Relay UE needs to satisfy any other AS layer conditions before acting as a Relay apart from the thresholds.

Proposal 1: 	U2U Remote UE(s) and U2U Relay UE are allowed to trigger discovery transmission when triggered by the upper layer. FFS whether the U2U Relay UE needs to fulfil any AS layer criterion in addition to upper layer trigger.

Similar to the case of U2N Relaying, there is no need for any combination of RRC state restriction between the Remote UE(s) and the Relay UE for U2U relaying operation and specifically, the Relay UE need not be in RRC_CONNECTED specifically for forwarding source Remote UE data. At the same time, we have an FFS from past meeting, related to checking for coverage and RRC state aspects as we have corresponding support for U2N relaying. From U2N relaying case, we understand that the L2 and L3 U2N Remote UE and Relay UE can be in any RRC state while in-coverage to obtain the configuration from the network and we can apply similar principle to U2U relaying. However, for out-of-coverage scenario, we have different behaviors from Remote UE and Relay UE as well as for L2 and L3 based cases. For L2 case, there is the condition that U2N relay UE has to always rely on network configuration (i.e. either via SIB or dedicated) while L3 U2N relay UE can rely on pre-configuration if SIB-based configuration is unavailable and L2/L3 U2N Remote UE can rely on pre-configuration if the network configuration is not available.

Since we already agreed that the gNB will not configure a Uu RSRP threshold for U2U Relay, it can potentially rely on pre-configuration while out-of-coverage to act as a U2U Relay UE (for both L2 and L3 cases) unlike the U2N relaying case. Therefore, the configuration for discovery can be either based on pre-configuration or provided by the network depending on the RRC state/coverage situation of the UEs.

Proposal 2a: 	In-coverage U2U Remote UE and in-coverage U2U Relay UE in RRC_CONNECTED can rely on network configuration provided via dedicated signalling for discovery configuration.

Proposal 2b: 	In-coverage U2U Remote UE and in-coverage U2U Relay UE in RRC_IDLE or RRC_INACTIVE can rely on network configuration provided via system information for discovery configuration.

Proposal 2c: 	Out-of-coverage U2U Remote UE and out-of-coverage U2U Relay UE (L2 and L3) neither in RRC_CONNECTED nor in RRC_IDLE or RRC_INACTIVE can rely on pre-configuration for discovery configuration.

For carrying the discovery message, we can consider that the same SL-SRB4 defined for U2N Relaying can be applicable in this case. And, similar to U2N relaying, there is no need for ciphering and integrity protection in PDCP layer for the discovery messages.

Proposal 3: 	SL-SRB4 defined in Rel-17 along with its parameters and PDCP PDU format can be reused for U2U discovery message as well (with fixed parameters and SCCH configuration).

Proposal 4: 	No ciphering and integrity protection in PDCP layer is needed for U2U relaying discovery messages.

Unlike U2N relaying, for U2U relaying, we think that the Relay UE need not be in RRC_CONNECTED state to forward packets from the U2U Remote UE and in general, both the UEs can be in any RRC state to be able to perform U2U relaying.

Proposal 5: 	There is no restriction on the RRC states of any UEs involved in UE-to-UE relaying and the U2U Relay UE need not be in RRC_CONNECTED state for specifically forwarding source Remote UE’s data towards the target UE.

Overall, we notice that the fundamental functionality of relay discovery operation will be similar between U2N relay discovery and U2U relay discovery and many aspects can be reapplied for the U2U relay discovery case. Depending on SA2 conclusion, the contents of the discovery message as well as whether the Relay UE performs its own discovery towards target UEs and then only the source UEs can find the Relay UEs correspondingly might change, but discovery over an individual UE-to-UE link will remain the same and many of the AS layer principles from prior work can be reused.

Considering the case when Relay/Remote UEs are in-coverage of the serving frequency in different RRC states, and performing SL communication on the same or different frequency, the following agreements were made for U2N relaying scenario related to discovery configuration. The corresponding specification details are provided in the annex.

RAN2 agree that for relay/remote UE in RRC IDLE/INACTIVE state, in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency:

- If there is Uu deployed coverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency.

- If there is no Uu deployed coverage at the concerned frequency, UE shall rely on pre-configuration.

RAN2 agree that for relay/remote UE in RRC IDLE/INACTIVE state, in-coverage on the serving frequency，if the serving frequency is shared with concerned SL frequency :

- If there is no discovery related SIB broadcasted on the serving carrier, UE does not perform SL discovery transmission/reception on the concerned frequency.

For the case of U2U relaying, we can follow the same principle when the UEs are in-coverage to allow inter-working of Uu deployment and SL service.

Proposal 6: 	U2U relaying to follow U2N relaying principles to obtain discovery configuration/related SIB for in-coverage U2U relay/remote UE in RRC_CONNECTED/RRC_IDLE/ RRC_INACTIVE states for a given serving frequency and SL frequency which may or may not be shared with the serving frequency.

At the same time, even though we think that it is not necessary for the Remote UE or Relay UE to enter RRC_CONNECTED in general to perform U2U relaying itself, however, for e.g. to acquire dedicated configuration on Tx resource pool where such configuration is absent in the SIB, we understand that we want to maintain the existing framework for discovery as in legacy V2X design as well as U2N relay discovery cases. The following agreements were made for the U2N relaying scenario and the same can be applicable for U2U relaying.

RAN2 agrees that for relay/remote UE in RRC IDLE/INACTIVE state, and in-coverage on the serving frequency, if there is discovery related SIB broadcasted on the serving frequency, and if the configuration of concerned SL frequency is included within the SIB of the serving frequency but the Tx resource pool configuration is absent, UE shall enter RRC CONNECTED state to acquire dedicated configuration on Tx resource pool.

RAN2 agree that RRC_CONNECTED relay/remote UE which are in-coverage on the serving frequency, if there is discovery related SIB broadcasted on the serving frequency, and if the configuration of concerned SL frequency is included within the SIB of the serving frequency, it can only use the SL discovery Tx resource configuration provided by dedicated signalling if provided, or not transmit discovery if not provided.

RAN2 agree that RRC_CONNECTED L3 relay/remote UE or layer 2 remote UE which are in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency,

If there is Uu coverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency.

Proposal 7. 	For UEs in-coverage interested in U2U relaying to rely on U2N principles for acquiring TX resource pool configuration for the purpose of discovery (as outlined above).

  - U2U Relay (Re)selection

As a baseline scenario, we can assume that upon triggering from upper layer, the source Remote UE performs necessary radio measurements and determines (or selects) a suitable U2U Relay UE for PC5 connection setup upon discovery after checking the higher layer criteria as well. Whether in-coverage or out-of-coverage, the source Remote UE then establishes a secure unicast PC5-RRC connection with the target UE (using NR V2X as baseline) through the U2U relay UE. The detailed CP procedure will be discussed separately in the future.

Figure 2. Relay (re)selection example scenario

    - AS layer criteria for Relay (re-)selection

U2U Relay reselection is similar to U2N relay reselection discussed and specified in Rel-17 as the Remote UE to Relay UE link is PC5-based. Except for the additional Uu based trigger from the U2N Relay UE, from Remote UE point of view, the measurement aspect and selecting a Relay based on candidate Relay UEs can be directly repurposed. Therefore, based on the background discussions in LTE ProSe relay, the Rel-17 study item on SL Relay and as part of the Rel-17 U2N relay specification, we can conclude the following procedures (based on [4]) as applicable for U2U relay (re)selection:

1) PC5 Link Quality Measurement: For relay(s) without unicast PC5 connection, source Remote UE uses RSRP measurements of sidelink discovery messages (i.e. SD-RSRP) to evaluate whether PC5 link quality of a Relay UE satisfies relay selection and reselection criterion and for relay(s) when there is an existing unicast PC5 connection, if there is active data, the source Remote UE could use SL-RSRP measurements.

2) Trigger of relay selection: Triggered at source remote UE by upper layer. Some companies wish to consider that there is an existing direct link between the source Remote UE and target Remote UE and the PC5 link quality of this link falling below a specified threshold can be considered as a trigger as well.

3) Trigger of relay reselection: Triggered at source remote UE when: a) PC5 measurement towards current relay UE is below a (pre)configured threshold; or b) there is an upper layer release message or similar indication from current relay UE; or c) triggered by upper layer

4) How to choose relay UE in relay (re)selection: Source remote UE searches for suitable U2U candidate relay UEs which meet all AS-layer & higher layer criteria. If multiple such candidate relay UEs available, it is up to Remote UE implementation to choose one Relay UE.

Proposal 8: 	Source Remote UE uses PC5 measurements (either SD-RSRP or SL-RSRP) as the primary AS layer criterion to trigger U2U relay (re)selection.

Proposal 9: 	SL-RSRP or SD-RSRP is used as the SL measurement quantity for U2U relay reselection trigger evaluation and it can be up to UE implementation to choose between the two (e.g. when no active data transmission between relay and remote UE, it can choose SD-RSRP).

Proposal 10:  	U2U Relay selection can be triggered at source Remote UE based on a) trigger from upper layer b) PC5 measurement towards target Remote UE being below a (pre)configured threshold.

Proposal 11:	U2U Relay reselection can be triggered at source Remote UE when: a) PC5 measurement towards current serving Relay UE is below a (pre)configured threshold; or b) there is an upper layer release message or similar indication from current serving Relay UE; or c) triggered by upper layer.

      - Link quality thresholds

The network defines the necessary thresholds and hysteresis values that the Remote UE uses for relay reselection. If the UE is out-of-coverage, it can rely on preconfiguration values of the thresholds. Furthermore, when SD-RSRP is used, Remote UE applies L3 filtering across the measurements on the DMRS of PSSCH carrying discovery message from the relay UE similar to U2N Relaying scenario [4].

Proposal 12: 	PC5 link quality threshold, L3 filter coefficient for SD-RSRP/SL-RSRP and hysteresis can be provided via SIB/RRC by gNB or obtained via pre-configuration.

Proposal 13: 	When SD-RSRP measurement from discovery message is used for relay (re)selection trigger and candidate relay evaluation, L3 filtering is applied across measurements on the DMRS of PSSCH transmission which carries discovery message from the concerned U2U relay.

      - Need for additional AS criteria

        - Relay load

One AS layer criterion that was discussed but not specified in Rel-17 for L2 U2N relay was the relay load. We could not arrive at a common methodology for specifying the relay load itself. Some of the requirements that had to be fulfilled to define relay load were simple and easy compute, less signalling overhead, low specification impact and reflects performance that the Remote UE could achieve if served by the Relay UE [3]. Multiple options were further discussed for what could be the relay load criterion itself including the number of PC5 connections at the Relay UE, number of Remote UEs supported by the Relay UE, resource pool usage of the Relay UE, etc. Although we think that it can be an important criterion, considering how discovery is modelled, we feel that the Relay UE can take its load into account in handling discovery when a new source Remote UEs request a path towards another target UE.

Proposal 14: 	Relay load is not considered as an additional AS layer criterion for U2U relay (re)selection and Relay UE uses discovery modeling to control its load (i.e. no specification impact is foreseen).

        - RLF

In Rel-17, we support notification message over PC5-RRC from U2N Relay UE to U2N Remote UE in case of handover, cell reselection or Uu RLF at Relay UE to inform Remote UE in advance to take appropriate action i.e. initiate relay reselection for example. This message is deemed applicable to both L2 and L3 Relay UE case. Similarly, we can support a notification message over sidelink between U2U Relay UE and U2U Remote UE to enable relay reselection when the U2U Relay UE loses connection towards the target UE or undergoes PC5 RLF.

Proposal 15:	 U2U Remote UE may trigger relay reselection if PC5 RLF with current relay UE is detected by the remote UE. The U2U Relay UE and the Remote UE may inform the upper layer to initiate PC5-S/unicast connection release.

Proposal 16: 	Support notification message over sidelink (e.g. PC5 RRC) from U2U Relay UE to Remote UE(s) for RLF. FFS whether to reuse the legacy message or introduce a new message.

Conclusion

In this contribution, we discussed aspects related to the discovery and re-selection details for L2/L3-based UE-to-UE relaying and have the following observation and proposals:

Proposal 1: 	U2U Remote UE(s) and U2U Relay UE are allowed to trigger discovery transmission when triggered by the upper layer. FFS whether the U2U Relay UE needs to fulfil any AS layer criterion in addition to upper layer trigger.

Proposal 2a: 	In-coverage U2U Remote UE and in-coverage U2U Relay UE in RRC_CONNECTED can rely on network configuration provided via dedicated signalling for discovery configuration.

Proposal 2b: 	In-coverage U2U Remote UE and in-coverage U2U Relay UE in RRC_IDLE or RRC_INACTIVE can rely on network configuration provided via system information for discovery configuration.

Proposal 2c: 	Out-of-coverage U2U Remote UE and out-of-coverage U2U Relay UE (L2 and L3) neither in RRC_CONNECTED nor in RRC_IDLE or RRC_INACTIVE can rely on pre-configuration for discovery configuration.

Proposal 3: 	SL-SRB4 defined in Rel-17 along with its parameters and PDCP PDU format can be reused for U2U discovery message as well (with fixed parameters and SCCH configuration).

Proposal 4: 	No ciphering and integrity protection in PDCP layer is needed for U2U relaying discovery messages.

Proposal 5: 	There is no restriction on the RRC states of any UEs involved in UE-to-UE relaying and the U2U Relay UE need not be in RRC_CONNECTED state for specifically forwarding source Remote UE’s data towards the target UE.

Proposal 6: 	U2U relaying to follow U2N relaying principles to obtain discovery configuration/related SIB for in-coverage U2U relay/remote UE in RRC_CONNECTED/RRC_IDLE/ RRC_INACTIVE states for a given serving frequency and SL frequency which may or may not be shared with the serving frequency.

Proposal 7. 	For UEs in-coverage interested in U2U relaying to rely on U2N principles for acquiring TX resource pool configuration for the purpose of discovery (as outlined above).

Proposal 8: 	Source Remote UE uses PC5 measurements (either SD-RSRP or SL-RSRP) as the primary AS layer criterion to trigger U2U relay (re)selection.

Proposal 9: 	SL-RSRP or SD-RSRP is used as the SL measurement quantity for U2U relay reselection trigger evaluation and it can be up to UE implementation to choose between the two (e.g. when no active data transmission between relay and remote UE, it can choose SD-RSRP).

Proposal 10:  	U2U Relay selection can be triggered at source Remote UE based on a) trigger from upper layer b) PC5 measurement towards target Remote UE being below a (pre)configured threshold.

Proposal 11:	U2U Relay reselection can be triggered at source Remote UE when: a) PC5 measurement towards current serving Relay UE is below a (pre)configured threshold; or b) there is an upper layer release message or similar indication from current serving Relay UE; or c) triggered by upper layer.

Proposal 12: 	PC5 link quality threshold, L3 filter coefficient for SD-RSRP/SL-RSRP and hysteresis can be provided via SIB/RRC by gNB or obtained via pre-configuration.

Proposal 13: 	When SD-RSRP measurement from discovery message is used for relay (re)selection trigger and candidate relay evaluation, L3 filtering is applied across measurements on the DMRS of PSSCH transmission which carries discovery message from the concerned U2U relay.

Proposal 14: 	Relay load is not considered as an additional AS layer criterion for U2U relay (re)selection and Relay UE uses discovery modeling to control its load (i.e. no specification impact is foreseen).

Proposal 15:	 U2U Remote UE may trigger relay reselection if PC5 RLF with current relay UE is detected by the remote UE. The U2U Relay UE and the Remote UE may inform the upper layer to initiate PC5-S/unicast connection release.

Proposal 16: 	Support notification message over sidelink (e.g. PC5 RRC) from U2U Relay UE to Remote UE(s) for RLF. FFS whether to reuse the legacy message or introduce a new message.

References

[1] RP-221262, New WID on NR Sidelink Relay Enhancements

[2] TR 38.836, NR study on Sidelink Relay, Release 17

[3] RAN2#114e chair notes

[4] RAN2#113bise chair notes

Annex

TS 38.331 for proposal 6

2>	else if the cell chosen for NR sidelink discovery transmission provides SIB12:

3>	if the UE is acting as NR sidelink U2N Relay UE and if the NR sidelink U2N Relay UE threshold conditions as specified in 5.8.x2.2 are met based on sl-RelayUE-ConfigCommon in SIB12; or

3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.x3.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

…

….

1>	else if out of coverage on the concerned frequency for NR sidelink discovery:

2>	if the UE is acting as L3 U2N Relay UE and if the NR sidelink U2N Relay UE threshold conditions as specified in 5.8.x2.2 are met based on sl-RelayUE-ConfigCommon in SidelinkPreconfigNR; or

2>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.x3.2 are met based on sl-RemoteUE-ConfigCommon in SidelinkPreconfigNR; or

Rel-17 WI on SL Relay extracted Agreements

The following agreements from the previous discussion on discovery are considered to be applicable for U2U Relay discovery mechanism as well.

Agreements:

Proposal 1:    RAN2 confirm that the following relay-discovery related agreements are also applicable to non-relay discovery.

One new SL-SRB4 is used for all discovery messages. Its parameters will be fixed and defined as SCCH configuration in 38.331. (FFS on the LCH priority in Proposal 8b)

No ciphering and integrity protection in PDCP layer is needed for the discovery messages.

Shared resource pool shall be the baseline for discovery message transmission/reception.

Relay UE and remote UE (IC) in RRC CONNECTED can use the discovery configuration provided via dedicated signalling if available.

Relay UE and remote UE (IC) in RRC IDLE or RRC INACTIVE shall use the discovery configuration provided via SIB if available.

L2 relay UE will always use the discovery configuration provided by gNB (either via SIB or dedicated signalling).

RAN2 confirm the SI conclusion that for L2 remote UE which is out-of-coverage, and is neither in RRC_CONNECTED nor RRC_IDLE/INACTIVE, it can rely on pre-configuration.

RAN2 confirm the SI conclusion that for L3 remote UE which is out-of-coverage, and is neither in RRC_CONNECTED nor RRC_IDLE/INACTIVE, it should follow pre-configuration.

RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE (i.e., either in RRC CONNECTED or RRC IDLE/INACTIVE), it should follow network configuration, i.e., SIB or dedicated signalling, if available.

RAN2 agree that for relay/remote UE in RRC IDLE/INACTIVE state, in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency:

If there is Uu deployed coverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency.

If there is no Uu deployed coverage at the concerned frequency, UE shall rely on pre-configuration.

RAN2 agree that for relay/remote UE in RRC IDLE/INACTIVE state, in-coverage on the serving frequency if the serving frequency is shared with concerned SL frequency:

If there is no discovery related SIB broadcasted on the serving carrier, UE does not perform SL discovery transmission/reception on the concerned frequency.

RAN2 agrees to reuse Rel-16 power control mechanism for transmission of discovery messages.

The same PDCP data PDU format as SL-SRB0 is used for sidelink discovery message (SL-SRB4), and the SDU type field is not used for SL-SRB4.

De-prioritize additional condition for discovery transmission/reception in Rel-17.

RAN2 agrees that for relay/remote UE in RRC IDLE/INACTIVE state, and in-coverage on the serving frequency, if there is discovery related SIB broadcasted on the serving frequency, and if the configuration of concerned SL frequency is included within the SIB of the serving frequency but the Tx resource pool configuration is absent, UE shall enter RRC CONNECTED state to acquire dedicated configuration on Tx resource pool.

RAN2 agree that RRC_CONNECTED relay/remote UE which are in-coverage on the serving frequency, if there is discovery related SIB broadcasted on the serving frequency, and if the configuration of concerned SL frequency is included within the SIB of the serving frequency, it can only use the SL discovery Tx resource configuration provided by dedicated signalling if provided, or not transmit discovery if not provided.

RAN2 agree that RRC_CONNECTED L3 relay/remote UE or layer 2 remote UE which are in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency:

If there is Uu coverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency.

RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE and in RRC IDLE/INACTIVE state, if the network configuration is not available, i.e., SIB, remote UE shall rely on pre-configuration to perform discovery.

RAN2 agrees to down-prioritize discovery specific resource allocation optimization in this release.

RAN2 agrees to down-prioritize the support of discovery gaps in this release.

RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE and in RRC CONNECTED state, if the network configuration is not available, i.e., SIB or dedicated signalling, remote UE shall rely on pre-configuration to perform discovery.

RAN2 agrees dedicated discovery resource pool is supported besides shared resource pool configuration, whether it is configured is based on network implementation. And PHY layer parameters and design shall reuse the Rel-16 legacy resource pool design (including resource allocation design).

RAN2 agrees to fix the priority value as 1 of sidelink discovery message in the specification.

Proposal 1a (modified): RAN2 to confirm SL CG type-1 (if configured) can be used for Non-relay discovery transmission.  This does not affect existing agreements restricting CG type 1 on the relay link.

Proposal 1b (modified): RAN2 to confirm SL CG type-1 (if configured) can be used for Relay discovery transmission in the following cases:

-    By remote UE before connecting with relay UE (including L2 remote/relay UE)

-    By relay UE before or after connecting with remote UE

-    Confirm that SL CG type-1 can always be used for L3 remote/relay UE if the gNB configures it

A NOTE can be added to the MAC spec to indicate P1a/P1b, subject to checking in the MAC CR discussion.

[Unanimous] Proposal 7: RAN2 confirms that CBR measurement is supported for NR SL discovery transmission.

The UE is expected to monitor the dedicated discovery pool if configured (as currently specified in RRC); we do not specify that the UE monitors only the dedicated discovery pool after it is configured.  Details to be resolved in the RRC CR discussion, i.e. whether something needs to be captured.

Proposal 8a: When UE is configured to transmit discovery, CBR measurements can be performed in dedicated discovery pool or shared pool, based on which pool the UE selects to transmit discovery.

Proposal 8b: When UE is configured to transmit discovery, in addition to dedicated discovery pool or shared pool, CBR measurements can also be performed in other pools (e.g. tx-PoolMeasToAddModList, sl-TxPoolExceptional) as legacy behaviour in sidelink communication.

Agreement:

Proposal: When receiving the discovery message or PC5-S signaling, UE can pass them to the upper layer along with an indication for differentiation, where a NOTE will be captured in PDCP spec and discussed in stage-3 CR drafting.

Agreements:

Proposal 2.1: [17/19] RAN2 assumes that discovery and data transmitted by a UE cannot be multiplexed into the same TB because they are always associated to different destination L2 IDs.  RAN2 sends this assumption in an LS to SA2.

Proposal 2.2: [18/19] For SL LCP procedure, only L2 destination IDs associated to discovery can be selected for grants from the dedicated discovery resource pool.

Proposal 2.3 (modified): [19/20] For SL LCP procedure, when the dedicated discovery pool is configured/used, only L2 destination IDs associated to communication can be selected for grants from the shared resource pool.  When the dedicated resource pool is not configured/used, this restriction is not applied.

Proposal 3.1: [19/19] The UE reports buffer status associated with discovery using the destination index associated to a discovery L2 ID (i.e. no impact to SL BSR MAC CE, or specific LCG ID is needed).

Proposal 3.2: [19/20] SUI includes an indication of whether a particular destination L2 ID is associated to discovery.

The UE can determine from SIB12 whether the gNB supports relay discovery and/or non-relay discovery.  Details (including whether SIB12 signalling can differentiate between support of relay vs. non-relay discovery and whether the support is indicated explicitly or implicitly) can be discussed as part of stage 3 CR drafting.

Proposal 4.3: [18/19] Whether gNB supports L2 relay is explicitly indicated in SIB12.

Proposal 4.5: [18/19] No additional indication in SIB12 is required to signal that operation as a L3 relay is not allowed.

Proposal 5.1: [20/20] HARQ feedback is not supported for SL discovery transmission.

Agreements:

Proposal 1.1: [12/18] The use of both dedicated and shared resource pools for discovery transmission, when both pools have been configured, is not supported in this release.

Whether L3 relaying support is signalled implicitly by indicating the support of discovery, or signalled independently from support of discovery, can be discussed in stage 3 drafting.

TR 38.836

5.2	Discovery

Model A and model B discovery model as defined in clause 5.3.1.2 of TS 23.303 [3] are taken as a working assumption for both UE-to-Network Relay and UE-to-UE Relay. The protocol stack of discovery message is similar or identical to PC5-S signalling as illustrated in Figure 16.9.2.1-2 of 38.300 [4].

Relay UE or Remote UE is allowed to transmit discovery message when triggered by upper layer.

Both Remote UE and Relay UE can rely on pre-configuration unless relevant radio configuration is provided by network, either via system information or dedicated signalling.

Resource pool to transmit discovery message can be either shared with or separated from resource pool for data transmission.

-	In case of shared resource pool a new LCID is introduced for discovery message i.e. discovery message is carried by a new SL SRB.

-	Within separated resource pool discovery messages are treated equally with each other during LCP procedure.

5.3	Relay (re-)selection criteria and procedure

The baseline solution for relay (re-)selection is as follow:

Radio measurements at PC5 interface are considered as part of relay (re)selection criteria.

-	Remote UE at least use the radio signal strength measurements of sidelink discovery messages to evaluate whether PC5 link quality of a Relay UE satisfies relay selection and reselection criterion.

-	When Remote UE is connected to a Relay UE, it may use SL-RSRP measurements on the sidelink unicast link to evaluate whether PC5 link quality with the Relay UE satisfies relay reselection criterion.

Further details on the PC5 radio measurements criteria, e.g., in case of no transmission on the sidelink unicast link can be discussed in WI phase.

For relay (re-)selection, Remote UE compares the PC5 radio measurements of a Relay UE with the threshold which is configured by gNB or preconfigured.  Higher layer criteria also need to be considered by Remote UE for relay (re-)selection, but details can be left to SA2 to decide. Relay (re-)selection can be triggered by upper layers of Remote UE.

Relay reselection should be triggered if the NR Sidelink signal strength of current Sidelink relay is below a (pre)configured threshold. Also, relay reselection may be triggered if RLF of PC5 link with current Relay UE is detected by Remote UE.

The above-described baseline for relay (re)selection apply to both L2 and L3 relay solutions. Additional AS layer criteria can be considered in WI phase for both L2 and L3 UE-to-UE relay solutions.

For relay (re-)selection, when Remote UE has multiple suitable Relay UE candidates which meet all AS-layer & higher layer criteria and Remote UE need to select one Relay UE by itself, it is up to UE implementation to choose one Relay UE.