3GPP TSG-RAN WG2 Meeting #120                                    R2-2211185

Toulouse, France, November 14-18, 2022

Agenda item:		6.0.4

Source:				Samsung

Title:					Further discussion on cell reselection priority handling on

coexistence of multiple features

Document for:		Discussion & Decision

1	Introduction

In the previous meeting, RAN2 made the common understanding on the intended UE behavior of reselection priorities handling but deferred the discussion whether there is any specification impact or not, as per below.

R2-2211025 Report of [AT119bis-e][005][NR17] Cell Reselection Frequency Prioritization (Kyocera) Kyocera noted - P1, P3, P4 are agreed: P1: For the cell reselection frequency prioritization, the MBS frequency may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation). P3’: For the cell reselection frequency prioritization, the frequency providing V2X/NR sidelink may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation). P4: For the cell reselection frequency prioritization, the HSDN cell shall be always considered as the highest priority, i.e., higher than the slice specific cell reselection priority even if configured and higher than MBS/V2X sidelink/NR sidelink frequency priority even if considered as the highest priority. It’s only applicable when the HSDN-capable UE is in High-mobility state, as it is today. Chair: Postpone finalization (checking of TS impact) to next meeting. If issues are found with the agreements, we can come back.

This contribution first clarifies how current specification has been written for reselection priority handling and then discusses any necessity on specification update.

2	Discussion

2.1	How TS 38.304 has been written for reselection priority handling

A general principle of reselection priorities handling is that the UE applies the priorities based on network configuration. If dedicated reselection priorities are provided in the RRCRelease message, they are applied;  otherwise common reselection priorities in the system information are applied.

Observation 1: A general principle of reselection priorities handling is that the UE applies the priorities based on network configuration (e.g. SIB or RRCRelease).

When V2X/NR sidelink feature was introduced in Rel-16, the following exception rule on reselection priorities handling was introduced in [1]:

If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority.

We understand that above rule does NOT mean that the V2X/NR sidelink capable UE applies it only. Instead, its intent is to clarify that the V2X/NR sidelink capable UE may consider a certain frequency to be the highest frequency priority according to UE preference by overriding reselection priorities provided in the RRCRelease message or in the system information. In other words, by default, the V2X/NR sidelink capable UE follows the general principle in mentioned in Observation 1.

Observation 2: Current TS 38.304 has been specified with the following understanding:

- By default, the V2X/NR sidelink capable UE applies reselection priorities according to network configuration (SIB or RRCRelease). But it may consider a certain frequency to be the highest frequency priority according to UE preference by overriding network-provided reselection priorities.

For MBS feature in Rel-17, the following exception rule on reselection priorities handling has been specified in [1]:

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.346 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service. NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in.

Similar to V2X/NR sidelink, the above statement was added by assuming that the MBS broadcast capable UE applies reselection priorities provided in the RRCRelease message or in the system information by default.

Observation 3: Current TS 38.304 has been specified with the following understanding:

- By default, the MBS broadcast capable UE applies reselection priorities according to network configuration (SIB or RRCRelease). But it may consider a certain frequency to be the highest frequency priority according to UE preference by overriding network provided reselection priorities.

For NR HSDN in Rel-17, the following exception rule on reselection priorities handling has been specified [1]:

When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities).

Again, the HSDN capable UE needs to apply reselection priorities according to network configuration, but it always prioritize HSDN cells to be the highest priority when it is in High-mobility. It means that such exception rule is applied regardless of network configuration and other user preferences (e.g. V2X/NR sidelink or MBS) due to the term 'shall always'.

Observation 4: Current TS 38.304 has been specified with the following understanding:

- By default, the HSDN capable UE applies reselection priorities according to network configuration (SIB or RRCRelease). But it shall always consider HSDN cells to be the highest frequency priority when it is in High-Mobility state regardless of network-provided reselection priorities and other user preferences (e.g. V2X/NR sidelink or MBS).

Lastly, for Slicing feature in Rel-17, clause 5.2.4.11 in [1] is introduced to specify how the dervies reselection priorities for slice-based cell reselection. We understand that it is more or less the same as in general principle (e.g. UE behabvior in Rel-15) except which kind of network configuration needs to be applied.

Observation 5: Current TS 38.304 has been specified with the following understanding:

- When UE is in camped normally state, if it supports slice-based cell reselection and has received NSAG(s) and their priorities, the UE shall derive slice-based reselection priorities according to network configuration (SIB or RRCRelease). It is more or less the same as in Observation 1 except which kind of network configuration is applied.

2.2	Necessity on specification update

In [1], the following statement is specified:

When UE is in camped normally state, if it supports slice-based cell reselection and has received NSAG(s) and their priorities from NAS, UE shall derive re-selection priorities according to clause 5.2.4.11.

Because of the term "shall derive", one may consider that the UE is NOT allowed to prioritize concerned MBS frequency or V2X/NR sidelink frequency to the highest priority on coexistence of MBS and Slicing or V2X/NR sidelink and Slicing. However, as clarified in Observations 2 and 3, the intent of reselection frequency prioritization for MBS or V2X/NR sidelink is to allow the UE to override network-provided reselection priorities according to user preference and it does not matter which kind of network configuration i.e. legacy configuration in Rel-15 or slice-based configuration in Rel-17. Thus, there is no specification impact to clarify P1 and P3 agreed in the last meeting.

Proposal 1: RAN2 to confirm that there is no specification impact to implement the following UE behavior:

- For the cell reselection frequency prioritization, the MBS frequency may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation)

- For the cell reselection frequency prioritization, the MBS frequency may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation).

As clarified in Observation 4, current specification is written as "the HSDN capable UE shall always consider HSDN cells to be the highest frequency priority when it is in High-Mobility state regardless of network-provided reselection priorities and other user preferences. Thus, there is no specification impact to clarify P4 agreed in the last meeting.

Proposal 2: RAN2 to confirm that there is no specification impact to implement the following UE behavior:

- For the cell reselection frequency prioritization, the HSDN cell shall be always considered as the highest priority, i.e., higher than the slice specific cell reselection priority even if configured and higher than MBS/V2X sidelink/NR sidelink frequency priority even if considered as the highest priority. It’s only applicable when the HSDN-capable UE is in High-mobility state, as it is today.

3	Conclusion

In section 2, the following observations are made:

Observation 1: A general principle of reselection priorities handling is that the UE applies the priorities based on network configuration (e.g. SIB or RRCRelease).

Observation 2: Current TS 38.304 has been specified with the following understanding:

- By default, the V2X/NR sidelink capable UE applies reselection priorities according to network configuration (SIB or RRCRelease). But it may consider a certain frequency to be the highest frequency priority according to UE preference by overriding network-provided reselection priorities.

Observation 3: Current TS 38.304 has been specified with the following understanding:

- By default, the MBS broadcast capable UE applies reselection priorities according to network configuration (SIB or RRCRelease). But it may consider a certain frequency to be the highest frequency priority according to UE preference by overriding network provided reselection priorities.

Observation 4: Current TS 38.304 has been specified with the following understanding:

- By default, the HSDN capable UE applies reselection priorities according to network configuration (SIB or RRCRelease). But it shall always consider HSDN cells to be the highest frequency priority when it is in High-Mobility state regardless of network-provided reselection priorities and other user preferences (e.g. V2X/NR sidelink or MBS).

Observation 5: Current TS 38.304 has been specified with the following understanding:

- When UE is in camped normally state, if it supports slice-based cell reselection and has received NSAG(s) and their priorities, the UE shall derive slice-based reselection priorities according to network configuration (SIB or RRCRelease). It is more or less the same as in Observation 1 except which kind of network configuration is applied.

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: RAN2 to confirm that there is no specification impact to implement the following UE behavior:

- For the cell reselection frequency prioritization, the MBS frequency may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation)

- For the cell reselection frequency prioritization, the MBS frequency may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation).

Proposal 2: RAN2 to confirm that there is no specification impact to implement the following UE behavior:

- For the cell reselection frequency prioritization, the HSDN cell shall be always considered as the highest priority, i.e., higher than the slice specific cell reselection priority even if configured and higher than MBS/V2X sidelink/NR sidelink frequency priority even if considered as the highest priority. It’s only applicable when the HSDN-capable UE is in High-mobility state, as it is today.

4	Reference

[1] 3GPP TS 38.304 v17.2.0	User Equipment (UE) procedures in Idle mode and RRC Inactive state