3GPP TSG RAN WG2 Meeting #120          	      	                	                  R2-2211375

Toulouse, FR, 14th – 18th Nov, 2022

Agenda Item:	8.12.3

Source:	Intel Corporation

Title:	PCI collision and TAC/RANAC update of mobile IAB-node

Document for:   Discussion and Decision

# Introduction

During RAN2 #119bis-e meeting, for full migration two logical DUs, RAN2 agreed with following agreement:

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

Moreover, RAN3 also agrees following agreement for PCI collision:

PCI Space Partitioning is performed by OAM and up to implementation. As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message. PCI-change on the IAB-node can be supported via handover of connected UEs between cells using old and new PCI, respectively. FFS for the PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs. PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node.

Furthermore, TAC/RANAC update is also not concluded in last RAN2 meeting.

In this contribution, we first discuss the need of supporting PCI collision and dynamic change, then discuss the RAN2 impact of supporting RAC/RANAC update of mobile IAB-node.

# Discussion

## PCI of mobile IAB-node

In RAN2 #119bis-e meeting, it was agreed that the two logical DUs of mobile IAB-node will use different PCIs if same carrier is used.

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

RAN3 agreed that PCI space partitioning can be supported via OAM by implementation. This PCI space partitioning approach can also support if two different PCIs are used for different physical cells at two logical DUs. This allows use of legacy HO procedure between the two cells.  OAM can simply reserve two PCIs for one mobile IAB-node. During full migration, the mobile IAB-node can use the 2nd PCI reserved by OAM as the PCI of the physical cell at the 2nd logical DU. The PCI of 1st logical DU can then be released and reused for the new established logical DU during mobile IAB-node’s next full migration. The two reserved PCIs can be switched between existing logical DU and new-established logical DU.

Observation 1: Two PCIs are required for the two cells of the two logical DUs during the full migration to allow HO between the two cells.  No other mechanism is necessary.

Proposal 1: Two PCIs are reserved to mobile IAB-node by OAM during PCI space partitioning and switch between two logical DUs during full migration.

Another aspect to discuss whether PCI collision detection and avoidance is needed. Even with split allocation of PCIs to fixed and mobile IAB nodes, it is possible that PCI collision may happen between two mIAB nodes due to the nature of their unpredictable mobility pattern. The conventional method of detecting PCI collision is when the network notices it during measurement reporting. It can be verified by using ANR reporting from UEs. While ANR can report the different NCGIs of the PCIs to detect PCI collision, it may not always do it reliably as it is a slow procedure and the period of collision due to mIAB mobility could be too short to detect it.  However, even if it is detected quickly, changing the PCI of a cell (as discussed below) to avoid the collision is likely to take even longer as all the UEs will need to be moved to the new PCI.  Hence the current ANR can be considered sufficient for PCI collision detection. There’s no need to introduce new UE reporting to improve PCI collision detection.

Proposal 2: Current ANR procedure is sufficient for PCI collision detection for mIAB. No need to introduce new UE reporting to improve PCI collision detection.

## TAC/RANAC for the mIAB node

During RAN3 #117bis-e meeting, TAC/RANAC issue had following agreement:

RAN3 to further discuss the following options for TAC/RANAC issue: - Option 1: The TAC/RANAC for the mobile IAB cell can be changed in order to reflect the physical location when the mobile IAB-node moves. - Option 2: Using static TAC/RANAC for mobile IAB when it moves. Involvement of SA2 may be needed

From UE point of view, both options can be well-supported by existing mechanisms as UE simply provides an update message when it sees that the TAC has changed.

Observation 2: RAN2 specifications support both options for TAC/RANAC for mIAB

For Option 2, the network can use this unique TAC/RNA and send paging messages to the corresponding UEs and UE will not need to preform tracking area update while it is camped on the mIAB node. However, the consequences of mapping this unique TAC to different geographic area due to mobile IAB-node’s mobility should be studied in SA2.

Since RNA is a subdivision of the TAC, its handling is dependent on the TAC design.  The design of RNA mapping to different geographic area can follow the same principle as TAC discussion in SA2.

Observation 3: How/whether to use a unique/unchanged TAC for a mIAB cell while moving in different geographic areas should be studied in SA2.

For Option 1, when the mobile IAB-node broadcasts the same TAC/RNA as its parent IAB-node or IAB-donor DU, for the UEs.  There are (at least) two areas to address.  One is about having to update the TAC/RNA in the SIB to match the TAC/RNA of the neighbouring cells while the mIAB node is moving around.  From Uu point of view, the frequent update of the TAC/RNA in SIB1 should not cause big problems.  RAN3/SA2 may need to evaluate how the neighbouring TAC/RNA information is updated in the mIAB node.

The other concern is that the group of UEs inside of mobile IAB-node may need to send tracking area update to the network at the same time, which may cause congestion. However, the same problem also occurs when considering massive number of UEs served by high-speed train performing the tracking area update simultaneously when a train arrives at a station and passengers come out of the train. For such scenario, the existing congestion control mechanism can support it well without further optimization, regardless the frequency of changing tracking area or simultaneous messages of tracking area update.

Observation 4: If mobile IAB-node broadcasts the same TAC/RNA as its parent IAB-node, the existing congestion control mechanism can well-support for frequent and large number of tracking area updates from group of served UEs.

Therefore, from RAN2 point of view, both changed/unchanged TAC/RNA can be supported without further optimization. How to design TAC of the mobile IAB-node is a SA2/RAN3 decision.

Proposal 3: RAN2 to conclude that both changed and unchanged TAC/RNA of mobile IAB-node can be supported by existing mechanism in RAN2 specifications. How to design TAC of the mobile IAB-node is left to SA2/RAN3.

# Conclusion

In this contribution, we mainly discussed RAN2 impacts of PCI allocation and collision. Additionally, we also discussed TAC/RANAC update RAN2 impact.

We propose following observations and proposals:

PCI

Observation 1: Two PCIs are required for the two cells of the two logical DUs during the full migration to allow HO between the two cells.  No other mechanism is necessary.

Proposal 1: Two PCIs are reserved to mobile IAB-node by OAM during PCI space partitioning and switch between two logical DUs during full migration.

Proposal 2: Current ANR procedure is sufficient for PCI collision detection for mIAB. No need to introduce new UE reporting to improve PCI collision detection.

TAC/RANAC

Observation 2: RAN2 specifications support both options for TAC/RANAC for mIAB

Observation 3: How/whether to use a unique/unchanged TAC for a mIAB cell while moving in different geographic areas should be studied in SA2.

Observation 4: If mobile IAB-node broadcasts the same TAC/RNA as its parent IAB-node, the existing congestion control mechanism can well-support for frequent and large number of tracking area updates from group of served UEs.

Proposal 3: RAN2 to conclude that both changed and unchanged TAC/RNA of mobile IAB-node can be supported by existing mechanism in RAN2 specifications. How to design TAC of the mobile IAB-node is left to SA2/RAN3.

# References