3GPP TSG-RAN WG2 Meeting #120	R2-2211548

# Toulouse, France, 14th - 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3613 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to FR2 UL Gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the field descriptions of ugl and ugrp the references to Table 9.1.7 in TS 38.133 are wrong. The values of ugl and ugrp are specified in Table 9.1.11-1. - There are editorial issues in the field descriptions of ugl and ugrp which need to be fixed. - The presence of field refFR2-ServCellAsyncCA-r17 is either conditionally mandatory or absent. But the latter is missing in the description of the condition AsyncCA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the field descriptions of ugl and ugrp the incorrect references to Table 9.1.7 in TS 38.133 have been fixed to Table 9.1.11-1. - The editorial issues in the field descriptions of ugl and ugrp have been fixed. - In the description of the condition AsyncCA the case when the field refFR2-ServCellAsyncCA-r17 is absent has been added. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: FR2 UL Gap Inter-operability: There are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in the specification of FR2 UL Gap remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.3.2	Radio resource control information elements

<Text omitted>

#### –	UL-GapFR2-Config

The IE UL-GapFR2-Config specifies the FR2 uplink gap configuration.

UL-GapFR2-Config information element

-- ASN1START

-- TAG-UL-GAPFR2-CONFIG-START

UL-GapFR2-Config-r17 ::=      SEQUENCE {

gapOffset-r17                 INTEGER (0..159),

ugl-r17                       ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms1},

ugrp-r17                      ENUMERATED {ms5, ms20, ms40, ms160},

refFR2-ServCellAsyncCA-r17    ServCellIndex                                                      OPTIONAL -- Cond AsyncCA

}

-- TAG-UL-GAPFR2-CONFIG-STOP

-- ASN1STOP

UL-GapFR2-Config field descriptions

gapOffset Value gapOffset is the gap offset of the FR2 UL gap pattern with UGRP indicated in the field ugrp. The value range is from 0 to ugrp-1.

refFR2-ServCellAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 UL gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

ugl Value ugl is the gap length in ms of the FR2 UL gap. The FR2 UL gap length is according to Table 9.1.11-1 in TS 38.133 [14]. Value ms0dot125 corresponds to 0.125 ms, ms0dot25 corresponds to 0.25 ms and so on.

ugrp Value ugrp is the gap repetition period in (ms) of the FR2 UL gap. The FR2 UL gap repetition period is according to Table 9.1.11-1 in TS 38.133 [14].

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring FR2 UL gap pattern to UE in: - (NG)EN-DC, NR SA, NE-DC or NR-DC without FR2-FR2 band combination, with asynchronous CA involving FR2 carriers. Otherwise, it is absent. |

End of changes