3GPP TSG RAN WG2 Meeting #120               	R2-2211918

# Toulouse, France, 14th 18th November 2022

Agenda Item: 	8.2.3

Source:		Sony

Title:			Considerations some aspects for integrity of RAT dependent positioning techniques

Document for:	Discussion & Decision

# Introduction

A new release-18 study item on expanded and improved NR positioning was approved in RANP#94e meeting in December 2021 [1].

The study item has several objectives, in which one of them is related to Integrity

Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

At RAN2#119e meeting [3], the following agreements were made;

Proposal 1: RAN2 to confirm the integrity principle of operation defined in the section 8.1.1a of TS38.305, including integrity definition (e.g., Error, Bound, Time to Alert, DNU, Residual Risk, irMinimum, irMaximum and Correlation Times; FFS if all parameters are needed in the RAT-dependent case), Equations for the GNSS integrity are reused for RAT dependent positioning methods. Proposal 2 (modified): RAN2 may add the mapping between Integrity definition/Fields (Integrity Alerts, error bounds (mean, StdDev), Residual Risks, Integrity correlation times ) and Error sources/assistance data for RAT-dependent positioning methods later once RAN1 identifies new error sources.

At RAN2#119bis e meeting [4], the following agreements were made;

Proposal 1-2. RAN2 study the usage of DNU flag for the RAT-dependent positioning integrity (assuming RAN1 agree to leave it to RAN2) and conclude on whether to indicate the DNU presence in the integrity principle equation.

Proposal 4. RAN2 will study the both UE-based and LMF-based integrity for RAT-dependent cases.

Proposal 7 (modified). RAN2 agree that R17 UE-based integrity mode signaling can be used as baseline with the following aspects:

UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure

LMF sends the assistance data for integrity calculation to UE for integrity of UE-based mode

LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode

UE sends integrity result to LMF using LPP location information Transfer message

LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

In this contribution, we discuss potential solutions for RAT dependent integrity.

# Discussion

## Background

The application layer of a UE may contain applications that are highly depending on certain accuracy being provided from a location system, but also other applications or users that may require an accurate positioning estimate of the UE. Furthermore, the location system may also need a view on to what level the positioning measurement are reliable.

In release 17 features to support GNSS integrity was included based on the GNSS integrity concepts. From SID [1] it was agreed to study integrity solutions for also a RAT dependant positioning. There concepts might be preferred to reuse as much as possible from also for RAT dependant integrity concept.

The basic definition of Positioning Integrity is that it is a measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts.

There is a need to have integrity to ensure the provided positioning estimate is fulfilling various levels of positioning requirements (accuracy, latency) for the intended service level.

In Rel-17 a number of error sources were identified and listed related to GNSS error sources, e.g. based on orbital, clock, code bias, phase bias Ionosperic or Tropospheric errors, that may affect the positioning measurements.

In this paper we make some reflections related to error sources, validity time for reporting and the possibility for adaptation of RAT-dependent positioning based on input from non-RAT dependent methods.

## Discussion on RAT dependent integrity

It has been a agreed in RAN2 to wait RAN1 evaluation related to error sources. One aspect worth mentioning meanwhile could be related to time aspect.

We see the time duration from request the positioning information to receiving integrity of the qualified positioning results as important. RAN1 has agreed that the reported measurement (e.g., timing based report (RSTD, RTOA)) is the source or error. The error should be properly modelled in certain distribution (e.g., Gaussian distribution). The time duration related to creation of a distribution of measurement error source can be defined. The application side which triggers the positioning request may know the positioning requirements and therefore it should be possible to request the positioning information to be reported from within a time window with a maximum time duration. The integrity metrics should be communicated to LCS as soon as ready within the maximum time duration (i.e., as limit).

Proposal 1: Consider a maximum predefined time window for modelling measurement error source.

Additionally, it could make sense to perform multiple measurement using the same or different positioning methods, then with a priority order of the required positioning methods to be performed to make sure the best positioning measurement is reported within the valid time duration. Furthermore, if one positioning method delivered a result fulfilling the expected level of trust there could be an option to abort the other methods and only report the selected result.

Proposal 2: Consider multiple positioning measurements in which the important/prioritized measurement data should be reported.

Additionally, there is also an opportunity of using integrity for adapting the RAT based positioning. We argue it is not useful to perform positioning estimations that are anyway not used by the application due to the trustworthiness of the estimations or any error source in the RAT based positioning system. Similarly, there might be less use of performing any RAN based localization if a better estimation of the position is provided by RAT-independent based method. Thereby we consider letting the UE have ability to adapt its position measurements and estimations based on other integrity metrics in a device. In other words, integrity KPIs/results from e.g GNSS could be used to set operating conditions for RAT based positioning. Thereby we propose to have the option to signal RAT-independent based integrity KPIs/results to RAN nodes to assist in positioning method selection/enable/disable or even act as input to resource scheduling for positioning reference signals.

Proposal 3: Consider RAT-independent integrity KPIs/results be available for adapting the RAT-dependent positioning methods and integrity reporting.

# Summary

In this contribution, we have discussed our view on RAT dependent integrity. Our observation and proposals are listed below:

Proposal 1: Consider a maximum predefined time window for modelling measurement error source.

Proposal 2: Consider multiple positioning measurements in which the important/prioritized measurement data should be reported.

Proposal 3: Consider RAT-independent integrity KPIs/results be available for adapting the RAT-dependent positioning methods and integrity reporting.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

- LPP, 37.355, “LTE Positioning Protocol (LPP), Rel-17

- Minutes RAN2#119e meeting

- Minutes RAN2#119bis e meeting