3GPP TSG-RAN WG2 #120	R2-2211958

Toulouse, France, November 2022

Agenda Item:	8.5.2.2

Source:	OPPO

Title:	Discussion on PDU prioritization

Document for:	Discussion, Decision

Introduction

This paper will provide our considerations on PDU prioritization, for which the previous RAN2 meetings reached the following agreements.

- RAN2 assumes that PDU Set based parameters and PDU Set related information may be used for better support of XR services. RAN2 can consider both UL and DL directions.

- RAN2 will study PDU Set based parameters and PDU Set related information handling in Network and UE

- RAN2 to adopt the current SA2 definition of PDU Set as an application media unit as working assumption, subjected to further guidance from SA2 and SA4.

- XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts.

- RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are).

- RAN2 can consider how PDU sets can be mapped to DRBs (FFS if SA2 discussion on PDU set mapping to QoS (sub-)flows impacts this)

Discussion

For the XR service, SA2 studies KI#4&5 which are related to the PDU Set support and concludes to deliver the following PDU Set QoS information to the AS layer for the PDU Set handling.

- 8.x.1.1	PDU Set QoS Parameters

PDU Set QoS treatment is determined using dynamic or non-dynamic PCC.

The following PDU Set QoS parameters are defined to support PDU Set handling:

-  PDU Set Error Rate: The PSER defines an upper bound for the ratio between the number of PDU Sets not successfully received and the total number of PDU Sets sent towards a recipient measured over a measurement window.

Editor’s Note: the criteria for determining whether a PDU Set is successfully delivered or not are FFS

-  PDU Set Delay Budget

Editor’s Note: The definitions of PSER and PSDB are FFS. For PSDB, it needs further study the impact due to N6 jitter.

-  Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication).

Editor’s Note: It is FFS “Whether a PDU Set is still valid in case PSDB is exceeded” is needed. It should be discussed together with the definition of PSDB, specially about the boundary of PSDB.

If PDU Set based QoS handling is used, PCF determines the above PDU Set QoS Parameters based on information provided by AF (described in 8.X.2) and/or local configuration. The PDU Set QoS parameters are sent to SMF as part of PCC rule, then SMF sends them to RAN

Although the definition of PSDB is unstable, it is clear that SA2 would like to introduce an upper bound for the time that a PDU Set may be delayed/transmitted. In other words, the maximum latency requirement would exist for each PDU Set. The logic we understood herein is that the decoder expects the PDUs of one PDU set to be received before the PSDB, otherwise, the correlated PDUs may be outdated. From this aspect, even if the packets are still handled per PDU, it is valuable if the transmitter considers the remaining time information in transmitting. For example, the transmitter prioritizes the packets of a PDU Set if the remaining latency of this PDU Set is less than a threshold. Or, the transmitter deprioritizes the packets of a PDU Set if the latency of this PDU Set already exceeds the PSDB. Thus, it would be good if the AS layer considers such latency requirements of the PDU Set and check whether the current AS mechanism suits.

In legacy, the prioritization mechanisms in the MAC layer include the LCP and grant prioritization.

The LCP procedure designed in MAC is to guarantee/prioritize data of high priority. When generating a new MAC PDU, the Multiplexing and assembly entity shall invoke the LCP procedure. The LCP procedure shall multiplex the available data into the allocated UL grant based on the LCH priority as well as fairness. The UL grant allows the data of LCHs multiplexed in strict decreasing priority order of the LCH priority. In addition, the LCHs to be multiplexed should satisfy the LCH mapping restrictions configured by RRC. In short, the current LCP procedure considers more on the logical channel priority and does not take the latency information into account. To simplify the spec impact and support the latency requirement of a PDU Set, we think the current LCP rule can be as a baseline (i.e. the LCP handling is based on LCH priority and the multiplexing is operated per RLC SDU/PDU), and the MAC layer should also consider the remaining latency of a PDU Set when performing the LCP procedure.

In the LCP procedure, RAN2 assumes the operation is per PDU but also considers taking the remaining latency of a PDU Set into account.

In the current 5G system, it could happen that multiple uplink grants/channels overlap and the overlapped grants/channels have individual transmission requirements. Rel-16 URLLC introduces the prioritization procedure to resolve this overlapping issue and lets the MAC layer prioritize the grant/channel of a high priority. The priority of an uplink grant is determined by the priority of the LCH which is multiplexed or to be multiplexed in the associated MAC PDU. Similar to the LCP procedure we analyzed above, besides the legacy rule, it would be good if the prioritization procedure can additionally take the PDU Set requirement into account when performing the grant selection. The PDU Set requirement includes the remaining latency of a PDU Set. Therefore, we propose RAN2 to assume the current prioritization rule is a baseline(i.e. prioritization decision considers the LCH priority), and the MAC layer should also consider the remaining latency of a PDU Set when performing the prioritization procedure.

RAN2 assumes that the MAC layer should also consider the remaining latency of a PDU Set when performing the prioritization procedure.

Usually, whether to enable a new functionality is controlled by the gNB. Thus, we prefer the gNB to indicate the UE whether to enable/disable the enhancement mentioned above.

Whether to enhance the current LCP/prioritization procedure can be controlled/indicated by the gNB.

Conclusion

We have the following proposals:

Proposal 1	In the LCP procedure, RAN2 assumes the operation is per PDU but also considers taking the remaining latency of a PDU Set into account.

Proposal 2	RAN2 assumes that the MAC layer should also consider the remaining latency of a PDU Set when performing the prioritization procedure.

Proposal 3	Whether to enhance the current LCP/prioritization procedure can be controlled/indicated by the gNB.

Reference

[1] RP-220285  New SID “Study on XR Enhancements for NR”

[2] 3GPP TR 23.700-60 Study on XR (Extended Reality) and media services