3GPP TSG-RAN WG2 #120	R2-2211960

Toulouse, France, November 2022

Agenda Item:	8.5.4.1

Source:	OPPO

Title:	Discussion on feedback enhancement

Document for:	Discussion, Decision

Introduction

This paper will provide our considerations on feedback enhancement, for which the previous RAN2 meetings reached the following agreements.

- XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts.

- RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are).

- 1: As starting point, RAN2 can further discuss the solutions in TR 38.838 that can impact on L2 operation (e.g., BSR, LCP, assistance information for scheduling, packet discarding, prioritization) for XR-specific capacity improvement. RAN2-specific solutions are not precluded (even if RAN1 hasn’t discussed them before).

- 1: Enhancement to SPS/CG should be justified for XR scheduling and should be evaluated against dynamic grant (DG) scheduling which should be considered as baseline. Should justify why enhancements are needed.

- RAN2 considers SPS enhancements may not be needed in Rel-18 XR since PDCCH capacity is not assumed to be a problem for XR. FFS if SPS has some power consumption benefits.

- 1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase).

- Delay information consists of at least “remaining time”.

- 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

- If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

Discussion

In RAN2#119bis-e meeting, RAN2 considers at least the remaining time useful for XR and one open issue is the necessity of dynamic reporting.

- 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

According to the SA2’s discussion, although the definition of PSDB is not decided, SA2 would like to use PSDB to represent an upper bound for the time that a PDU Set may be delayed/transmitted. The part debated is the PSDB is evaluated starting from the first or the last packet of a PDU Set. Please note that PSDB is a statistics value which can not reflect the latency/delay of a specific/individual PDU Set. In reality, for each PDU Set in one QoS flow, even if a PDU set has the same PSDB value as others, the actual waiting time of this specific PDU Set in the L2 buffer is probably different from other PDU Sets since the radio link quality is time-varying and the uplink grant may not always well-match the arrival time/packet size of each PDU Set. In addition, the packet arrival time of a PDU set has a jitter range, thus the gNB can not know the actual arrival time of each PDU Set accordingly. As a result, we understand that the PSDB can not replace the actual remaining time, i.e. dynamic reporting of the remaining time of a PDU Set is necessary.

Since the grant allocation may take data burst information into account and one data burst can associate with multiple PDU Sets which may arrive in a very short time, we thus understand that the granularity of dynamic reporting of the remaining time can be per data burst. To further reduce the reporting overhead, the granularity of such dynamic reporting can be softened per LCH.

- PSDB is a statistics value which cannot reflect the actual remaining time of an individual PDU Set.

RAN2 confirms that the remaining time needs to be dynamically reported from a UE to the gNB. FFS on the reporting granularity, i.e. per PDU Set or data burst.

In legacy, BSR is designed to report the gNB how much data are buffered at the UE side, thus only per-LCG data volume information is explicitly contained in the BSR MAC CE, i.e. no time-related information is included in the BSR MAC CE. Also, the current BSR can only be triggered under certain conditions, which implies the gNB can not use BSR reporting to estimate the data arrival time or the remaining time of the packets which are buffered at the UE side (see below).

A BSR shall be triggered if any of the following events occur for activated cell group:

-	UL data, for a logical channel which belongs to an LCG, becomes available to the MAC entity; and either

-	this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or

-	none of the logical channels which belong to an LCG contains any available UL data.

in which case the BSR is referred below to as 'Regular BSR';

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR';

-	retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR';

-	periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'.

Thus, the most straightforward way is to introduce a new MAC CE to contain the remaining time information.

Considering the current BS table may not be accurate for XR traffic transmission, RAN2 already achieves the following agreement. To reduce the reporting overhead, we understand the new MAC CE can also contain the data volume information of the packets for which the delay value is to be reported.

- 1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase).

- 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

RAN2 considers to introduce a new MAC CE to at least contain the remaining time.

The following triggering criteria of the new reporting would be introduced, e.g.

- A report is triggered when a fresh PDU Set/data burst is available;

- A report is triggered when the latency of a PDU Set/data burst buffered in the MAC layer approaches the PDSB.

Conclusion

We have the following observations:

Observation 1	PSDB is a statistics value which cannot reflect the actual remaining time of an individual PDU Set.

We have the following proposals:

Proposal 1	RAN2 confirms that the remaining time needs to be dynamically reported from a UE to the gNB. FFS on the reporting granularity, i.e. per PDU Set or data burst.

Proposal 2	RAN2 considers to introduce a new MAC CE to at least contain the remaining time.

Reference

[1] RP-220285  New SID “Study on XR Enhancements for NR”

[2] 3GPP TR 23.700-60 Study on XR (Extended Reality) and media services