3GPP TSG-RAN WG2 Meeting #120	R2-2212103

November 14 – November 20  2022, Toulouse, France

Agenda item:	8.4.3 - NR-DC with selective activation cell of groups

Source:	Nokia, Nokia Shanghai Bell

Title:	Further analysis on the solution aspects for selective activation

WID:	NR_Mob_enh2 - Release 18

Document for:	Discussion and Decision

# Introduction

In the last meeting, the following agreements have been made for selective activation of SCG (SAS) [5]:

Baseline procedure to support subsequent secondary cell group change

- (FFS if UE keeps all configurations or if those are indicated by the network,

- FFS support of nested configs):

a.	Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

b.	Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

c.	Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

- Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

- Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

- RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In the previous meeting, the following agreements have been made for SAPC (Selective activation for PSCell-change) [4] and is not treated:

- FFS how many subsequent conditional changes are targeted (and what is the impact of such assumption).

In this document, we provide further inputs on the remaining open issues related to maintaining the configurations across successive executions related to delta-configuration, security issues and measurement evaluations. We also analyze other open issues related to limitations on a number of cell changes and impacts of configuration changes during selective activation.

# 2	Discussion

## 2.1 Delta Configuration management for selective activation

In RAN2-119bis-e, RAN2 agrees to support delta configuration for selective activation. Following is the RAN2 agreement details for the same.

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

In current specifications for classic PSCell-change and also for CPAC, the conditional configurations are prepared as a delta configuration with reference to current SCG configuration. UE applies the delta configuration over the current source configuration at the time of CPAC execution. UE may reconfigure or re-establish the corresponding layers’ functionality as configured by the delta configuration.

When the delta configurations prepared for CPAC are stored for consecutive CPAC execution there may be conflict or mismatch of the configuration as explained below.

- As part of CPAC preparation, UE receives delta configurations for target PSCells B, C and D with serving PSCell A.

- When the UE moves from A to B, the UE applies the delta configuration of B over the current configuration A. After successful CPAC execution, the operating configuration is B’. B’ is derived by modification of source configuration A by replacing the parameters given in B.

- If the UE moves from B to C, as per current specification, the UE applies the delta configuration of C over current configuration B’. This may result in mismatch of configuration for PSCell C at UE and NW. Because the new configuration for C (referred as C’) is the update of B’ with delta configuration of C where-as the configuration at NW (referred as C’’) is update of A with C.

In the above case C’ and C’’ will be different if the parameter set affected by configuration B and C differs at-least by one parameter.

Observation 1: Use of delta configuration may result in configuration mismatch for selective activation for successive cell-changes if the affected parameters target delta configurations are different.

Observation 2: The configuration mismatch during successive cell-change should be resolved to support delta-configuration for SAPC.

This problem can be solved in multiple ways as described below.

Candidate Solution 1: Target configurations as a combination of common reference configuration and cell-specific delta configuration.

From the above observation, it is clear that if all the delta configurations of target cells are modifying the same set of parameters it is possible to use them for consecutive CPAC executions. This can be realized by identifying the parameters whose values are common across a set of target cells and dividing the target configuration into two parts namely reference configuration and cell-specific delta configurations.

The signalling procedure of this solution consists of the following key steps.

- When target cells are prepared for selective activation, the target SN may prepare multiple target cells using common reference configuration and delta configuration containing cell-specific and UE-specific parameters.

- In case if the target cells belong to a different SN, the reference configuration used by different SN should be the same for this procedure to work. This will require coordination of reference configuration for selective activation across involved Secondary nodes.

- The RRC Reconfiguration message sent to UE will contain one reference configuration and multiple conditional configurations which are delta configurations. These delta configurations should contain the same set of parameters and the values will be set for all these parameters.

When UE switches across cells when configured with reference and delta configurations, UE needs to only swap the delta configuration of the target cell.  As all the configurations are consistently made up of reference and delta configurations based on the common template. The configuration mismatch issue indicated in the observation will not happen.

Candidate Solution 2: Generation of modified delta configuration which can be used after successive cell changes based on current configuration

The UE stores the current cell configuration (at the time of SAS configuration) as a baseline configuration for the delta configuration. In this case for every successive cell change, the UE can first restore the baseline configuration and apply the delta configuration over this baseline configuration over this configuration. This two-step procedure at UE will enable application of the correct target cell configuration after mobility. But the UE need to restore to the baseline configuration as a ‘Full-configuration’. This may lead to additional interruption and delay in switching to new configuration which is equivalent to applying the full configuration of target cell.

It is possible to avoid above problem of applying full configuration of baseline configuration, if the UE can generate a modified delta configuration of all target cells that consists of all the parameters affected by all the delta configuration when it receives the delta configurations. The modified delta configuration would be source cell specific and can be applied directly on the current cell configuration at the time of SAS execution. This solution is illustrated using the following example.

Example scenario:

- UE is in dual connectivity with serving PSCell A.

- Based on measurement report, the target PSCells B, C and D are prepared as delta configuration as per current specifications.

- The parameter list of each delta-configurations is given here as example

  - Target cell B: Modifies parameter X and Y with value X1 and Y1.

    - The current value in source configuration is X0 and Y0.

  - Target cell C: Modifies parameter U and V with value U1 and V1.

    - The current value in source configuration is U0 and V0.

  - Target cell D:  Modifies parameter M and N with value M1 and N1.

    - The current value in source configuration is M0 and N0.

Based on the affected parameters of each delta configuration, UE can generate set of parameters to be used for generating new delta-configuration which can be referred as delta-configuration-sel-activation as below.  The delta-configuration-sel-act will consists of parameters X, Y, U, V, M and N.

- The parameter list of each delta-configuration-sel-act is given here as example

  - Delta for Target cell B:

    - Modifies parameter X and Y with value X1 and Y1.

    - Modifies parameter U and V with value U0 and V0.

    - Modifies parameter M and N with value M0 and N0.

  - Delta for Target cell C:

    - Modifies parameter X and Y with value X0 and Y0.

    - Modifies parameter U and V with value U1 and V1.

    - Modifies parameter M and N with value M0 and N0.

  - Delta for Target cell D:

    - Modifies parameter X and Y with value X0 and Y0.

    - Modifies parameter U and V with value U0 and V0.

    - Modifies parameter M and N with value M1 and N1.

The new values in these parameters are populated from the received delta configurations. Remaining values are taken from the current source configuration.

The modified delta-configurations generated in this approach will contain same set of parameters. Now if the UE applies this configuration after initial cell-change, there will not be a mismatch because the source configuration parameters which are over-written due to cell change is restored in the new delta configuration containing the source configuration values.

Candidate Solution 3: Nested CPAC configuration.

The use of delta configuration for selective activation can also be supported if each Target cell prepares another CPAC configuration containing delta configurations with the target cell as reference configuration. In this case towards UE there will be at least N*N target configurations assuming all the target cells are maintained for selective activation for each target cell. This increases the signalling overhead and also storage overhead at UE for selective activation.  Within network, for Inter-SN scenario the nested configuration approach requires multiple preparation triggered from each target cell towards other cells. Network also needs to maintain multiple configurations at each SN for incoming mobility from all other target cells.

The pros and cons including specification impacts of the candidate solutions are captured in below Table.

|  | Advantages | Challenges | Specification Impacts |
| --- | --- | --- | --- |
| Solution 1 | The simple solution for successive execution for selective activation as UE only modifies the delta configuration during cell switching | For Inter-SN scenario, the common reference configuration needs to be coordinated across nodes. Restriction in configuration changes | Inter-node coordination for reference configuration. Minor changes to RAN2 specification on use of reference configuration |
| Solution 2 | Minimum or no impact to network as the preparation is similar to CPAC | UE needs to generate modified delta configuration which can be used for successive cell-changes. Only minor impacts to UE specification | UE specification impacts |
| Solution 3 | Reuse of existing CPAC signalling procedure. | Nested preparations needed within network for selective activation. Increases the signaling procedures and delay in preparation of target cell configurations. UE storage impacts as large number of configurations needed for same target-cell. Change of RRC-Reconfiguration requires significant change in repreparing the nested configurations. | Minor changes to RAN2. RAN3 spec impacts to enable multiple preparation of same target cells. |

As per the above analysis the functionality and performance impacts of nested configuration is very high compared to other two solutions, we propose to exclude the nested CPAC configuration as solution for supporting delta-configuration.

Proposal 1: Nested CPAC configuration is not considered for selective activation

Proposal 2: RAN2 to down-select between the use of common reference configuration (Solutio-1) and UE-modification of delta-configuration (Solution-2) as the solution for using delta configurations for successive cell changes.

## 2.2 Maintenance of candidate cells of selective activation

The target cells for selective activation are selected based on UE measurement report. For SN initiated case, the measurement report provides suitable target cells which are better than the source cell (based on A3 event) suitable for selective activation. In case of MN-initiated selective activation, the target cells are derived based on measurement-reports received at MN.  In either case, all these initial list of cells need not be maintained for selective activation after initial cell-change. Only sub-set of these target cells are required for successive cell changes.

Each target cells prepared for CPAC can indicate the potential target cells needs to be maintained for immediate cell change as part of preparation. The target cells can provide this assistance information based on the mobility history related to PSCell changes and also the measurements of other target cells compared to its own radio condition reported in the measurements.

Observation 3: The target cells for selective activation after initial CPAC can be different from the CPAC target cells.

Observation 4: Each Target cell of CPAC preparation can provide assistance information on their preferred target cells for selective activation based on mobility history and measurement results.

Proposal 3: Assistance information of preferred cells for selective activation from each target-cell is indicated from SN to MN for selective activation.

Based on the assistance information received on the preferred cells for selective activation from each target cell, MN can indicate the list of configurations to be maintained after the first CPAC execution against each Target cell. When UE executes the first cell change from the current serving cell to new cell, the UE only maintains the configurations of target cells given for this cell as a selective activation set and releases other target cell configurations.

Proposal 4: The selective activation set to be maintained after initial CPAC is indicated to UE for each Target-cell.

## 2.3 Measurements and Evaluation for selective activation

The execution condition for PSCell change differs across the cells based on the coverage radius, overlapping region, and interference situations. For example, one cell may not have much interference in the cell edges and RSRP-based measurements will be sufficient for mobility whereas in another cell which has higher inter-cell interference at cell-edges RSRQ based measurements may be required. Even the threshold for conditional events and priority for execution may differ in each cell.

Observation 5:  The CPC execution conditions for the prepared target cells can be different depending on the serving PSCell.

For changing evaluation conditions after UE switches to new cell, another RRC Reconfiguration to modify the measurement configuration will be needed. This will delay the consecutive execution until this new configuration is enabled at UE.  Additional mechanisms may be required to pre-configure these cell-specific execution conditions for SAS.

Proposal 5: For SAS target cell-specific execution conditions should be considered. FFS the details on enabling it without reconfiguration of the conditions after cell change.

In Rel-17 CPAC, the source SCG configuration is replaced by the target configuration as part of CPAC execution and it is not preserved. In the case of selective activation, the target cell configurations for which subsequent CPAC execution is possible should be maintained after initial execution. As the initial serving cell can also be a potential candidate cell for consecutive CPAC execution in some mobility scenarios, the UE should maintain the source configuration also for selective activation after initial execution. While UE keeps the source SCG for SAS the UE shouldn’t do any extra measurements to evaluate this SAS configuration.

For SAS UE might be configured with all SAS candidate cells under the same PCell. Some of the configured cells may not be a direct neighbour to the serving PSCell. Thus, UE will never execute PSCell change to that PSCell from the current PSCell. From an energy efficiency point of view, UE might not have an execution condition for such cells.

Proposal 6: Target cell-specific execution condition can indicate no execution condition, i.e., for the source SCG configuration. FFS if no execution condition can be configured as part of target cell specific execution condition for any SAS configuration.

## 2.4 Security Key management for selective activation

On the security key management for selective activation following agreement is made in RAN2-119bis-e.

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

As per the above agreement the SN counter value which needs to be changed as part of successive cell changes of selective activation is pre-configured at the time of preparation for selective activation.  The pre-configured values of SN counters needs to ensure the following criteria of SN key changes.

- The SN counter value is not reused across successive SN change scenarios. This is needed to avoid security re-keying issue during PSCell change scenario.

- The SN counter values are increased in monotonous way across cell changes. The SN counter value used for Nth cell change should be larger than the SN counter value used in the previous cell-changes upto N-1.

It is possible to achieve the above criteria for SN counter change for selective activation if list of SN counter values are a range of SN counter values are configured along with the configuration for selective activation. Whenever the UE changes across SN it uses the first un-used counter value from the list. After configuring the new security keys using this counter value UE marks this value as used and points to the next value in the list for the next cell change.  MN can modify the SN counter list depending on the remaining counter values via the RRC Reconfiguration message to extend the selective activation for a longer time.

Proposal 7: The RRC configuration for selective activation includes SN-counter-list for security key management.  The selection of SN counter value from this list for cell change will be specified.

## 2.5 Other issues

### 2.5.1 Selective activation for CPA

Support for selective activation for CPA is confirmed in RAN2-119bis-e.  This means when multiple target cells are prepared for CPA for adding SCG for single connectivity UE, these target cell configurations (all of them or sub-set) can be maintained for selective activation for successive cell changes after initial CPA execution to one of the prepared cells. The CPA measurement configuration of other candidate cells can be reused for CPC. So that after SCG addition (by CPA) the preserved measurement configuration can be reused for consequent CPC without additional signalling towards candidate cells. In case the CPC execution condition needs to consider the state of the current serving PSCell, UE can be configured to switch to a CPC specific execution condition for the same conditional configuration.

As the target cell configurations are prepared with Full configurations for CPA, if the UE can maintain them after initial CPA these full configurations of target cells can be used as such for successive cell changes. But the successive cell changes are not optimum compared to the case where the target cells for selective activation are prepared after the first CPA as delta configurations.  Because the configuration changes and impacts to user plane interruption with delta configuration is lesser compared to full configuration.  This issue can be mitigated if the target configurations for CPA is prepared as a combination of reference and cell-specific delta configurations. With this change after the first CPA, the UE only needs to replace the delta configuration for subsequent cell changes.

For UE based solution, it is possible to support selective activation for CPA if the UE generates the modified delta configurations based on the full configuration after first CPAC execution.

Proposal 8: The solution selected for delta-configuration management for successive cell changes is also considered for CPA selective activation.

When the MN releases the SCG for UE configured with selective activation for SCG, the stored configurations for selective activations are released if the existing principles of CPAC is followed. But the UE may need to restart the complete procedure of CPA and selective activation when it is later configured for CPA from the same radio coverage of the target cells of selective activation. It is beneficial to extend the preservation of selective activation at UE and MN after SCG release with certain conditions such as a timer or measurement reports. If the UE supports SCG deactivation, the configurations for selective activation can be maintained after SCG deactivation for the same benefits.

Proposal 9: RAN2 to consider maintaining the selective activation configuration after SCG Release or SCG Deactivation

### 2.5.2 Number of consecutive cell changes for selective activation for PSCell-change (SAPC)

The last meeting has made the following agreement with respect to the number of subsequent CPC executions:

Agreement 6: FFS how many subsequent conditional changes are targeted (and what is the impact of such assumption).

We assume that the SAS configurations are maintained at the UE side unless explicitly indicated by the network to be de-configured. Thus, it is not relevant to discuss the number of allowed subsequent changes, i.e., the UE can perform CPC executions until the CPC configurations are released by the network.

As the CPC configurations are kept by the UE after CPC execution for SAS, the resources are reserved at the target nodes for longer time than Rel. 17 CPC procedure. In such cases, the specific target cells may pre-empt or release the resources reserved from CPAC for some high priority admission control. When the target node releases the configuration, UE should be notified via RRC-Reconfiguration to release the corresponding stored configuration. Whenever the UE moves outside the initial list of cells for selective activation, new target cells can be added and earlier cells can be released dynamically using existing procedures.

Observation 6: SAS configurations can be added and released depending on resource situation at prepared target cells using mechanisms for the addition/release of configuration similar to conditional configuration.

Based on the above observations we don’t foresee the number of subsequent changes have impact on the selective activation functionality.

Proposal 10: The number of subsequent conditional changes is not restricted for SAS. Existing mechanisms of target cell configuration management can be used to release the prepared cells.

# 3	Conclusion

In this document we analyse the configuration management and signalling procedures of selective activation. Following are the observations proposals based on the same.

Support of Delta-configuration for SAPC

Observation 1: Use of delta configuration may result in configuration mismatch for selective activation for successive cell-changes if the affected parameters’ target delta configurations are different.

Observation 2 : The configuration mismatch during successive cell changes should be resolved to support delta configuration for SAPC.

Proposal 1: Nested CPAC configuration is not considered for selective activation

Proposal 2: RAN2 to down-select between the use of common reference configuration (Solutio-1) and UE-modification of delta-configuration (Solution-2) as the solution for using delta configurations for successive cell changes.

Observation 3: The target cells for selective activation after initial CPAC can be different from the CPAC target cells.

Observation 4: Each Target cell of CPAC preparation can provide assistance information on their preferred target cells for selective activation based on mobility history and measurement results.

Candidate cells for SAPC

Proposal 3: Assistance information of preferred cells for selective activation from each target cell is indicated from SN to MN for selective activation.

Proposal 4: The selective activation set to be maintained after initial CPAC is indicated to UE for each Target-cell.

Observation 5:  The CPC execution conditions for the prepared target cells can be different depending on the serving PSCell.

Measurements and Evaluations for SAPC

Proposal 5: For SAPC target cell-specific execution conditions should be considered. FFS the details on enabling it without reconfiguration of the conditions after cell change.

Proposal 6: Target cell-specific execution condition can indicate no execution condition, i.e., for the source SCG configuration. FFS if no execution condition can be configured as part of the target cell-specific execution condition for any SAS configuration.

Security Key Management for SAPC

Proposal 7: The RRC configuration for selective activation includes SN-counter-list for security key management.  The selection of SN counter value from this list for cell change will be specified.

Other Issues for SAPC

Proposal 8: The solution selected for delta-configuration management for successive cell changes is also considered for CPA selective activation.

Proposal 9: RAN2 to consider maintaining the selective activation configuration after SCG Release or SCG Deactivation.

Observation 6: SAS configurations can be added and released depending on the resource situation at prepared target cells using mechanisms for the addition /release of a configuration similar to conditional configuration.

Proposal 10: The number of subsequent conditional changes is not restricted for SAPC. Existing mechanisms of target cell configuration management can be used to release the prepared cells whenever needed.

# References

- TS 37.340, E-UTRA and NR; Multi-Connectivity; Stage 2 (Release 17).

- TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2 (Release 17).

- RP-221799 Revised WID on Further NR mobility enhancements 3GPP TSG RAN Meeting #96 Budapest, Hungary, June 6-9, 2022

- R2-220XXX, Report of 3GPP TSG RAN WG2 meeting #119-e, Online, . clause 8.4.2

- R2-220XXX, Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online, . clause 8.4.3