3GPP TSG-RAN WG2 Meeting #120                                                 	R2-2212145

Toulouse, France, 14th - 18th Nov, 2022

Source:	CATT

Title:	Measurement Reporting for NR UAV

Agenda Item:	8.8.2

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e, the following issues were discussed:

- Details on flight path plan reporting

- Using height (H1/H2) for measurement reporting and parameter adjustments

- Details of reporting when numberOfTriggeringCells is met and other means to avoid excessive reporting

In this contribution, we discuss some of the leftover issues and provide our view.

Discussion

Flight path reporting

In the last meeting, the below agreements were reached for flight path reporting aspect:

Agreements:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

For the ffs on flight path details (waypoints and what is time information), it already captured the below description for flight path reporting for LTE UAV in [1].

E-UTRAN can request a UE to report flight path information consisting of a number of waypoints defined as 3D locations as defined in TS 36.355. A UE reports up to configured number of waypoints if flight path information is available at the UE. The report can consist also time stamps per waypoint if configured in the request and if available at the UE.

Hence, we think the similar mechanism can be applied in NR. It is proposed that:

Proposal 1: A location list of waypoints (3D location information) is adopted as the basic content of flight path report.

For the time information, we can follow the LTE principle as baseline, hence the below proposal is raised:

Proposal 2: Absolute time information is used as baseline for time stamp.

For the ffs on the details for flight path update, we also raise that NR UAV can follow LTE UAV’s mechanism as baseline.

Proposal 3: A request message for flight path information from serving gNB is introduced as baseline. Use UE information request response message to response.

For the fss on the granularity of flight path, considering it is transferred by RRC signalling and the payload is not short board, we would like to keep some flexibility, hence we prefer to leave it to UE implementation.

Proposal 4: For the granularity of flight path, it is left to UE implementation.

Using height for measurement reporting and parameter adjustments

In the last meeting, RAN2 discussed the issue for using height for measurement reporting and parameter adjustments without consensus.

- Continue to study height-depending scaling, triggering and combinations

In order to push forward, rapporteur raised to discuss the scenarios where NW-configured height-dependent parameter adjustment is beneficial. From our point of view, it is still unclear why to introduce the NW-configured height-dependent parameter adjustment. For example, whether it is introduced to enhance the mobility performance (HOF) or to limit the report quantity?

Proposal 5: RAN2 to discuss the purpose of using height for measurement reporting and parameter adjustments firstly.

Beam-level measurements

We reached agreement that the numberOfTriggeringCells was introduced to avoid frequent measurement reporting and ffs on whether to introduce numberOfTriggeringBeams similar to numberOfTriggeringCells. The stuck point is the purpose to using beam measurement for NR UAV. Then it is reasonable to further discuss whether to introduce numberOfTriggeringBeams similar to numberOfTriggeringCells or not.

Proposal 6: RAN2 to discuss the purpose of using beam measurement for NR UAV.

reportOnLeave enhancement

In last RAN2 meeting, two contributions[2]-[3] raised different solutions for reportOnLeave enhancement. In [2], it indicates that the number of triggering cells applies to the measurement report for a leaving condition. Its purpose is to reduce excessive measurement reporting caused by the leaving condition. In [3], it indicates that the UE sends the MR to the NW only when the cell which is leaving the cellsTriggeredList has been reported to the NW beforehand. Its purpose is to revise the unreasonable point for the current reportOnLeave achievement. Both of the two enhancements are important for further push forward on reportOnLeave enhancement.

Proposal 7: RAN2 to discuss whether the current reportOnLeave achievement is reasonable and whether the number of triggering cells applies to the measurement report for a leaving condition.

CHO-related UAV enhancements

In LTE UAV, similar issue had been discussed in RAN2#101 meeting with typical contributions [4] and [5]. In [4], it proposed to postpone the discussion on CHO for aerial UEs with the below two reasons:

- Due to the signaling overhead and the requirement of relay like X2 communication issues with Conditional Handover, further study and discussion are required.

- For the aerial UEs especially for those with high altitude, the fluctuation of the measurement can pre-maturely trigger the CHO at wrong timing. This will compromise the HO reliability and increase the signaling overhead. Then more time is required for further study.

In [5], it stated that as a way to resolve these difficulties in aerial radio condition, it is suggested to consider handover procedure based on a configured mobility condition i.e., conditional handover.

But due to time limitation, the conditional handover for Aerial UE was not specified in LTE spec.

The motivation for the conditional handover procedure is to reduce the time for transmission of measurement reporting and reception of handover command. So that it would be possible to reduce the handover failure caused by not receiving handover command at a proper time. The disadvantage of intruding CHO is signalling overload in some scenario. But with location and path information, one balance can be reached. Hence, we suggest introducing CHO for NR UAV.

Proposal 8: It is proposed to introduce CHO for NR UAV.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: A location list of waypoints (3D location information) is adopted as the basic content of flight path report.

Proposal 2: Absolute time information is used as baseline for time stamp.

Proposal 3: A request message for flight path information from serving gNB is introduced as baseline. Use UE information request response message to response.

Proposal 4: For the granularity of flight path, it is left to UE implementation.

Proposal 5: RAN2 to discuss the purpose of using height for measurement reporting and parameter adjustments firstly.

Proposal 6: RAN2 to discuss the purpose of using beam measurement for NR UAV.

Proposal 7: RAN2 to discuss whether the current reportOnLeave achievement is reasonable and whether the number of triggering cells applies to the measurement report for a leaving condition.

Proposal 8: It is proposed to introduce CHO for NR UAV.

- Reference

- TS 36.300 V16.6.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 16)

- R2-2210648 Measurement Report Enhancement	LG Electronics Finland	discussion	Rel-18

- R2-2210504	Potential issues and enhancements for UAV measurements	Huawei, HiSilicon

- R2-1802665 Discussion on Conditional HO Huawei, HiSilicon

- R2-1802709 General aspects of Conditional HO in Aerial LG