3GPP TSG-RAN WG2 #120	R2-2212467

Toulouse, 2022-11-14- 2022-11-18

Agenda Item:	8.4.3 NR-DC with selective activation cell of groups

Source:	Ericsson

Title:	NR-DC with selective activation

Document for:	Discussion, Decision

# 1	Introduction

The rel-18 work on further NR mobility enhancements is described in the WID in [1]. The WID includes an objective related to enhancements for PSCell mobility in case of NR-DC configurations (objective 2). At the RAN2#119bis-e meeting the following agreements were reached for this objective:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

- Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

- Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

- Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In this contribution we discuss different aspects and open issues for objective 2 in the WID on rel-18 work on further NR mobility enhancements (in [1]).

# 2	Discussion

## 2.1	Keeping CPC configurations at cell changes

A baseline procedure for how to support subsequent SCG changes, after execution of a PSCell addition or change procedure, was agreed at the RAN2#119bis-e meeting. It consists of that the UE does not release conditional configurations for other candidate PSCells (for subsequent CPC). Whether the UE should always keep all those configurations or whether it should be possible for the network to indicate what configurations to keep or to release was however left as FFS “FFS if UE keeps all configurations or if those are indicated by the network”.

In rel-17, the conditional configurations (CHO, CPC and CPA) are released at execution of another conditional configuration (or legacy cell change procedure). By introducing the possibility to keep CPC configurations in rel-18, it needs to be clear between the UE and the network how stored CPC configurations are to be handled at cell group changes, i.e. if they are to be stored or released. It should also be considered that, as the UE is moving into new cells/areas, the CPC configurations for the UE may need to be updated. There is therefore a need for a possibility for the network to indicate to the UE whether CPC configurations should be kept or released at a cell change procedure.

There is sometimes a need to update the CPC configurations for a UE, e.g. if the UE is moving into a different area.

It should be possible for the network to indicate to the UE whether to keep or release stored CPC configurations when performing a cell group change.

The following alternatives can then be identified:

- The network indicates to the UE after the successful cell group change procedure which of the CPC configurations to keep and which to release.

- The UE is, before the cell group change, provided with an indication about which CPC configurations to keep. Such indication could e.g. be included as part of the configuration that triggered the cell group change procedure, or as part of the CPC configuration that is to be kept or released.

The alternative to let the network (the target node) indicate to the UE after the cell group change procedure which CPC configurations to keep and which to release would mean that there is a need for signaling after each cell group change. This signaling can probably be kept small compared to retransmitting the whole CPC configurations, but it would still not be possible to perform a subsequent PSCell change without any reconfiguration.

To let the UE determine whether to keep or release a CPC configuration based on previous indication from the network does not require any signalling after each cell change procedure and thus enables subsequent conditional PSCell changes without any reconfiguration. We therefore have a slight preference for this alternative, i.e. that the UE should keep/make use of a stored CPC configuration based on information received before the execution of the cell group change. Since it should be the target SN node that decides what  CPC configurations that the UE should have after the cell group change, it should also be the target SN that decides what or what type of CPC configurations that the UE should keep after the cell group change.

The UE should keep (i.e. not release) a stored CPC configuration after cell group change so that subsequent signaling after the cell group change is not needed.

The UE should keep (i.e. not release) a stored CPC configuration after cell group change based on information from the target SN of the cell group change procedure, e.g. in the CPC configuration or PSCell change configuration that triggered the cell group change.

The serving SN always needs to be aware of the SN initiated CPC configurations that the UE has stored so that the configurations are consistent. For a target SN of an inter-SN cell group change, where the UE stores CPC configurations that it has received earlier, there is therefore a need to get information about all SN initiated CPC configurations that the UE has stored. In case of an inter-SN PSCell change the MN should therefore inform the target SN about which SN initiated CPC configurations that the UE has stored.

In case of a PSCell change the target SN needs to have the information about the SN initiated CPC configurations that the UE has stored.

In case of an inter-SN PSCell change the MN should inform the target SN about which SN initiated CPC configurations that the UE has stored.

## 2.2	Use cases for Conditional PSCell Addition

The following agreement related to Conditional PSCell Addition (CPA) was reached at the RAN2#119bis-e meeting:

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

According to the agreement at RAN2#119bis-e meeting, it should thus be possible for the UE to keep a CPA configuration that it has received. Use cases that can be foreseen are then that it should be possible for the UE to keep a CPA configuration if:

- the UE performs an MCG mobility procedure (i.e. a PCell handover) where the CPA configuration is still valid in the target PCell

- an PSCell/SCG is setup and then released again, where the CPA configuration then can be used for a subsequent PSCell addition.

For the case of PCell mobility, whether it is possible to keep the CPA configuration depends on whether the target PCell belongs to the same MN as the source PCell. To prepare a DC operation between the MN and the SN, for configuration of CPA, the SN Addition Request procedure is triggered by the MN towards the candidate SN. In case the UE moves to a new MN the corresponding preparation of DC operation would then not be done between that new MN and the candidate SN (for a CPA configuration that was configured by the source MN). Since the UE does not know when it changes MN, whether to keep or release a stored CPA configuration at PCell handover should therefore be indicated to the UE by the network.

There can be different reasons why an SCG is released for a UE. The typical reason to release the SCG/PSCell is that the amount of data is not enough to make use of the SCG. Another reason to why the SCG disappears could be that the radio conditions have changed so that the UE is out of coverage for the possible PSCell(s). In case CPA configurations are kept by the UE when the SCG is released by the network due to e.g. too little data, the UE should then not start evaluation of any CPA configuration since the execution of CPA then would lead to an undesired PSCell addition. In such case it would therefore be needed to make sure that the CPA configurations are released when the SCG is released, e.g. through an indication from the network at the SCG release. The CPA configurations could however still be useful later on, in case the amount of data increases so that there is a need to configure an SCG again. In that case it could be considered to support that the CPA configuration is not evaluated by the UE after the SCG is released (i.e. that the CPA configurations are “deactivated”) and that the network then can activate the CPA configurations again when there is a need for the SCG.

It can be noted that the execution conditions for CPA are based on A4 events, i.e. that the radio conditions for the candidate target PSCell are good enough. These conditions are however not useful for PSCell mobility since the radio conditions for the serving PSCell then need to be taken into account as well. If the network wants to configure the UE with a CPC configuration towards the same cell as there is a CPA configuration, this needs to be a separate configuration.

In case of an inter-MN PCell handover, it is not possible to use a CPA configuration that was configured by the source MN.

It should be possible for the network to indicate whether stored CPA configurations should be kept (or deactivated) by the UE at a PCell handover, at setup of the SCG and at release of the SCG.

## 2.3	Handling of execution conditions and measurement configurations

When the UE stores the CPC configuration after a cell group change, as discussed in section 2.3.1, the corresponding execution conditions and conditional measConfig parts should also be kept in order to avoid the need for reconfigurations.

When the CPC configuration is stored after cell group change, the UE should keep the corresponding execution conditions and conditional measConfig.

In some cases the stored execution conditions and conditional measConfig might not be optimal when the UE performs a PSCell change to a new target SN. For SN initiated CPC configurations, the target SN therefore needs to have the possibility to update the execution conditions and corresponding measConfig. This, in turn, requires that the target SN is aware of the execution conditions and conditional measConfig that the UE has stored.

The following alternatives for the target SN to get hold of the execution conditions and related conditional measConfig for SN initiated CPC configurations can be identified:

- The MN informs the target SN about the execution conditions and conditional measConfig stored by the UE. This information can e.g. be provided when the MN sends to the target SN information about the SN initiated CPC configurations that the UE has stored, as discussed in section 2.3.1.

- The UE sends to the target SN (via the MN) the stored the execution conditions and conditional measConfig when executing the PSCell change procedure (e.g. in RRCReconfigurationComplete).

To let the MN provide the execution conditions and conditional measConfig to the target SN requires that the MN keeps an up to date list of the execution conditions and the SCG measConfig and then transmits these to the target SN at the cell change procedure. The alternative where the UE sends the information about the execution conditions and conditional measConfig to the target SN would increase the signalling over the air interface, which should preferably be avoided. We therefore prefer the solution where the MN provides the stored execution conditions and the related conditional measConfig to the target SN, together with the corresponding SN initiated CPC configurations.

The MN should, at PSCell change procedures, provide the execution conditions and the related conditional measConfig for SN initiated CPC configurations (together with the information about which SN initiated CPC configurations the UE has stored) to the target SN.

## 2.4	Handling of SN security

There has been discussions both in RAN2#119-e and in RAN2#119bis-e related to whether there might be any security issues due to that CPC configurations are kept in the UE when one PSCell change is executed. The following agreement was reached at the RAN2#119bis-e meeting:

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

The Sk-Counter is sent by the MN to the SN and the SN uses it to derive base keys. These keys can be used for a number of reconfigurations. When the SN is changed the MN sends a new Sk-Counter to the UE. The MN is aware of when it needs to send a new Sk-Counter and there is in principle no difference between the legacy case or the case when the reconfigurations are done in advance as is the case for conditional reconfigurations. The network can include new Sk-Counters in the CPC configurations in the cases when it is needed. This is not an issue as the Sk-Counters do not have to be used in any particular order.

There does not seem to be any security issue related to subsequent conditional reconfigurations.

## 2.5	Nested CPC configurations

How to achieve subsequent SCG changes was discussed at RAN2#119bis-e with an agreement that the baseline procedure is that the UE does not release conditional configuration of other candidate PSCells after finishing a PSCell addition or change. The following FFS was however also part of the agreement “FFS support of nested configs”, i.e. whether a subsequent SCG change could also be achieved through inclusion of CPC configurations within a CPC/CPA configuration (or within a PSCell change/addition command).

Today it is not allowed to include the field conditionalReconfiguration in the condRRCReconfig, i.e. a new conditional reconfiguration within the conditional reconfiguration. This can be seen in the field description for condRRCReconfig in 38.331:

condRRCReconfig

The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config. For CPA and for CPC, the RRCReconfiguration message contained in condRRCReconfig cannot contain the field scg-State.

It is also described in 37.340 (clause 10.6) that CPC configurations cannot be included in conditional reconfigurations:

CPC configuration in HO command, in PSCell addition/change command or in conditional configuration (i.e CPA, CPC or CHO configuration) is not supported.

The objective 4 in the WID [1] is to specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC. It can thus be assumed that there then will be support for a CHO configuration that includes CPA/CPC. By adding support for CPC/CPA configurations within a CHO configuration, the above restrictions regarding inclusion of a CPC configuration within a conditional reconfiguration would be removed. The restriction for CPC configurations within a CPC or a CPA configuration would then also be removed unless new, more specific, restrictions are to be introduced in the specification.

In case support for CPC or CPA within a CHO configuration is specified, the possibility to also support a CPC configuration within a CPC or CPA configuration may be supported without additional specification work.

A benefit with so called “nested CPC configurations”, i.e. where one CPC configuration includes other CPC configurations, is that the target SN for the first CPC configuration can configure new CPC configurations that are relevant in that PSCell, which the UE can start evaluating as soon as it enters that PSCell. What CPC configurations that are relevant will typically differ when the UE moves between different cells/areas in the network. By including other CPC configuration within one CPC configurations allows that the candidate configurations for NR-DC with selective activation can be updated as the UE moves in the network without additional signalling. It should then thus be possible to keep also these “inner” CPC configurations at later cell change procedures.

To include CPC configurations with CPA configurations will in a similar way allow that the UE is initially configured with CPC configurations that are relevant in the PSCell that is added.

Inclusion of a CPC configurations within a CPC or CPA configuration should be supported. It should also be possible to keep these included (nested) CPC configurations at later cell change procedures.

According to the sentence in 37.340, 10.6, it is today also not supported to include a CPC configuration in a PSCell addition command or a PSCell change command:

CPC configuration in HO command, in PSCell addition/change command or in conditional configuration (i.e CPA, CPC or CHO configuration) is not supported.

Considering the need to support cases with frequent SCG changes in FR2, this restriction will however mean that the UE may not yet have any CPC configurations in time for the first PSCell mobility procedure after the first PSCell has been added (through a normal PSCell change command). The same is applicable for the case where the UE enters an area where e.g. the solution for NR-DC with selective activation should be used, i.e. when a normal PSCell change command thus is used when the UE enters such an area. It should therefore be possible to include CPC configurations in the normal PSCell addition command and the normal PSCell change command. It should also be possible to keep these CPC configurations at further cell change procedures, i.e. be used for the NR-DC with selective activation.

CPC configurations should be supported within normal PSCell addition and PSCell change commands. It should also be possible to keep these CPC configurations at later cell change procedures.

## 2.6	Supported types of CPC configurations

There are three different types of CPC configurations that have been specified in rel-16 and rel-17:

- Rel-16 intra-SN CPC (without MN involvement)

- Rel-17 SN initiated inter-SN CPC

- Rel-17 MN initiated inter-SN CPC

At the RAN3#117bis-e meeting RAN3 has taken an agreement that Inter-CU and Intra-CU cases should be considered with equal priority (with a possibility to revisit based on RAN2 progress). We see no specific reason to change the RAN3 agreement from a RAN2 point of view and we thus think that both intra-SN and inter-SN Conditional PSCell Changes should be targeted in the rel-18 work on NR-DC with selective activation.

For PSCell mobility it is the SN that should be in control. The SN should e.g. always be in control of its own resources and there is e.g. only the SN initiated procedure for intra-SN CPC (from rel-16). This should then be the case also for inter-SN PSCell changes. The SN initiated CPC should therefore be supported in the rel-18 solution for NR-DC with selective activation.

In addition, the MN initiated inter-SN CPC today suffers from that, when MCG measurement configuration is used, the events (execution conditions) that compare the candidate target cell with the source cell (A3/A5) then compare the candidate target PSCell with the current PCell (i.e. not with the current PSCell). These events are thus not useful for MN initiated inter-SN CPC. The A4 event does not have this issue since it only looks at the candidate target PSCell, but it is then not useful for mobility use cases, which is targeted with CPC. The issue with A3/A5 events thus means that the rel-17 MN initiated CPC solution is not useful as such and we therefore think that this issue needs to be corrected (in rel-18) in case we should target MN initiated CPC in the rel-18 work.

Both intra-SN and inter-SN Conditional PSCell Changes should be targeted in the rel-18 work.

SN initiated inter-SN Conditional PSCell Changes should be supported in the rel-18 solution.

In case MN initiated inter-SN CPC is to be targeted in the rel-18 work, A3/A5 events where the candidate target PSCell is compared with the current/serving PSCell should be supported for MN initiated inter-SN CPC.

In case both intra-SN and inter-SN CPC configurations can be kept by the UE when performing cell change procedures, a question is however how to handle the case when a UE has an intra-SN CPC configuration stored and then performs an inter-SN PSCell change. Since the intra-SN CPC configuration correspond to a Rel-16 intra-SN CPC configuration, i.e. without MN involvement, it can then not be used by the UE after moving to another SN, even though the included candidate target PSCell (and the configuration) may still be a relevant candidate.

The same question is valid also for the case where the UE is configured with an inter-SN CPC configuration, i.e. Rel-17 SN-initiated inter-SN CPC or Rel-17 MN-initiated inter-SN CPC, and the UE performs a PSCell change to another PSCell in the same SN as the candidate target PSCell that is included in the CPC configuration. In that case the candidate target SN will be the same as the source SN for that configuration.

It is unclear how to handle an intra-SN CPC configuration if the UE performs an inter-SN PSCell change.

It is unclear how to handle an inter-SN CPC configuration if the UE performs a PSCell change to a cell in the same SN as the candidate target PSCell in the CPC configuration.

For the handling of a rel-16 intra-SN CPC configuration at an inter-SN PSCell change one could consider to somehow reuse the configuration also when the UE has moved to a different SN since the target PSCell and perhaps also the target configuration may be valid for CPC. However, specification of such solution would probably be rather complex, e.g. due to the need to somehow convert a CPC configuration in SN format (involving one SN) into another one in MN format (which then would involve both another SN and the MN). A simpler solution is then that intra-SN CPC configurations instead are released in case the UE performs an inter-SN PSCell change.

To handle the case that an (rel-17) inter-SN CPC configuration can be kept in case the UE moves into a PSCell that belongs to the same SN as the candidate target PSCell in the CPC configuration, would be possible by letting such an inter-SN CPC configuration be supported also for intra-SN candidate PSCells. In that case, it should also be supported to configure CPC for intra-SN PSCells using the rel-17 CPC configuration in MN format, i.e. that the candidate target PSCell also belongs to the S-SN and that the S-SN is the candidate T-SN in that configuration. Such CPC configuration could then be kept in case the UE performs a PSCell change to another SN.

In case the UE performs an inter-SN PSCell change, it should release any rel-16 intra-SN CPC configurations.

For NR-DC with selective activation, it should be supported to configure CPC for an intra-SN candidate target PSCell using the rel-17 inter-SN CPC solution, i.e. to configure it in MN format.

## 2.7	Support for delta configurations

The following agreement related to delta configuration was reached at the RAN2#119bis-e meeting:

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

The use of the delta configuration implies that the UE determines a reference configuration, which is used as the configuration towards which a delta configuration is applied.

The delta configuration implies that the UE determines a reference configuration, which is used as the configuration towards which a delta configuration is applied.

Different means can be used to determine a reference configuration, however two of them can be considered for further investigation:

- The reference configuration is provided separately by the network in the RRC Reconfiguration message. When received, the UE stores it and, at handover execution, the received delta configuration is applied on top of it. The benefit of this solution is that it enables reconfigurations of the reference configuration.

- Use the current UE’s SCG configuration at the moment when the UE receives the RRC Reconfiguration message from the MN, as reference configuration. That is, when the UE receives the RRC Reconfiguration message from the MN, the current SCG configuration is stored and, at handover execution, the received delta configuration is applied on top of it. This solution has the benefit to reduce the size of the reconfiguration message, since the reference configuration is not included. However further aspects, e.g. how to modify the reference configuration may need to be investigated.

RAN2 should analyse and discuss the pros and cons of these two solutions and agree on which one to use, considering a possible harmonization with LTM solution.

RAN2 to discuss options for the reference configuration, considering the pros and cons and a possible harmonization with LTM solution:
- The reference configuration is provided separately by the network in the RRC Reconfiguration message.
- The UE uses its current SCG configuration at the moment when the UE receives the RRC Reconfiguration message from the MN, as reference configuration.

## 2.8	Candidate configuration for the serving cell

A rather typical use case for the frequent SCG changes in FR2 includes that the UE goes back to any of the previous PSCells. This means that the UE should be able to go back to the source cell based on a CPC configuration and that it should be possible, at least in some cases, to reuse the same CPC configuration more than once.

This means that when configuring CPC candidates for NR-DC selective activation, i.e. CPC configurations that can be kept at cell change procedures, it should be possible to also configure CPC for the current serving cell. When the UE performs a PSCell change procedure (normal or CPC) to a PSCell for which the UE has a CPC configuration, it should be possible for the UE to keep that CPC configuration. This includes that it should be possible for the UE to keep a CPC configuration that has just been executed.

In any of these cases, when the candidate target PSCell in a CPC configuration is the same as the current serving PSCell, the UE should however not perform any evaluation of that CPC configuration, or it should at least not perform any execution of the configuration.

It should be possible to have a CPC configuration for the serving PSCell, i.e. where the current serving PSCell is the candidate target PSCell.

A CPC configuration where the candidate target PSCell is the same as the current serving PSCell should not be evaluated, or at least not executed, by the UE.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There is sometimes a need to update the CPC configurations for a UE, e.g. if the UE is moving into a different area.

Observation 2	In case of a PSCell change the target SN needs to have the information about the SN initiated CPC configurations that the UE has stored.

Observation 3	In case of an inter-MN PCell handover, it is not possible to use a CPA configuration that was configured by the source MN.

Observation 4	There does not seem to be any security issue related to subsequent conditional reconfigurations.

Observation 5	In case support for CPC or CPA within a CHO configuration is specified, the possibility to also support a CPC configuration within a CPC or CPA configuration may be supported without additional specification work.

Observation 6	It is unclear how to handle an intra-SN CPC configuration if the UE performs an inter-SN PSCell change.

Observation 7	It is unclear how to handle an inter-SN CPC configuration if the UE performs a PSCell change to a cell in the same SN as the candidate target PSCell in the CPC configuration.

Observation 8	The delta configuration implies that the UE determines a reference configuration, which is used as the configuration towards which a delta configuration is applied.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It should be possible for the network to indicate to the UE whether to keep or release stored CPC configurations when performing a cell group change.

Proposal 2	The UE should keep (i.e. not release) a stored CPC configuration after cell group change so that subsequent signaling after the cell group change is not needed.

Proposal 3	The UE should keep (i.e. not release) a stored CPC configuration after cell group change based on information from the target SN of the cell group change procedure, e.g. in the CPC configuration or PSCell change configuration that triggered the cell group change.

Proposal 4	In case of an inter-SN PSCell change the MN should inform the target SN about which SN initiated CPC configurations that the UE has stored.

Proposal 5	It should be possible for the network to indicate whether stored CPA configurations should be kept (or deactivated) by the UE at a PCell handover, at setup of the SCG and at release of the SCG.

Proposal 6	When the CPC configuration is stored after cell group change, the UE should keep the corresponding execution conditions and conditional measConfig.

Proposal 7	The MN should, at PSCell change procedures, provide the execution conditions and the related conditional measConfig for SN initiated CPC configurations (together with the information about which SN initiated CPC configurations the UE has stored) to the target SN.

Proposal 8	Inclusion of a CPC configurations within a CPC or CPA configuration should be supported. It should also be possible to keep these included (nested) CPC configurations at later cell change procedures.

Proposal 9	CPC configurations should be supported within normal PSCell addition and PSCell change commands. It should also be possible to keep these CPC configurations at later cell change procedures.

Proposal 10	Both intra-SN and inter-SN Conditional PSCell Changes should be targeted in the rel-18 work.

Proposal 11	SN initiated inter-SN Conditional PSCell Changes should be supported in the rel-18 solution.

Proposal 12	In case MN initiated inter-SN CPC is to be targeted in the rel-18 work, A3/A5 events where the candidate target PSCell is compared with the current/serving PSCell should be supported for MN initiated inter-SN CPC.

Proposal 13	In case the UE performs an inter-SN PSCell change, it should release any rel-16 intra-SN CPC configurations.

Proposal 14	For NR-DC with selective activation, it should be supported to configure CPC for an intra-SN candidate target PSCell using the rel-17 inter-SN CPC solution, i.e. to configure it in MN format.

Proposal 15	RAN2 to discuss options for the reference configuration, considering the pros and cons and a possible harmonization with LTM solution: - The reference configuration is provided separately by the network in the RRC Reconfiguration message. - The UE uses its current SCG configuration at the moment when the UE receives the RRC Reconfiguration message from the MN, as reference configuration.

Proposal 16	It should be possible to have a CPC configuration for the serving PSCell, i.e. where the current serving PSCell is the candidate target PSCell.

Proposal 17	A CPC configuration where the candidate target PSCell is the same as the current serving PSCell should not be evaluated, or at least not executed, by the UE.

# References

RP-221799, Revised WID on Further NR mobility enhancements, MediaTek Inc., 3GPP TSG RAN Meeting #96, June 6-9, 2022