3GPP TSG-RAN WG2 Meeting #112		R2-2212503

Toulouse, France, November, 2022

Agenda Item:	8.4.4

Source: 	LG Electronics

Title:  	Discussion on CHO with CPAC

Document for:	Discussion

Introduction

For Rel-18 Further NR mobility enhancements, RAN2 has discussed supporting CHO with CPA/CPC and has made the following agreements in RAN2#119e [1]:

Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17. CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported. FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

In this contribution, we discuss supporting CHO with CPA/CPC based on the agreements and FFS.

Discussion

At the RAN2#119 meeting, the discussion of the evaluation rule for CHO and CPA/CPC included in CHO remains as FFS.

First, we would like to check the related objective to consider scenarios for CPA/CPC evaluation. Below is a related objective [2]:

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

Objective 4 clearly describes the scenario where the CHO configuration includes the CPA/CPC configuration. According to objective 4, for CPA/CPC execution, CHO execution needs to be first. Thus, we can exclude the scenario where the UE performs CPA/CPC execution before CHO execution, i.e. the UE cannot perform CPA/CPC evaluation until CHO execution.

Then, we consider another scenario that the UE performs CPA/CPC evaluation before CHO is complete.

Some smart or high-performance UEs can perform CPA/CPC evaluation while applying CHO configuration as soon as the target PCell is selected for CHO. Performing CPA/CPC evaluation before CHO completion can be helpful for the UE because the UE can quickly activate SCG and can quickly perform data transmission via SCG based on the CPA/CPC evaluation results.

However, the CPA/CPC configuration included in the CHO is likely to be dependent on the MCG configuration (i.e. CHO configuration) in most cases. Since the SCG configuration may affect the MCG configuration in some UE capabilities related parameters (e.g. MIMO layer, power class, cross-carrier scheduling, etc.), the UE cannot perform CPAC execution before CHO is complete.

In that sense, we can conclude that some UEs can selectively perform CPA/CPC evaluation before CHO completion for the fast process, but eventually, the UE cannot perform CPA/CPC execution before CHO completion. Also, according to the current spec, the UE is allowed to start the evaluation of CPA/CPC when CHO configuration is applied (i.e. before CHO is complete) if CPA/CPC configuration is included in the CHO confirmation.

Based on the above reasons, we don't think it's necessary to specify additional UE behaviours to start/stop CPA/CPC evaluation and we propose the following:

Proposal 1.	RAN2 should not specify a new rule to start/stop CPAC evaluation for the case that CHO configuration includes CPAC configuration, i.e. leave the additional rule of evaluation as the UE implementation.

As the next issue, as some companies has raised in the contribution papers for RAN2#119e, we consider whether there is a scenario in which the same candidate cell is configured simultaneously considering CHO and CPA/CPC configuration. This is because the UE now may receive many candidate cells for multiple purposes of conditional mobility, e.g. CHO, CPA/CPC for current SN, CPA/CPC for target MN’s SN. If the same candidate cell is provided to the UE for other mobility purposes, the candidate cell may be chosen by another serving cell that is already being used for other nodes, which may lead to connection failure and also resource waste from the network point of view.

Then, we consider a scenario in which CHO includes the same candidate cell(s) as the candidate(s) for CPA/CPC included in the CHO. For this scenario, we would like to conclude RAN2 doesn’t need to specify new UE behaviour for this scenario. This is because candidate cells for CHO and candidate cells for CPA/CPC cannot be the same. In the case of Inter SN CPC and CPA, the target MN should involve for the CPA/CPC configuration, and the target MN can prevent the same candidate cell is configured. In the case of Intra SN CPC, the target MN cannot be the same with any candidate cell for CPC since a candidate cell should be prepared within a single node and the MN and SN cannot be the same node in real cell deployment scenario.

Thus, we propose the following:

Proposal 2.	RAN2 doesn’t need to consider the UE behavior for a scenario the same candidate cell is configured in both CHO and CPA/CPC included in CHO.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1.	RAN2 should not specify a new rule to start/stop CPAC evaluation for the case that CHO configuration includes CPAC configuration, i.e. leave the additional rule of evaluation as the UE implementation.

Proposal 2.	RAN2 doesn’t need to consider the UE behavior for a scenario the same candidate cell is configured in both CHO and CPAC included in CHO.

Reference

- R2-2209301, “RAN2 Chairman Notes,” MediaTek, Aug 2022

- RP-222332, “Revised WID on Further NR mobility enhancements,” MediaTek, Sep 2022