3GPP TSG-RAN2 #120	R2- 2212851

# Toulouse, France, November 14th – 18th, 2022

Agenda item:		8.3.2 (FS_Netw_Energy_NR)

Source:	LG Electronics Inc.

Title: 	Discussion on DTX/DRX mechanism

Document for:	Discussion and Decision

1.	Introduction

This document discusses open issues for DTX/DRX mechanism for NES. The agreements in R2-119bis-e related to DTX/DRX mechanism are as follows.

… => RAN2 assumes that the options above are possible for gNB DTX/DRX behavior and discuss UE RAN2 behavior/impact during the DTX/DRX. => For the purpose of our discussion we will focus on a single UE behavior at any point in time. FFS if we allow multiple configuration of NW DRX/DTX behaviors … => Periodic DTX is assumed as a baseline. The gNB provides indication to UE about NW DTX mode/configuration via dedicated dynamic L1/L2 signaling. => Dynamic L1/L2 group signalling from NW to provide NW DTX mode/configuration is also considered in RAN2 => It is beneficial to align UE DRX with network DTX and DRX alignment among multiple UEs. Details are FFS, including UE transmission/reception behavior during DTX. RAN2 to study the alignment. …

During a post email discussion ([POST119bis][303][NES] TP on NW DTX/DRX), remaining issues has been discussed and listed up. The list includes clarification of L1/L2 signaling, configuration and signaling, UE behavior during Cell DRX Off duration, and alignment of DRX/DTX.

2.	Discussion

During the post email discussion ([POST119bis][303][NES]), the following issue is raised. Alt-2 is proper for DTX/DRX operation.

Whether L1/L2 signalling can be used to configure the DTX/DRX pattern, or only be used to activate the RRC-configured DTX/DRX pattern: - Alt-1: Allow both periodic pattern (configured by RRC) and one-shot pattern (configured by L1/L2 signalling) - Alt-2: Allow periodic pattern, configured by RRC and activated by L1/L2 signalling

DRX/DTX operation may continue for considerable time. If one-shot pattern configured by L1/L2 signaling is used, NW should send the L1/L2 signaling every DRX/DTX period. It will entail a lot of signaling. Periodic DRX/DTX will save signaling used for one-shot pattern. To efficiently change DRX/DTX pattern, multiple patterns can be pre-configured and at least one of them is activated by L1/L2 signaling.

Proposal 1. Confirm Alt-2: Allow periodic pattern, configured by RRC and activated by L1/L2 signalling.

For Cell DTX operation, a UE does not need to know Cell DTX configuration because UE DRX can be used for supporting Cell DTX. NW can configure UE DRX and is aware of the Active Time of each UE DRX. When the Active Time of each UE DRX is covered by On duration of Cell DTX, Cell DTX can be achieved. The figure below depicts how to use UE DRX for supporting Cell DTX.

Figure 1 UE DRX for supporting Cell DTX

Observation 1. UE DRX can be used for supporting Cell DTX.

Proposal 2. Cell DTX configuration is not informed to a UE.

For Cell DRX operation, a Cell DRX configuration needs to be known to a UE because the UE needs to know when the cell can receive its UL transmissions. To keep On duration of Cell DRX compact, a single Cell DRX configuration is shared by UEs.

Proposal 3. A single Cell DRX configuration is informed to a UE.

Cell DRX configuration needs to be separate from Cell DTX operation because uplink traffic generation on UE side is not synchronized with downlink traffic transmission. If uplink traffic pattern and downlink traffic pattern are aligned, same pattern can be used for Cell DRX and Cell DTX. In general, NW should consider different Cell DRX pattern from Cell DTX pattern for handling uplink traffic generation outside of Cell DTX On duration.

Proposal 4. Cell DRX configuration is separate from Cell DTX pattern.

It is beneficial that NW have multiple Cell DTX patterns to cope with traffic changes efficiently. In case that Cell DTX is achieved by UE DRX, a UE can be configured with multiple DRX configurations, NW selects a proper UE DRX configuration to cope with traffic change along with energy saving and activates the UE DRX configuration via L1/L2 signaling.

Proposal 5. Allow multiple UE DRX configurations to support multiple Cell DTX patterns.

In addition, whether to use a combination of UE DRX configurations at a time or not can be discussed. If a combination of UE DRX configuration can be used, it is beneficial to reduce pre-configurations because lots of new UE DRX patterns can be created from the pre-configured UE DRX configurations by the combinations.

Proposal 6. Allow multiple UE DRX configurations to be active at the same time.

For CA in Cell DTX/DRX, it is a possible scenario that the maximum data rate to be supported during the UE Active Time or Cell DTX On duration is higher than the data rate supported by a single serving cell. In legacy UE DRX, CA is supported in UE DRX. Similarly, CA can be supported in Cell DTX/DRX.

Observation 2. CA is supported in UE DRX.

Proposal 7. Allow to keep CA in Cell DTX/DRX.

The following examples on Cell DTX/DRX behavior are captured in TR 38.864.

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods. - Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals) - Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS). - Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

Although Example 1 has the highest power saving gain, the impact is also higher than other examples according to discussion about SSB adaptation/SIB-less cells. Without solutions for SSB adaptation/SIB-less cells, Example 1 does not seem to work properly. Therefore, Example 1 can be discussed later depending on the result from discussion about SSB adaptation/SIB-less cells.

Example 2 and Example 3 turn off all data transmission and dynamic data transmission respectively. In Example 3, SPS/CG transmission is allowed during Off duration of Cell DTX/DRX. Example 4 is not much different with Example 2 because a UE does not transmit CSI reporting and SRS during being not in Active Time in the legacy UE DRX. So, Example 2 and Example 3 can be mainly discussed. Since Example 3 is more aligned with the legacy UE DRX behavior, Example 3 can be a baseline for Cell DTX/DRX behavior and turning off configured data transmission (Example 2) can be further discussed.

Proposal 8. Consider Example 3 (turning off dynamic data transmission only during Off duration of Cell DTX/DRX) as a baseline for Cell DTX/DRX behaviour.

There has been a discussion about whether/how to align UE DRX with network DTX and whether/how to align DRX alignment among multiple UEs. The alignment does not need to mean that Cell DTX pattern and all UE DRX patterns are exactly matched. For example, a UE may be less active and its Active Time can be shorter than others, or vice versa. However, all UE DRX patterns need to be covered by Cell DTX pattern. It can be achieved by network implementation because NW configures UE DRX, NW is aware of the Active Time of each UE DRX, and NW can adjust Cell DTX accordingly.

Proposal 9. Confirm that Cell DTX/UE DRX alignment means that Cell DTX pattern covers all UE DRX patterns.

3.	Conclusion

This document discusses open issues for DTX/DRX mechanism for NES.

Proposal 1. Confirm Alt-2: Allow periodic pattern, configured by RRC and activated by L1/L2 signalling.

Observation 1. UE DRX can be used for supporting Cell DTX.

Proposal 2. Cell DTX configuration is not informed to a UE.

Proposal 3. A single Cell DRX configuration is informed to a UE.

Proposal 4. Cell DRX configuration is separate from Cell DTX pattern.

Proposal 5. Allow multiple UE DRX configurations to support multiple Cell DTX patterns.

Proposal 6. Allow multiple UE DRX configurations to be active at the same time.

Observation 2. CA is supported in UE DRX.

Proposal 7. Allow to keep CA in Cell DTX/DRX.

Proposal 8. Consider Example 3 (turning off dynamic data transmission only during Off duration of Cell DTX/DRX) as a baseline for Cell DTX/DRX behaviour.

Proposal 9. Confirm that Cell DTX/UE DRX alignment means that Cell DTX pattern covers all UE DRX patterns.