3GPP TSG RAN WG2 Meeting #121                                                   R2-2300433

# Athens, Greece, 27th February-3rd March 2023

Agenda item:	8.5.4

Source:	Intel Corporation

Title:	Enhancements to Buffer Status Reporting for XR Traffic

Document for:	Discussion

# Introduction

In the last RAN2 meeting #120, the topic of XR specific capacity improvements was discussed and the following relevant agreements were made [1].

- RAN2 thinks we need one or more additional BSR table(s) for XR. FFS whether these are static (=specified) or dynamic (e.g. generated, differs according to some RRC parameter), can be discussed in WI phase. - RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay. - RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

This document discusses possible enhancements to buffer status reporting for XR traffic. Particularly, we discuss how BSR could be enhanced for XR traffic for the following cases: (1) Additional BSR tables, (2) Delay Reporting and (2) BSR in the event of PDU discard.

# Discussion

## Additional BSR tables

The need for additional BSR table(s) for XR is agreed in RAN2 with the purpose of reducing quantization errors for typical high volume of data in UL buffer which can be expected for XR applications with high bit rates. It was left FFS in the last meeting whether these additional BSR tables will be static or dynamic, i.e UE dynamically constructed BSR tables based on some configuration e.g step size, shape etc. Dynamic BSR tables may be more complicated with respect to UE implementation, while static tables could be limiting. One other approach which could be adopted is to define these BSR tables semi-statically, i.e to have pre-defined BSR tables with finer granularity, where RRC configuration could be used by the gNB to select the desired BSR table. Another aspect to consider in RAN2 would be whether such semi-static BSR tables are configured per MAC entity or per LCG, and whether the BSR tables support linear or exponential quantization. In our view, it will be straightforward to have per MAC entity configuration of such semi-static BSR tables in the interest of time, however RAN2 could consider per DRB/LCG configuration if needed.

- Additional BSR table(s) are defined for XR. UE determines which BSR table to use based on configuration received from network.

  - Configuration of semi-static BSR table(s) for XR can be provided per MAC entity via RRC signalling.

  - RAN2 to discuss which quantization algorithm is used for additional BSR table(s) for XR e.g linear or exponential.

## Delay Reporting

### Aggregation level and reported value

SA2 has agreed to introduce PDU set information in user plane e.g PDU Set sequence number, end PDU of PDU set, and also new QoS parameters of PDU Set Delay Budget (PSDB) can be optionally communicated to RAN for downlink [3]. It can be assumed that similar information may also be available in uplink. It must be noted that at any given time there may be multiple PDU sets stored in the UL buffer, and each PDU set can have its own PSDB value. The gNB may not have a guaranteed way of keeping track of all PDU-sets that the UE has started transmitting, and whether the remaining budget for data stored in the UL buffer is limited/expiring. In some cases, the PDU set may be transmitted altogether, hence all PDUs in the PDU set face similar delay circumstances. However, based on the UL buffer and the size of the PDU set, it is also possible that the PDU set is transmitted sequentially over different bursts (i.e. different occasions in time). Depending on whether the PDUs of a data burst arrive simultaneously in the AS or sequentially, the gNB may also not be able to reliably track the PDU set transmission progress (e.g how many PDUs and delay budget remains for any given PDU set).

- gNB cannot reliably keep track of remaining time especially if multiple PDU-sets are undergoing transmission or are stored in the UL buffer, or if a PDU set is transmitted over multiple data bursts.

Since the UE and gNB are cognizant of PSDB value per PDU set, or PDB value per PDU, especially for delay-sensitive data in the UL buffer, the UE could enhance L2 procedures. The definition of remaining time for PDUs or data stored in the UL buffer will vary according to the delay budget value of their associated PDU sets, however reporting delay information of remaining time per PDU set could result in significant overhead, since also considering that at any given time, multiple PDU sets may be stored in the UL buffer. RAN2 therefore needs to decide on an aggregation level over which individual delay values (or remaining time) can be reported e.g over all PDUs/PDU-sets stored in the UL buffer, or per sub-group of PDU-sets in which case it could be defined as the worst-case or average value, that is the smallest or mean value of remaining delay budget among PDU-sets in that aggregated group. If the BSR is used to convey this aggregated delay information to the gNB, an appropriate aggregation level could be per logical channel group (LCG), such that the delay information is reported as the mean or worst-case value across multiple PDU sets of a given LCG.

- Delay information based on the remaining time is determined/reported by UE in the BSR at an aggregation level per logical channel group.

  - RAN2 to discuss which value of remaining time per aggregation level (e.g per LCG) is reported, considering for example, worst-case or average value, that is the smallest or mean value of remaining delay budget among PDU-sets over that aggregate.

### Remaining Time Based Configuration and Trigger

If delay values (discussed in previous section) are communicated using the BSR,  network should indicate to a UE when it is allowed to provide it (e.g. configuring a delayReportEnabled parameter to UE for delay critical applications)  and a corresponding new trigger can be introduced for when to provide the information associated with the remaining delay. For data in the UL buffer, the UE can determine the remaining time before expiration of preconfigured timer based on the maximum delay tolerable by a PDU set (e.g based on PDB/PSDB value, or a configured delay parameter). If a new trigger condition for reporting delay value is defined, it also needs to be considered that a delay reporting BSR is not triggered ever so often. A straightforward approach could be that the delay report is triggered if the remaining time for any given PDU set falls below a certain threshold criterion. However, this could potentially lead to multiple reporting triggers.  This could be easily limited for example, by using that delayReportEnabled parameter to limit the number of reports, i.e. network could disable or stop allowing the reporting. This configuration could be per DRB or per MAC entity and if delayReportEnabled is supported and enabled, the UE reports the aggregated remaining time value per LCG and the associated data volume in the BSR.

- Network can indicate to a UE when delay information (based on the remaining time) feature can or cannot be used (e.g. via a new delayReportEnabled parameter defined for delay critical applications).

- When the delay information feature is configured, a new threshold trigger condition is defined for UE to determine when to include this information the BSR. For example, if delayReportEnabled is configured AND the remaining time for any given PDU set falls below a certain configurable threshold criterion, then UE reports the aggregated remaining time value per LCG and the associated data volume .

### Reporting Format

To reflect delay in BSR, a new BSR mapping table for delay values similar to Table 6.1.3.1-1 and Table 6.1.3.1-2 in TS 38.321 could be created. A new MAC CE for enhanced BSR could be defined to carry delay information for XR traffic, where this delay information is provided per logical channel group as the smallest or mean value of remaining delay budget of data carried by any given logical channel in a particular logical channel group. An example of such MAC CE is shown in Figures 1a and 1b below. In the example MAC CE in Figure 1a, it is assumed that for any LCG both the buffer size and the delay value is reported. In the example MAC CE in Figure 1b, it is assumed that LCG-ext is set to 1 for only those LCGs for which a delay value is reported.

Figure 1a: BSR MAC CE with Remaining Time Information reported for all LCGs

Figure 1b: BSR MAC CE with Remaining Time Information reported for some LCGs

- A new MAC CE for enhanced BSR is defined to carry delay information for XR traffic, where this delay information is provided per logical channel group.

## BSR Enhancements for Packet Discard

### Impact of Packet discard on resource allocation

The topic of PDU discard is discussed in our companion paper [2]. If some PDUs within a PDU set are lost/corrupted/delayed during UL transmission, the transmitting entity (i.e. the UE) may decide to discard remaining PDUs within the PDU set to avoid unnecessary transmission e.g if the PDU set integrated handling indication (PSIHI) is flagged, or PDU sets could be discarded based on their type (high/low importance). This decision could be made by the transmitter (i.e. UE) for example, based on some delay budget related timer, or the type of PDU set. A problem in relation to feedback and BSR from the UE side, which could arise in the case of PDU discard, is that network might have outdated BSR information when UE discards packets without knowledge of the network. In some cases, the network may have allocated UL grants for packets that are discarded by UE before gNB has received an updated BSR. In other cases, the network might have not allocated a grant yet but it can still be useful to know about a reduction in UE’s buffered data volume e.g. for future scheduling, resource usage/planning. An example of such scenario is shown in Figure 2 below.

Figure 2: Updated BSR after PDU discard

- Network has outdated BSR information of a given UE when the UE transmitter discards packets autonomously i.e. without knowledge of the network.

- It is beneficial for in the UE to report to the network about a reduction in the data volume of the UE’s buffer. This happens when PDU discard is performed on UL data stored in UE’s buffers (i.e., which was previously reported in a BSR to the network).

### BSR Trigger in the event of PDU Discard

To avoid/minimize resource wastage in the event of packet discard, RAN2 can discuss methods of efficient buffer status reporting of XR traffic in uplink. A new BSR triggering condition can be introduced such that when UE performs the discard of packets, it sends an updated BSR to the gNB with the updated data volume in the buffer (after subtracting the discarded packets’ volume), i.e the UE reports a reduction in UL data volume via a BSR. When the UE learns of transmission failure for some (or all) transmitted PDUs, it could trigger the discard of all the remaining packets in the UL buffer for that PDU set and it is also possible that there may not be data left in the UL buffer for that LCG. The BSR transmitted in the event of PDU discard can potentially use the existing BSR MAC CE to report the data in the UL buffer, however some triggering conditions for BSR may be required. Transmitting a BSR ever so often, however, can lead to power consumption at the UE and cause signalling overhead. A trade-off is required on how often such PDU discard triggered BSRs can be generated. To this end, some additional condition(s) might need to be met in connection with the discard of the data for the UE.

- A timer can be introduced such that a BSR once triggered in the event of PDU set discard, can only be re-triggered after a preconfigured time duration.

- In the event of PDU-set discard, the initial discard update BSR is only triggered if the volume of discarded data is more than configured threshold

- PDU discard at UE transmitter triggers a BSR to report reduction in UL buffer data volume.

- RAN2 to discuss additional conditions to be met for discard triggered buffer status reporting e.g on the amount of discarded data, or time elapsed between discard triggered BSRs.

# Conclusion

The observations and proposals are captured as follows:

Observation 1.	gNB cannot reliably keep track of remaining time especially if multiple PDU-sets are undergoing transmission or are stored in the UL buffer, or if a PDU set is transmitted over multiple data bursts.

Observation 2.	Network has outdated BSR information of a given UE when the UE transmitter discards packets autonomously i.e. without knowledge of the network.

Observation 3.	It is beneficial for in the UE to report to the network about a reduction in the data volume of the UE’s buffer. This happens when PDU discard is performed on UL data stored in UE’s buffers (i.e., which was previously reported in a BSR to the network).

Proposal 1.	Additional BSR table(s) are defined for XR. UE determines which BSR table to use based on configuration received from network.

Proposal 1.1.	Configuration of semi-static BSR table(s) for XR can be provided per MAC entity via RRC signalling.

Proposal 1.2.	RAN2 to discuss which quantization algorithm is used for additional BSR table(s) for XR e.g linear or exponential.

Proposal 2.	Delay information based on the remaining time is determined/reported by UE in the BSR at an aggregation level per logical channel group.

Proposal 2.1.	RAN2 to discuss which value of remaining time per aggregation level (e.g per LCG) is reported, considering for example, worst-case or average value, that is the smallest or mean value of remaining delay budget among PDU-sets over that aggregate.

Proposal 3.	Network can indicate to a UE when delay information (based on the remaining time) feature can or cannot be used (e.g. via a new delayReportEnabled parameter defined for delay critical applications).

Proposal 4.	When the delay information feature is configured, a new threshold trigger condition is defined for UE to determine when to include this information the BSR. For example, if delayReportEnabled is configured AND the remaining time for any given PDU set falls below a certain configurable threshold criterion, then UE reports the aggregated remaining time value per LCG and the associated data volume .

Proposal 5.	A new MAC CE for enhanced BSR is defined to carry delay information for XR traffic, where this delay information is provided per logical channel group.

Proposal 6.	PDU discard at UE transmitter triggers a BSR to report reduction in UL buffer data volume.

Proposal 7.	RAN2 to discuss additional conditions to be met for discard triggered buffer status reporting e.g on the amount of discarded data, or time elapsed between discard triggered BSRs.

# References

- R2-22xxxxx, Report of 3GPP TSG RAN WG2 meeting #120, Online, November 2022.

- R2-2300430, Criteria and Mechanism of PDU Discard for XR traffic, RAN2 121

- TR 23.700-60 (v1.2.0), Study on XR (Extended Reality) and media services, SA2, Rel-18, Dec 2022.