3GPP TSG-RAN2 Meeting #121	R2-2300797

Athens, Greece, Feb 27- Mar 4, 2023

Agenda Item:	8.20.2 - TEI proposals by RAN2

Source: 	Qualcomm, Ericsson, Verizon, FirstNet

Title:  	RedCap CFR for MBS broadcast

Document for:	Discussion and Decision

# Introduction

Rel-17 introduced Multicast and Broadcast Services (MBS) for NR.

RAN received LS from SA2 (RP-221041) where one of the questions for MBS broadcast was:

“Question 1: Whether, similarly to eMBMS case for eMTC/NB-IoT, would it be useful for NG-RAN to receive from 5GC information on NR UE capabilities (e.g. RedCap) of the target recipients of MBS data in MBS broadcast mode.”

Support of MBS by RedCap UEs was discussed in RAN#96, where following was proposed (see RP-221335):

- confirm that a RedCap UE supporting FG33-1 supports MBS broadcast

- revise the Rel-18 MBS WI to enable separate CFR configuration for RedCap UEs

In the offline discussion during RAN#96, it was proposed to postpone the discussion of potential Rel-18 enhancements until RAN#97 (see offline report in RP-221782).

A reply LS to SA2 was approved (see RP-221861) indicating:

“RAN#96 has concluded that Rel-17 specifications do not prevent any UE, including RedCap UEs, to support MBS.”

In the meantime, SA2 concluded the KI#5 on “Coexistence with existing power saving mechanisms for capability-limited devices” and captured the following (see TR 23.700-47 section 8.5)

“How to support NR capability-limited (RedCap) UEs in MBS will be decided in normative phase considering possible related decisions of RAN WGs”

And the following objective is included in the SA2 work item (S2-2211458):

“Support of the enhancement of the multicast and broadcast communication service within 5G system, specifically:

…

- Support of coexistence with existing power saving mechanisms for capability-limited devices.”

Sourcing companies propose this item to be addressed in RAN2 by introducing a separate RedCap CFR for MBS broadcast.

# Discussion

The locationAndBandwidthBroadcast in SIB20 indicates the configured bandwidth for both MCCH and MTCH, i.e. it indicates the bandwidth used for all MBS broadcast sessions in the cell. If a RedCap UE does not support the configured bandwidth in SIB20 then the RedCap UE cannot receive MBS broadcast.

Observation 1: A RedCap UE cannot receive MBS broadcast when the bandwidth of the configured CFR in SIB20 exceeds the UE’s capability.

But a RedCap UE could indicate in MII message the frequencies/services it is interested to receive. And the gNB could use the MII signalling of a RedCap UE to lower the bandwidth of the default CFR in SIB20, if needed. But this approach has the following drawbacks/limitations/problems:

- If the PCell includes SIB20 and the configured bandwidth of the CFR exceeds the RedCap capabilities, then the Redcap UE would not be able to acquire the MCCH and the UE can only indicate interest in services based on SIB21 and USD info.

- A RedCap UE only sends MII signalling in connected mode when interested to receive MBS broadcast, i.e. the gNB does not know anything about RedCap UEs in RRC_IDLE or RRC_INACTIVE.

- It is not obvious that when a single RedCap UE in RRC_CONNECTED indicates to be interested in an MBS broadcast service, that the gNB would lower the configured bandwidth for all UEs and all MBS broadcast services. Furthermore a change of the CFR configuration interrupts the reception of MBS broadcast for all UEs/services, and it may also violate the QoS (e.g. required bitrate) of those services.

The gNB does not have an accurate view of the RedCap UEs interested in MBS services, and it may not be desirable to lower the bandwidth for all UEs and all MBS broadcast services to support (some) RedCap UEs:

Observation 2: The gNB does not have an accurate view of the interest of RedCap UEs in MBS services, and it may not be desirable to lower the bandwidth for all UEs and all MBS broadcast services to support some RedCap UEs.

To support group communications when some UEs have limited capabilities is complicated, and it is difficult to make it perfect for everybody. But we think it would be beneficial if the NW has the option to configure a separate RedCap CFR with limited bandwidth for RedCap UEs and not be required to lower the bandwidth for all UEs and services:

Proposal 1: Introduce a separate RedCap CFR which can be used when the configured bandwidth for the default CFR in SIB20 exceeds the RedCap capability.

The introduction of a RedCap CFR does not have impact on other UEs supporting MBS broadcast.

A RedCap UE uses the RedCap CFR when configured, otherwise it uses the default CFR provided that the UE supports the required bandwidth.

The gNB does not configure a RedCap CFR when the default CFR is also supported by RedCap UEs or when the session is not intended for RedCap UEs.

For reliability and coverage purposes the gNB may transmit the MCCH/MTCH with maximum power and bandwidth (similar as for Paging/System Information). Thus when a separate RedCap CFR is configured, then also a separate mcch-ConfigRedCap is required, because the MCCH/MTCH transmissions for a RedCap cannot occur in the same slot(s) as for “normal” UEs.

# Summary

RAN2 is asked to introduce RedCap CFR for MBS broadcast based on the following observations. The proposal impacts only RAN2 specification, as shown in the TP in appendix.

Observation 1: A RedCap UE cannot receive MBS broadcast when the bandwidth of the configured CFR in SIB20 exceeds the UE’s capability.

Observation 2: The gNB does not have an accurate view of the interest of RedCap UEs in MBS services, and it may not be desirable to lower the bandwidth for all UEs and all MBS broadcast services to support some RedCap UEs.

Proposal 1: Introduce a separate RedCap CFR which can be used when the configured bandwidth for the default CFR in SIB20 exceeds the RedCap capability.

# Appendix: TP for TS 38.331

#### –	SIB20

SIB20 contains the information required to acquire the MCCH configuration for MBS broadcast.

SIB20 information element

-- ASN1START

-- TAG-SIB20-START

SIB20-r17 ::=	SEQUENCE {

mcch-Config-r17                MCCH-Config-r17,

cfr-ConfigMCCH-MTCH-r17        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...,

[[

cfr-ConfigMCCH-MTCH-RedCap-r17 CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

mcch-ConfigRedCap-r17          MCCH-Config-r17         OPTIONAL   -- Cond CFR-RedCap

]]

}

MCCH-Config-r17 ::= SEQUENCE {

mcch-RepetitionPeriodAndOffset-r17   MCCH-RepetitionPeriodAndOffset-r17,

mcch-WindowStartSlot-r17             INTEGER (0..79),

mcch-WindowDuration-r17              ENUMERATED {sl2, sl4, sl8, sl10, sl20, sl40,sl80, sl160}     OPTIONAL, -- Need S

mcch-ModificationPeriod-r17          ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, r2048, rf4096, rf8192, rf16384, rf32768, rf65536}

}

MCCH-RepetitionPeriodAndOffset-r17 ::= CHOICE {

rf1-r17                                INTEGER(0),

rf2-r17                                INTEGER(0..1),

rf4-r17                                INTEGER(0..3),

rf8-r17                                INTEGER(0..7),

rf16-r17                               INTEGER(0..15),

rf32-r17                               INTEGER(0..31),

rf64-r17                               INTEGER(0..63),

rf128-r17                              INTEGER(0..127),

rf256-r17                              INTEGER(0..255)

}

-- TAG-SIB20-STOP

-- ASN1STOP

SIB20 field descriptions

cfr-ConfigMCCH-MTCH Common frequency resource used for MCCH and MTCH reception. If the field is absent, the CFR for broadcast has the same location and size as CORESET0 and PDSCH configuration of MCCH is the same as PDSCH configuration provided in initialDownlinkBWP in SIB1.

cfr-ConfigMCCH-MTCH-RedCap Common frequency resource used for MCCH and MTCH reception for RedCap UEs. If the field is absent, the RedCap UE can use cfr-ConfigMCCH-MTCH if the UE supports the configured bandwidth.

mcch-WindowDuration Indicates, starting from the slot indicated by mcch-WindowStartSlot, the duration in slots during which MCCH may be scheduled. Absence of this field means that MCCH is only scheduled in the slot indicated by mcch-WindowStartSlot. The network always configures mcch-WindowDuration to be shorter or equal to the length of MCCH repetition period.

mcch-ModificationPeriod Defines periodically appearing boundaries, i.e. radio frames for which SFN mod mcch-ModificationPeriod = 0. The contents of different transmissions of MCCH information can only be different if there is at least one such boundary in-between them. Value rf2 corresponds to two radio frames, value rf4 corresponds to four radio frames and so on.

mcch-RepetitionPeriodAndOffset Defines the length and the offset of the MCCH repetition period. rf1 corresponds to a repetition period length of one radio frame, rf2 corresponds to a repetition period length of two radio frames and so on. The corresponding integer value indicates the offset of the repetition period in the number of radio frames. MCCH is scheduled in the MCCH transmission window starting from each radio frame for which: SFN mod repetition period length = offset of the repetition period.

mcch-WindowStartSlot Indicates the slot in which MCCH transmission window starts.

| Conditional presence | Explanation |
| --- | --- |
| CFR-RedCap | The field is optionally present, Need R, if the configured bandwidth in cfr-ConfigMCCH-MTCH exceeds the RedCap UE capability. It is absent otherwise. |