3GPP TSG RAN WG2 Meeting #121                                                        	R2-2301107

Athens, Greece, 27th Feb – 3rd March 2023

Agenda item:	8.16.2

Source:	Samsung

Title:	Data Collection for AI/ML

Document for:	Discussion and Decision

# 1. Introduction

In RAN2#120 meeting, RAN2 discussed the Rel-18 study on AI/ML for NR air interface [1] and concluded the following assumptions and agreements on the AI/ML methods [2]:

### 8.16.2 AIML methods R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions

Moreover, RAN2 briefly discussed whether to consider a new framework and/or network entities for data collection for AI/ML model training and monitoring. Eventually, this discussion was moved to the following email:

[Post120][054][AIML18] Data Collection (Ericsson / vivo)

Scope: Long email discussion for next meeting, on data collection (focus on monitoring and training), on to what extent existing methods can be useful including also identifying these existing methods and their potential extensions

Intended outcome: Report

In this contribution, we discuss the proposals of the above email [Post120][054][AIML18] Data Collection [3].

# 2. Discussion

In RAN2#120 meeting, RAN2 briefly discussed data collection for AI/ML and whether existing data collection methods can be used as a baseline in this case or there is a need to introduce new framework and network entities for collecting required data for AI/ML. For example, data collection for purpose of training or monitoring of a AI/ML model (or models).

RAN2 discussion on data collection was moved to the email [Post120][054][AIML18] Data Collection [3]. In the following we analyse questions and concluded proposals provided in [3].

Q1) Based on the above: Do you agree that RAN2 should start discussing the solutions and specification impact analysis centered around UE measurements/reporting and data collection for gNB/LMF-sided models? Q2) Would it be possible to agree on the following: RAN2 assumes that for gNB/LMF-sided AIML models, the data collection information is terminated in the gNB/LMF? RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity. Study RAN2 implications of data collection for all concerning LCM purpose, e.g., model training/monitoring/selection/update/inference/etc. RAN2 to separately analyse the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted.

In general, the above proposals 1, 2, and 3 can be supported, considering that at this stage, RAN2 aim is to confirm the scope of discussion on data collection requirements and solutions for model training, performance monitoring, and other LCM procedures, for both the UE-side and the Network-side models. However, as a next stage, RAN2 still needs to address the following remaining open points:

- Whether existing data collection methods and network entities can address the identified data collection requirements, or RAN2 would need to introduce new methods and entities for AI/ML.

- Whether RAN2 should define where the data collection information is terminated for gNB/LMF.

Observation 1: RAN2 to discuss whether to introduce new methods and network entities for data collection for LCM procedures.

Observation 2: RAN2 to discuss whether there is a need to define where the data collection information is terminated for gNB/LMF.

Proposal 1: RAN2 to start discussion on whether to introduce new methods and network entities specific for data collection process related to LCM procedures.

Q3) Do companies agree to start by focusing on data collection requirements related to: a) content of the data, b) data size, c) latency, periodicity, or “efficiency”, d) configuration-related requirements? Q4) Companies are invited to comment on the above requirements, and whether these could be considered for designing data collection solutions for NW-sided beam management use cases. Q5) Companies are invited to comment on the above requirements and whether these could be considered for designing data collection solutions for NG-RAN node assisted positioning with gNB-sided model? Q6) Companies are invited to comment on the above requirements and whether these could be considered for designing data collection solutions for UE-assisted/LMF-based positioning with LMF-sided model? Q7) Companies are invited to comment on the above requirements and whether these could be considered for designing data collection solutions for spatial-frequency domain CSI compression? Wait for RAN1 requirements before discussing specific data collection solutions for use cases and for the related (LCM) procedures. In the meantime, RAN2 can summarize the implementation of existing frameworks while focusing on different performance metrics. When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy.

In general, the above proposals 4 and 5 can be supported, since at this stage, RAN2 can start with a high-level discussion of the different categories/requirements of data collection methods for AI/ML, while postpone any discussion on details of data collection solutions pending RAN1 input.

Observation 3: RAN2 may start a high level discussion on different requirements of data collection methods, while postponing discussion on solutions’ details pending RAN1 input.

However, one aspect which was missed in the email conclusion is companies input on the need to separate the discussion on data collection for model training and model monitoring, and possibly data collection related to other LCM procedures. For example, in our understanding, data collection could be separated into:

- Case 1: Real-time purposes, e.g., model monitoring, inference, selection, switching, etc.

- Case 2: Non-real-time purposes, e.g., model training, update

Observation 4: RAN2 to consider separating the discussion on data collection methods for different LCM procedures.

Q8) Do you agree to consider the above candidate frameworks as starting points to be considered for data collection in this SI? Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed. Upon receiving specific (RAN1) requirements, RAN2 to decide whether the existing frameworks can be reused/extended, or whether a new framework is required.

In general, proposals 6 and 7 can be supported, since RAN2 would need to decide whether existing data collection frameworks and methods are sufficient for data collection for LCM purposes. However, regarding proposal 7, RAN2 may start, at least from a high-level, to discuss whether to introduce new methods and network entities for data collection for different LCM procedures.

Observation 5: RAN2 to start high level discussion on the need for new data collection methods and network entities in relation to different LCM procedures.

Q9) Is there any aspect(s) missing in this part that should be addressed by RAN2? Do companies have further views concerning the data collection mechanism(s) that should be used? Proposal 5 When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy.

Regarding the issue of security/privacy that was raised by some companies in the email discussion, we think that RAN2 may discuss any data collection requirements related preserving user privacy/security.

Q10) What approach do you think RAN2 should adopt concerning the collaboration with RAN1? Do you think RAN2 should ask for RAN1 input in particular areas? Do you think RAN2 should simply keep progressing and inform of concerning agreements to RAN1 when necessary? Should another or no different approach as the one followed now should be taken? For data collection, RAN2 will simply keep progressing and will inform of concerning agreements to RAN1 when necessary.

We support proposal 8, considering it is aligned with agreement in RAN#98 in [4]:

Proposal 3: Leave the work arrangement about what to be discussed or what to be prioritized to WGs.

# 3. Conclusion

In this contribution, we discussed the proposals of the email discussion [Post120][054][AIML18] Data Collection [3]. The following are the observations and proposals in this document:

Observation 1: RAN2 to discuss whether to introduce new methods and network entities for data collection for LCM procedures.

Observation 2: RAN2 to discuss whether there is a need to define where the data collection information is terminated for gNB/LMF.

Observation 3: RAN2 may start a high level discussion on different requirements of data collection methods, while postponing discussion on solutions’ details pending RAN1 input.

Observation 4: RAN2 to consider separating the discussion on data collection methods for different LCM procedures.

Observation 5: RAN2 to start high level discussion on the need for new data collection methods and network entities in relation to different LCM procedures.

Proposal 1: RAN2 to start discussion on whether to introduce new methods and network entities specific for data collection process related to LCM procedures.

Proposal 2: RAN2 to support proposals in [Post120][054][AIML18] Data Collection [3].

# 4. Reference

RP-221348 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

RAN2#120 Chairman report

R2-2301440 Outcome of [Post120][054][AIML18] Data Collection (Ericsson / vivo)

RP-223507 Moderator’s summary for discussion [98e-13-R18-AIML-Air].

# 5. Annex A ([Post120][054][AIML18] Data Collection [3])

[Post120][054][AIML18] Data Collection (Ericsson / vivo) Scope: Long email discussion for next meeting, on data collection (focus on monitoring and training), on to what extent existing methods can be useful including also identifying these existing methods and their potential extensions Intended outcome: Report Deadline: Long

3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity.

Proposal 2	Study RAN2 implications of data collection for all concerning LCM purpose, e.g., model training/monitoring/selection/update/inference/etc.

Proposal 3	RAN2 to separately analyse the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted.

Proposal 4	Wait for RAN1 requirements before discussing specific data collection solutions for use cases and for the related (LCM) procedures. In the meantime, RAN2 can summarize the implementation of existing frameworks while focusing on different performance metrics.

Proposal 5	When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy.

Proposal 6	Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed.

Proposal 7	Upon receiving specific (RAN1) requirements, RAN2 to decide whether the existing frameworks can be reused/extended, or whether a new framework is required.

Proposal 8	For data collection, RAN2 will simply keep progressing and will inform of concerning agreements to RAN1 when necessary.