3GPP TSG-RAN WG2 Meeting #121-bis-e	R2-2302633

# Online Apr 17th – 26th  2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0427 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the descriptions in Positioning methods IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The wrong IE name is used within the field description “additionalInformation”. - There lacks field description on the error cause “incorrectDataValue”. - The wrong IE name “deltaSFN” is used. - Typo in the field description multiPrbNprs. - The RSRP or RSRQ measurement results for E-CID positioning method reported by UE is a mapping value, but not the real value, and the mapping table is defined in the TS36.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 6.4.2 - Correct the IE name. - In clause 6.4.2 - Adding description for the error cause code “incorrectDataValue”. - In clause 6.5.1.5, 6.5.1.6, 6.5.1.7, 6.5.1.5, 6.5.5.2 - Correct the IE name. - In clause 6.5.1.7 - Correct the typo. - In clause 6.5.3.2 E CID Location Information Elements - Add “Measurement report mapping is according to TS 36.133 [18].” in the field description of IE rsrp-Result and rsrq-Result. Impact analysis Architecture options SA, NSA Impacted functionality: Positioning Inter-operability: If only the network is implemented according to the CR and the UE is not, no interoperability problems are foreseen. If only the UE is implemented according to the CR and the network is not, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may misundertand that the reporing rsrp and rsrq are real values, and other mistakes still present in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2, 6.5.1.5, 6.5.1.6, 6.5.1.7, 6.5.3.2, 6.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First of change

6.4.2	Common Positioning

#### –	CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

scheduledLocationTime-r17

ScheduledLocationTime-r17	OPTIONAL,	-- Need ON

targetIntegrityRisk-r17

TargetIntegrityRisk-r17		OPTIONAL	-- Need ON

]]

}

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...

}

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

TriggeredReportingCriteria ::=		SEQUENCE {

cellChange							BOOLEAN,

reportingDuration					ReportingDuration,

...

}

ReportingDuration ::=				INTEGER (0..255)

AdditionalInformation ::= ENUMERATED {

onlyReturnInformationRequested,

mayReturnAditionalInformation,

...

}

QoS ::= SEQUENCE {

horizontalAccuracy			HorizontalAccuracy		OPTIONAL,	-- Need ON

verticalCoordinateRequest	BOOLEAN,

verticalAccuracy			VerticalAccuracy		OPTIONAL,	-- Need ON

responseTime				ResponseTime			OPTIONAL,	-- Need ON

velocityRequest				BOOLEAN,

...,

[[	responseTimeNB-r14		ResponseTimeNB-r14		OPTIONAL	-- Need ON

]],

[[	horizontalAccuracyExt-r15	HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalAccuracyExt-r15		VerticalAccuracyExt-r15			OPTIONAL	-- Need ON

]]

}

HorizontalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

VerticalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

HorizontalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

VerticalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

ResponseTime ::= SEQUENCE {

time								INTEGER (1..128),

...,

[[	responseTimeEarlyFix-r12		INTEGER (1..128)		OPTIONAL		-- Need ON

]],

[[	unit-r15				ENUMERATED { ten-seconds, ... , ten-milli-seconds-v1700 }

OPTIONAL		-- Need ON

]]

}

ResponseTimeNB-r14 ::= SEQUENCE {

timeNB-r14							INTEGER (1..512),

responseTimeEarlyFixNB-r14			INTEGER (1..512)		OPTIONAL,		-- Need ON

...,

[[	unitNB-r15				ENUMERATED { ten-seconds, ... }	OPTIONAL		-- Need ON

]]

}

Environment ::= ENUMERATED {

badArea,

notBadArea,

mixedArea,

...

}

MessageSizeLimitNB-r14 ::= SEQUENCE {

measurementLimit-r14				INTEGER (1..512)		OPTIONAL,		-- Need ON

...

}

ScheduledLocationTime-r17 ::= SEQUENCE {

utcTime-r17			UTCTime											OPTIONAL,	-- Need ON

gnssTime-r17		SEQUENCE {

gnss-TOD-msec-r17		INTEGER (0..3599999),

gnss-TimeID-r17			GNSS-ID

}												OPTIONAL,	-- Need ON

networkTime-r17		CHOICE {

e-utraTime-r17			SEQUENCE {

lte-PhysCellId-r17			INTEGER (0..503),

lte-ArfcnEUTRA-r17			ARFCN-ValueEUTRA,

lte-CellGlobalId-r17		CellGlobalIdEUTRA-AndUTRA

OPTIONAL,	-- Need ON

lte-SystemFrameNumber-r17	INTEGER (0..1023)

},

nrTime-r17				SEQUENCE {

nr-PhysCellID-r17		NR-PhysCellID-r16,

nr-ARFCN-r17			ARFCN-ValueNR-r15,

nr-CellGlobalID-r17		NCGI-r15	OPTIONAL,		-- Need ON

nr-SFN-r17				INTEGER (0..1023),

nr-Slot-r17				CHOICE {

scs15-r17				INTEGER (0..9),

scs30-r17				INTEGER (0..19),

scs60-r17				INTEGER (0..39),

scs120-r17				INTEGER (0..79)

}										OPTIONAL	-- Need ON

},

...

}												OPTIONAL,	-- Need ON

relativeTime-r17	INTEGER (1..1024)								OPTIONAL	-- Need ON

}

TargetIntegrityRisk-r17 ::=		INTEGER (10..90)

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ECID | The field is optionally present, need ON, if E-CID or NR E-CID is requested. Otherwise it is not present. |

CommonIEsRequestLocationInformation field descriptions

locationInformationType This IE indicates whether the server requires a location estimate or measurements. For 'locationEstimateRequired', the target device shall return a location estimate if possible, or indicate a location error if not possible. For 'locationMeasurementsRequired', the target device shall return measurements if possible, or indicate a location error if not possible. For 'locationEstimatePreferred', the target device shall return a location estimate if possible, but may also or instead return measurements for any requested position methods for which a location estimate is not possible. For 'locationMeasurementsPreferred', the target device shall return location measurements if possible, but may also or instead return a location estimate for any requested position methods for which return of location measurements is not possible.

triggeredReporting This IE indicates that triggered reporting is requested and comprises the following subfields: - cellChange: If this field is set to TRUE, the target device provides requested location information each time the primary cell has changed. - reportingDuration: Maximum duration of triggered reporting in seconds. A value of zero is interpreted to mean an unlimited (i.e. "infinite") duration. The target device should continue triggered reporting for the reportingDuration or until an LPP Abort or LPP Error message is received. The triggeredReporting field should not be included by the location server and shall be ignored by the target device if the periodicalReporting IE or responseTime IE or responseTimeNB IE is included in CommonIEsRequestLocationInformation.

periodicalReporting This IE indicates that periodic reporting is requested and comprises the following subfields: - reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 1, 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is 'infinite/indefinite', the target device shou-ld continue periodic reporting until an LPP Abort message is received. The value 'ra1' shall not be used by a sender. - reportingInterval indicates the interval between location information reports and the response time requirement for the first location information report. Enumerated values ri0-25, ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64 correspond to reporting intervals of 1, 2, 4, 8, 10, 16, 20, 32, and 64 seconds, respectively. Measurement reports containing no measurements or no location estimate are required when a reportingInterval expires before a target device is able to obtain new measurements or obtain a new location estimate. The value 'noPeriodicalReporting' shall not be used by a sender.

additionalInformation This IE indicates whether a target device is allowed to return additional information to that requested. If this IE indicates 'onlyReturnInformationRequested' then the target device shall not return any additional information to that requested by the server. If this IE indicates 'mayReturnAditionalInformation' then the target device may return additional information to that requested by the server. If a location estimate is returned, any additional information is restricted to that associated with a location estimate (e.g. might include velocity if velocity was not requested but cannot include measurements). If measurements are returned, any additional information is restricted to additional measurements (e.g. might include E-CID measurements if A-GNSS measurements were requested but not E-CID measurements).

qos This IE indicates the quality of service and comprises a number of sub-fields. In the case of measurements, some of the sub-fields apply to the location estimate that could be obtained by the server from the measurements provided by the target device assuming that the measurements are the only sources of error. Fields are as follows: - horizontalAccuracy indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracy' corresponds to the encoded uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - verticalCoordinateRequest indicates whether a vertical coordinate is required (TRUE) or not (FALSE) - verticalAccuracy indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracy' corresponds to the encoded uncertainty altitude as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - responseTime - time indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. If the periodicalReporting IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - responseTimeEarlyFix indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFix IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the time IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFix IE. A server should set the responseTimeEarlyFix IE to a value less than that for the time IE. A target shall ignore the responseTimeEarlyFix IE if its value is not less than that for the time IE. - unit indicates the unit of the time and responseTimeEarlyFix fields. Enumerated value 'ten-seconds' corresponds to a resolution of 10 seconds. Enumerated value 'ten-milli-seconds' corresponds to a resolution of 0.01 seconds. If this field is absent, the unit/resolution is 1 second. Enumerated value 'ten-milli-seconds' is only applicable for NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning. If the enumerated value 'ten-milli-seconds' is included for methods others than NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning the target device shall ignore the unit field. - velocityRequest indicates whether velocity (or measurements related to velocity) is requested (TRUE) or not (FALSE). - responseTimeNB If the periodicalReporting IE or responseTime IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - timeNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. - responseTimeEarlyFixNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFixNB IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the timeNB IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFixNB IE. A server should set the responseTimeEarlyFixNB IE to a value less than that for the timeNB IE. A target shall ignore the responseTimeEarlyFixNB IE if its value is not less than that for the timeNB IE. - unitNB indicates the unit of the timeNB and responseTimeEarlyFixNB fields. Enumerated value 'ten-second' corresponds to a resolution of 10 seconds. If this field is absent, the unit/resolution is 1 second. - horizontalAccuracyExt indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the horizontalAccuracy field is included in QoS. - verticalAccuracyExt indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the verticalAccuracy field is included in QoS. All QoS requirements shall be obtained by the target device to the degree possible but it is permitted to return a response that does not fulfill all QoS requirements if some were not attainable. The single exception is time and timeNB which shall always be fulfilled – even if that means not fulfilling other QoS requirements. A target device supporting NB-IoT access shall support the responseTimeNB IE. A target device supporting HA GNSS shall support the HorizontalAccuracyExt, VerticalAccuracyEx, and unit fields with enumerated value 'ten-seconds'. A target device supporting NB-IoT access and HA GNSS shall support the unitNB field.

environment This field provides the target device with information about expected multipath and non line of sight (NLOS) in the current area. The following values are defined: - badArea: possibly heavy multipath and NLOS conditions (e.g. bad urban or urban). - notBadArea: no or light multipath and usually LOS conditions (e.g. suburban or rural). - mixedArea: environment that is mixed or not defined. If this field is absent, a default value of 'mixedArea' applies.

locationCoordinateTypes This field provides a list of the types of location estimate that the target device may return when a location estimate is obtained by the target.

velocityTypes This fields provides a list of the types of velocity estimate that the target device may return when a velocity estimate is obtained by the target.

messageSizeLimitNB This field provides an octet limit on the amount of location information a target device can return. - measurementLimit indicates the maximum amount of location information the target device should return in response to the RequestLocationInformation message received from the location server. The limit applies to the overall size of the LPP message at LPP level (LPP Provide Location Information), and is specified in steps of 100 octets. The message size limit is then given by the value provided in measurementLimit times 100 octets.

segmentationInfo This field indicates whether this RequestLocationInformation message is one of many segments, as specified in clause 4.3.5

scheduledLocationTime This field indicates that the target device is requested to obtain location measurements or location estimate valid at the scheduledLocationTime T and comprises the following subfields: - utcTime provides T in UTC in the form of YYMMDDhhmmssZ. - gnssTime provides T in GNSS system time of the GNSS indicated by gnss-TimeID. - gnss-TOD-msec specifies the GNSS TOD in 1-milli-second resolution rounded down to the nearest millisecond unit. - networkTime provides T in E-UTRA or NR network time. - lte-PhysCellId, lte-ArfcnEUTRA, lte-CellGlobalId identifies the reference cell (E-UTRA) that is used for the network time. - lte-systemFrameNumber specifies the system frame number in E-UTRA. - nr-PhysCellID, nr-ARFCN , nr-CellGlobalID identifies the reference cell (NR) that is used for the network time. - nr-SFN specifies the system frame number in NR. - nr-Slot specifies the slot number in NR for the indicated subcarrier spacing (SCS). The total NR network time is given by nr-SFN + nr-Slot. - relativeTime provides T in seconds from current time, where current time is defined as the time the CommonIEsRequestLocationInformation was received. NOTE 1: A location estimate returned to an LCS Client, AF or UE for a scheduled location time can be treated by the LCS Client, AF or UE as an estimate of the location of the UE at the scheduled location time (see TS 23.273 [42]). NOTE 2: If this field is present, at least one of utcTime, gnssTime, networkTime, or relativeTime shall be present.

targetIntegrityRisk This field indicates the TIR for which the PL is requested. The TIR is calculated by P=10-0.1n [hour-1] where n is the value of targetIntegrityRisk and the range is 10-1 to 10-9 per hour.

Next change

–	CommonIEsError

The CommonIEsError carries common IEs for an Error LPP message Type.

-- ASN1START

CommonIEsError ::= SEQUENCE {

errorCause		ENUMERATED {

undefined,

lppMessageHeaderError,

lppMessageBodyError,

epduError,

incorrectDataValue,

...,

lppSegmentationError-v1450

}

}

-- ASN1STOP

CommonIEsError field descriptions

errorCause This IE defines the cause for an error. 'lppMessageHeaderError', 'lppMessageBodyError' and 'epduError' is used if a receiver is able to detect a coding error in the LPP header (i.e., in the common fields), LPP message body or in an EPDU, respectively. ‘incorrectDataValue’ is used if a receiver is not able to detect a correct LPP message Type. 'lppSegmentationError' is used if a receiver detects an error in LPP message segmentation.

Next change

#### 6.5.1.5	OTDOA Location Information Elements

#### –	OTDOA-SignalMeasurementInformation

The IE OTDOA-SignalMeasurementInformation is used by the target device to provide RSTD measurements to the location server. The RSTD measurements are provided for a neighbour cell and the RSTD reference cell, both of which are provided in the IE OTDOA-ProvideAssistanceData. The RSTD reference cell may or may not be the same as the assistance data reference cell provided in OTDOA-ReferenceCellInfo or OTDOA-ReferenceCellInfoNB. If the target device stops reporting inter-frequency RSTD measurements, where the inter-frequency RSTD measurement is an OTDOA RSTD measurement with at least one cell on a frequency different from the serving cell frequency, the LPP layer shall inform lower layers that inter-frequency RSTD measurements are stopped.

NOTE 1:	If there are more than 24 NeighbourMeasurementElement to be sent, the target device may send them in multiple ProvideLocationInformation messages, as described under clause 5.3.

NOTE 2:	If NPRS/PRS antenna ports are quasi co-located, the target device provides a single RSTD measurement for the quasi co-located antenna ports of NPRS/PRS.

-- ASN1START

OTDOA-SignalMeasurementInformation ::= SEQUENCE {

systemFrameNumber		BIT STRING (SIZE (10)),

physCellIdRef			INTEGER (0..503),

cellGlobalIdRef			ECGI					OPTIONAL,

earfcnRef				ARFCN-ValueEUTRA		OPTIONAL,		-- Cond NotSameAsRef0

referenceQuality		OTDOA-MeasQuality		OPTIONAL,

neighbourMeasurementList	NeighbourMeasurementList,

...,

[[ earfcnRef-v9a0		ARFCN-ValueEUTRA-v9a0	OPTIONAL		-- Cond NotSameAsRef1

]],

[[ tpIdRef-r14			INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer0

prsIdRef-r14		INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer1

additionalPathsRef-r14

AdditionalPathList-r14	OPTIONAL,

nprsIdRef-r14		INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer2

carrierFreqOffsetNB-Ref-r14

CarrierFreqOffsetNB-r14	OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14		BIT STRING (SIZE (10))	OPTIONAL		-- Cond H-SFN

]],

[[

motionTimeSource-r15		MotionTimeSource-r15		OPTIONAL

]]

}

NeighbourMeasurementList ::= SEQUENCE (SIZE(1..24)) OF NeighbourMeasurementElement

NeighbourMeasurementElement ::= SEQUENCE {

physCellIdNeighbour		INTEGER (0..503),

cellGlobalIdNeighbour	ECGI					OPTIONAL,

earfcnNeighbour			ARFCN-ValueEUTRA		OPTIONAL,		-- Cond NotSameAsRef2

rstd					INTEGER (0..12711),

rstd-Quality			OTDOA-MeasQuality,

...,

[[ earfcnNeighbour-v9a0	ARFCN-ValueEUTRA-v9a0	OPTIONAL		-- Cond NotSameAsRef3

]],

[[ tpIdNeighbour-r14	INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer0

prsIdNeighbour-r14	INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer1

delta-rstd-r14		INTEGER (0..5)			OPTIONAL,

additionalPathsNeighbour-r14

AdditionalPathList-r14	OPTIONAL,

nprsIdNeighbour-r14	INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer2

carrierFreqOffsetNB-Neighbour-r14

CarrierFreqOffsetNB-r14	OPTIONAL		-- Cond NB-IoT

]],

[[

delta-SFN-r15			INTEGER (-8192..8191)	OPTIONAL

]]

}

AdditionalPathList-r14 ::= SEQUENCE (SIZE(1..maxPaths-r14)) OF AdditionalPath-r14

maxPaths-r14	INTEGER ::= 2

MotionTimeSource-r15 ::= SEQUENCE {

timeSource-r15				ENUMERATED {servingCell, referenceCell, gnss, mixed,

other, none, ...}

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsRef0 | The field is absent if the corresponding earfcnRef-v9a0 is present. Otherwise, the target device shall include this field if the EARFCN of the RSTD reference cell is not the same as the EARFCN of the assistance data reference cell provided in the OTDOA assistance data. |
| NotSameAsRef1 | The field is absent if the corresponding earfcnRef is present. Otherwise, the target device shall include this field if the EARFCN of the RSTD reference cell is not the same as the EARFCN of the assistance data reference cell provided in the OTDOA assistance data. |
| NotSameAsRef2 | The field is absent if the corresponding earfcnNeighbour-v9a0 is present. Otherwise, the target device shall include this field if the EARFCN of this neighbour cell is not the same as the earfcnRef for the RSTD reference cell. |
| NotSameAsRef3 | The field is absent if the corresponding earfcnNeighbour is present. Otherwise, the target device shall include this field if the EARFCN of this neighbour cell is not the same as the earfcnRef for the RSTD reference cell. |
| ProvidedByServer0 | The target device shall include this field if a tpId for this transmission point is included in the OTDOA-ProvideAssistanceData. Otherwise the field is absent. |
| ProvidedByServer1 | The target device shall include this field if a prsID for this transmission point is included in the OTDOA-ProvideAssistanceData. Otherwise the field is absent. |
| ProvidedByServer2 | The target device shall include this field if an nprsID for this cell is included in the OTDOA-ProvideAssistanceData and if this cell is a NB-IoT only cell (without associated LTE PRS cell). Otherwise the field is absent. |
| NB-IoT | The target device shall include this field if the cell is a NB-IoT only cell (without associated LTE PRS cell). Otherwise the field is absent. |
| H-SFN | The target device shall include this field if it was able to determine a hyper SFN of the RSTD reference cell. |

OTDOA-SignalMeasurementInformation field descriptions

systemFrameNumber If the delta-SFN and motionTimeSource fields are not present, this field specifies the SFN of the RSTD reference cell containing the starting subframe of the PRS or NPRS positioning occasion if PRS or NPRS are available on the RSTD reference cell, or subframe of the CRS for RSTD measurements if PRS and NPRS are not available on the RSTD reference cell during which the most recent neighbour cell RSTD measurement was performed. In the case of more than a single PRS configuration on the RSTD reference cell, the first PRS configuration is referenced. If the delta-SFN and motionTimeSource fields are present, this field specifies the SFN of the RSTD reference cell when the TOA measurement for the RSTD reference cell has been made.

physCellIdRef This field specifies the physical cell identity of the RSTD reference cell.

cellGlobalIdRef This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the RSTD reference cell. The target shall provide this IE if it knows the ECGI of the RSTD reference cell.

earfcnRef This field specifies the EARFCN of the RSTD reference cell.

referenceQuality This field specifies the target device′s best estimate of the quality of the TOA measurement from the RSTD reference cell, TSubframeRxRef, where TSubframeRxRef is the time of arrival of the signal from the RSTD reference cell. When delta-SFN and motionTimeSource are both included, the target device shall not include measurement errors caused by motion of the target device in referenceQuality (e.g. the target device may assume the target device was stationary during OTDOA measurements).

neighbourMeasurementList This list contains the measured RSTD values for neighbour cells together with the RSTD reference cell, along with quality for each measurement.

tpIdRef This field specifies the transmission point ID of the RSTD reference cell.

prsIdRef This field specifies the PRS-ID of the first PRS configuration of the RSTD reference cell.

additionalPathsRef This field specifies one or more additional detected path timing values for the RSTD reference cell, relative to the path timing used for determining the rstd value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nprsIdRef This field specifies the NPRS-ID of the RSTD reference cell.

carrierFreqOffsetNB-Ref This field specifies the offset of the NB-IoT channel number to EARFCN given by earfcnRef as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper SFN as defined in TS 36.331 [12] of the RSTD reference cell for the systemFrameNumber.

motionTimeSource This field provides reference information concerning the movement of the target device and comprises the following subfields: - timeSource specifies the external time source to which UE time was locked during the OTDOA measurements. Enumerated value "mixed" indicates that UE time was locked to more than one external time source during OTDOA measurements (e.g. is applicable to a change in serving cell when the serving cell was used as a time source). The value "other" indicates some other external time source. The value "none" indicates that UE time was not locked to an external time source. If this field is present, the target device shall also provide the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

physCellIdNeighbour This field specifies the physical cell identity of the neighbour cell for which the RSTDs are provided.

cellGlobalIdNeighbour This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the neighbour cell for which the RSTDs are provided. The target device shall provide this IE if it was able to determine the ECGI of the neighbour cell at the time of measurement.

earfcnNeighbour This field specifies the EARFCN of the neighbour cell used for the RSTD measurements.

rstd This field specifies the relative timing difference between this neighbour cell and the RSTD reference cell, as defined in TS 36.214 [17]. Mapping of the measured quantity is defined as in TS 36.133 [18] clause 9.1.10.3.

rstd-Quality This field specifies the target device′s best estimate of the quality of the measured rstd. When delta-SFN and motionTimeSource both included, the target device shall not include measurement errors caused by motion of the target device in rstd-Quality (e.g. the target device may assume the target device was stationary during OTDOA measurements).

tpIdNeighbour This field specifies the transmission point ID for the neighbour cell for which the RSTDs are provided.

prsIdNeighbour This field specifies the PRS-ID of the first PRS configuration of the neighbour cell for which the RSTDs are provided.

delta-rstd This field specifies the higher-resolution RSTD RSTD as defined in TS 36.133 [18] clause 9.1.10.4. Mapping of the measured quantity is defined as in TS 36.133 [18] clause 9.1.10.4.

additionalPathsNeighbour This field specifies one or more additional detected path timing values for the neighbour cell, relative to the path timing used for determining the rstd value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nprsIdNeighbour This field specifies the NPRS-ID of the neighbour cell for which the RSTDs are provided.

carrierFreqOffsetNB-Neighbour This field specifies the offset of the NB-IoT channel number to EARFCN given by earfcnNeighbour as defined in TS 36.101 [21].

delta-SFN This field provides information concerning the movement of the target device: Together with systemFrameNumber specifies the measurementSFN of the RSTD reference cell when the TOA measurement for this neighbour cell has been made for determining the rstd. The measurementSFN is given by systemFrameNumber + delta-SFN. (The actual SFN is the measurementSFN modulo 1024.). The measurementSFN is used in IE Sensor-MotionInformation to provide movement information corresponding to the TOA measurement time. If this field is present, the target device shall also provide the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

Next change

#### 6.5.1.6	OTDOA Location Information Request

#### –	OTDOA-RequestLocationInformation

The IE OTDOA-RequestLocationInformation is used by the location server to request OTDOA location measurements from a target device. Details of the required measurements (e.g. details of assistance data reference cell and neighbour cells) are conveyed in the OTDOA-ProvideAssistanceData IE in a separate Provide Assistance Data message.

-- ASN1START

OTDOA-RequestLocationInformation ::= SEQUENCE {

assistanceAvailability		BOOLEAN,

...,

[[

multipathRSTD-r14		ENUMERATED { requested }	OPTIONAL,		-- Need ON

maxNoOfRSTDmeas-r14		INTEGER (1..32)				OPTIONAL		-- Need ON

]],

[[

motionMeasurements-r15	ENUMERATED { requested }	OPTIONAL		-- Need ON

]]

}

-- ASN1STOP

OTDOA-RequestLocationInformation field descriptions

assistanceAvailability This field indicates whether the target device may request additional OTDOA assistance data from the server. TRUE means allowed and FALSE means not allowed.

multipathRSTD This field, if present, indicates that the target device is requested to report additional detected path timing information per RSTD reference and neighbour cell.

maxNoOfRSTDmeas This field, if present, indicates the maximum number of NeighbourMeasurementElement fields (i.e., RSTD measurements) the target device can provide in OTDOA-SignalMeasurementInformation.

motionMeasurements This field, if present, indicates that the target device is requested to report the motion measurements (delta-SFN and motionTimeSource) in OTDOA-SignalMeasurementInformation as well as the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

Next change

#### 6.5.1.7	OTDOA Capability Information

#### –	OTDOA-ProvideCapabilities

The IE OTDOA-ProvideCapabilities is used by the target device to indicate its capability to support OTDOA and to provide its OTDOA positioning capabilities to the location server.

-- ASN1START

OTDOA-ProvideCapabilities ::= SEQUENCE {

otdoa-Mode		BIT STRING {	ue-assisted				(0),

ue-assisted-NB-r14		(1),

ue-assisted-NB-TDD-r15	(2) } (SIZE (1..8)),

...,

supportedBandListEUTRA		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA		OPTIONAL,

supportedBandListEUTRA-v9a0	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v9a0

OPTIONAL,

interFreqRSTDmeasurement-r10		ENUMERATED { supported }						OPTIONAL,

additionalNeighbourCellInfoList-r10	ENUMERATED { supported }						OPTIONAL,

prs-id-r14							ENUMERATED { supported }						OPTIONAL,

tp-separation-via-muting-r14		ENUMERATED { supported }						OPTIONAL,

additional-prs-config-r14			ENUMERATED { supported }						OPTIONAL,

prs-based-tbs-r14					ENUMERATED { supported }						OPTIONAL,

additionalPathsReport-r14			ENUMERATED { supported }						OPTIONAL,

densePrsConfig-r14					ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsBandwidth-r14		ENUMERATED { n6, n15, n25, n50, n75, n100, ...}	OPTIONAL,

prsOccGroup-r14						ENUMERATED { supported }						OPTIONAL,

prsFrequencyHopping-r14				ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsConfigs-r14			ENUMERATED { c2, c3 }							OPTIONAL,

periodicalReporting-r14				ENUMERATED { supported }						OPTIONAL,

multiPrbNprs-r14					ENUMERATED { supported }						OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }							OPTIONAL,

numberOfRXantennas-r14				ENUMERATED { rx1, ... }							OPTIONAL,

motionMeasurements-r15				ENUMERATED { supported }						OPTIONAL,

interRAT-RSTDmeasurement-r15		ENUMERATED { supported }						OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupport-r17			OPTIONAL

}

maxBands INTEGER ::= 64

SupportedBandEUTRA ::= SEQUENCE {

bandEUTRA							INTEGER (1..maxFBI)

}

SupportedBandEUTRA-v9a0 ::=		SEQUENCE {

bandEUTRA-v9a0						INTEGER (maxFBI-Plus1..maxFBI2)		OPTIONAL

}

maxFBI								INTEGER	::=	64	-- Maximum value of frequency band indicator

maxFBI-Plus1						INTEGER ::= 65	-- lowest value extended FBI range

maxFBI2								INTEGER ::= 256	-- highest value extended FBI range

-- ASN1STOP

OTDOA-ProvideCapabilities field descriptions

otdoa-Mode This field specifies the OTDOA mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the particular OTDOA mode is supported; a zero value means not supported. A zero-value in all bit positions in the bit string means OTDOA positioning method is not supported by the target device. ue-assisted: Bit 0 indicates that the target device supports UE-assisted OTDOA and LTE PRS. ue-assisted-NB: Bit 1 indicates that the target device supports UE-assisted OTDOA and NB-IoT NPRS. ue-assisted-NB-TDD: Bit 2 indicates that the target device supports UE-assisted OTDOA and NB-IoT NPRS for TDD.

SupportedBandEUTRA This field specifies the frequency bands for which the target device supports RSTD measurements. One entry corresponding to each supported E-UTRA band as defined in TS 36.101 [21]. In the case the target device includes bandEUTRA-v9a0, the target device shall set the corresponding entry of bandEUTRA (i.e. without suffix) to maxFBI.

interFreqRSTDmeasurement This field, if present, indicates that the target device supports inter-frequency RSTD measurements within and between the frequency bands indicated in SupportedBandEUTRA.

additionalNeighbourCellInfoList This field, if present, indicates that the target device supports up to 3×24 OTDOA-NeighbourCellInfoElement in OTDOA-NeighbourCellInfoList in OTDOA-ProvideAssistanceData without any restriction for the earfcn in each OTDOA-NeighbourCellInfoElement as specified in clause 6.5.1.2.

prs-id This field, if present, indicates that the target device supports PRS generation based on the PRS-ID as specified in TS 36.211 [16] and support for TP-ID in OTDOA-ReferenceCellInfo and OTDOA-NeighbourCellInfoList.

tp-separation-via-muting This field, if present, indicates that the target device supports RSTD measurements for cells which have associated transmission points (e.g., Remote Radio Heads) within the cell coverage and where these associated transmission points have the same physical cell identity as the associated cell, and where these transmission points are identified via a different muting pattern. The field also indicates support for TP-ID in OTDOA-ReferenceCellInfo and OTDOA-NeighbourCellInfoList.

additional-prs-config This field, if present, indicates that the target device supports additional PRS configurations. The additional PRS configuration in PRS-Info IE comprise: - support for prs-ConfigurationIndex > 2399; - support for NPRS values in addition to 1, 2, 4 and 6 (add-numDL-Frames in PRS-Info); - support for muting bit string lengths > 16 bits.

prs-based-tbs This field, if present, indicates that the target device supports RSTD measurements for PRS-only TPs.

additionalPathsReport This field, if present, indicates that the target device supports reporting of timing information for additional detected paths for RSTD reference and each neighbour cell.

densePrsConfig This field, if present, indicates that the target device supports a subset of the additional PRS configurations associated with capability additional-prs-config which comprises: - support for prs-ConfigurationIndex > 2404; - support for NPRS values of 10, 20, 40, 80 and 160 (in addition to 1, 2, 4 and 6). In the case additional-prs-config is present, this field is not present.

maxSupportedPrsBandwidth This field, if present, indicates the maximum PRS bandwidth supported by the target device. Enumerated value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If this field is not present, the target device is assumed to support the PRS bandwidth associated with the target device type, which for LTE devices including Cat-M1/M2 is 100 resource blocks and for NB-IoT devices is 1 resource block.

prsOccGroup This field, if present, indicates that the target device supports PRS occasion groups, which implies that each bit of a configured muting pattern applies per PRS occasion group.

prsFrequencyHopping This field, if present, indicates that the target device supports PRS occasion frequency hopping, as specified in TS 36.211 [16].

maxSupportedPrsConfigs This field, if present, indicates that the target device supports multiple PRS configurations per cell. Enumerated value c2 indicates support for up to 2 configurations; c3 indicates support for up to 3 configurations.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting of RSTD measurements. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

multiPrbNprs This field, if present, indicates that the target device supports NPRS configuration in more than one resource block (i.e., maxCarrier in PRS-Info-NB greater than 1).

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform RSTD measurements.

numberOfRXantennas This field is not applicable to NB-IoT devices. This field, if present, indicates the number of UE downlink receive antennas for RSTD measurements (see TS 36.133 [18]). Enumerated value rx1 indicates a single antenna receiver. If this field is absent, the target device is assumed to support two RX antennas for RSTD measurements.

motionMeasurements This field, if present, indicates that the target device supports reporting of motion measurements (delta-SFN and motionTimeSource) in OTDOA-SignalMeasurementInformation. The presence of this field implies presence of sensor-MotionInformationSup in IE Sensor-ProvideCapabilities.

interRAT-RSTDmeasurement This field, if present, indicates that the target device supports inter-RAT RSTD measurements (TS 38.215 [36]); i.e., E-UTRA RSTD measurements when the target device is served by an NR cell.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

Next change

#### 6.5.3.2	E-CID Location Information Elements

#### –	ECID-SignalMeasurementInformation

The IE ECID-SignalMeasurementInformation is used by the target device to provide various UE-measurements to the location server.

-- ASN1START

ECID-SignalMeasurementInformation ::= SEQUENCE {

primaryCellMeasuredResults	MeasuredResultsElement	OPTIONAL,

measuredResultsList			MeasuredResultsList,

...

}

MeasuredResultsList ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement

MeasuredResultsElement ::= SEQUENCE {

physCellId						INTEGER (0..503),

cellGlobalId					CellGlobalIdEUTRA-AndUTRA	OPTIONAL,

arfcnEUTRA						ARFCN-ValueEUTRA,

systemFrameNumber				BIT STRING (SIZE (10))		OPTIONAL,

rsrp-Result						INTEGER (0..97)				OPTIONAL,

rsrq-Result						INTEGER (0..34)				OPTIONAL,

ue-RxTxTimeDiff					INTEGER (0..4095)			OPTIONAL,

...,

[[	arfcnEUTRA-v9a0				ARFCN-ValueEUTRA-v9a0		OPTIONAL		-- Cond EARFCN-max

]],

[[	nrsrp-Result-r14			INTEGER (0..113)			OPTIONAL,

nrsrq-Result-r14			INTEGER (0..74)				OPTIONAL,

carrierFreqOffsetNB-r14		CarrierFreqOffsetNB-r14		OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14				BIT STRING (SIZE (10))		OPTIONAL

]],

[[

rsrp-Result-v1470			INTEGER (-17..-1)			OPTIONAL,

rsrq-Result-v1470			INTEGER (-30..46)			OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding arfcnEUTRA (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| NB-IoT | The field is mandatory present if the measured cell is a NB-IoT cell. Otherwise it is not present. |

ECID-SignalMeasurementInformation field descriptions

primaryCellMeasuredResults This field contains measurements for the primary cell (if the primary cell is a E-UTRA or NB-IoT cell), when the target device reports measurements for both primary cell (E-UTRA or NB-IoT) and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell (E-UTRA or NB-IoT) only, in which case the measurements for the primary cell (E-UTRA or NB-IoT) is reported in the measuredResultsList. This field shall be omitted when the primary cell is not a E-UTRA or NB-IoT cell.

measuredResultsList This list contains the E-CID measurements for up to 32 E-UTRA or NB-IoT cells.

physCellId This field specifies the physical cell identity of the measured cell.

cellGlobalId This field specifies cell global ID of the measured cell. The target device shall provide this field if it was able to determine the ECGI of the measured cell at the time of measurement.

arfcnEUTRA This field specifies the ARFCN of the measured E-UTRA carrier frequency, as defined in TS 36.331 [12]. In the case the target device includes arfcnEUTRA-v9a0, the target device shall set the corresponding arfcnEUTRA (i.e. without suffix) to maxEARFCN.

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

rsrp-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 36.331 [12], TS 36.214 [17]. In the case the target device includes rsrp-Result-v1470, the target device shall set the corresponding rsrp-Result (i.e. without suffix) to value 0. Measurement report mapping is according to TS 36.133 [18].

rsrq-Result This field specifies the reference signal received quality (RSRQ) measurement, as defined in TS 36.331 [12], TS 36.214 [17]. In the case the target device includes rsrq-Result-v1470, the target device shall set the corresponding rsrq-Result (i.e. without suffix) to value 0 or 34. Measurement report mapping is according to TS 36.133 [18].

ue-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 36.214 [17]. It is provided only for measurements on the UE's primary cell. Measurement report mapping is according to TS 36.133 [18].

nrsrp-Result This field specifies the narrowband reference signal received power (NRSRP) measurement, as defined in TS 36.214 [17]. Measurement report mapping is according to TS 36.133 [18].

nrsrq-Result This field specifies the narrowband reference signal received quality (NRSRQ) measurement, as defined in TS 36.214 [17]. Measurement report mapping to the value defined in TS 36.133 [18]. Values 0..29 map to values NRSRQ_-30..NRSRQ_-1. Values 30..62 map to NRSRQ_01..NRSRQ_33. Values 63..74 map to NRSRQ_35..NRSRQ_46. The UE does not report NRSRQ_00 nor NRSRQ_34.

carrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to ARFCN given by arfcnEUTRA as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper-SFN of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the hyper-SFN of the cell at the time of measurement.

Next change

#### 6.5.5.2	Sensor Location Information Elements

#### –	Sensor-MotionInformation

The IE Sensor-MotionInformation is used by the target device to provide UE movement information to the location server. The movement information comprises an ordered series of points. This information may be obtained by the target device using one or more motion sensors.

-- ASN1START

Sensor-MotionInformation-r15 ::= SEQUENCE {

refTime-r15					DisplacementTimeStamp-r15,

displacementInfoList-r15	DisplacementInfoList-r15,

...

}

DisplacementInfoList-r15 ::= SEQUENCE (SIZE (1..128)) OF DisplacementInfoListElement-r15

DisplacementInfoListElement-r15 ::= SEQUENCE {

deltaTimeStamp-r15			DeltaTime-r15,

displacement-r15			Displacement-r15			OPTIONAL,

...

}

DisplacementTimeStamp-r15 ::= CHOICE {

utcTime-r15					UTC-Time-r15,

gnssTime-r15				MeasurementReferenceTime,

systemFrameNumber-r15		SFN-r15,

measurementSFN-r15			INTEGER(-8192..9214),

...

}

DeltaTime-r15 ::= CHOICE {

deltaTimeSec-r15			INTEGER (1..16384),

deltaTimeSFN-r15			INTEGER (1..4096),

...

}

SFN-r15 ::= SEQUENCE {

sfn-r15						BIT STRING (SIZE (10)),

hyperSFN-r15				BIT STRING (SIZE (10))		OPTIONAL,

...

}

Displacement-r15 ::= SEQUENCE {

bearing-r15					INTEGER (0..3599),

bearingUncConfidence-r15	INTEGER (0..100)				OPTIONAL,

bearingRef-r15				ENUMERATED { geographicNorth, magneticNorth, local },

horizontalDistance-r15		INTEGER (0..8191),

horizontalDistanceUnc-r15	INTEGER (0..255)				OPTIONAL,

horizontalUncConfidence-r15	INTEGER (0..100)				OPTIONAL,

verticalDirection-r15		ENUMERATED{upward, downward}	OPTIONAL,

verticalDistance-r15		INTEGER(0..8191)				OPTIONAL,

verticalDistanceUnc-r15		INTEGER (0..255)				OPTIONAL,

verticalUncConfidence-r15	INTEGER (0..100)				OPTIONAL,

...

}

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

...

}

-- ASN1STOP

Sensor-MotionInformation field descriptions

refTime This field provides the reference time t0 associated with the starting position of the first displacement in the displacement list.

displacementInfoList This field provides an ordered series of direction and distance travelled by the target device and comprises the following subfields: - deltaTimeStamp specifies the time between tn-1 and tn, were n corresonds to the order of entry in the DispacementInfoList (n=0 correspond to the time provided in refTime). - displacement provides the direction and distance travelled between time tn-1 and tn.

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

gnssTime This field provides the time stamp of the refTime in GNSS time.

systemFrameNumber This field provides the time stamp of the refTime in serving cell SFN time.

measurementSFN This field provides the time stamp of the refTime in form of the measurement SFN as defined in delta-SFN in IE OTDOA-SignalMeasurementInformation. This field may be included when OTDOA measurements are included.

deltaTimeSec This field provides the time between tn-1 and tn in units of milliseconds.

deltaTimeSFN This field provides the time between tn-1 and tn in units of system frame numbers.

bearing This field specifies the direction (heading) of the horizontal displacement measured clockwise from bearingRef. Scale factor 0.1 degree.

bearingRef This field specifies the reference direction for the bearing. Enumerated value 'geographicNorth' indicates that the bearing is measured clockwise from the Geographic North; 'magneticNorth' indicates that the bearing is measured clockwise from the Magnetic North; 'local' indicates that the bearing is measured clockwise from an arbitrary (undefined) reference direction.

horizontalDistance This field specifies the horizonal distance travelled between time tn-1 and tn. Scale factor 1 cm.

horizontalDistanceUnc, horizontalUncConfidence This field specifies the horizontal uncertainty of the displacement (corresponding to tn). horizontalDistanceUnc correspond to the encoded high accuracy uncertainty as defined in TS 23.032 [15]. horizontalUncConfidence corresponds to confidence as defined in TS 23.032 [15].

verticalDistance This field specifies the vertical distance travelled between time tn-1 and tn. Scale factor 1 cm.

verticalDistanceUnc, verticalUncConfidence This field specifies the vertical uncertainty of the displacement (corresponding to tn). verticalDistanceUnc correspond to the encoded high accuracy uncertainty as defined in TS 23.032 [15]. verticalUncConfidence corresponds to confidence as defined in TS 23.032 [15].

End of change