3GPP TSG-RAN WG2 Meeting #121bis-e 	R2-2302682

eMeeting, April 17-26, 2023

Agenda item:	7.8.4

Work Item:	NR_UAV-Core

Source: 	Qualcomm Incorporated

Title: 	Subscription-based Aerial-UE Identification in NR

Document for:     Discussion/Decision

# Introduction

One of the objectives of NR UAV WI [1] is as follows:

2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

3GPP studied support of Aerial Vehicles during Rel-15 LTE and documents the finding in a TR [2]. During the normative phase, support for subscription-based aerial-UE identification was specified. The above objective is intended for adapting the LTE solution to NR and developing NR-specific enhancements if needed.

In this contribution, we discuss the LTE UAV enhancements and their applicability in NR. Further, we will discuss a potential NR specific enhancement which can be beneficial for the efficient support of NR UAVs.

# Background

In LTE, support for subscription-based aerial-UE identification was specified as follows by SA2, RAN2 and RAN3.

SA2 specification TS 23.401 section 4.3.31 describes the following:

### 4.3.31 Subscription handling for Aerial UEs This clause describes subscription information handling in order to support operating Aerial UE function over E-UTRAN as defined in TS 36.300 [5] and TS 36.331 [37]. The eNodeB supporting Aerial UE function handling uses the per user information supplied by the MME to determine whether or not to allow the UE to use Aerial UE function. Support of Aerial UE function is stored in the user's subscription information in HSS. HSS transfers this information to the MME via Update Location message during Attach and Tracking Area Update procedures. Home Operator may revoke user's subscription authorisation for operating Aerial UEs at any time. MME that supports Aerial UE function provides the user's subscription information on Aerial UE authorisation to the eNodeB via the S1 AP Initial Context Setup Request during Attach, Tracking Area Update and Service Request procedures. For the intra and inter MME S1 based handover (intra RAT) or Inter-RAT handover to E-UTRAN, the Aerial UE subscription information for the user is included in the S1-AP UE Context Modification Request message sent to the target eNodeB after the handover procedure. For X2-based handover, the Aerial UE subscription information for the user is sent to target eNodeB as follows: - If the source eNodeB supports Aerial UE function and the user's Aerial UE subscription information is included in the UE context, the source eNodeB shall include the information in the X2-AP Handover Request message to the target eNodeB. - The MME shall send the Aerial UE subscription information to the target eNodeB in the Path Switch Request Acknowledge message. If the Aerial UE subscription information has changed, the updated Aerial UE subscription information is included in the S1-AP UE Context Modification Request message sent to the eNodeB.

RAN2 captured in stage-2 the overall use of this information as shown below from TS 36.300:

### 23.17.2 Subscription based identification of Aerial UE function Support of Aerial UE function is stored in the user's subscription information in HSS. HSS transfers this information to the MME during Attach, Service Request and Tracking Area Update procedures. The subscription information can be provided from the MME to the eNB via the S1-AP Initial Context Setup Request during Attach, Tracking Area Update and Service Request procedures. The subscription information can also be updated via the S1-AP UE Context Modification Request message. In addition, for X2-based handover, the source eNodeB can include the subscription information in the X2-AP Handover Request message to the target eNodeB. For the intra and inter MME S1 based handover, the MME provides the subscription information to the target eNB after the handover procedure.

As explained above, “Aerial UE subscription information” can be included in:

- INITIAL CONTEXT SETUP REQUEST (S1-AP: MME -> eNB)

- UE CONTEXT MODIFICATION REQUEST (S1-AP: MME -> eNB)

- HANDOVER REQUEST (S1-AP: MME -> target eNB; X2-AP: source eNB -> target eNB)

- PATH SWITCH REQUEST ACKNOWLEDGE (S1-AP: MME -> eNB)

- RETRIEVE UE CONTEXT RESPONSE (X2-AP: source eNB -> target eNB)

RAN3 specifications provide the signaling details for transferring the subscription information between the network nodes, which is captured in TS 36.413 (for S1-AP) and TS 36.423 (for X2-AP) and defined as follows:

### 9.2.129	Aerial UE subscription information

This information element is used by the eNB to know if the UE is allowed to use aerial UE function, refer to TS 23.401[12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aerial UE subscription information | M |  | ENUMERATED (allowed, not allowed, ...) |  |

As can be seen, the Aerial UE subscription information is a 1-bit flag indicating whether UAV specific features are ‘allowed’ or ‘not allowed’ for the UE.

In LTE, the Aerial UE subscription information is a 1-bit flag indicating whether the UE is ‘allowed’ or ‘not allowed’ to use the aerial UE functions.

# Different UAV types

It is well accepted view that it is beneficial for the RAN to know whether a UE is allowed to use UAV functions/enhancements or not.

We further think RAN knowing that a certain aerial UE is of a certain type can be more beneficial so that RAN can provide tailored services based on the specific type. For example, a large drone carrying heavy items (e.g. delivery drone) may get services with higher priority over a lightweight amateur drone taking pictures. Another example scenario: an emergency services drone (e.g. delivering medicine) may get higher priority services than an e-commerce delivery drone.

So, higher granularity of information related to the UAV type/subscription can be beneficial to RAN e.g. to prioritize mobility handling, admission control during congestion scenarios etc.

Higher granularity of information related to the UAV type/subscription can be beneficial to RAN e.g. to prioritize mobility handling, admission control during congestion etc.

## Signalling different UAV types

Now, the question is how to provide additional information about UAV type (e.g. emergency drone, service vehicle, commercial drone, delivery drone, amateur drone, etc.) to the RAN node?

One option is for RAN2 to directly define such information e.g. using RRC signalling from the UE. This allows changing the UAV type more dynamically, e.g. based on mission types, without going to IDLE or doing a TAU update. For instance, emergency drone inbound to the scene may be higher priority but returning after the mission may be normal priority. In this case, the RAN-node can check with the serving AMF if the indicated type is authorized by checking against the subscription information.

On the other hand, given that LTE already defined a framework to provide a one-bit flag about UAV subscription, it should be straightforward to enable a more granular information using the same method.

A more granular UAV type/subscription information can be enabled using the same method as in LTE.

Since RAN3 would need to enable signalling mechanisms for this objective, it is essential that RAN2 agree to the principle that it would be beneficial from RAN2 point of view for RAN to have the information about the different UAV types, subscription, and/or mission types. Details can be discussed further once the principle is agreed.

Once RAN2 agrees that it is beneficial, RAN2 can ask RAN3 and SA2 to define the required signalling details.

Therefore, we propose:

From RAN2 point of view, it is beneficial for RAN to have the information about different UAV types, UAV subscription types, and/or UAV mission types.

Inform RAN3 and SA2 of the RAN2 agreement.

# Conclusion

In this contribution, we made the following observations on the subscription based aerial UE identification:

Observation 1.	In LTE, the Aerial UE subscription information is a 1-bit flag indicating whether the UE is ‘allowed’ or ‘not allowed’ to use the aerial UE functions.

Observation 2.	Higher granularity of information related to the UAV type/subscription can be beneficial to RAN e.g. to prioritize mobility handling, admission control during congestion etc.

Observation 3.	A more granular UAV type/subscription information can be enabled using the same method as in LTE.

Observation 4.	Once RAN2 agrees that it is beneficial, RAN2 can ask RAN3 and SA2 to define the required signalling details.

Based on the discussion and observations, we propose:

Proposal 1.	From RAN2 point of view, it is beneficial for RAN to have the information about different UAV types, UAV subscription types, and/or UAV mission types.

Proposal 2.	Inform RAN3 and SA2 of the RAN2 agreement.

# References

[1] RP-230782, Revised WID: NR support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

[2] TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles (Release 15), Technical Report