3GPP TSG-RAN WG2 Meeting #121b	 R2-2302768

Online, April 17 – April 26, 2023

Agenda Item:	6.2.3

Source: 	NEC, LG Electronics Inc, Nokia, Nokia Shanghai Bell, Samsung

Title:  	Discussion on the correction for cfr-ConfigMulticast and Multicast DRX

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#121 meeting, the following agreement is made [1]:

UE doesn’t need to report CSI if cfr-ConfigMulticast is not included in the current active BWP, even if the allowCSI-SRS-Tx-MulticastDRX-Active-r17 is configured. FFS spec impact.

Furthermore, how to capture this spec is agreed in post email discussion [2] as described below. In this contribution, we further analyse the spec impact.

- 2 Discussion

- 2.1 Background

In RAN2#112 meeting, the issue of CSI reporting is raised in email discussion [3]. Based on the current MAC spec, if UE is configured with allowCSI-SRS-Tx-MulticastDRX-Active, and the multicast DRX is in active time, UE will report CSI based on CSI report configuration.

There could be an issue that currently the multicast CFR configuration (i.e., cfr-ConfigMulticast) is configured per BWP. If the current active BWP of the UE does not fully include the bandwidth of CFR, according to RAN1 spec (TS 38.211), the UE is not expected to receive PDSCH or PDCCH associated with MBS transmissions scheduled with G-RNTI, G-CS-RNTI, which means UE is not receiving multicast service in the current active BWP, thus there is no need to report CSI during multicast DRX active time for gNB multicast scheduling.

However, based on the current RRC spec, the enabler for CSI reporting for MBS multicast (i.e., allowCSI-SRS-Tx-MulticastDRX-Active) is configured in the MAC configuration (i.e., MAC-CellGroupConfig) which is common for all configured BWP(s). That is to say network may configure the allowCSI-SRS-Tx-MulticastDRX-Active-r17 to a UE although the UE may not be able to receive the multicast service in the current BWP due to absence of cfr-ConfigMulticast. Then unnecessary CSI reporting is caused.

Based on the analysis above, in post email discussion [2], the following CR is agreed in MAC spec 5.7 (note that there are 3 parts related to CSI reporting captured this description):

- 2.2 More issues for the spec impact

Based on the current correction for CSI reporting, UE only report CSI when allowCSI-SRS-Tx-MulticastDRX-Active is configured AND cfr-ConfigMulticast is configured in active BWP AND multicast DRX is in Active Time. We admit this is a correct behaviour but there could be one more issue.

drx-ConfigPTM is also configured in MAC configuration which is common for all configured BWP(s), likewise, UE can still run multicast DRX even though there is no CFR configured in the current active BWP. Although based on RAN1 spec if no CFR is configured for any active BWP(s), UE is not required to monitor PDCCH regardless of multicast DRX configuration, this is unreasonable/abnormal configuration. Honestly, this UE behaviour is not aligned with the traditional definition of DRX. If the UE does not monitor PDCCH, the UE actually did not follow the DRX pattern, since DRX includes both UE active and non-active time. If the UE intends not to monitor PDCCH, this means there is only non-active time for the UE based on the DRX pattern.

Therefore, in essence, if the current active BWP of UE does not fully include CFR (i.e., UE is not receiving multicast service), not only CSI reporting is not needed, but also multicast DRX should not be started. If we fix the latter one, then naturally CSI report will not be triggered due to no multicast DRX Active Time.

And the suggested proposals in MAC spec clause 5.7b (provided in our CR [4]) is shown below (note that the previously agreed CR in MAC spec clause 5.7 should be removed, details see CR [4]):

Observation 1: if the current active BWP of UE does not fully include CFR (i.e., UE is not receiving multicast service), not only CSI reporting is not needed, but also multicast DRX should not be started.

Proposal 1: RAN2 is suggested to re-consider the spec impact on CSI reporting, i.e., if CFR is not configured in any of the active BWP(s) of the Serving Cell(s), the UE should not start multicast DRXes.

Proposal 2: if proposal 1 is agreed, adopt CR [R2-2302767] to specify this issue.

- 3 Conclusions

Observation 1: if the current active BWP of UE does not fully include CFR (i.e., UE is not receiving multicast service), not only CSI reporting is not needed, but also multicast DRX should not be started.

Proposal 1: RAN2 is suggested to re-consider the spec impact on CSI reporting, i.e., if CFR is not configured in any of the active BWP(s) of the Serving Cell(s), the UE should not start multicast DRXes Active Time.

Proposal 2: if proposal 1 is agreed, adopt CR [R2-2302767] to specify this issue.

- 4 References

[1] R2-2301907	Report from MBS breakout session						session chair

[2] R2-2302095	38321 MBS MAC Corrections							vivo

[3] R2-2302090	Report of [AT121][604][MBS-R17] Remaining UP issues	vivo

[4] R2-2302767	Corrections on cfr-ConfigMulticast and Multicast DRX		NEC, LG Electronics Inc, Nokia, Nokia Shanghai Bell, Samsung