3GPP TSG-RAN WG2 #121-bis-e 	R2-2302818

Online, 17th – 26th April, 2023

Title:		Discussion on RAN1 led positioning topics

Source:	Huawei, HiSilicon

Agenda item:	7.2.5

Document for:	Discussion and Decision

# Introduction

A new WID on positioning was approved in RAN#98-e [1] and it has the following objectives related to RedCap positioning, carrier phase positioning and bandwidth aggregation, with R1 as the leading group for discussion:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4]. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4]. - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

We discuss each topic and present the enhancements needed in RAN2.

# Discussion

## 2.1	RedCap positioning

RAN1 made considerable progress on frequency hopping in RAN1-112 meeting. We present RAN1 agreements in the following sections and discuss the RAN2 impact.

### 2.1.1	Potential enhancements in RAN2 to support frequency hopping

RAN1 made the following agreements in RAN1-112 meeting:

DL-PRS Rx frequency hopping agreements: Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported. Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

For DL PRS Rx hopping, RAN1 is still evaluating if there is specification impact. Hence, RAN2 can wait with discussing DL PRS enhacements.

SRS Tx frequency hopping agreements: Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

For SRS Tx frequency hopping, RAN1 may discuss the potential enhancement to achieve the decoupling of pos-SRS configuration from the existing BWP configuration and decide whether to support SRS Tx hopping within an SRS resource or across SRS resources.

Considering RAN1 still needs to finalize the details on DL PRS and UL SRS frequency hopping, we suggest that RAN2 wait for the progress in RAN1.

Proposal 1: RAN2 wait for RAN1 progress on PRS and SRS frequency hopping before studying the enhancements required in RAN2.

### 2.1.2	Awareness of RedCap UE capability at the LMF

During the SI phase, RAN2 made the following agreements in RAN2-119bis-e [2]:

- Information on RedCap UE capability from the UE to the LMF (e.g., restricted bandwidth capability) can be discussed in WI phase. - RAN2 wait for RAN1 conclusion on the signalling design for assistance data for frequency hopping.

In the current specification, the capabilities in an LPP context refer to supporting of different positioning methods defined for LPP, different aspects of a particular positioning method (e.g. different types of assistance data for A-GNSS) and common features not specific to only positioning method (e.g. ability to handle multiple LPP transactions). Even though LMF is the controlling entity for the positioning procedures, it’s not clear why it should know the RedCap UE capability either to recommend PRS/SRS configuration or to determine the positioning method. The gNB knows the RedCap UE type and it can determine SRS configuration. For PRS configuration, RAN1 agreed to use only Rx frequency hopping (i.e., wideband PRS transmission). Hence, there is no need for LMF to know the RedCap UE capability for the PRS configuration.

Proposal 2: For either SRS or PRS configuration, LMF does not need to know the RedCap UE capability.

## 2.2	Carrier Phase Positioning

RAN1 made the following agreements in RAN1-112 meeting:

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements.

According to the agreements, new measurements will be introduced to support carrier phase positioning (CPP). The potential new measurements may include DL carrier phase and/or DL carrier phase difference for UE-based and UE-assisted CPP, and UL carrier phase for NG-RAN node-assisted CPP. Signaling enhancements are needed for LPP to support the new measurements reporting from the UE to the LMF. Since RAN1 are still discussing the details, it is proposed to wait for RAN1 progress.

Proposal 3: Signaling enhancements are needed to support the new measurements introduced by CPP method. Wait for RAN1 progress to specify the measurements.

## 2.3	Bandwidth Aggregation

### 2.3.1 Aggregation of PRS resources

For downlink positioning, all configuration information is transmitted by the LMF to the UE through LPP signaling. The following figure shows the structure of the PRS configuration information：

Figure 2.3.1 The structure of PRS configuration from LMF to UE

As shown in Figure 2.3.1, PRS resources are configured based on the hierarchical relationship among PFL (Positioning Frequency Layer), TRPs, Resource sets, and Resources. And, the maximum number of Positioning frequency layers can be configured is 4.

From the perspective of RAN2, the preceding configuration architecture does not need to be modified for PRS bandwidth aggregation-based positioning. The network only needs to notify the UE of the following additional information:

- When does the UE perform PRS bandwidth aggregation-based measurement?

- Which PFLs should the UE measure at the same time?

From the perspective of signaling,

- For the first problem, a new field can be added to the Location Measurement Request to indicate that PRS bandwidth aggregation-based measurement is required for the UE.

- For the second problem, a simple way is directly adding the indicator in the PRS configuration information. For example, the configuration information can be enhanced to indicate the PRS resource set-3 belonging to the PFL-2 is associated with the PRS resource set-1 belonging to the PFL-1 in the Figure 2.3.1. After receiving the configuration, the UE would know that the PRS resource set-3 and PRS resource set-1 can be aggregated.

However, the above issues should be discussed in RAN1 first, RAN2 can further specify the signaling content after the conclusion of RAN1 is reached.

Proposal 4: Signaling enhancements are needed to support the new measurements introduced by PRS bandwidth aggregation-based measurement. Wait for RAN1 progress to specify the measurements.

### 2.3.2 Aggregation of SRS resources

A difference between the SRS configuration process and the PRS configuration process lies in that the SRS is completely configured by the serving gNB, and the configuration information of the SRS is included in the configuration information of the BWP, as shown in the following figure：

Figure 2.3.2 The structure of SRS configuration from gNB to UE

As shown in Figure 2.3.2, the current SRS resources are configured in a BWP of a cell. For the positioning method based on uplink SRS bandwidth aggregation, the current configuration procedure does not need to be changed, but how to instruct the UE to send SRS resources in multiple UL carriers with the same port at the same time needs to be considered. A simple solution is to add indication information to SRS resource set configuration information of a cell, to indicate the association to the SRS resource set configuration of another carrier. After receiving the configuration, the UE would know those SRS resources need to transmitted with the same port at the same time.

Actually, RAN1 will also discusses this problem, and RAN2 may define specific signalling content after RAN1 has a conclusion.

Proposal 5: Signaling enhancements are needed to support the new measurements introduced by SRS bandwidth aggregation. Wait for RAN1 progress to specify the measurements.

### 2.3.3 Decouple the positioning BW aggregation with communication CA

Based on the current description of the existing signalling architecture, to support PRS/SRS bandwidth aggregation, the UE first needs to support carrier aggregation. Carrier aggregation is a concept introduced in data communication. It aims to transmit and receive data on multiple carriers at the same time to greatly increase the communication data rate. On the other hand, the bandwidth aggregation in positioning is used to improve positioning accuracy by using larger reference signal bandwidth.

However, considering actual application scenarios, some scenarios do not require a very high communication data rate, but may require very high positioning precision, such as AGV navigation, autopilot, etc. In addition, the UE generally does not perform communication when performing a positioning operation. Therefore, positioning and communication are independent processes from the perspective of requirements and services, and thus carrier aggregation used for communication and bandwidth aggregation used for positioning should be decoupled so that both UE/NW could design the baseband/RF tailored to the communication and positioning individually. For example, a UE may not support carrier aggregation, but may support bandwidth aggregation of positioning reference signals to reduce the cost of baseband processing and L1/L2 buffering. There is no need to bundle carrier aggregation and positioning bandwidth aggregation.

For DL-PRS, the configuration of DL-PRS is not based on the serving Cell/BWP configuration. So, the PRS bandwidth aggregation and communication carrier aggregation are already decoupled from the view of configuration.

For UL-SRS, because the SRS is configured in the BWP of the serving cell, the SRS bandwidth aggregation depends on the communication carrier aggregation. The decoupling of the SRS bandwidth aggregation and communication carrier aggregation means that a separate UE capability for SRS carrier/BW aggregation should be defined independent from UE communication CA capabilities, e.g. bandwidth class.

Proposal 6: Support decoupling the positioning SRS bandwidth aggregation with communication carrier aggregation and wait for RAN1 progress to specify the signaling.

# 3.	Conclusion

In this contribution, we discussed RedCap positioning, carrier phase positioning and bandwidth aggregation and proposed the following:

Proposal 1: RAN2 wait for RAN1 progress on PRS and SRS frequency hopping before studying the enhancements required in RAN2.

Proposal 2: For SRS configuration, LMF does not need to know the RedCap UE capability. For PRS configuration, RAN2 wait for RAN1 conclusion on PRS frequency hopping details.

Proposal 3: Signaling enhancements are needed to support the new measurements introduced by CPP method. Wait for RAN1 progress to specify the measurements.

Proposal 4: Signaling enhancements are needed to support the new measurements introduced by PRS bandwidth aggregation-based measurement. Wait for RAN1 progress to specify the measurements.

Proposal 5: Signaling enhancements are needed to support the new measurements introduced by SRS bandwidth aggregation. Wait for RAN1 progress to specify the measurements.

Proposal 6: Support decoupling the positioning PRS/SRS bandwidth aggregation with communication carrier aggregation and wait for RAN1 progress to specify the signaling.

# 4.	References

- 3GPP RP-223549, “New WID on Expanded and Improved NR positioning”

- RAN2 Chair’s Notes RAN2#119bis-e, RAN2#119bis-e, e-Meeting Oct, 2022.

- 3GPP TR 38.859 v18.0.0, “Study on expanded and improved NR positioning (Release 18)”.

- 3GPP TS 38.133 v16.1.0, “Requirements for support of radio resource management”