3GPP TSG RAN WG 2 Meeting #121bis electronic		TDoc R2-2302945

17th - 26th April, 2023

Title:	[Draft] Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM

Response to:	LS R2-2302053/R1-2302194 on L1 measurement RS configuration and PDCCH ordered RACH for LTM from RAN1

Release:	Release 18

Work Item:	NR_mob_enh2-Core

Source:	Fujitsu, CATT (to be RAN2)

To:	RAN1, RAN3

Cc:

Contact person:	Takako Sanda

sanda.takako<atmark>fujitsu.com

Rui Zhou

Zhourui<atmark>catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN2 thanks RAN1 for their LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM, based on the discussion in RAN2.

RAN2 would like to provide the following feedback for RAN1’s question:

RAN 1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN 3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in the agreement described in section B.

- From RAN2 perspective, all options are feasible.

- The following aspects will be potential RAN2 specs impact.

<Without RAR>

- It needs to be specified whether/how the UE knows the Msg.1 is successfully received by the gNB or failed.

- RAN2 understands RAN1 will decide it, and RAN2 will specify the decision in RAN2 specs.

<With RAR>

- RAN2 needs to discuss how to maintain the TA of candidate cell at UE side, i.e.,

-	The needs of TA timer.

-	Whether the TA of candidate is maintained after LTM

<With RAR and the RAR is received from the serving cell >

- RAN2 needs to discuss how to decide the start of the RAR window at UE side, and the UE behaviour if no RAR is received within the RAR window, and specify the result of the discussion.

- For inter-DU case, the preambles used for LTM in the candidate DU may be different from the serving DU. Therefore:

-	RAN2 needs to wait RAN1 decision whether additional information for LTM RAR is introduced or not, e.g., whether a specific information to distinguish LTM RAR from normal RAR is introduced.

-	If the additional information is introduced, legacy RAR cannot be used, RAN2 needs to discuss the format of the RAR and the UE behaviour to receive the RAR, and specify the result of the discussion

-	If the additional information is not introduced, the legacy RAR can be used. Although RAN2 needs to discuss how to handle unnecessary information (e.g., UL grant, Temporary C-RNTI) in RAR and specify the result of the discussion, spec impact may be smaller than the former case.

-	RAN2 (and/or RAN3) needs to discuss how to avoid potential preamble confliction and specify the result of the discussion.

- The confliction may occur when PDCCH ordered RACH for LTM is performed by a UE, another UE in the same serving cell performs CBRA using the same preamble.

<With RAR and the RAR is received from the candidate cell >

- RAN2 needs to discuss whether the caused data transmission interruption at the source is acceptable, or any specific handling is necessary to avoid it, and specify the result of the discussion.

2	Actions

To RAN1 and RAN3 groups

ACTION: 	RAN2 kindly asks RAN1 and RAN3 to take the above feedback into consideration.

3	Dates of next TSG RAN WG 2 meetings

TSG RAN WG2 Meeting #122	22 – 26 May 2023	Incheon, KR

TSG RAN WG2 Meeting #123	21 – 25 August 2023	Toulouse, FR